3GPP TSG-RAN WG3 Meeting #130                                                                 R3-258676

Dallas, US, 17 – 21, Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1330 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UHI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation,China Telecom, Pengcheng Laboratory |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core,TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current Rel-18 TS 38.423 specification, the UE History Information (UHI), as defined in TS 38.413, is carried in XnAP messages (e.g., HANDOVER REQUEST, SN-NODE ADDITION REQUEST) to support Ping-Pong avoidance for L3 handover, or to prepare conditional candidate cells for CHO or SCPAC. To complete the UHI, including information related to the MCG and SCG, the SCG UHI may also be exchanged from the SN to the MN over XnAP. LTM is introduced in Rel-18 for connected mobility, with support limited to intra-CU LTM. For the DC scenario, intra-MN MCG LTM and intra-SN SCG LTM are supported in Rel-18, but simultaneous PCell and PSCell change is not supported. According to the definition of UHI in TS 38.413 in Rel-18, the UHI is defined as the latest visited cells, including PCells and PSCells. Therefore, when a UE visits a cell due to an MCG or SCG LTM cell switch, that cell should also be recorded in the UHI. Furthermore, the UHI represents a continuous record of the cells visited by the UE, without gaps, i.e., it does not filter out specific types of cell visits. Based on the current definition of UHI, it may cause the target node to misinterpret all visited cells including LTM cells in the UHI as legacy L3 mobility cells. Therefore, the UHI should be enhanced to enable the receiving node to differentiate cells recorded as a result of legacy L3 mobility from those due to LTM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To add the MobilityType IE into the Last Visited NG-RAN Cell Information IE and the Last Visited PSCell Information IE for UHI, in order to indicate whether the UE accessed the cell due to LTM. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has isolated impact with the previous version of the specification (same release) because it only impacts the UHI information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MRO for Ping-Pong or the configuration of candidate cells for conditional mobility may be improperly applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.97, 9.3.1.235 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - V1 :Resubmit of R3-256753 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| Mobility Type | O |  | ENUMERATED ( LTM, …) | This IE is present only if the UE accesses the cell due to LTM. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |
| Mobility Type | O |  | ENUMERATED ( LTM, …) | This IE is present only if the UE accesses the cell due to LTM. |

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

id-ExtendedBackupAMFName,

id-MobilityType

,

maxnoofAllowedAreas,

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastVisitedPSCellList	CRITICALITY ignore	EXTENSION LastVisitedPSCellList	PRESENCE optional }|

{ ID id-MobilityType	CRITICALITY ignore	EXTENSION MobilityType	PRESENCE optional },

...

}

LastVisitedPSCellList ::= SEQUENCE (SIZE(1..maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCellInformation

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MobilityType	CRITICALITY ignore	EXTENSION MobilityType	PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

MobilityType ::=ENUMERATED {

LTM,

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-TAIMBSSupportList									ProtocolIE-ID ::= 441

id-ExtendedBackupAMFName    							ProtocolIE-ID ::= 442

id-ExtendedOldAMF			    						ProtocolIE-ID ::= 443

id-MobilityType			    							ProtocolIE-ID ::= xxx

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>