3GPP TSG-RAN WG3 Meeting #130	R3-258682

Dallas, US, 17 – 21 November 2025

Agenda Item:	13.2

Source: 	CEWiT

Title: 	Discussion paper on ISAC network architecture

Document for:	Discussion and Decision

# Introduction

RAN#108 agreed study on ISAC which is updated in the RAN#109 [1]. The study is divided between the evaluation of the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case and study the procedures, signalling between RAN and CN to support ISAC along with network architecture for gNB-based mono-static sensing for UAV sensing target use cases. The latter objective is relevant from RAN 3 perspectives based on the RAN 1 input for the prior objective. This relevant objective regarding for ISAC from RAN 3 perspective is as below,

Study the procedures, signalling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

Further a stage 1 work on ISAC has been completed by SA1 including the use cases and requirement aspects for ISAC. TR 22.837 identifies a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137, in particular, detection and/or tracking of UAV (Uncrewed Aerial Vehicles). SA2 also has an ongoing Rel-20 study on Architecture Enhancement to support Integrated Sensing and Communication (FS_Sensing_ARC, SP-250401). In Release 19, a channel modelling framework for ISAC to enable evaluation of sensing techniques for such use cases was added to the existing communication models in TR38.901.

In the last meeting of RAN 3 a pCR to 38.765 [2] is agreed giving the logical architecture for ISAC. The pCR test is the following.

# 7. Network architecture Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied. >>>>>>>>>>Start of Change<<<<<<<<<< This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture. Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF. Figure 7.1 Logical architecture for ISAC This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF. Editor’s Note: The details of "Nx" interface need further discussion and decision.

Further the agreed guidance for this meeting is “To continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.”

Further it is agreed that “RAN3 focus on sensing protocol in coordination with SA2”.

Considering these aspects and the defined objective, this contribution highlights necessary aspects of the network architecture for ISAC considering mono static sensing as baseline but can be extended to bistatic sensing too.

# ISAC Network architecture

SA2 is working on the different key issues to define the sensing architecture, sensing function, sensing entity, discovery of sensing functions & sensing entity, sensing data collection, sensing parameters configuration provisioning and sensing result exposure. Whereas RAN 1 is working on the physical layer parameter evaluation for sensing purpose. Further the scope of the study limits to mono static sensing for detection and/or tracking of Uncrewed Aerial Vehicles (UAV). It is agreed to extend the same framework to bistatic sensing too without specific enhancement related to it. From RAN 3 perspectives, the interim agreement can be considered for initial discussion purpose and details can be updated once the inputs are available from SA2.

For the initial discussion ISAC perspective, a logical architecture is agreed as mentioned in above section.

SA2 is discussing their conclusion on under key issue 4 “Sensing Data and the Associated Information Collection and Transport” sensing data collection mechanism and involved sensing nodes/functions. Interim agreements are captured in TR 23.700-14 section 7.1.4.

TR 23.700-14 7.1.4 The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one-time, periodical Sensing Data to the Sensing Function. Editor’s Note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

Further SA2 is considering using both direct and indirect way of sensing data collection mechanism. In direct method will be through AMF.

RAN 3 also should consider both direct and indirect architecture for sensing data/result transfer to the sensing function (SF).

Proposal 1: RAN3 should study both direct and indirect architectures for sensing data/result transfer.

The RAN 3#129b meeting [3] brings out the three options of the architecture. They are

Option 1: New interface solution

Option 2: Positioning like solution

Option 3: Ambient-IoT like solution

Option 1 necessitates that the gNB shall connect directly with the Sensing Function, and a new interface between the gNB and the Sensing Function needs to be defined. Whereas option 2 is similar to NRPPa where new protocol for sensing will be defined as contained over NGAP. Option 3 is similar to A-IoT where the NGAP elementary procedures are added from A-IoT service perspective e.g. Inventory request, Command Request A-IoT session release, A-IoT Session Release Request, Inventory report etc. But the NG-RAN maintains a separate direct connection NGAP connection towards the A-IoT-CN node.

Out of these methods, option 1 and option 3 are direct architecture while option 2 is indirect architecture. As SA2 is working on both possible options considering control signaling as well as data signaling, it is important to study these options considering the possibility that direct and indirect architecture is necessary to support.

Observation 1: SA2 considers ISAC control and data signaling using direct and indirect methods. SA2 is expected to conclude this in the November meeting. RAN 3 should consider this while discussing sensing protocol stacks.

Proposal 2: RAN 3 should study following architecture options,

Direct Architecture:

Option 1: New interface with new protocol for sensing

Option 2: Enhance NGAP to support direct interface between NG-RAN and SF

Option 3: Defined a new protocol as container in NGAP to support direct interface between NG-RAN and SF

Indirect Architecture:

Option 4: Define new protocol as container in the NGAP considering NRPPa as baseline.

# Protocol stack for ISAC RAN -CN interface:

In this option, the protocol stack based on above options can be designed, as below,

Option 1: For option this option a new sensing protocol (SPa) should be defined but the underline TNL can be proprietor.

Figure 1: Potential protocol for stack option 1

Option 2: For this option NGAP will be modified to define the sensing procedures and SF will be end point.

Figure 2: Potential protocol for stack option 3 enhancing NGAP for ISAC

Option 3: For this option new sensing protocol will be used over NGAP, Spa defined as container in the NGAP. The NGAP end point will be sensing function directly.

Figure 3: Potential protocol for stack option 3 SPa over NGAP

Option 4: For the direct method NRPPa stack should be adopted considering new sensing protocol as shown below.

Figure 4: Potential protocol stack for indirect method option 4

Proposal 3: For the proposed 4 ISAC network architecture options, RAN 3 should consider the protocol stacks as mentioned in figure 1 to 4 respectively.

# Conclusion

This contribution provides the ISAC network architecture options. The contribution provides following proposals for the discussion.

Proposal 1: RAN3 should study both direct and indirect architectures for sensing data transfer.

Observation 1: SA2 is considering ISAC control and data signalling using direct and indirect methods. SA2 is expected to conclude on this in the November meeting. RAN 3 should consider this while discussion sensing protocol stacks.

Proposal 2: RAN 3 should study following ISAC architecture options,

Direct Architecture:

Option 1: New interface with new protocol for sensing

Option 2: Enhance NGAP to support direct interface between NG-RAN and SF

Option 3: Defined a new protocol as container in NGAP to support direct interface between NG-RAN and SF

Indirect Architecture:

Option 4: Define a new protocol as a container in the NGAP considering NRPPa as baseline.

Proposal 3: For the proposed 4 ISAC network architecture options, RAN 3 should consider the protocol stacks as mentioned in figure 1 to 4 respectively.

# References

[1] RP-252819: Revised SID: Study on Integrated Sensing and Communication (ISAC) for NR

[2] R3-257325 (TP to BL pCR to 38.765) Network Architecture

[3] R3-256558 Discussion on Network Architecture for ISAC, ZTE

[4] TR 22.837 : Feasibility Study on Integrated Sensing and Communication (Stage 1)

[5] TS 22.137 : Service requirements for Integrated Sensing and Communication.

Stage 1 (Release 19)

[6] TR 38.901 : Study on channel model for frequencies from 0.5 to 100 GHz (Release 19)