3GPP TSG-RAN WG3 Meeting #130															R3-258698

Dallas, US, 17 – 21 November 2025

Agenda Item:	9.2.2

Source:	ZTE Corporation

Title:	Response to R3-258173

Document for:	Discussions & Approval

1	Introduction

This contribution is to response to R3-258173 [1] about the continuous MDT collection.

It is worth noting that most companies, including operators, hope to conclude RAN3's work on Continuous MDT collection at this meeting, and have provided CRs for TS 38.423 [2], TS 38.300 [3], TS 37.320 [4], and TS 38.401 [5].

2	Discussion

In the last meeting, because of concerning on the following issues captured in the chairman notes, the prepared CR to finish RAN3 work on Continuous MDT collection is blocked.

- It is discussed whether Signalling Based MDT can be always prioritized with respect to Continuous MDT. It should be checked whether it is possible to de-configure a UE from Management Based Immediate and Logged MDT to give priority to the Signalling Based MDT configuration.

- How can a UE be de-configured from Logged Continuous MDT.

4.1.5	MDT Trace selection conditions

In Immediate MDT, both in case of signalling based and management based MDT trace activation, it is always the network that evaluates all selection conditions for activating the MDT measurements and deactivating the MDT measurements (this evaluation is done continuously during the selected call session). The network activates and deactivates the MDT measurements toward the UE accordingly via RRC.

In Logged MDT and Logged MBSFN MDT, the network configures UEs for MDT tracing that are eligible based on the specified selection criteria. If area based condition is specified in the trace configuration, it is sent to the UE at configuration time and the UE will evaluate the area condition as it moves in the network in idle mode.

Immediate MDT, Logged MDT and Logged MBSFN MDT measurements shall always be configured as separate trace sessions.

In cases of overlapping MDT configuration request the signaling based request shall override the management based request. For logged MDT and Logged MBSFN MDT, prior to re configuring, the eNB shall retrieve the MDT logs from the UE.

For the first bullet whether Signalling Based MDT can be always prioritized with respect to Continuous MDT, it is explicitly statement in TS 32.422 about the priority of management-based MDT and signalling based MDT.

In cases of overlapping MDT configuration requests, the signalling-based request shall override the management-based request. This means the signalling-based MDT configuration will override the management-based MDT configuration. This principle still applies to continuous management-based MDT. That is, if the gNB receives the signalling-based MDT for that UE, the previous management-based MDT configuration will be replaced by the signalling-based MDT configuration. There is nothing special in C-MDT case.

- In cases of overlapping MDT configuration requests, the signalling-based request shall override the management-based request.

- The priority of signalling-based MDT is always higher than that of management-based MDT, including C-MDT.

Another concern “How can a UE be de-configured from Logged Continuous MDT” raised by companies, which was also raised in R3-258173 [1], proposes two solutions to solve the problem:

-	Solution 1 – “endless” continuity: OAM cancels / stops the entire C-MDT Trace job in all the involved gNBs in the configured area.

-	Solution 2 – two-hour continuity: Only immediate MDT (and not Logged MDT) is reconfigured in the UE in case of handover or in case of transition from RRC_Inactive to RRC_Connected.

Continuous management-based MDT: a management-based MDT functionality that enables continuous collection of MDT data, including both Immediate MDT and Logged MDT, within a specified area.

NOTE:	This MDT data collection captures UE transitions between RRC states (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED), as well as UE mobility between participating NG-RAN nodes. The MDT data collection on selected UEs is stopped when both Immediate MDT and Logged MDT are deactivated.

Management based MDT: MDT data is collected from UEs in a specified area. The area is defined as a list of cells (UTRAN or E-UTRAN) or as a list of tracking/routing/location areas. The management based MDT is an enhancement of the management based trace functionality. Management based MDT can be either a logged MDT or Immediate MDT.

First, we want to emphasize that SA5 has specified in their agreed CR how to stop Continuous MDT collection, as quoted below.

Our understanding is that the deactivation of C-MDT is the same as management based MDT which is triggered and controlled by OAM and captured in SA5 specification.

- SA5 has already specified the termination of MDT data collection, which also applies to Continuous MDT collection.

The issue is highlighted in the above in that the rationale is to provide an optional mechanism to update (renew) the TRSR if required for security reasons. However, with this approach the 3GPP standard becomes incomplete because there has been no evaluation whether TRSR renewal is required or not, and therefore there is no mandate or recommendation in the standard relative to the TRSR renewal. However, security is a critical aspect, and it is therefore  essential to have such mandate or recommendation if TRSR renewal is needed for security reasons in Solution 1. Otherwise, it could imply that the same identifier “TR/TRSR” is sent over the air, both in the UL (when UE uploads the logged MDT reports to the network) and in the DL (when the UE is reconfigured with logged MDT). As a general principle, once a permanent identifier is transmitted over the air it cannot be reused in order to avoid security risks to the UE. Therefore, SA3 analysis is needed on this point if Solution 1 is maintained.

Regarding the solution 1, security issues has been raised in R3-258173:

RAN3 would like to ask SA5 to take the above agreements and descriptions from RAN3 into account and to specify a solution for continuous MDT accordingly. RAN3 asks SA5 to check the security aspects of the Continuous MDT solution derived.

From our perspective, the renewal of TRSR during Continuous MDT collection is aimed at providing security guarantees. The renewal of TRSR is to avoid treating TR and TRSR as UE identifiers, thereby violating the anonymity principle of management-based MDT in RAN. Additionally, the security issues of the solution are not within the scope of RAN3's work. RAN3 has already sent the LS [6] to SA5 when specifying the solution and checking its security.

RAN3 should believe that the solution derived in SA5 has also checked the security issues. Thus, SA5 provided the agreed CRs to RAN3 in the last meeting. If any company believes there is a security issue, the proper procedure is to raise the security issue in SA5 rather than discussing it in RAN3, because RAN3 has already stated its own requirements.

- RAN3 provided the requirement to SA5 to check the security aspects of the Continuous MDT solution derived before, and SA5 has also check the security issues.

- No security issue should be discussed in RAN3 for continuous MDT collection.

Regarding Solution 2, the proposing company believes that only Immediate MDT (and not Logged MDT) is reconfigured in the UE in the case of handover or transition from RRC_INACTIVE to RRC_CONNECTED. This will lead to other problems: if the time for the previous Logged MDT measurement has expired, the previous logged MDT configuration in the UE becomes invalid, requiring reconfiguration of the Logged MDT configuration; otherwise, the continuity is effectively interrupted.

Therefore, the best way is to reconfigure the logged MDT configuration and the Immediate MDT configuration for the UE after UE transitions into RRC_CONNECTED, which can ensure that subsequent Continuous MDT collection across RRC states is supported.

- Reconfigure the logged MDT configuration and the Immediate MDT configuration for the UE after UE transitions into RRC_CONNECTED.

There is only one RAT-specific logged measurement configuration for Logged MDT in the UE. When the network provides a configuration, any previously configured logged measurement configuration will be entirely replaced by the new one. Moreover, logged measurements corresponding to the previous configuration will be cleared at the same time. It is left up to the network to retrieve any relevant data before providing a new configuration.

Additionally, we want to emphasize that when the UE receives a new logged MDT configuration, it shall discard the ongoing logged MDT measurement, as specified in TS37.320.

Therefore, when UE transitions into RRC_CONNECTED, and is reconfigured by RAN for continuous MDT collection with logged MDT configuration and the Immediate MDT configuration, the new logged configuration will replace the old one.

- Only one RAT-specific logged measurement configuration for Logged MDT in the UE. When the network provides a configuration, any previously configured logged measurement configuration will be entirely replaced by the new one.

For solution 1, UEs that are in idle mode when the OAM system stops C-MDT will still indicate report availability when they transition to connected mode. The gNBs will retrieve the logs by the UEs and forward them to the TCE. However, there will be no immediate MDT configuration available correlated to these logs. The network will therefore collect discontinuous logged MDT (i.e. without corresponding immediate MDT in connected mode) during 2 hours.

For solution 2, if the logged MDT duration expires when the UE is in idle mode, the gNB will not be aware of the expiry of the C-MDT session when the UE reconnects. So, the gNB will still reconfigure immediate MDT, and also transfer the Continuous MDT Indicator to the target node in case of Xn Handover or during transition from RRC_Inactive to RRC_Connected. Solution 2 therefore implies that the immediate MDT measurements collected during the last connection will not represent continuity with logged MDT measurements.

Also, R3-258173 also mentioned the drawbacks of its proposed solution 1& solution 2:

The drawback is that the continuity with MDT measurements is interrupted. Actually, we understand that Continuous MDT collection is for AI/ML training, and the current solution employs a best-effort approach to collect data.

For the continuity interruption issue in Solution 1, if the OAM has already deactivated the TR for this Continuous MDT, the continuity requirement has actually been canceled by the consumer, so there is no continuity interruption problem.

For the continuity interruption issue in Solution 2, if the logged MDT measurement times expired, leading to a gap between the Logged MDT measurement and subsequent Immediate measurements resulting in discontinuous data, this can be solved by setting proper logged MDT duration value as implementation. During data collection, the OAM ultimately performs screening and data cleaning, and cannot guarantee that every data collection will be continuously usable—this is a necessary step in AI/ML data processing.

Furthermore, in the legacy logged MDT procedure, if the UE's storage memory for data is full, the UE will also stop the logged MDT measurement. This will also lead to the continuity of Logged MDT measurements being interrupted.

- The current solution to support Continuous MDT collection is a best-effort approach to ensure continuity of MDT data across RRC states. It can not be guaranteed that every-time data collection will always be continuous, considering factors such as improper logging duration, UE memory size, and so on.

Anyway, the issue proposed in R3-258173 is not a reason to prevent finalizing RAN3 work on Continuous MDT collection. We agree with the CRs in TS 38.423 [2], TS 38.300 [3], TS 37.320 [4], and TS 38.401 [5].

- Agree with the CRs in TS 38.423 [2], TS 38.300 [3], TS 37.320 [4], and TS 38.401 [5].

3	Conclusion

Following is our observations and proposals:

- In cases of overlapping MDT configuration requests, the signalling-based request shall override the management-based request.

- The priority of signalling-based MDT is always higher than that of management-based MDT, including C-MDT.

- SA5 has already specified the termination of MDT data collection, which also applies to Continuous MDT collection.

- RAN3 provided the requirement to SA5 to check the security aspects of the Continuous MDT solution derived before, and SA5 has also check the security issues.

- No security issue should be discussed in RAN3 for continuous MDT collection.

- Reconfigure the logged MDT configuration and the Immediate MDT configuration for the UE after UE transitions into RRC_CONNECTED.

- Only one RAT-specific logged measurement configuration for Logged MDT in the UE. When the network provides a configuration, any previously configured logged measurement configuration will be entirely replaced by the new one.

- The current solution to support Continuous MDT collection is a best-effort approach to ensure continuity of MDT data across RRC states. It can not be guaranteed that every-time data collection will always be continuous, considering factors such as improper logging duration, UE memory size, and so on.

- Agree with the CRs in TS 38.423 [2], TS 38.300 [3], TS 37.320 [4], and TS 38.401 [5].

4	Reference

- R3-258173, Discussion on Continuous Management Based MDT, Nokia, Qualcomm

- R3-258433, Continuous MDT support, Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon, ZTE, Huawei, Telecom Italia, NEC, Samsung, CATT, China Telecom, China Unicom

- R3-258216, Support for Continuous Management-based MDT, ZTE Corporation, Huawei, CATT, CMCC, Samsung, China Unicom, China Telecom, Ericsson, FiberCop, Deutsche Telekom, Jio Platforms

- R3-258458, Support for continuous management-based MDT, CATT, Ericsson, ZTE, CMCC, Samsung, NEC

- R3-258685, Support for Continuous Management-based MDT in split architecture Samsung, CATT, ZTE, CMCC, China Telecom, Huawei, Ericsson, FiberCop, Deutsche Telekom, China Unicom, NEC

- R3-253958, Reply LS to LS on Continuous MDT, Ericsson