3GPP TSG-RAN WG3 Meeting #129-bis	R3-257224

Prague, Czech Republic, 13th ~17th Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1633 | Rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia, ZTE, LGE, NEC, Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following three IEs in the RRC LTM-Candidate IE are still missing in F1AP which cause related functions not workable: - ltm-NZP-CSI-RS-ResourceSet - ltm-CSI-IM-Resource - ltm-CSI-IM-ResourceSet |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the following IEs in LTM preparation related procedures. - ltm-NZP-CSI-RS-ResourceSet - ltm-CSI-IM-Resource for periodic and semi-persistent - ltm-CSI-IM-ResourceSet for periodic and semi-persistent |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some functions of inter-CU LTM are not workable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.360, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.423CR 1583 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | v0, R3-257129 |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 9.3.1.360		CSI-RS Resource Configuration

This IE contains the CSI-RS resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodic CSI-RS Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8]. |
| Semi Persistent CSI-RS Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8]. |
| CSI-RS Resource Configuration To Release List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| Periodic CSI-RS Resource Set Configuration To AddModList | O |  | OCTET STRING | Includes ltm-NZP-CSI-RS-ResourceSetToAddModList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| Semi-Persistent CSI-RS Resource Set Configuration To AddModList | O |  | OCTET STRING | Includes ltm-NZP-CSI-RS-ResourceSetToAddModList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| Periodic CSI-IM Resource Configuration To AddModList | O |  | OCTET STRING | Includes ltm-CSI-IM-ResourceToAddModList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| Semi-Persistent CSI-IM Resource Configuration To AddModList | O |  | OCTET STRING | Includes ltm-CSI-IM-ResourceToAddModList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| Periodic CSI-IM ResourceSet to AddMod List | O |  | OCTET STRING | Includes ltm-CSI-IM-ResourceSetToAddModList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| Semi-Persistent CSI-IM ResourceSet to AddMod List | O |  | OCTET STRING | Includes ltm-CSI-IM-ResourceSetToAddModList contained in the LTM-Config IE as defined in TS 38.331 [8]. |

<< Next Change >>

CSI-RSResourceConfig ::= SEQUENCE  {

periodicCSI-RSResourceConfigurationToAddModList		OCTET STRING	OPTIONAL,

spCSI-RSResourceConfigurationToAddModList			OCTET STRING	OPTIONAL,

cSI-RSResourceConfigurationToReleaseList			OCTET STRING	OPTIONAL,

periodicCSI-RSResourceSetConfigurationToAddModList	OCTET STRING	OPTIONAL,

spCSI-RSResourceSetConfigurationToAddModList		OCTET STRING	OPTIONAL,

periodicCSI-IMResourceConfigurationToAddModList		OCTET STRING	OPTIONAL,

spCSI-IMResourceConfigurationToAddModList			OCTET STRING	OPTIONAL,

periodicCSI-IMResourceSetConfigurationToAddModList	OCTET STRING	OPTIONAL,

spCSI-IMResourceSetConfigurationToAddModList		OCTET STRING	OPTIONAL,

iE-Extensions										ProtocolExtensionContainer { { CSI-RSResourceConfig-ExtIEs} }	OPTIONAL,

...

}

CSI-RSResourceConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<< End of Changes >>