3GPP TSG-RAN WG3 Meeting #129bis	R3-257300

Prague, CZ, 13th-17th, Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1606 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Semi-Persistent CSI-RS activation with TCI state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, ZTE, China Telecom, Ericsson, LG Electronics, Samsung, Huawei, NEC, ZTE, Google, LG Electronics, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For each periodic CSI-RS resource, the candidate gNB-DU provides each CSI-RS associated with pre-configured QCL-info (via qcl-InfoPeriodicCSI-RS, which indicates a TCI state ID). However, for SP CSI-RS, this is not possible. The reason is that Semi-Persistent CSI-RSs are pre-configured but activated dynamically, based on needs determined from periodic SSB or CSI-RS measurements. Therefore, when the source gNB-DU selects an SP CSI-RS for activation, it also needs to determine the appropriate TCI State and indicate it to the candidate gNB-DU(s), this ensures that the Semi-Persistent CSI-RS can be properly activated with the correct TCI State. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TCI State Information List IE in DU-CU CSI-RS COORDINATION REQUEST message and CU-DU CSI-RS COORDINATION REQUEST message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Candidate gNB-DU is unable to know the TCI State for SP CSI-RS activation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.12.2, 8.3.13.2, 9.2.2.18, 9.2.2.20, 9.4.5 |  |  |  |  |  |  |  |  |
| .4 |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR1540 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: R3-256872 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8.3.12	DU-CU CSI-RS Coordination

8.3.12.1	General

The purpose of the DU-CU CSI-RS Coordination procedure is e.g. to enable the gNB-DU to request the gNB-CU to activate/deactivate the SP CSI-RS transmissions from specific cells. The procedure uses UE-associated signalling.

8.3.12.2	Successful Operation

Figure 8.3.12.2-1: DU-CU CSI-RS Coordination procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CSI-RS COORDINATION REQUEST message.

If the TCI State Information List IE is included in the DU-CU CSI-RS COORDINATION REQUEST message, the gNB-DU shall, if supported, use it for Semi-Persistent CSI-RS activation.

8.3.12.3	Unsuccessful Operation

Not applicable.

8.3.12.4	Abnormal Conditions

Not applicable.

8.3.13	CU-DU CSI-RS Coordination

8.3.13.1	General

The purpose of the CU-DU CSI-RS Coordination procedure is e.g. to enable the gNB-CU to request the gNB-DU to activate/deactivate the SP CSI-RS transmission from specific cells. The procedure uses UE-associated signalling.

8.3.13.2	Successful Operation

Figure 8.3.13.2-1: CU-DU CSI-RS COORDINATION procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CSI-RS COORDINATION REQUEST message.

If the TCI State Information IE is included in the CU-DU CSI-RS COORDINATION REQUEST message, the gNB-DU shall, if supported, use it for Semi-Persistent CSI-RS activation.

8.3.13.3	Unsuccessful Operation

Not applicable

8.3.13.4	Abnormal Conditions

Not applicable

9.2.2.18	DU-CU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-DU to request the gNB-CU e.g. to activate/deactivate the SP CSI-RS transmissions from specific cells.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Gnb-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Gnb-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Resource Coordination List |  | 1 |  |  | YES | ignore |
| >CSI-RS ResourceCoordination Request Item |  | 1 .. <maxnoofLTMCSI-RSResourceConfig> |  |  | - |  |
| >>LTM CSI Resource Configuration ID | M |  | INTEGER (0..111) |  | - |  |
| >>Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | - |  |
| >>TCI State Information List |  | 0..1 |  | Indicates the TCI states where the semi persistent CSI-RS resource transmits. The mapping between the CSI-RS Resource indicated by the LTM CSI Resource Configuration ID IE and the TCI state is defined in TS 38.321 [12]. | - |  |
| >>>TCI State Information Item |  | 1 .. < maxnoofLTM-CSI-ResourcesPerSet> |  |  | - |  |
| >>>>Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCSI-RSResourceConfig | Maximum number of LTM CSI-Resource Configurations. Value is 112. |
| maxnoofLTM-CSI-ResourcesPerSet | Maximum number of LTM CSI-RS resource per set. Value is 512. |

9.2.2.19	DU-CU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-CU e.g. to inform the gNB-DU about the SP CSI-RS transmissions activation/deactivation result.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Coordination Result List |  | 0..1 |  |  | YES | ignore |
| >CSI-RS Coordination Result Item IEs |  | 1 .. < maxnoofLTMCSI-RSResourceConfig > |  |  | - |  |
| >>LTM CSI Resource Configuration ID | M |  | INTEGER (0..111) |  | - |  |
| >>Transmission Status | M |  | ENUMERATED(activated, deactivated, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCSI-RSResourceConfig | Maximum number of LTM CSI-Resource Configurations. Value is 112. |

9.2.2.20	CU-DU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-CU e.g. to coordinate the gNB-DU to activate/deactivate the SP CSI-RS transmissions from specific cells.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Coordination Request List |  | 1 |  |  | YES | ignore |
| >CSI-RS Coordination Request Item |  | 1 .. <maxnoofLTMCSI-RSResourceConfig> |  |  | - |  |
| >>LTM CSI Resource Configuration ID | M |  | INTEGER (0..111) |  | - |  |
| >>Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | - |  |
| >>TCI State Information List |  | 0..1 |  | The mapping between LTM CSI Resource and TCI state as defined in TS 38.321 [12]. | - |  |
| >>>TCI State Information Item |  | 1 .. < maxnoofLTM-CSI-ResourcesPerSet> |  |  | - |  |
| >>>>Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCSI-RSResourceConfig | Maximum number of LTM CSI-Resource Configurations. Value is 112. |
| maxnoofLTM-CSI-ResourcesPerSet | Maximum number of LTM CSI-RS resource per set. Value is 512. |

9.2.2.21	CU-DU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-DU e.g. to coordinate the gNB-CU about the SP CSI-RS transmissions activation/deactivation result.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Coordination Result List |  | 0..1 |  |  | YES | ignore |
| >CSI-RS Coordination Result Item IEs |  | 1 .. < maxnoofLTMCSI-RSResourceConfig > |  |  | - |  |
| >>LTM CSI Resource Configuration ID | M |  | INTEGER (0..111) |  | - |  |
| >>Transmission Status | M |  | ENUMERATED(activated, deactivated, …) |  | - |  |

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Coverage-Modification-Cause,

-------skipped------

#### -- C

CAGID ::= BIT STRING (SIZE(32))

Cancel-all-Warning-Messages-Indicator ::= ENUMERATED {true, ...}

Candidate-SpCell-Item ::= SEQUENCE {

candidate-SpCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { Candidate-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Candidate-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CandidateCellwithBeamInfo	::= SEQUENCE {

nRCGI				NRCGI,

sSBIndex			SSBIndex,

iE-Extensions		ProtocolExtensionContainer { { CandidateCellwithBeamInfo-ExtIEs } }	OPTIONAL

}

CandidateCellwithBeamInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

-------skipped------

CSI-RSCoordinationRequestList	::= SEQUENCE (SIZE(1.. maxnoofLTMCSI-RSResourceConfig)) OF CSI-RSCoordinationRequest-Item

CSI-RSCoordinationRequest-Item	::= SEQUENCE {

ltmCSIResourceConfigurationID		INTEGER (0..111),

transmissionRequest					ENUMERATED{activate, deactivate},

tci-State-InformationList			Tci-State-InformationList 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CSI-RSCoordinationRequest-Item-ExtIEs} }	OPTIONAL,

...

}

CSI-RSCoordinationRequest-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TAI ::= SEQUENCE {

pLMN-Identity				         PLMN-Identity,

fiveGS-TAC					         FiveGS-TAC,

iE-Extensions					ProtocolExtensionContainer { {TAI-ExtIEs} } OPTIONAL,

...

}

TAI-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TAAssistanceInfo ::=  ENUMERATED{zero, ...}

FiveGS-TAC ::= OCTET STRING (SIZE(3))

Configured-EPS-TAC ::= OCTET STRING (SIZE(2))

TagIDPointer ::= OCTET STRING

TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

-------skipped------

Tci-State-InformationList	::=  SEQUENCE (SIZE(1.. maxNrofLTM-CSI-ResourcesPerSet)) OF Tci-State-Information-Item

Tci-State-Information-Item	::= SEQUENCE {

jointorDLTCIStateID			JointorDLTCIStateID,

iE-Extensions		ProtocolExtensionContainer { { Tci-State-Information-Item -ExtIEs } }	OPTIONAL,

...

}

Tci-State-Information-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNR							INTEGER ::= 12

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 5120

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels					INTEGER ::= 65536

maxnoofRoutingEntries					INTEGER ::= 1024

maxnoofIABSTCInfo						INTEGER ::= 45

maxnoofSymbols							INTEGER ::= 14

maxnoofServingCells						INTEGER ::= 32

maxnoofDUFSlots							INTEGER ::= 320

maxnoofHSNASlots						INTEGER ::= 5120

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofSSBarea							INTEGER ::=64

maxnoofChildIABNodes					INTEGER ::= 1024

maxnoofNonUPTrafficMappings				INTEGER ::= 32

maxnoofTLAsIAB							INTEGER ::= 1024

maxnoofMappingEntries					INTEGER ::= 67108864

maxnoofDSInfo							INTEGER ::= 64

maxnoofEgressLinks						INTEGER ::= 2

maxnoofULUPTNLInformationforIAB			INTEGER ::= 32678

maxnoofUPTNLAddresses					INTEGER ::= 8

maxnoofSLDRBs							INTEGER ::= 512

maxnoofQoSParaSets						INTEGER ::= 8

maxnoofPC5QoSFlows						INTEGER ::= 2048

maxnoofSSBAreas							INTEGER ::=	64

maxnoofPhysicalResourceBlocks			INTEGER ::= 275

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRAReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

maxnoofRLCDuplicationState				INTEGER ::=	3

maxnoofCHOcells							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::=	16

maxnoofCAGsupported						INTEGER ::= 12

maxnoofNIDsupported						INTEGER ::= 12

maxnoofNRSCSs							INTEGER ::= 5

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofPosMeas							INTEGER ::=	16384

maxnoofTRPInfoTypes						INTEGER ::=	64

maxnoofTRPs								INTEGER ::=	65535

maxnoofSRSTriggerStates					INTEGER ::= 3

maxnoofSpatialRelations					INTEGER ::= 64

maxnoBcastCell							INTEGER ::= 16384

maxnoofAngleInfo						INTEGER ::= 65535

maxnooflcs-gcs-translation				INTEGER ::= 3

maxnoofPath								INTEGER ::= 2

maxnoofMeasE-CID						INTEGER ::= 64

maxnoofSSBs								INTEGER ::= 255

maxnoSRS-ResourceSets					INTEGER ::= 16

maxnoSRS-ResourcePerSet					INTEGER ::= 16

maxnoSRS-Carriers						INTEGER ::= 32

maxnoSCSs								INTEGER ::= 5

maxnoSRS-Resources						INTEGER ::= 64

maxnoSRS-PosResources					INTEGER ::= 64

maxnoSRS-PosResourceSets				INTEGER ::= 16

maxnoSRS-PosResourcePerSet				INTEGER ::= 16

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofSuccessfulHOReports				INTEGER ::= 64

maxnoofNR-UChannelIDs					INTEGER ::= 16

maxServedCellforSON						INTEGER ::= 256

maxNeighbourCellforSON					INTEGER ::= 32

maxAffectedCells						INTEGER ::= 32

maxnoofMRBs								INTEGER ::= 32

maxnoofMBSQoSFlows						INTEGER ::= 64

maxnoofMBSFSAs 							INTEGER ::= 256

maxnoofUEIDforPaging 					INTEGER ::= 4096

maxnoofCellsforMBS						INTEGER ::= 512

maxnoofTAIforMBS						INTEGER ::= 512

maxnoofMBSAreaSessionIDs				INTEGER ::= 256

maxnoofMBSServiceAreaInformation		INTEGER ::= 256

maxnoofIABCongInd						INTEGER ::= 1024

maxnoofNeighbourNodeCellsIAB			INTEGER ::= 1024

maxnoofRBsetsPerCell					INTEGER ::= 8

maxnoofRBsetsPerCell-1					INTEGER ::= 7

maxnoofMeasPDC							INTEGER ::= 16

maxnoARPs								INTEGER ::=	16

maxnoofULAoAs							INTEGER ::= 8

maxNoPathExtended						INTEGER ::= 8

maxnoTRPTEGs							INTEGER ::= 8

maxFreqLayers							INTEGER ::= 4

maxNumResourcesPerAngle					INTEGER ::= 24

maxnoAzimuthAngles						INTEGER ::= 3600

maxnoElevationAngles					INTEGER ::= 1801

maxnoofPRSTRPs							INTEGER ::= 256

maxnoofQoEInformation					INTEGER ::= 16

maxnoofUuRLCChannels					INTEGER ::= 32

maxnoofPC5RLCChannels					INTEGER ::= 512

maxnoofSMBRValues						INTEGER ::= 8

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

maxnoofUETypes							INTEGER ::= 8

maxnoofLTMCells							INTEGER	::= 8

maxnoofTAList							INTEGER	::= 8

maxnoofLTMgNB-DUs						INTEGER	::= 8

maxnoofUEsInQMCTransferControlMessage	INTEGER ::= 512

maxnoofUEsforRAReportIndications		INTEGER ::= 64

maxnoofSuccessfulPSCellChangeReports	INTEGER ::= 64

maxnoofPeriodicities					INTEGER ::= 8

maxnoofThresholdMBS-1					INTEGER ::= 7

maxMBSSessionsinSessionInfoList			INTEGER ::= 1024

maxnoofLBTFailureInformation			INTEGER ::= 64

maxnoofRSPPQoSFlows						INTEGER ::= 2048

maxnoVACell								INTEGER ::= 32

maxnoAggregatedSRS-Resources			INTEGER ::= 3

maxnoAggregatedPosSRSResourceSets		INTEGER ::= 3

maxnoAggregatedPosPRSResourceSets		INTEGER ::= 3

maxnoofTimeWindowSRS					INTEGER ::= 16

maxnoofTimeWindowMea					INTEGER ::= 16

maxnoPreconfiguredSRS					INTEGER ::= 16

maxnoHopsMinusOne						INTEGER ::= 5

maxnoAggCombinations					INTEGER ::= 2

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

maxnoofCandidateCells					INTEGER ::= 8

maxnoofSSBIndices						INTEGER ::= 64

maxnoofPreambleIndex					INTEGER ::= 64

maxnoofThresholds						INTEGER ::= 8

maxnoofNZP-CSI-RS-ResourcesPerSet		INTEGER ::= 64

maxnoofSRS-Resources					INTEGER ::= 64

maxnoofCellsinUEHistoryInfo				INTEGER ::= 16

maxnoofLTMCSI-RSResourceConfig			INTEGER ::= 112

maxnoofCSI-RSs							INTEGER ::= 192

maxnoofTAs								INTEGER ::= 2

maxnoofChannelRes						INTEGER ::= 24

maxNeighbourCellReport					INTEGER ::= 512

maxnoofLTM-CSI-ResourcesPerSet			INTEGER ::= 512