3GPP TSG-RAN WG3 Meeting #129-bis	R3-257240

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	10.3.1

Source:	Huawei (moderator)

Title:	(TP to TR 38.760-3) RAN-CN interface principles and functions

Document for:	Agreement

1	Introduction

This contribution provides the TP to the TR 38.760-3 on the RAN-CN interface principles and functions based on the discussion of the following CB:

CB: # 20_6GRAN-CNinf

- TP for section 6.1.1 & 6.1.2 (RAN-CN interface general principles and functions)

- Capture open issues for next meeting

- Introduce new sections in the TR, if agreeable

(Huawei - moderator)

2	TP to TR 38.760-3

---------------Start of the Change------------------

6.1		RAN-CN Interface

6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

[Easy ones, same as NG]

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer;

-	the 6G RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions;

-	the 6G RAN-CN interface is decoupled with the possible RAN deployment variants;

[Easy ones_new principles proposed by the online treated papers]

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing.

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN interface supports reliable signalling transmission;

-	the 6G RAN-CN interface supports large scale of RAN nodes deployment;

-	the 6G RAN-CN interface supports clear boundaries between RAN and CN;

[Need more discussion]

-	The 6G RAN-CN interface uses different procedures for different functionalilties, as needed;

-	the 6G RAN-CN interface, from logical standpoint, is a point-to-point interface between an RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the RAN node and CN node;

NOTE:	It is FFS on new principles for the new services (e.g., AI and sensing, etc), can be added after the progress of these topics.

-	The 6G RAN-CN interface shall be friendly to flexible new services deployment and on-demand infrastructure, e.g., API.

6.1.2	RAN-CN Interface Functions

RAN-CN control plane interface supports following functions:

[Easy ones, same as NG]

-	Interface management: The functionality to manage the RAN-CN Control Plane interface;

-	UE context management	: The functionality to manage the UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE;

-	PDU Session Management: The functionality to establish, manage and remove PDU sessions and respective RAN resources.

NOTE:	It is FFS on the PDU Session, subject to progress in SA2.

-	Configuration Transfer: The functionality to transfer the RAN configuration information (e.g., transport layer addresses for establishment of interface between two RAN nodes) between two RAN nodes via the CN.

-	Warning Message Transmission: The functionality to transfer warning messages via RAN-CN interface or cancel ongoing broadcast of warning messages;

[Need more discussion]

-	Paging: The functionality to send paging requests to the RAN nodes;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN;

NOTE:	It is FFS on the Paging function and the UE Moblility management function, subject to futher progress of the discussion on Mobility.

-	New service management: The functionality to manage the new services (e.g., AI and sensing, etc) between the RAN and CN;

6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

6.1.4	Evaluation of RAN-CN Interface Options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 6.1.3.

---------------End of the Change------------------