3GPP TSG-RAN WG3 Meeting #132	R3-262526

Dalian, China, 18th May – 22th May 2026

Agenda Item:	14.3

Source:	CMCC

Title:	(TP for 38.410 BL CR) Discussion on Topology 1 for Device C

Document for:	Discussion and Decision

# 1	Introduction

This TP for TS 38.410 is used to capture the RAN3 agreements made for Rel-20 Ambient IoT.

# TP for TS 38.410

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 5.7	NAS Node Selection function

The interconnection of an NG-RAN node to multiple AMFs is supported in the 5GS architecture.

Therefore, a NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE, based on the UE's temporary identifier, which was assigned to the UE by the AMF. When the UE's temporary identifier has not been yet assigned or is no longer valid the NG-RAN node may instead take into account other information (e.g. slicing information, onboarding indication) to determine the AMF. When the NG-RAN node is configured to ensure that the selected AMF serves the country where the UE is located, as described in TS 23.501 [8], the NG-RAN node takes into account UE location information, if available, when determining the AMF.

This functionality is located in the NG-RAN node and enables proper routing via the NG interface. On the NG interface, no specific procedure corresponds to the NAS Node Selection Function.

When an NR Femto node connects to an NR Femto GW, selection of an AMF at UE attachment is performed by the NR Femto GW.

The functionality is located in the NG-RAN node to determine the A-IoT CN node for the A-IoT device, based on the AIOTF routing information received from the A-IoT device.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 6.28	AIoT procedures

The following procedures are used for AIoT services:

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT Session Release (AIoT CN Initiated);

-	A-IoT Session Release Request (gNB initiated).

-	A-IoT Downlink NAS Transport;

-	A-IoT Uplink NAS Transport;

<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>