3GPP TSG-RAN WG3 Meeting #129          	          R3-255771

Bengaluru, India, Aug 25th- Aug 29th, 2025

Agenda item:	16.2

Source:	CMCC

Title:	(TP to BLCR for TS 38.300) Introduction of Ambient IoT

Document for:	Approval

# 1	Introduction

This contribution provides the TP to BLCR for TS 38.300 for all Ambient IoT related aspects based on the discussion paper.

# Text Proposal to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or multiple readers.

-	A gNB is a between A-IoT Areas and readers by means of OAM configuration.

-	An A-IoT Area may include readers belonging to the same or different gNB.

-	One reader only belongs to one gNB.

-	One reader can be mapped to one or multiple A-IoT Areas.

-	As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE 1:	A deployment with both direct connectivity and indirect connectivity is not expected.

-	In this version of the specification, A-IoT specific information is transported between the gNB and the A-IoT  CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of readers. The A-IoT Area is represented by an A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

The AIOTF is aware of the mapping relationship among gNBs, readers and A-IoT areas by means of OAM configuration, as needed.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>