3GPP TSG-RAN WG3 Meeting #129	R3-255959

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	10.3.1

Source:	ZTE Corporation, Huawei, Ericsson, Nokia, CATT, Samsung

Title:	(TP to MDT BL CR for 38.423) MDT Enhancement for NTN

Document for:	Discussion

1  Introduction

This TP is to introduce the geographical area scope of MDT for NTN.

2  TP to BL CR for 38.423 on MDT enhancement for NTN

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.420: "NG-RAN; Xn General Aspects and Principles".

[4]	3GPP TS 38.422: "NG-RAN; Xn Signalling Transport".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP) ".

[6]	3GPP TS 25.921: "Guidelines and principles for protocol description and error handling".

[7]	3GPP TS 23.501: "System Architecture for the 5G System".

[8]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[9]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) Protocol specification".

[11]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[12]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[13]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[14]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[15]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[16]	ITU-T Recommendation X.680 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[17]	ITU-T Recommendation X.681 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[18]	3GPP TS 29.281: "General Packet Radio Service (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[19]	3GPP TS 38.424: "NG-RAN; Xn data transport".

[20]	3GPP TS 38.414: "NG-RAN; NG data transport".

[21]	3GPP TS 38.412: "NG-RAN; NG Signalling Transport".

[22]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[23]	3GPP TS 32.422: "Trace control and configuration management".

[24]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[25]	3GPP TS 36.104: "Base Station (BS) radio transmission and reception ".

[26]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[27]	3GPP TS 36.101: "User Equipment (UE) radio transmission and reception".

[28]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[29]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[30]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[31]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[32]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[33]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[34]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[35]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[36]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[37]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[38]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[39]	3GPP TS 38.211: "NR; Physical channels and modulation".

[40]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[41]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[42]	3GPP TS 38.314: "NR; Layer 2 measurements".

[43]	3GPP TS 37.320: " Radio measurement collection for Minimization of Drive Tests (MDT),"

[44]	3GPP TS 36.423: " Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol (X2AP)".

[45]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[46]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[47]	3GPP TS 26.247: “Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)”.

[48]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[49]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[50]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[51]	3GPP TS 37.213: "NR; Physical layer procedures for shared spectrum channel access".

[52]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[53]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[54]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[55]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[56]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[57]	3GPP TS 23.527: "5G System; Restoration procedures".

[xx]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.1	Handover Preparation

8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

Unchanged part is omitted

If the DL LBT Failure Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the DL LBT failure information of the UE in the target cell during handover.

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the target NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, and the Geographical Area IE contains the MDT PLMN List IE, the target NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.4	Retrieve UE Context

8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission. The procedure can also be used to transfer the authorization status information of the mobile IAB-node.

The procedure uses UE-associated signalling.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

Unchanged part is omitted

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN. Upon reception of the PNI-NPN Area Scope of MDT IE, the new NG-RAN node shall consider that the area scope for MDT measurement collections of PNI-NPN areas is defined only by the areas included in the PNI-NPN Area Scope of MDT IE.

If the UE is a mobile IAB-node, the old NG-RAN node shall include the Mobile IAB Authorization Status IE in the RETRIEVE UE CONTEXT RESPONSE message. If the Mobile IAB Authorization Status IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, consider that the UE is a mobile IAB-node, then store it and use it accordingly as defined in TS 38.401 [2].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the new NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, and the Geographical Area IE contains the MDT PLMN List IE, the new NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

Interaction with the Retrieve UE Context Confirm procedure

If the UE Context Reference at the S-NG-RAN node IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node may use it to establish dual connectivity with the S-NG-RAN node and shall trigger the Retrieve UE Context Confirm procedure to the old NG-RAN node when the UE successfully resumes on the new NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.14	Trace Start

#### 8.3.14.1	General

The purpose of the Trace Start procedure is to allow the M-NG-RAN node to request the S-NG-RAN node to initiate a trace session for a UE. The procedure uses UE-associated signalling.

#### 8.3.14.2	Successful Operation

Figure 8.3.14.2-1: Trace Start, successful operation

The Trace Start procedure is initiated by the M-NG-RAN sending the TRACE START message to the S-NG-RAN for that specific UE. Upon reception of the TRACE START message, the S-NG-RAN node shall initiate the requested trace session as described in TS 32.422 [23].

Unchanged part is omitted

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the TRACE START message, the S-NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN. Upon reception of the PNI-NPN Area Scope of MDT IE, the S-NG-RAN node shall consider that the area scope for MDT measurement collection of PNI-NPN areas is defined only by the areas included in the PNI-NPN Area Scope of MDT IE.

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the TRACE START message, the S-NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the S-NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the TRACE START message, and the Geographical Area IE contains the MDT PLMN List IE, the S-NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.3.126	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Immediate MDT and Trace, Logged MDT only, ...) |  | – |  |
| CHOICE Area Scope of MDT-NR | O |  |  |  | – |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT-NR |  | 1 .. <maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. | – |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1 |  |  | – |  |
| >>>TAI List for MDT Item |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>>TAC | M |  | 9.2.2.5 |  | – |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.2.3.191 |  | – |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | – | - |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>TAC | M |  | 9.2.2.5 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >> SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >Geography Based MDT |  |  |  | The geographical area scope can be used with NTN deployment. |  |  |
| >>Geographical Area |  |  | 9.2.3.y |  |  |  |
| CHOICE MDT Mode | M |  |  |  | – |  |
| >Immediate MDT-NR |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [43]. First Bit = M1, Second Bit= M2, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 3. | – |  |
| >>M1 Configuration | C-ifM1 |  | 9.2.3.128 |  | – |  |
| >>M4 Configuration | C-ifM4 |  | 9.2.3.129 |  | – |  |
| >>M5 Configuration | C-ifM5 |  | 9.2.3.130 |  | – |  |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [43]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". | – |  |
| >>M6 Configuration | C-ifM6 |  | 9.2.3.131 |  | – |  |
| >>M7 Configuration | C-ifM7 |  | 9.2.3.132 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >Logged MDT-NR |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 , ms61440, infinity, ...) | Corresponds to information provided in the LoggingInterval IE as defined in TS 38.331 [10]. The value "infinity" represents one shot logging, i.e., only one log per event in the logged MDT report. | – |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90, 120) | Corresponds to information provided in the LoggingDuration IE as defined in TS 38.331 [10]. Unit: [minute]. | – |  |
| >>CHOICE Report Type | M |  |  |  | – |  |
| >>>Periodical |  |  |  |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Logged Event Trigger Config | M |  | 9.2.3.137 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >>Area Scope of Neighbour Cells | O |  | 9.2.3.140 |  | – |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [10]. | – |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | – |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  |  | YES | Ignore |
| >CAG List for MDT | M |  | 9.2.3.191 |  | – |  |
| Network Slice Area Scope of MDT | O |  | 9.2.3.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is 16. |

| Condition | Explanation |
| --- | --- |
| ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to "1". |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to "1". |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1". |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1". |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1". |

#### 9.2.3.x	Network Slice Area Scope of MDT

This IE is used to identify the list of network slices for MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  |
| --- | --- | --- | --- | --- | --- |
| Network Slice List for MDT |  | 1 |  |  |  |
| >Network Slice Item for MDT |  | 1..<maxnoofMDTPLMNs> |  |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |  |
| >Slice MDT List |  | 1 |  |  |
| >>Slice MDT Item |  | 1..<maxnoofSliceItemsforMDT > |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | Maximum no. of PLMNs in the MDT PLMN list. Value is 16. |
| maxnoofSliceItemsforMDT | Maximum no. of S-NSSAIs for MDT area scope. Value is 1024. |

#### 9.2.3.y	Geographical Area

This IE is used to indicate the geographical area scope for NTN MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NTN Geographical Area |  | 1 |  |  |
| >NTN Geographical Area Item |  | 1..<maxnoofAreaNTNforMDT> |  |  |
| >>CHOICE Area Type | M |  |  |  |
| >>>Circle |  |  |  |  |
| >>>>Reference Location | M |  | OCTET STRING | ReferenceLocation-r17 as defined in TS 38.331[10]. |
| >>>>Distance Radius | M |  | INTEGER(1..65535) | Each step represents 50m distance. |
| >>> Polygon |  |  |  |  |
| >>>> Polygon |  |  | OCTET STRING | The first/leftmost bit of the first octet contains the most significant bit, as defined in TS 37.355 [xx]. |
| >MDT PLMN List | O |  | 9.2.3.133 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAreaNTNforMDT | Maximum no. of the geographical area configurations. Value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<unchanged is omitted>>>

id-MN-only-MDT-collection,

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-NetworkSliceAreaScopeofMDT,

id-GeographicalArea,

id-GeographyBasedMDT

maxEARFCN,

maxnoofAllowedAreas,

<<<unchanged is omitted>>>

maxnoofMDTSNPNs,

maxnoofSecurityConfigurations,

maxnoofRSPPQoSFlows,

maxnoofSliceItemsforMDT,

maxnoofNTNAreasforMDT,

<<<unchanged is omitted>>>

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

tAIBased					TAIBasedMDT,

...,

choice-extension		ProtocolIE-Single-Container { {AreaScopeOfMDT-NR-ExtIEs} }

}

AreaScopeOfMDT-NR-ExtIEs XNAP-PROTOCOL-IES ::= {

{ ID id-PNI-NPNBasedMDT			CRITICALITY ignore	TYPE PNI-NPNBasedMDT			PRESENCE mandatory}|

{ ID id-SNPN-CellBasedMDT		CRITICALITY ignore	TYPE SNPN-CellBasedMDT			PRESENCE mandatory}|

{ ID id-SNPN-TAIBasedMDT		CRITICALITY ignore	TYPE SNPN-TAIBasedMDT			PRESENCE mandatory}|

{ ID id-SNPN-BasedMDT			CRITICALITY ignore	TYPE SNPN-BasedMDT			PRESENCE mandatory}|

{ID id-GeographyBasedMDT       CRITICALITY ignore	TYPE GeographyBasedMDT			PRESENCE mandatory},

...

}

<<<unchanged is omitted>>>

CHO-Candidate-PSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CHO-Candidate-PSCells-Item

CHO-Candidate-PSCells-Item ::= SEQUENCE {

pscell-id								NR-CGI,

target2source-NG-RANNode-Container		OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { {CHO-Candidate-PSCells-Item-ExtIEs}}	OPTIONAL,

...

}

CHO-Candidate-PSCells-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

Circle ::= SEQUENCE {

referenceLocation			OCTECT STRING,

distanceRadius				INTEGER(1..65536, ...),

iE-Extensions				ProtocolExtensionContainer { { Circle-ExtIEs} }	OPTIONAL,

...

}

Circle-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<unchanged is omitted>>>

GBRQoSFlowInfo ::= SEQUENCE {

maxFlowBitRateDL			BitRate,

maxFlowBitRateUL			BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			ENUMERATED {notification-requested, ...}				OPTIONAL,

maxPacketLossRateDL			PacketLossRate											OPTIONAL,

maxPacketLossRateUL			PacketLossRate											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GBRQoSFlowInfo-ExtIEs} }	OPTIONAL,

...

}

GBRQoSFlowInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	},

...

}

GeographyBasedMDT::= SEQUENCE {

geographicalArea		GeographicalArea,

iE-Extensions		ProtocolExtensionContainer { {GeographyBasedMDT-ExtIEs} } OPTIONAL,

...

}

GeographyBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GeographicalArea ::= SEQUENCE {

nTNGeographicalArea		NTNGeographicalArea,

nTNPLMNList				MDTPLMNList											OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {GeographicalArea-ExtIEs} }	OPTIONAL,

...

}

GeographicalArea-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<unchanged is omitted>>>

NRUESidelinkAggregateMaximumBitRate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {NRUESidelinkAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

NRUESidelinkAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NSAG-ID ::= INTEGER (0..255, ...)

NTNGeographicalArea ::= SEQUENCE (SIZE(1.. maxnoofAreaNTN)) OF NTNGeographicalAreaItem

NTNGeographicalAreaItem ::= CHOICE {

circle						Circle,

polygon				        Polygon,

cHOICE-Extensions			ProtocolIE-SingleContainer { {NTNGeographicalAreaItem-ExtIEs} }

}

NTNGeographicalAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

<<<unchanged is omitted>>>

#### -- P

Polygon ::= OCTET STRING

PositioningInformation ::= SEQUENCE {

requestedSRSTransmissionCharacteristics				RequestedSRSTransmissionCharacteristics,

routingID											RoutingID,

nRPPaTransactionID									INTEGER (0..32767),

iE-Extension 				ProtocolExtensionContainer { { PositioningInformation-ExtIEs} } OPTIONAL,

...

}

PositioningInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<unchanged is omitted>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

<<<unchanged is omitted>>>

maxnoofSecurityConfigurations				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofSliceItemsforMDT						INTEGER ::= 1024

maxnoofAreaNTNforMDT						INTEGER ::= 32

<<<unchanged is omitted>>>

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-NetworkSliceAreaScopeofMDT																		ProtocolIE-ID ::= XX1

id-GeographicalArea        																		  ProtocolIE-ID ::= XX2

id-GeographyBasedMDT	      																      ProtocolIE-ID ::= XX3

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>