3GPP TSG-RAN WG3 Meeting #129	R3-255811

Bengaluru, India, 25th ~29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0133 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of LTE-based 5G Broadcast Phase 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Qualcomm Incorporated, EBU, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to WID RP-250794, the time/frequency interleaver is introduced for LTE-based 5G Terrestrial Broadcast. The M2AP should be enhanced to support the the time/frequency interleaver. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the PMCH Configuration IE, introduce MCH Scheduling Period Extended 3 IE, Time Interleaving Parameters IE and Frequency Interleaving Indicator IE to support time/frequency interleaver. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support LTE-based 5G Broadcast Phase 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2, 9.2.1.8, 9.2.1.xx(new), 9.2.1.yy(new), 9.3.4, and 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: add co-source company, fix typos. |  |  |  |  |  |  |  |  |

CHANGES START

## 8.4	MBMS Scheduling Information

### 8.4.1	General

The purpose of the MBMS Scheduling Information Procedure is to provide MCCH related information, and optional session suspension decision to the eNB.

The procedure uses non MBMS-Service-associated signalling.

### 8.4.2	Successful Operation

Figure 8.4.2-1. MBMS Scheduling Information procedure. Successful operation.

The MCE initiates the procedure by sending the MBMS SCHEDULING INFORMATION message to the eNB. The eNB shall store the MBSFN Area Configuration Item IE, apply the MCCH update from the modification period defined in the MCCH Update Time IE, and transmit the MCCH according to the MCCH configuration for the given MBSFN area indicated by the MCE. If an empty PMCH Configuration List IE is contained in this message for an MBSFN area, the eNB shall update the content of the corresponding MCCH so as not to include PMCH related information over the corresponding MCCH. If the Modulation and Coding Scheme 2 IE is included, the eNB shall ignore the Modulation and Coding Scheme IE and use the Modulation and Coding Scheme 2 IE instead. If the received PMCH Configuration IE contains the MCH Scheduling Period Extended IE, the eNB shall take its value into account instead of the value signalled in the MCH Scheduling Period IE. If the received PMCH Configuration IE contains the MCH Scheduling Period Extended 2 IE, the eNB shall take its value into account instead of the value signalled in the MCH Scheduling Period IE. If the received PMCH Configuration IE contains the MCH Scheduling Period Extended 3 IE, the eNB shall take its value into account instead of the value signalled in the MCH Scheduling Period IE. The eNB shall schedule the MBMS services in the MCCH according to the order defined in the MBMS Session List per PMCH IE. If the MBMS Suspension Notification List IE is included in this message for an MBSFN area, the eNB shall broadcast the suspension decision over the air interface from the radio frame defined by the SFN IE, until the end of the Modification Period just before the "MCCH Update Time". If the received PMCH Configuration IE contains the Frequency Interleaving Indicator IE, the eNB shall, if supported, store it and take it into account. If the received PMCH Configuration IE contains the Time Interleaving Indicator IE, the eNB shall, if supported, store it and take it into account.

If the MBMS SCHEDULING INFORMATION message contains the Subframe Allocation Extended IE included in the MBSFN Subframe Configuration IE, the eNB shall, if supported, store it and take it into account.

### 8.4.3	Abnormal Conditions

Void.

NEXT CHANGE

#### 9.2.1.8	PMCH Configuration

This information element provided PMCH configuration related content for MCCH.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Allocated Subframes End | M |  | INTEGER (0..1535) | Encoded as the sf-AllocEnd IE in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme | M |  | INTEGER (0..28) | Encoded as the dataMCS IE in TS 36.331 [11]. | − | − |
| MCH Scheduling Period | M |  | ENUMERATED (rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024) | Encoded as the mch-SchedulingPeriod IE in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme 2 | O |  | INTEGER (0..27) | Encoded as the t2 in the dataMCS IE in TS 36.331 [11]. If this IE is present, the value signalled in the Modulation and Coding Scheme IE is ignored. | YES | reject |
| MCH Scheduling Period Extended | O |  | ENUMERATED (rf4, …) | Encoded as the mch-SchedulingPeriod-r12 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| MCH Scheduling Period Extended 2 | O |  | ENUMERATED (rf1, rf2, …) | Encoded as the mch-SchedulingPeriod-v14x0 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| MCH Scheduling Period Extended 3 | O |  | ENUMERATED (rf7, rf14, rf28, rf53, rf56, rf108, rf112, rf212, rf424, …) | Encoded as the mch-SchedulingPeriod-v19xy contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| Frequency Interleaving Indicator | O |  | 9.2.1.xx |  | YES | reject |
| Time Interleaving Parameters | O |  | 9.2.1.yy |  | YES | reject |

NEXT CHANGE

#### 9.2.1.xx	Frequency Interleaving Indicator

This IE indicates whether frequency interleaving is enabled or not.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Frequency Interleaving Indicator | M |  | ENUMERATED (enabled, …) | Encoded as the pmch-FreqInterleaving-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |

#### 9.2.1.yy	Time Interleaving parameters

This IE defines the time interleaving parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Value M | M |  | ENUMERATED (sf4, sf8, sf16, sf32, …) | Encoded as the pmch-TimeInterleavingM-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Value N | M |  | ENUMERATED (n2, n4, n8, n16, …) | Encoded as the pmch-TimeInterleavingN-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Scaling factor Beta | M |  | ENUMERATED (one32th, one5th, one3rd, three8th, five12th, onehalf, five8th, two3rd, five6th, one, …) | Encoded as the scalingFactorBeta-r19 in the softBufferSizeParameters-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Reference UE Category | M |  | INTEGER (4..26, ...) | Encoded as the refUE-CategoryDL-r19 in the softBufferSizeParameters-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Value M-Last MTCH | O |  | ENUMERATED (sf4, sf8, sf16, sf32, …) | Encoded as the pmch-TimeInterleavingM-LastMTCH-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Value N-Last MTCH | O |  | ENUMERATED (n2, n4, n8, n16, …) | Encoded as the pmch-TimeInterleavingN-LastMTCH-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Cyclic Shift Alpha | O |  | ENUMERATED (alphaOne, alphaOther, …) | Encoded as the pmch-CyclicShiftAlpha-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331[11]. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

M2AP-Ies {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) m2ap (4) version1 (1) m2ap-Ies (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MCH-Scheduling-PeriodExtended,

id-MCH-Scheduling-PeriodExtended2,

id-Modification-PeriodExtended,

id-Modulation-Coding-Scheme2,

id-Repetition-PeriodExtended,

id-Subcarrier-SpacingMBMS,

id-SubframeAllocationExtended,

id-SubframeAllocationFurtherExtension,

id-AdditionalConfigParameters,

id-FrequencyInterleavingIndicator,

id-TimeInterleavingParameters,

id-MCH-Scheduling-PeriodExtended3,

maxnoofMBSFNareas,

maxnoofPMCHsperMBSFNarea,

maxnoofCells,

maxnoofMBMSServiceAreasPerCell,

maxnoofSessionsPerPMCH,

maxnooferrors,

maxnoofCellsforMBMS

FROM M2AP-Constants

### //skip unchanged part

ENBname ::= PrintableString (SIZE (1..150,...))

EUTRANCellIdentifier ::= BIT STRING (SIZE (28))

#### -- F

#### FrequencyInterleavingIndicator ::= ENUMERATED {enabled, ...}

#### -- G

GBR-QosInformation ::= SEQUENCE {

mBMS-E-RAB-MaximumBitrateDL			BitRate,

mBMS-E-RAB-GuaranteedBitrateDL		BitRate,

iE-Extensions						ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

### //skip unchanged part

#### -- M

MBMS-Cell-List ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBMS)) OF ECGI

MBMS-E-RAB-QoS-Parameters ::= SEQUENCE {

qCI								QCI,

gbrQosInformation				GBR-QosInformation		OPTIONAL,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

iE-Extensions					ProtocolExtensionContainer { { MBMS-E-RAB-QoS-Parameters-ExtIEs} }	OPTIONAL,

...

}

MBMS-E-RAB-QoS-Parameters-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

MBMS-Service-associatedLogicalM2-ConnectionItem ::= SEQUENCE {

eNB-MBMS-M2AP-ID				ENB-MBMS-M2AP-ID OPTIONAL,

mCE-MBMS-M2AP-ID				MCE-MBMS-M2AP-ID OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MBMS-Service-associatedLogicalM2-ConnectionItemExtIEs} } OPTIONAL,

...

}

MBMS-Service-associatedLogicalM2-ConnectionItemExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

MBMS-Service-Area ::= OCTET STRING

MBMS-Service-Area-ID-List ::= SEQUENCE (SIZE(1..maxnoofMBMSServiceAreasPerCell)) OF MBMS-Service-Area

MBMS-Session-ID ::= OCTET STRING (SIZE (1))

MBMSsessionListPerPMCH-Item ::= SEQUENCE (SIZE(1..maxnoofSessionsPerPMCH)) OF SEQUENCE {

tmgi							TMGI,

lcid							LCID,

iE-Extensions					ProtocolExtensionContainer { { MBMSsessionListPerPMCH-Item-ExtIEs} } OPTIONAL,

...

}

MBMSsessionListPerPMCH-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

MBMSsessionsToBeSuspendedListPerPMCH-Item ::= SEQUENCE (SIZE(1..maxnoofSessionsPerPMCH)) OF SEQUENCE {

tmgi							TMGI,

iE-Extensions					ProtocolExtensionContainer { { MBMSsessionsToBeSuspendedListPerPMCH-Item-ExtIEs} } OPTIONAL,

...

}

MBMSsessionsToBeSuspendedListPerPMCH-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

MBSFN-Area-ID ::= INTEGER (0..255)

MBSFN-SynchronisationArea-ID ::= INTEGER (0..65535)

MBSFN-Subframe-Configuration ::= SEQUENCE {

radioframeAllocationPeriod		ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset		INTEGER (0..7),

subframeAllocation				CHOICE {

oneFrame						BIT STRING (SIZE (6)  ),

fourFrames						BIT STRING (SIZE (24) ) },

iE-Extensions					ProtocolExtensionContainer { { MBSFN-Subframe-Configuration-ExtIEs} } OPTIONAL,

...

}

MBSFN-Subframe-Configuration-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

{ID id-SubframeAllocationExtended			CRITICALITY reject	EXTENSION SubframeAllocationExtended			PRESENCE optional}|

{ID id-SubframeAllocationFurtherExtension	CRITICALITY reject	EXTENSION SubframeAllocationFurtherExtension		PRESENCE optional},

...

}

MCCH-Update-Time ::= INTEGER (0..255)

MCCHrelatedBCCH-ConfigPerMBSFNArea-Item ::= SEQUENCE {

mbsfnArea						MBSFN-Area-ID,

pdcchLength						ENUMERATED {s1, s2, ...},

repetitionPeriod				ENUMERATED {rf32, rf64, rf128, rf256},

offset							INTEGER (0..10),

modificationPeriod				ENUMERATED {rf512, rf1024},

subframeAllocationInfo			BIT STRING (SIZE(6)),

modulationAndCodingScheme		ENUMERATED {n2, n7, n13, n19},

cellInformationList				Cell-Information-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MCCHrelatedBCCH-ConfigPerMBSFNArea-Item-ExtIEs} } OPTIONAL,

...

}

MCCHrelatedBCCH-ConfigPerMBSFNArea-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-14 to support MCCH repetition period values –-

{ID id-Repetition-PeriodExtended			CRITICALITY reject	EXTENSION Repetition-PeriodExtended	PRESENCE optional}|

-- Extension for Rel-14 to support MCCH modification period values –-

{ID id-Modification-PeriodExtended			CRITICALITY reject	EXTENSION Modification-PeriodExtended	PRESENCE optional}|

{ID id-Subcarrier-SpacingMBMS				CRITICALITY reject	EXTENSION Subcarrier-SpacingMBMS		PRESENCE optional},

...

}

MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item ::= SEQUENCE {

mbsfnArea						MBSFN-Area-ID,

repetitionPeriodExpanded		ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, ...},

offset							INTEGER (0..10),

modificationPeriodExpanded		ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024, ...},

subframeAllocationInfoExpanded	BIT STRING (SIZE(10)),

modulationAndCodingScheme		ENUMERATED {n2, n7, n13, n19},

subcarrier-SpacingMBMSExpanded	ENUMERATED {khz-7dot5, khz-2dot5, khz-1dot25, khz-0dot37, ..., kHz-15},

timeSeparation					ENUMERATED {sl2, sl4, ...}		OPTIONAL,

cellInformationList				Cell-Information-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item-ExtIEs} } OPTIONAL,

...

}

MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-17 to support PMCH Bandwidth values –-

{ID id-AdditionalConfigParameters	CRITICALITY reject	EXTENSION AdditionalConfigParameters	PRESENCE optional},

...

}

MCE-ID ::= OCTET STRING (SIZE(2))

MCE-MBMS-M2AP-ID ::= INTEGER (0.. 16777215)

MCEname ::= PrintableString (SIZE (1..150,...))

MCH-Scheduling-Period ::= ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024}

MCH-Scheduling-PeriodExtended ::= ENUMERATED {rf4, ...}

MCH-Scheduling-PeriodExtended2 ::= ENUMERATED {rf1, rf2, ...}

MCH-Scheduling-PeriodExtended3 ::= ENUMERATED {rf7, rf14, rf28, rf53, rf56, rf108, rf112, rf212, rf424, ...}

Modulation-Coding-Scheme2 ::= INTEGER (0..27)

Modification-PeriodExtended ::= ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, ...}

#### -- N

#### -- O

#### -- P

PLMN-Identity ::= OCTET STRING (SIZE(3))

PMCH-Bandwidth ::= ENUMERATED {n40, n35, n30, ...}

PMCH-Configuration ::= SEQUENCE {

allocatedSubframesEnd			AllocatedSubframesEnd,

dataMCS							INTEGER (0..28),

mchSchedulingPeriod				MCH-Scheduling-Period,

iE-Extensions					ProtocolExtensionContainer { {PMCH-Configuration-ExtIEs} } OPTIONAL,

...

}

PMCH-Configuration-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-12 to support 256QAM for MTCH –

{ID id-Modulation-Coding-Scheme2			CRITICALITY reject	EXTENSION Modulation-Coding-Scheme2		PRESENCE optional}|

-- Extension for Rel-12 to support shorter MCH scheduling period –

{ID id-MCH-Scheduling-PeriodExtended		CRITICALITY reject	EXTENSION MCH-Scheduling-PeriodExtended	PRESENCE optional}|

-- Extension for Rel-14 to support shorter MCH scheduling period values –

{ID id-MCH-Scheduling-PeriodExtended2		CRITICALITY	reject	EXTENSION MCH-Scheduling-PeriodExtended2	PRESENCE optional}|

{ID id-MCH-Scheduling-PeriodExtended3		CRITICALITY	reject	EXTENSION MCH-Scheduling-PeriodExtended3	PRESENCE optional}|

{ID id-FrequencyInterleavingIndicator		CRITICALITY	reject	EXTENSION FrequencyInterleavingIndicator	PRESENCE optional}|

{ID id-TimeInterleavingParameters			CRITICALITY	reject	EXTENSION TimeInterleavingParameters		PRESENCE optional},

...

}

Common-Subframe-Allocation-Period ::= ENUMERATED {rf4, rf8, rf16, rf32, rf64, rf128, rf256}

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel ::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

### //skip unchanged part

#### -- T

TimeInterleavingParameters ::= SEQUENCE {

valueM					ENUMERATED {sf4, sf8, sf16, sf32, ...},

valueN					ENUMERATED {n2, n4, n8, n16, ...},

scalingfactorBeta		ENUMERATED {one32th, one5th, one3rd, three8th, five12th, onehalf, five8th, two3rd, five6th, one, ...},

referenceUECategory		INTEGER (4..26,...),

valueM-LastMTCH			ENUMERATED {sf4, sf8, sf16, sf32, ...}								OPTIONAL,

valueN-LastMTCH			ENUMERATED {n2, n4, n8, n16, ...}									OPTIONAL,

cyclicShiftAlpha		ENUMERATED {alphaOne, alphaOther, ...}								OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { TimeInterleavingParameters-ExtIEs} } OPTIONAL,

...

}

TimeInterleavingParameters-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TMGI ::= SEQUENCE {

pLMNidentity			PLMN-Identity,

serviceID				OCTET STRING (SIZE (3)),

iE-Extensions			ProtocolExtensionContainer { {TMGI-ExtIEs} } OPTIONAL,

...

}

TMGI-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

TNL-Information ::= SEQUENCE {

iPMCAddress						IPAddress,

iPSourceAddress					IPAddress,

gTP-TEID						GTP-TEID,

iE-Extensions					ProtocolExtensionContainer { {TNL-Information-ExtIEs} } OPTIONAL,

...

}

TNL-Information-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

NEXT CHANGE

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

### //skip unchanged part

id-MaximumDataBurstVolume 								ProtocolIE-ID ::= 432

id-MN-only-MDT-collection								ProtocolIE-ID ::= 433

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-FrequencyInterleavingIndicator						ProtocolIE-ID ::= xxx

id-TimeInterleavingParameters							ProtocolIE-ID ::= yyy

id-MCH-Scheduling-PeriodExtended3						ProtocolIE-ID ::= zzz

END

-- ASN1STOP

CHANGES END