3GPP TSG-RAN WG3 #129	R3-205783

Bengaluru India, 25-29 August, 2025

Agenda Item:	13.3

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on Conditional LTM

Document for:	Approval

# Introduction

CB: # MobilityEnh_CLTM

- further check with the open issues and capture the agreements.

- TAT transfer issues.

- update and check whit the TPs and BLCRs.

(moderator-Nokia)

Summary of offline discussion in R3-255783

# For the Chairman’s Notes

To be updated

# Discussion

## TAT Handling after Cell Switch

There is RAN2 agreement to maintain C-LTM TA values and corresponding TA timers for candidate cells after LTM switch. In legacy LTM TA values were provided via CELL SWITCH NOTIFICATION, however for C-LTM this message is not triggered from source after LTM mobility

Alternative A: Contributions [R3-255427, R3-255270, R3-255151, R3-255376] propose to have the Source DU provide this information after realizing a successful cell change as follows:

- Firstly, when Source DU receives UE CONTEXT MODIFICATION REQUEST message which indicates that the UE has successfully changed cell, the Source DU signals the valid (a) TA values and (b) remaining time of the TA timers to the CU in the UE CONTEXT MODIFICATION RESPONSE message.

- Secondly, the CU forwards the TA values and remaining TA timers to the Candidate DU via the UE CONTEXT MODIFICATION message.

- Note: In [R3-255376] it is proposed to additionally include the Tag ID IE

Alternative B: Contributions [R3-255606] proposes not to introduce changes to F1 specification to transfer TA and remaining TA timers from Source DU to Candidate DU (via CU) after CLTM cell change.

Moderator summary

Moderator suggests to support RAN2 decision and agree the following proposals.

Proposal: The source DU sends the TA values and the remaining time of the TA timers of candidate cells and target cell to the CU by the UE CONTEXT MODIFICATION RESPONSE message after receiving the successful cell change notification from the CU.

Proposal: The CU reuses the UE Context Modification procedure to transfer the TA values and the remaining time of the TATs to the target DU.

Discuss further whether Tag ID is needed.

## Stage 3 issues

The following Stage 3 issues/required changes were highlighted

| Company | Comment |
| --- | --- |
| NEC [R3-255199] | There are two similar IEs in the baseline CR to 38.473 (R3-254011->R3-255065): “9.3.1.XXX L1 Execution Condition List” and “9.3.1.XXX Conditional LTM Execution Condition List” Need to remove the “9.3.1.XXX Conditional LTM Execution Condition List”. |
| LGE [R3-255616] | To inform the previous serving DU and stop data transmission after Inter-DU C-LTM execution happens, re-use the existing Transmission Action Indicator IE (used for S-CPAC) in UE CTXT MOD REQ message. |

Moderator summary

Moderator suggests to incorporate the changes above into Stage 3 BL CR.

## BL CRs

Moderator summary

Moderator suggests BLCR allocation as follows to capture online and offline agreements

- F1AP 38.473 (Huawei) revision of R3-255427

- Stage 2 38.401 (ZTE) revision of R3-255151

# Conclusion, Recommendations [if needed]

If needed

# References

| R3-255270 | (TP to CLTM BL CR for TS 38.473, TS 38.401) – Completion of Intra-CU Conditional LTM (Ericsson) | Other |
| --- | --- | --- |
| R3-255427 | (TP for LTM BLCR for TS38.473, TS38.401): Intra-CU conditional LTM (Huawei) | other |
| R3-255140 | Discussion on Conditional LTM in split architecture (Nokia) | discussion |
| R3-255151 | (TP to BL CR for TS 38.401, 38.473) TAT transfer for Conditional LTM (ZTE Corporation) | other |
| R3-255199 | (TP to BL CR for 38.473 on conditional intra-CU LTM) Remaining issues of Rel-19 intra-CU Conditional LTM (NEC) | other |
| R3-255685 | Remaining issues on Intra-CU Conditional LTM (Samsung) | discussion |
| R3-255606 | (TP to 38.473) Discussion for C-LTM (CATT) | discussion |
| R3-255302 | Signalling enhancements for Intra-CU Conditional LTM (Qualcomm India Pvt Ltd) | discussion |
| R3-255616 | (TPs for NR_Mob_Ph4 TS 38.473 and TS 38.401) Discussions on the remaining aspects of Conditional Intra-CU LTM (LG Electronics Inc.) | other |
| R3-255376 | (TP to TS38.401) On support of intra-CU Conditional LTM (China Telecom) | other |