3GPP TSG-RAN WG4 #116	R4-2509033

Bengaluru, India, August 25-29, 2025

Agenda item:	7.20.6		Demodulation performance and CSI reporting requirements

Source:	Qualcomm Incorporated

Title:	Additional demodulation performance requirements for Rel-19 MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #115, the work on Rel-19 MIMO Phase 5 demod requirements has been started and several agreements have been made [1].

In this paper we provide our views for the open issues identified in the way forward in [1]

# Discussion

## General Considerations

## UE Reporting Enhancements for CJT

In RAN4 #115 it was discussed whether demod requirements in 38.101-4 should be introduced but no conclusion could be reached.

Issue 1-1-11: Clarify if CSI reporting requirements are needed for UE reporting enhancements for CJT

Way forward:

-	Option 1: RAN4 do not introduce CSI reporting requirements for UE reporting enhancements for CJT

-	Option 2: RAN4 introduce new requirements for CJT calibration offset reporting for the time offset (TO), the carrier frequency offset (CFO) and the phase offset (PO) scenarios. Further study exact implementation of the new requirements.

-	RAN4 do not introduce new CQI/RI and dedicated PMI reporting requirements but consider PMI calculation as part of the CJT calibration reporting requirements.

-	Introduce a new subsection in TS 138 101-4 to cover introduction of CJT calibration offset reporting requirements.

The difficulty in introducing demod requirements is that it is not straight forward how the reported time, frequency and phase offsets will be applied by the gNB. This requires detailed further discussion and specification of an algorithm that has to be implemented in the test equipments. In addition, also more discussions are required how to model time, frequency and phase offset. In particular for frequency and phase offsets we believe this is not straightforward since it depends on the behavior of the oscillators in case of frequency offset reporting and the Tx/Rx frontends in case of phase offset reporting.

In the other side, we see more and more algorithms that require specification of algorithms running in the test equipment. Another example is SRS port grouping, also being discussed in this paper. Here specification of an beamforming algorithm for the TE is needed that aims to orthogonalize the two codewords sent to those antennas that belong to a SRS port group.

Generally, we see value in a discussion whether RAN4 should start introducing simple algorithms for the TE to allow better testing of features that involve device and gNB. In some form we already do this in case of two-sided AI/ML model for CSF compression. However, we believe that this discussion requires more time and should not be done anymore for 5G. Instead, we think that such discussions should take place in the 6G study item.

Observation 1: Demod requirements for CJT enhancements require detailed discussion and specification of an algorithm that has to be implemented in the test equipments. In addition, also more discussions are required how to model time, frequency and phase offset.

Observation 2: More and more features require specification of algorithms running in the test equipment. Another example is SRS port grouping, also being discussed in MIMO Phase 5.

Observation 3: Generally, RAN should discuss whether RAN4 should start introducing simple algorithms for the TE to allow better testing of features that involve device and gNB. In some form RAN4 already does this in case of two-sided AI/ML model for CSF compression. However, we believe that this discussion requires more time and should not be done anymore for 5G. Instead, we think that such discussions should take place in the 6G study item.

Proposal 1: RAN4 should not introduce demod requirements for reporting enhancements of CJin Rel-19.

## 128 CSI-RS Ports

In RAN4#115, only PMI requirements with 64 and/or 128 ports with SinglePanel TypeI precoders was agreed. In this section, we provide views on the open issues in [1].

Issue 1-1-7: Clarify if PMI reporting requirements are needed for eTypeII-r19

Way forward:

-	Option 1: RAN4 introduce PMI reporting requirements for eTypeII-r19

-	Option 2: RAN4 do not introduce PMI requirements for eTypeII-r19

2.3.1 PMI requirements with eTypeII based codebooks

The feature was introduced for deeper coverage, higher spectral efficiency achievable with large antenna arrays. To specify requirements for these aspects, PMI requirements with Type I precoders are sufficient. Additional PMI requirements with eTypeII based precoders are redundant in our view.

eTypeII Doppler codebook based precoders were to enable precoders to track channel variations over time with limited feedback. Moreover, in Rel-18 too, lot of effort was required to arrive at a channel to demonstrate improvement over Rel-16 eTypeII codebook. Hence, with limited TU available for Rel-19, PMI requirements with these precoders are unnecessary in our view.

Proposal 2: RAN4 to not introduce PMI requirements with eTypeII based precoders.

Proposal 3: RAN4 to not consider PMI requirements with eTypeII Doppler-r19 codebook.

Issue 1-2-13: portMappingMethod

Agreement:

-	Select one for requirements. FFS on which one next meeting.

2.3.2 Test settings with SinglePanel TypeI based codebook

Port mapping method effectively selects the subpanel used by each CSI-RS resource. For 64 port case, (N1,N2) options are (16,2) and (8,4). If Method1 is used for (16,2) and Method2 is picked for (8,4), then the subpanel used by each CSI-RS resource is of size (8,2). This implies size of subpanel used by each CSI-RS resource is the same and therefore there is no need to pursue both panel sizes since the performance is expected to be the same. A single port mapping method be chosen for both panel sizes.

Observation 4: Choosing different port mapping methods result in similar performance for both panel sizes for 64 ports.

Proposal 4: RAN4 to choose a single port mapping method, i.e., either Method1 or Method2 for 64 port case.

Issue 1-2-14: Number of slots for CSI-RS transmission

Agreement:

-	One slot for 64 ports. FFS for 128 ports.

Since CSI-RS in a single slot is the basic UE capability supporting this feature, we propose a single slot for CSI-RS transmission.

Proposal 5: RAN4 to only have a single slot CSI-RS transmission for both 64 and 128 ports.

Issue 1-2-15: CQI/RI/PMI delay

-	Option 1: RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing capabilities.

-	Option 2: Re-discuss the CSI-RS scheduling and CSI reporting pattern to match the capability 2 of timeline to support up to 128 CSI-RS ports

Proposal 6: RAN4 to use row#18 of Tabel 7.4.1.5.3-1 for 128 port CSI-RS transmission.

Since basic UE capability is to support the feature with capability 2, we support option 2.

Proposal 7: RAN4 to discuss CSI-RS scheduling and reporting to match capability 2 based timeline as test setting for PMI requirements.

## SRS Port Grouping

In RAN4#115, the group could not discuss in detail the unique characteristics of the receiver supporting this feature and how it is not straightforward to reuse an existing requirement.

Issue 1-1-9: Clarify if UE demodulation/CSI reporting requirements are needed for SRS port grouping

Way forward:

-	Option 1: RAN4 introduce PDSCH demodulation requirements for SRS port grouping

-	Option 1A: RAN4 to discuss reuse of existing demod requirements for SRS port grouping scheme.

-	Option 1B: RAN4 to discuss the new performance requirements for R19 8Rx UE supporting SRS port grouping, FFS the test setup.

-	Option 2: RAN4 do not introduce UE demodulation/CSI reporting requirements for SRS port grouping

The SRS port grouping was introduced to enable a low-complexity receiver that can support up to 8layers. This feature assumes gNB does interference suppression between codewords for 6L or 8L cases. Zero forcing based precoding at gNB requires knowledge of channel information based on SRS in TDD systems. To come up with a requirement for such a case imposes lot of assumption on TE too. We also note that there is no precedence on demodulation requirements with gNB precoding based on SRS transmissions.

Observation 4: There is no 6-layer demodulation requirement for an 8Rx device in Rel-18.

Since the feature essentially has two sets of 4Rx receivers within a device, and to have a demodulation requirement with 6 or 8 layers, we suggest reusing existing rank3 requirement of Rel-17 can be extended. For example, the test set up can be: 6-layer transmission from gNB with random precoding on non-overlapping ports between codewords, each codeword is mapped to 3-layers, and two codewords are transmitted. All the 6 ports of TE are connected to 8 Rx ports of UE. Effectively, each 4Rx only receives 3-layer transmission, and 6-layers are received concurrently. With only 4Rx used to decode each codeword, 3-layer Rel-17 requirement can be enforced on each codeword of the 6-layer transmission.

Proposal 8: RAN4 to discuss to reuse an existing demodulation requirement with a simple set up for SRS port grouping feature.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #115. Our observations and proposals are summarized as follows:

UE Reporting Enhancements for CJT:

Observation 1: Demod requirements for CJT enhancements require detailed discussion and specification of an algorithm that must be implemented in the test equipment. In addition, also more discussions are required how to model time, frequency and phase offset.

Observation 2: More and more features require specification of algorithms running in the test equipment. Another example is SRS port grouping, also being discussed in MIMO Phase 5.

Observation 3: Generally, RAN should discuss whether RAN4 should start introducing simple algorithms for the TE to allow better testing of features that involve device and gNB. In some form RAN4 already does this in case of two-sided AI/ML model for CSF compression. However, we believe that this discussion requires more time and should not be done anymore for 5G. Instead, we think that such discussions should take place in the 6G study item.

Proposal 1: RAN4 should not introduce demod requirements for reporting enhancements of CJT in Rel-19.

128 CSI-RS Ports

Proposal 2: RAN4 to not introduce PMI requirements with eTypeII based precoders.

Proposal 3: RAN4 to not consider PMI requirements with eTypeII Doppler-r19 codebook.

Proposal 4: RAN4 to choose a single port mapping method, i.e., either Method1 or Method2 for 64 port case.

Proposal 5: RAN4 to only have a single slot CSI-RS transmission for both 64 and 128 ports.

Proposal 6: RAN4 to use row#18 of Tabel 7.4.1.5.3-1 for 128 port CSI-RS transmission.

Proposal 7: RAN4 to discuss CSI-RS scheduling and reporting to match capability 2 based timeline as test setting for PMI requirements.

SRS Port Grouping

Observation 4: There is no 6-layer demodulation requirement for an 8Rx device in Rel-18.

Proposal 8: RAN4 to discuss to reuse an existing3-layer, 4Rx demodulation requirement with a simple set up for SRS port grouping feature.

# References

- R4P-2508627, Way Forward for [115][329] NR_MIMO_demod, May 2025