3GPP TSG-RAN WG4 Meeting # 116											R4-2509059

Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.20.1

Source:	Moderator (Samsung)

Title:	Topic summary for [116][216] NR_MIMO_Ph5_Part1

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for first part of Rel-19 RRM for NR MIMO Phase 5:

- 7.20	NR MIMO Phase 5

- 7.20.4 RRM core requirements

  - 7.20.4.1 Enhancement for UE-initiated/event-driven beam management

- 7.20.5 RRM performance requirements

Recommended issues to online discussion:

1-1

1-3

1-4

1-5

3-1

1-7

1-2

1-6

  - a (offline firstly, come back to online after 1-1)

Topic #1: RRM core requirements for UE-initiated/event-driven beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509031 | Qualcomm Technologies Ireland | Proposal 1: RAN4 should discuss whether an LS should be sent to RAN1 summarizing the RAN4 agreements for the measurement periods and asking for further clarity of the RAN1 agreement. Observation 1: In case no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested. Proposal 2: RAN4 should not define requirements for the window-based approach whether the event has occurred M times. Observation 2: Any discussion about enhancing the TCI state switch latency should assume known TCI states and focus on removing Tfirst_SSB + TSSB-proc from the minimum requirement. Observation 3: Removal of the term Tfirst_SSB + TSSB-proc from the minimum requirement can already be ensured by the network if it is ensured that the target TCI state is already in the active TCI state list. Proposal 3: RAN4 should not introduce reduced TCI state switch requirements in Rel-19 for UE-initiated/event-driven beam management. Observation 4: It is difficult to define a meaningful test case for a TCI state change during measurement time in case the device has a measurement period that is significantly lower than the RAN4 requirements. Proposal 4: It should be left to implementation to reset the measurement in case the TCI state changes during measurement period. Observation 5: L1-RSRP is part of channel state feedback and RAN1 has already a framework for the number of supported CSI calculations per component carrier and across all component carriers. RAN1 framework fully covers the control of the UE load. Proposal 5: No additional UE capability for support of event triggering and reporting criteria is needed in RAN4. Proposal 6: Since we propose that no UE capability is introduced for event triggering and reporting criteria, no discussion is needed whether the UE capabilities are commonly or separately defined for Rel-19 and Rel-19 MIMO. |
| R4-2509145 | China Telecom | Proposal 1: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19. Proposal 2: For the scenario when the indicated TCI state is changed during the L1 measurement time, no need to capture this scenario in RAN4 requirements. Proposal 3: For event trigger L1 reporting, do not define such capability in parallel per category like L3. |
| R4-2509162 | Nokia | Observation 1: In the case of window-based approach, the UE can have only one timer running per beam at a time. Observation 2: For UE to determine that a report is to be triggered in the window-based approach, M events for the same beam should occur before the expiry of the timer. Proposal 1: Define reporting delay requirement also for the window/timer based approach, where the delay considers that the UE shall send the report after M event instances have occurred within the window/timer. Proposal 2: For the window/timer based approach, when the timer is started for the first time and if the reporting condition for a beam is met before the expiry of the first timer, reporting delay is the time from when the condition is met over the air until the first PUCCH transmission. Observation 3: The M event instances may not be consecutive, and if the timer expires without a report being triggered, the UE can start another timer for the same beam when the first timer expires. Proposal 3: If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from the start of a subsequent timer during which reporting condition is met until the first PUCCH transmission. Proposal 4: The reporting delay for the window-based approach shall not be larger than the evaluation delay + the time until the first PUCCH transmission after M event occasions have occurred within one timer. Proposal 5: If the indicated TCI state changes during the L1 measurement time, the evaluation of the event trigger determination shall start after the TCI state switch is completed. However, there is no need to capture this scenario in RAN4 requirements. Proposal 6: Support DL synchronization based on UEIBM report. Proposal 7: If UE initiated DL synchronization based on UEIBM report is supported, TOk = 0 (DL sync is skipped) in the TCI state switching delay, if the TCI state switching command is received after reception of the first SSB after sending the report. Proposal 8: Introduce the number of supported UEIBM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4. Proposal 9: Reuse the agreement under NR_Mob_Ph4 regarding coordination between mobility and MIMO events: RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19. |
| R4-2509484 | Apple | Reporting Delay Requirements Observation #1: It is not feasible to define core requirement for reporting delay with eventInstanceCount-r19 >1 Observation #2: The purpose of defining requirements based on evaluation window/ counter is unclear. Proposal 1: Measurement reporting delay requirement for UE initiated beam report is defined for eventInstanceCount-r19 =1. Proposal 2: To extend the requirements for case when eventInstanceCount-r19 >1 is configured based on UE capability, add note longer delay is expected if eventInstanceCount-r19 >1 is configured. Applicability Proposal 3: Measurement reporting delay requirement applicability - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period Capabilities Proposal 4: For event trigger L1 reporting, do not define such capability in parallel per category like L3. |
| R4-2509629 | LG Electronics Inc. | Proposal 1: Regarding window-based event when eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 is configured , event triggered reporting delay can be reused Proposal 2: RAN4 to specify minimum interval requirement between two event instance, when eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 is configured Proposal 3: Regarding minimum interval requirement between two event instance, RAN4 to consider as follows: - 1ms as lower bound of minimum interval for FR2 - Scaling factor to handle the case when the current beam and new beam is overlapped on the same symbol Proposal 4: For unified TCI switching delay requirements, no further enhancements in Rel-19 Proposal 5: For the case when indicated TCI states is changed during the L1 measurement time, prefer Option3 (add the applicablility rule of event triggered L1 measurement) |
| R4-2509767 | Xiaomi | Observation 1: From RAN1, the same evaluation periodicity is defined for all events. Proposal 1: The RAN1 agreement on a common evaluation periodicity does not affect RAN4’s existing agreement that the L1 measurement delay component is bounded by the maximum of the per-beam measurement periods. Proposal 2: RAN4 to Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19. Proposal 3: Don’t specify new unified TCI switching delay requirements. Proposal 4: If indicated TCI states is changed during the L1 measurement time, the L1 evaluation time should restart. |
| R4-2509833 | Samsung | Proposal 1: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19. Proposal 2: For the requirements, the measurement period is at least be larger than max{eventInstanceCount-r19 * measurement period (for basic), Ttime_window} Proposal 3: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. Proposal 4: For event trigger L1 reporting, do not define such capability in parallel per category like L3. Proposal 5: If P4 is agreed, no need to consider such capability compatibility with LTM. Proposal 6: Even if there may be capabilities in Issue 1-8. In Rel-15, these capabilities are mandatory in spec. No need to add UE features for this. Proposal 7: For Rel-19 MIMO Phase 5 RRM, no need to add dedicated RAN4 UE feature list for UEIBM, CJTC reporting, Asymmetric DL sTRP/UL mTRP. They have been covered in RAN1 feature list. |
| R4-2510142 | CMCC | Proposal 1: for UEIBM, it is proposed to define RAN4 requirements for FR1, since according to RAN1 feature list, FR1 is consiered for UE-initiated/event-driven beam management. Proposal 2: it is proposed to specify requirements to cover the case of no time window and the case with time window with configurable eventInstanceCount-r19. Proposal 3: if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is proposed to define a separate capability for L1 measurement, not prefer to mix L1 measurement reporting capability with legacy L3 measurement reporting capability. Proposal 4: if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO. |
| R4-2510600 | Huawei, HiSilicon | Observation 1: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading. Proposal 1: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering Observation 2: The motivation of introducing window-based UEIBR is to have reliable event report to mitigate the impact of instant channel fading (may happens for non-window-based case). Proposal 2: The requirements shall be defined to verify both following purpose: - The event is trigger timely after number of event instances have been evaluated. - The event is not wrongly/too early when the number of event instances have not been evaluated. Proposal 3: When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event no later than TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, and no earlier than TL1-RSRP_event triggered_measurement_Period * (C-1) + Tfirst UL channel, where C is the configured counter. Observation 3: Different behaviour on choosing the results for evaluation may results different evaluation results (e.g. different number of event instance counting), and it is not clear for gNB to configure the number of conter. Proposal 4: For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance. Proposal 5: No need to consider the scenario that TCI state is changed during the L1 measurement time. Observation 4: Based on current RAN1 feature list, UE can explicitly indicate the supported event (event 1/2/7) and the supported number of new beams for each event. Proposal 6: Do not define RAN4 capabilities on support of Event Triggering and Reporting Criteria. |
| R4-2510682 | ZTE Corporation, Sanechips | Proposal 1: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. For the optional Case 2, the sample number M and timer X should be considered. Proposal 2: UE is assumed to store the QCL properties of the SSB associated with new beam in the latest report. When UE receives the TCI state switching command and the new beam is indicated by the command, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored. Proposal 3: Confirm the case that the indicated TCI state is changed during the L1 measurement time. The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. |
| R4-2510701 | Ericsson | Proposal 1: RAN4 to define UE IBR requirements when the eventInstanceCount-r19 is configured. Proposal 2: RAN4 to agree on following as the method to specify the requirement when the eventInstanceCount-r19 is configured: - After the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window Proposal 3: RAN4 to agree TFirst UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time, where Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1. Proposal 4: For event 7, the L1-RSRP measurement period for each of the activated TCI state refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0. Proposal 5: TCI state activation delay enhancements are not feasible without RAN1 signalling support. Proposal 6: When the indicated TCI state is changed, counter resetting mechanism is defined in RAN1 and RAN4 not to define additional applicability rules when the indicated TCI state changes. Proposal 7: Do not see a need for a separate capability like the L3 event evaluation category in the RAN4 specification. If a capability for the number of events to be supported is needed, it shall be defined by RAN1. |
| R4-2511609 | MediaTek inc. | Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. If timeRestrictionForChannelMeasurement is not configured, measurement period is based on 3 sample. Proposal 2: RAN4 to consider the legacy sharing factor (P) for L1-RSRP measurement as baseline: - P value is same for intra-cell SSB resources included both current and new beam - P value is different for inter-cell SSB resources included both current and new beam - P value is different for CSI-RS resources included both current and new beam Proposal 3: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”: Proposal 4: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window. Proposal 5: For the case of time window with configurable M (event instances threshold): - Option 1: Measurement reporting delay can re-use the same principle as L3 event triggered reporting defined in legacy. - Option 2: Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where, - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW. - Ttime_window: time window is configured by NW. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. Proposal 6: No requirement is applied if indicated TCI states is changed during the L1 measurement time. Proposal 7: Define UE reported capabilities for numbers of event refer to legacy L3 (like legacy L3 in section 9.1.4). - For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9]. |

Open issues summary

Sub-topic 1-1

- Issue 1-1: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

- Proposals

  - Option 1: (Qualcomm, Apple)

    - Do not define requirements for the window-based approach whether the event has occurred M times.

    - Option 1b: (Apple)

      - Add note longer delay is expected if eventInstanceCount-r19 >1 is configured

  - Option 2: (China Telecom, Nokia, LGE, Xiaomi, Samsung, CMCC, Huawei, ZTE, Ericsson, MediaTek)

    - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

- Recommended WF

  - First decide from two options:

    - Option 1: No requirements for with time window based with eventInstanceCount-r19>1

    - Option 2: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

  - Suggest to agree on Option 2 based on majority views

- Issue 1-1-a: How to specify requirements for “with time window based with eventInstanceCount-r19” if option 2 in Issue 1-1-1 is supported?

- Proposals

  - Option 1: (Nokia)

    - For the window/timer based approach, when the timer is started for the first time and if the reporting condition for a beam is met before the expiry of the first timer, reporting delay is the time from when the condition is met over the air until the first PUCCH transmission.

    - If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from the start of a subsequent timer during which reporting condition is met until the first PUCCH transmission.

    - The reporting delay for the window-based approach shall not be larger than the evaluation delay + the time until the first PUCCH transmission after M event occasions have occurred within one timer.

  - Option 2: (LGE)

    - event triggered reporting delay can be reused

  - Option 3: (Samsung)

    - The measurement period is at least be larger than max{eventInstanceCount-r19 * measurement period (for basic), Ttime_window}

  - Option 4: (Huawei)

    - When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event no later than TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, and no earlier than TL1-RSRP_event triggered_measurement_Period * (C-1) + Tfirst UL channel, where C is the configured counter.

    - For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

  - Option 5: (Ericsson)

    - After the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window

  - Option 6: (MediaTek)

    - Alt1. Measurement reporting delay can re-use the same principle as L3 event triggered reporting defined in legacy.

    - Alt2. 	Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

      - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

      - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW.

      - Ttime_window: time window is configured by NW.

      - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st  PUCCH.

- Recommended WF

  - If we can get consensus on Issue 1-1, to discuss:

    - Option 1&3&4&6 have similar idea but with different details,

    - Option 2 reuse the same as no window based.

    - Option 5 use fixed T_window length

- Issue 1-2: Whether to specify new unified TCI switching delay requirements?

Background: Agreement in RAN4#115 - The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management

- Proposals

  - Option 1: (Qualcomm, LGE, Xiaomi, Ericsson)

    - Keep the previous agreement and no further additional enhancements

  - Option 2: (Nokia, ZTE)

    - Yes. Further enhancements are needed

    - Option 2a: (Nokia)

      - Support DL synchronization based on UEIBM report. If UE initiated DL synchronization based on UEIBM report is supported, TOk = 0 (DL sync is skipped) in the TCI state switching delay, if the TCI state switching command is received after reception of the first SSB after sending the report.

    - Option 2b: (ZTE)

      - UE is assumed to store the QCL properties of the SSB associated with new beam in the latest report. When UE receives the TCI state switching command and the new beam is indicated by the command, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

- Recommended WF

  - Suggest to agree on Option 1 based on majority views. Discuss on Option 2 if time allows. According to Chair’s guidance: Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion.

- Issue 1-3: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

- Proposals

  - Option 1: (Qualcomm, China Telecom, Nokia, Huawei, Ericsson, MediaTek)

    - No need to capture this scenario in RAN4 requirements

  - Option 2: (Xiaomi, Samsung, ZTE)

    - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

  - Option 3: (Apple, LGE)

    - Add the applicability rule of event triggered L1 measurement:

      - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

      - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

- Recommended WF

  - Choose one of option 1, 2, 3 in this meeting. Suggest to agree on Option 1 based on majority views.

- Issue 1-4: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

- Proposals

  - Option 1: (Qualcomm, China Telecom, Apple, Samsung, Huawei, Ericsson)

    - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

  - Option 2: (Nokia)

    - Introduce the number of supported UEIBM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.

  - Option 3: (MediaTek)

    - For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

  - Option 4: (CMCC)

    - if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is proposed to define a separate capability for L1 measurement, not prefer to mix L1 measurement reporting capability with legacy L3 measurement reporting capability.

- Recommended WF

  - Choose one of option 1, 2, 3 in this meeting. Suggest to agree on Option 1 based on majority views.

- Issue 1-5: Compatibility of capabilities with LTM

- Proposals

  - Option 1: (Qualcomm, China Telecom, Samsung, Huawei, Ericsson)

    - No need to consider such capability compatibility with LTM

  - Option 2: (Nokia)

    - RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

  - Option 3: (CMCC)

    - if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO..

- Recommended WF

  - Choose one of option 1, 2 in this meeting. Suggest to agree on Option 1 based on majority views.

- Issue 1-6 (New): Whether RAN4 to specify minimum interval requirement between two event instances?

- Proposals

  - Option 1: (LGE)

    - Yes. RAN4 to consider as follows:

    - 1ms as lower bound of minimum interval for FR2

    - Scaling factor to handle the case when the current beam and new beam is overlapped on the same symbol

  - Option 2:

    - No

- Recommended WF

  - TBA

- Issue 1-7: Whether RAN4 to send LS to RAN1 for further clarification of the case - eventInstanceCount-r19 > 1?

- Proposals

  - Option 1: (Qualcomm)

    - Yes. Details can be further discussed.

  - Option 2:

    - No

- Issue 1-8: UE-initiated/event-driven beam management for FR1

- Proposals

  - Option 1: (CMCC)

    - for UEIBM, it is proposed to define RAN4 requirements for FR1, since according to RAN1 feature list, FR1 is considered for UE-initiated/event-driven beam management.

  - Option 1a: (Samsung, Apple)

    - Add FR1 support in R4-2509236 & R4-2509485

- Recommended WF

  - Agree for FR1 support and check the update in draft CRs R4-2509236 & R4-2509485 directly.

- Issue 1-9: L1-RSRP measurement period for event-7

- Proposals

  - Option 1: (Ericsson)

    - For event 7, the L1-RSRP measurement period for each of the activated TCI state refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0.

  - Option 2: (Samsung, Apple)

    - Add event-7 support in R4-2509236 & R4-2509485

- Recommended WF

  - Check the update in draft CRs R4-2509236 & R4-2509485 directly and wording refinement can be in CR sub-threads.

Sub-topic 1-2: Draft CRs

| T-doc number | Company |  |
| --- | --- | --- |
| R4-2509163 | Nokia | DraftCR for UEIBM requirements |
| R4-2509236 | Samsung | Draft CR on event trigger Reporting based on L1-RSRP measurements for serving cell |
| R4-2509485 | Apple | Draft CR on Event triggered L1 reporting - SSB from different PCI |
| R4-2509631 | LG Electronics Inc. | Draft CR for minimum requirement for L1 indication for event trigger Reporting based on L1-RSRP measurements |
| R4-2510179 | CMCC | DraftCR on event trigger Reporting based on L1-RSRP measurements |
| R4-2510702 | Ericsson | Draft CR for event-triggered reporting for MIMO |

Topic #2: RRM performance requirements for Rel-19 MIMO

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509394 | Qualcomm Technologies Ireland | Proposal 1: RAN4 should define test cases for event-2. Whether to define test cases for event-7 and event-1 can be further discussed. Proposal 2: RAN4 should not define test cases for CSI-RS based inter-cell beam management. Proposal 3: No performance requirements should be defined for eventInstanceCout-r19 > 1. Performance requirements should only be defined for the case when no time window is defined. Proposal 4: RAN4 should define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F and CJTC-P. Proposal 5: RAN4 should define accuracy requirements for time offset and frequency offset reporting. The number of samples will be defined in the core requirements. Proposal 6: RAN4 should not define core and performance requirements in Rel-19 for phase offset reporting. Proposal 7: RAN4 should define test cases for FR1 and FR2. Proposal 8: Also, both scenarios #1 and #2 should be covered by test cases for FR1. For FR2, only test cases for scenario #2 should be defined. |
| R4-2509487 | Apple | Observation #1: Introducing test case for Event 2 is sufficient and doesn’t test anything different with test cases for Event 1 or Event 7. Proposal 1: Introduce test case for Event 2 alone. Observation #2: Given the ending point of first PUCCH transmission, the requirements are the same for Mode A and Mode B. Proposal 2: Introduce test case for either Mode-A (preferred) or Mode-B. Proposal 3: Introduce test cases for (1) CSI-RS based event trigger and (1) inter-cell SSB based event trigger. Proposal 4: Do not introduce test cases for time window-based evaluation with eventInstanceCount-r19 > 1. Proposal 5: Introduce test cases in FR2 alone. Proposal 6: Do not introduce test cases for TCI state switching/ beam switching delay for UE-initiated/event-triggered beam management. Proposal 7: RAN4 to define mapping tables for time offset, frequency offset, phase offset reporting based on the respective of range and resolution for CJT calibration reporting. Observation #3: Based on the results collected for FO/ TO accuracy the results are very diverse. Proposal 8: Test cases for phase offset reporting need not be discussed as there are no core requirements. Proposal 9: RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part. Proposal 10: RAN4 to discuss defining test cases for timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP. Proposal 11: Test cases to cover both scenario 1 and scenario 2. Proposal 12: Do not introduce test cases for TCI state switching for asymmetrical DL sTRP/ UL mTRP. Proposal 13: Introduce test cases for scenario 1 in FR1 and scenario 2 in FR2. |
| R4-2509769 | Xiaomi | Observation 1: In legacy RAN4, there will be performance test to make sure that UE correctly detect the channel variation and report within required delay. Precision = 90% is used. Observation 2: RAN1 introduce a new reporting quantity: the Condition Met Indicator (CMI). Proposal 1: RAN4 to define requirement to verify the performance of condition met indicator(CMI) in the report. Observation 3: With the introduction of CMI, it is necessary to reconsider how the correct rate for event-triggered L1 reporting is defined. Proposal 2: RAN4 to discuss how to define performance metric for event triggered reporting, whether to include CMI together or independently. |
| R4-2509835 | Samsung | Observation 1: The performance requirements focus on other 3 objectives including UEIBM, CJTC reporting, enhancement for asymmetric DL sTRP/UL mTRP scenarios. Observation 2: The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements. Observation 3: The testability of Event-7 has issue by using existing chamber for more AoAs. Proposal 1: RAN4 to add new test cases to cover: - sTRP serving cell SSB based L1-RSRP - sTRP serving cell CSI-RS based L1-RSRP - sTRP additional PCI cell SSB based L1-RSRP Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1. No TCs are added for event-7. TCs are defined to cover both Mode A and Mode B. FFS on whether to define TC if eventInstanceCount-r19 > 1. We support to define such TC no matter there is core requirements or not. Specify the test cases for FR2 test. Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P. Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F. Proposal 4: RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F. Proposal 5: RAN4 not to add accuracy requirements and TCs for CJTC-P. Proposal 6: RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching. |
| R4-2510144 | CMCC | Proposal 1: for UE-initiated/event-driven beam reporting, it is proposed to define test cases for: - SSB based intra-cell UEIBM, - SSB based inter-cell UEIBM, - CSI-RS based intra-cell UEIBM (CSI-RS is QCLed to the SSB of serving cell), - CSI-RS based inter-cell UEIBM (CSI-RS is QCLed to the SSB of additionPCI cell) Proposal 2: it is proposed to define test cases to cover Event-2, Event-1 and Event-7. Proposal 3: it is proposed to define test cases for both mode A and mode B. Proposal 4: if it is agreed to specify requirements to cover both no time window based and with time window based, it is proposed to define test cases for the two cases. Proposal 5: it is proposed to define test cases for L1 measurement reporting delay, since Tfirst UL channel is newly introduced as part of L1 measurement reporting delay. Proposal 6: for CJT calibration, it is proposed to define test cases for delay and frequency offset reporting for - measurement period - scheduling restriction (if core parts impact are identified) Proposal 7: for UE reporting enhancement for CJT calibration, it is proposed to define report mapping for delay offset reporting, frequency offset reporting and phase offset reporting. Proposal 8: for asymmetric DL sTRP/UL mTRP scenarios, it is proposed to define test cases for timing requirements (e.g. two TA enhancements) for scenario #1 and scenario #2. |
| R4-2510551 | Huawei, HiSilicon | Proposal 1: For UEIBR, define TC as follows: - SSB-based UEIBR for Event-2. - CSI-RS based UEIBR for Event-2. Note 1: At least define TC when timeRestrictionForChannelMeasurement is configured. Note 2: At least define TC for intra-cell. Proposal 2: RAN4 to discuss whether to define TC for inter-cell UEIBR. Proposal 3: RAN4 to discuss whether to define TC for other Events (i.e. Event 1 and Event 7) Proposal 4: It is possible to establish the accuracy requirement based on one sample as the baseline while waiting to finalise the core requirements for the CJTC. Proposal 5: For updated 2AT requirements for Scenario #1, the legacy timing accuracy test case can be reused without any new updates. |
| R4-2510683 | ZTE Corporation, Sanechips | Proposal 1: To cover all the applicable cases in UEIBM, the following cases should be verified: - Intra-cell SSB based L1-RSRP UEIBM in FR2; - Inter-cell SSB based L1-RSRP UEIBM in FR2; - Intra/inter-cell CSI-RS based L1-RSRP UEIBM in FR2. Proposal 2: As long as the separate requirements for Case 1(no time window) and Case 2(with time window) are specified, both of them should be verified. Proposal 3: Both Mode A and Mode B should be verified. Proposal 4: RAN4 approved to define requirements for Event-2, Event-1 and Event-7. So all of these event types should be verified. Proposal 5: If additional enhancements allowed, the TCI state switching should be verified. |
| R4-2510694 | Nokia | Proposal 1: RAN4 to at least define a test case for Event-2. Proposal 2: Include reporting Mode A and Mode B as optional test configurations under each UEIBM test. No need to define separate test cases for Mode A and Mode B. Proposal 3: UE to be tested with reporting Mode A or Mode B, if UE supports both options, else UE to be tested with reporting Mode A (basic mode of operation). Proposal 4: RAN4 shall define test cases for Event-trigger for Intra-cell BM for CSI-RS and SSB and Inter Cell BM for SSB. Test case for no time window and eventInstanceCount-r19 > 1: Window-based reporting requirements are still under discussion, and RAN4 should agree about the relevant test cases for this option if RAN4 defines the corresponding core requirements. Proposal 5: RAN4 to define test case for window based approach if window based reporting requirements are defined. Proposal 6: RAN4 to define test case for UE initiated DL synchronization based on UEIBM report if requirements for this scenario are defined. Proposal 7: RAN4 to not define a test case for the condition met indicator (CMI). Observation 1: A UE supporting 2 TAs for asymmetric DL sTRP/UL mTRP (Rel 19) is not required to support at the same time also 2 TAs for multi-DCI (Rel 18). Proposal 8: Define test cases for asymmetric DL sTRP/UL mTRP with 2 TAs for both a. Scenario#1, UL-only TRP does not transmit SSBs, and b. Scenario#2, UL-only TRP transmits SSBs. Proposal 9: For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#2 (UL-only TRP transmits SSBs) define a. A Transmit Timing Test for FR1 extending the Rel-18 multi-TRP multi-DCI 2 TAs Transmit Timing Test defined in A.6.4.1.2 of TS 38.133 and b. A Transmit Timing Test for FR2 extending the Rel-18 multi-TRP multi-DCI 2 TAs Transmit Timing Test defined in A.7.4.1.3 of TS 38.133. Proposal 10: For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#2 (UL-only TRP transmits SSBs), include different configurations depending on the different UE MRTD capabilities in the Transmit Timing Tests. Proposal 11: For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#1 (UL-only TRP does not transmit SSBs) define a. Transmit Timing Test for FR1 and for FR2, and b. Timing Advance Adjustment Accuracy Test for FR1 and for FR2, including configurations for the TACs that depend on the MTTD requirements, if MTTD requirements are introduced. Proposal 12: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P. Proposal 13: Use SNR=-3 dB for defining the accuracy requirements for CJTC reporting. Proposal 14: RAN4 to define test cases for CJTC reporting: a. to verify CJTC-Dd measurement period b. to verify CJTC-f measurement period c. to verify CJTC-Dd measurement accuracy d. to verify CJTC-f measurement accuracy Other issues: Invalid measurements impact on the test. Proposal 15: Define the conditions where the UE may report invalid measurements. . Proposal 16: Discuss about the impact of invalid measurements in the CJTC reporting test and possible ways to verify the conditions for invalid reports for CJT calibration. |
| R4-2510771 | Ericsson | Proposal 1: For defining measurement accuracy for delay and frequency offset, we suggest a 50° total phase error budget for CJT coherent combining, with 30° allocated to UE measurement error and the remainder to other contributing factors. Observation 1: Accuracy requirements depend on signal level (SNR) and number of samples. Proposal 2: Develop accuracy requirements, at least for delay and frequency offset UE measurement reports for: - Different number of samples, namely 3 samples for the frequency offset requirements for SNR > -3dB and 5 samples for the delay offset requirements for SNR > 0 dB. Proposal 3: Develop test cases for delay, frequency UE measurement reports to test core and accuracy requirement. Observation 2: We are not against report mapping as such but believe that we need all the quantization steps defined by RAN1. The defined number of quantization steps are based on RAN1 and RAN4 analytical calculations and simulations. Proposal 4: Any defined report table should reflect all quantization steps. Proposal 5: RAN4 to define the test cases for Event-2 and Event-7 Proposal 6: Define test cases with time window-based and non-time window-based configurations. Proposal 7: For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state). Proposal 8: For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state). Proposal 9: RAN4 to specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset). Proposal 10: No MAC CE-based TCI state switching test case is needed with event-driven reporting. Proposal 11: For DCI-based TCI state switch test cases, wait for further RAN1 progress. |
| R4-2511611 | MediaTek inc. | Proposal 1: RAN4 to prioritize event-2 for testing each of - Event-trigger for Intra-cell BM for SSB - Event-trigger for Intra-cell BM for CSI-RS - Event-trigger for Inter-cell BM for SSB Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F. Observation 1: At low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased. Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F based on the number of samples that will be agreed in the core part. Proposal 4: RAN4 to consider testing CJTC reporting for delay offset and frequency offset. Proposal 5: RAN4 to prioritize testing Scenario#1 for asymmetric DL sTRP / UL mTRP in the performance part. |

Open issues summary

Sub-topic 2-1: Performance requirements for UE-initiated/event-driven beam management

- Issue 2-1-1: Test scope

|  | QC | Apple | Samsung | CMCC | Huawei | ZTE | Nokia | Ericsson | MediaTek |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2: Y Event-1/Event-7: FFS | Event-2: Y Event-1/Event-7: No | Event-2: Y Event-1: limited TC Event-7: No | All | Event-2 FFS on others | All | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? |  | Either A or B | Both | Both |  | Both | Mode A&B as Test configurations. Can be skip depends on UE capability | Mode A&B as Test configurations. Can be skip depends on UE capability |  |
| Event-trigger for Intra-cell BM for SSB | Y | N | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y | Y | FFS | Y | Y | ? | Y |
| Event-trigger for Inter-cell BM for CSI-RS | N | N | N | Y |  | Y | N |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | Y | Y |  | Y | Y | Y |  |
| Frequency Range – FR2? |  | Y. No FR1 | Y. FR2 |  |  | Y. FR2 |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI | N |  |  |  | FFS on core req. | FFS on core req. | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |  |
| L1 measurement reporting delay |  |  |  | Y |  |  |  |  |  |
| CMI |  |  |  |  |  |  | N |  |  |

- Recommended WF

  - In high level, RAN4 to define test cases for UEIBM. For the test case list, to discuss based on table above.

- Issue 2-1-3: Whether to add requirement to verify the performance of CMI?

- Proposals

  - Option 1: (Xiaomi)

    - RAN4 to define requirement to verify the performance of condition met indicator(CMI) in the report.

    - RAN4 to discuss how to define performance metric for event triggered reporting

- Recommended WF

  - TBA

Sub-topic 2-2: Performance requirements for CJTC reporting

- Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

- Proposals

  - Option 1: (Qualcomm, Apple, Samsung, CMCC, Nokia, Ericsson, MediaTek)

    - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

  - Option 1a (Ericsson)

    - Any defined report table should reflect all quantization steps.

- Recommended WF

  - Agree option 1 and Option 1a

- Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

- Proposals

  - Option 1: (Qualcomm, Samsung, Nokia, MediaTek)

    - RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F. No accuracy requirements for CJTC-P

    - Option 1a (Nokia)

      - Use SNR=-3dB

  - Option 2: (Apple)

    - RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part.

  - Option 3: (Huawei)

    - It is possible to establish the accuracy requirement based on one sample as the baseline while waiting to finalise the core requirements for the CJTC.

  - Option 3: (Ericsson)

    - Develop accuracy requirements for delay, frequency UE measurement reports for:

      - 50° total phase error budget for CJT coherent combining, with 30° allocated to UE measurement error and the remainder to other contributing factors.

      - Different number of samples, namely 3 samples for the frequency offset requirements for SNR > -3dB and 5 samples for the delay offset requirements for SNR > 0 dB.

- Recommended WF

  - TBA

- Issue 2-2-3: Test scope

| High level: | QC | Apple | Samsung | CMCC | Nokia | Ericsson | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify CJTC-Dd, measurement period | Y | FFS | Y | Y | Y | Y | Y |
| Whether to define TCs to verify CJTC-f，measurement period | Y | FFS | Y | Y | Y | Y | Y |
| Whether to define TCs to verify CJTC-P， (To be verify?) | N | N | N |  |  |  |  |
| Whether to define TCs to verify scheduling restriction |  |  |  | Y (if core req.) |  |  |  |
| Whether to define TCs to verify CJTC-Dd, CJTC-f，accuracy |  |  |  |  | Y | Y |  |

- Recommended WF

  - For the test case list, to discuss based on table above.

Sub-topic 2-3: Performance requirements for enhancement for Asymmetric DL sTRP / UL mTRP

- Issue 2-3-1: Test scope

| High level: | QC | Apple | Samsung | CMCC | Huawei | Nokia | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify timing requirements? |  | Y | Y | Y | Y | Y |  |
| Whether to define TCs to verify TCI state switching requirements? | Y |  | N |  |  |  |  |
| FR1 and/or FR2? | both |  |  |  |  |  |  |
| Whether to cover Scenario#1 and Scenario#2? | Both for FR1. Scenario#2 for FR2 | Both |  | Both | Scenarios#1 | both | Scenarios#1 |

- Recommended WF

  - For the test case list, to discuss based on table above.

Topic #3: UE feature list in RAN4

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509833 | Samsung | Proposal 7: For Rel-19 MIMO Phase 5 RRM, no need to add dedicated RAN4 UE feature list for UEIBM, CJTC reporting, Asymmetric DL sTRP/UL mTRP. They have been covered in RAN1 feature list. |

Open issues summary

- Issue 3-1: Whether to need RAN4 dedicated UE feature list for Rel-19 MIMO RRM?

- Proposals

  - Option 1: (Samsung)

    - For Rel-19 MIMO Phase 5 RRM, no need to add dedicated RAN4 UE feature list for UEIBM, CJTC reporting, Asymmetric DL sTRP/UL mTRP. They have been covered in RAN1 feature list.

  - Option 1a: (Qualcomm, Huawei)

    - Clearly indicate that no need to add dedicated RAN4 UE capabilities for UEIBM.

- Recommended WF

  - Check option 1 is agreeable.