3GPP TSG-RAN WG4 Meeting #116	R4-2509172

Bengaluru, 25-29 August, 2025

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for SCM for UE demod for Alignment

Agenda item:	 7.12.3

Document for:	Information

- Introduction

In the previous RAN4 Meeting #115, we contributed to the SCM Study Item sharing considerations on the Study Item on the use spatial channel model for demodulation performance requirements.

According to the outcome of the discussion during the meeting and based on the approved Way Forward, we provide in this contribution simulation results for information.

Proposal 1: RAN4 to consider the simulation results included in this document for the purpose of SCM alignment.

# Simulation Results for PDSCH Alignment

| Channel Model |  | UE Speed/ Doppler | PMI | Test point - % of Rel TP | CW0 | CW1 |
| --- | --- | --- | --- | --- | --- | --- |
| 4x4 w/ 4 layers |  |  |  |  |  |  |
| CDL Option 1 (Non truncated) | AAV option 3 | 3 kph | Random | 30% | 5.0 |  |
|  |  |  | Random | 70% | 12.5 |  |
| CDL Option 2 (Truncated) |  |  | Random | 30% | 5.9 |  |
|  |  |  | Random | 70% | 14.7 |  |
| Enh TDL Option 2 |  |  | Fixed (i11=4) | 30% | 6.3 |  |
|  |  |  | Fixed (i11=4) | 70% | 14.4 |  |
| 8x8 w/ 8 layers |  |  |  |  |  |  |
| CDL Option 1 (Non truncated) | AAV option 3 | 3 kph | Random | 30% | 6.7 | 8.8 |
|  |  |  | Random | 70% | 16.4 | 23.2 |
| CDL Option 2 (Truncated) |  |  | Random | 30% | 6.9 | 12.3 |
|  |  |  | Random | 70% | 17.2 | 23.4 |
| Enh TDL Option 2 |  |  | Fixed (i11=4) | 30% | 6 | 10.3 |
|  |  |  | Fixed (i11=4) | 70% | 13.6 | 17.8 |

# Simulation Results for PMI Alignment

| Channel Model |  | UE Speed/ Doppler | Codebook | Metric (% of Max TP) | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
| 8x4 w/ 4 layers |  |  |  |  |  |
| CDL Option 1 (Non truncated) | AAV option 3 | 3 kph | Type I | 70% | 7.8 |
|  |  |  |  | 90% | 10.4 |
|  |  |  | eType II | 70% | 6.7 |
|  |  |  |  | 90% | 8.9 |
| CDL Option 2 (Truncated) |  |  | Type I | 70% | 9.5 |
|  |  |  |  | 90% | 12.0 |
|  |  |  | eType II | 70% | 8.2 |
|  |  |  |  | 90% | 10.6 |
| Enh TDL Option 2 |  |  | Type I | 70% | 11.4 |
|  |  |  |  | 90% | 13.4 |
|  |  |  | eType II | 70% | 10.9 |
|  |  |  |  | 90% | 12.8 |