3GPP TSG-RAN WG4 Meeting #116										 	R4-2509227

Bengaluru, 25th - 29th August, 2025

Agenda Item:	7.6.5.2

Source:	MediaTek Inc.

Title:	Discussion on RRM NTN testing for NGSO

WID/SID:        NR_IoT_NTN_req_test_enh

Document for:	Discussion

# 1 Introduction

R19 specifies a TE-emulated channel model with varying Doppler and delay shifts. This paper discusses its application to RRM tests and the impact on specifications.

# 2 Principle of NTN testing for NGSO satellite link channel with varying Doppler and delay shifts

In R17/R18, the SAN RRM test cases use an artificial channel with fixed/constant Delay and Doppler shift, as described in B.5 of TS38.133 (see Appendix), because a time-varying model was not yet defined.

R19 introduces a TE-emulated channel model with varying Doppler and delay shifts, so the testing procedure (B.5) should reference the new channel model, likely defined in 38.101-5 for R19 UEs.

Proposal 1: For R19 38.133, update the testing procedure (B.5) to reference the new channel model.

In addition, testing the new NGSO satellite on legacy R17/R18 UEs is beneficial, as it better reflects real channel conditions. To prevent duplicate testing with different channels, a testing principle should allow R17/R18 NTN UEs that supports NGSO to skip constant Delay and Doppler shift tests if they pass the tests with varying Delay and Doppler shift, as defined in R19 38.101-5.

Proposal 2: For R17/R18 UEs that supports NGSO, if the UE passes the tests with the varying delay and Doppler shift channel, as defined in R19 38.101-5, it can skip the tests with constant delay and Doppler shift channel.

# References

Proposal 1: For R19 38.133, update the testing procedure (B.5) to reference the new channel model.

Proposal 2: For R17/R18 UEs that supports NGSO, if the UE passes the tests with the varying delay and Doppler shift channel, as defined in R19 38.101-5, it can skip the tests with constant delay and Doppler shift channel.

# Appendix

# B.5	High level test procedure for SAN RRM tests

The following high level steps are conducted for test cases for SAN defined in clause A.14.

-	A set of ephemeris information are pre-defined for each satellite corresponding to respective epoch times in TS 38.508-1 [38].

-	The same ephemeris information will be maintained during the test (constant ephemerisInfo in all SIB19 updates), i.e. SAN RRM test cases are defined with fixed constant Delay and Doppler shift from Satellite access node to UE unless otherwise stated.

The range from which the constant Delay is selected is as follows:

-	For GSO an altitude of 35,786km is considered. The range of the one-way delay between UE and satellite is from 119.375 ms to 128.79 ms.

-	For NGSO an altitude of 600km and 1200km on a circular orbit are considered. The range of the one-way delay between UE and satellite is from 2 ms (lowest value for LEO orbit 600km) to 6.67 ms (highest value for LEO orbit 1200km).

-	The constant Doppler shift (i.e. frequency offset) is derived from the same ephemeris (i.e. orbit emulation) as the Delay.

-	UE location is determined for the test. During the test, the test system shall provide the UE location to the DUT using AT commands.

-	The ephemeris and the UE location should be designed such that elevation angle relative to the UE position shall not be smaller than 30 deg during entire test time.

-	Test equipment adjusts the time and frequency of transmission according to the pre-defined ephemeris (constant in all SIB19 updates) and UE location.

## A.3.36	Testing related to Satellite access

## A.3.36.1	Introduction

### A.3.36.1	Introduction

In annex A test cases are defined for verifying various type of RRM requirements related to satellite access.

### A.3.36.2	Principle of testing GSO and NGSO scenarios

In annex A, RRM test cases related to satellite access are defined for both GSO and NGSO. The testing principle for these test cases is as follows:

-	A UE capable of GSO only is required to pass the test cases with GSO.

-	A UE capable of NGSO only is required to pass the test cases with NGSO.

-	A UE capable of both GSO and NGSO is required to pass the test cases with NGSO only.

Support of GSO and NGSO scenario is indicated via ntn-ScenarioSupport-r17.

### A.3.36.2	Principle of testing different RRM requirements

In annex A, RRM test cases related to satellite access are defined for all applicable RRM requirements. The testing principle for these test cases is as follows:

-	A UE capable of NTN only is required to pass all the test cases defined in clause A.14.

-	A UE capable of both TN and NTN is required to pass the test cases for NTN specific requirements in table A.3.36.2-1.

Table A.3.36.2-1: Test cases for NTN specific requirements

| Clause | Test case slogan |
| --- | --- |
| A.14.1.2 | Cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.3 | Time-based cell reselection to FR1 intra-frequency NR cell |
| A.14.1.4 | Location-based cell reselection to FR1 intra-frequency NR cell |
| A.14.1.7 | Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.8 | Time-based Cell reselection to FR1 inter-frequency NR satellite access case |
| A.14.1.9 | Location-based Cell reselection to FR1 inter-frequency NR satellite access case |
| A.14.2.1.3 | Intra-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.4 | Inter-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.5 | Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.2.1.6 | Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.3.1.1 | NR UE Transmit Timing Test for FR1 |
| A.14.5.1.1 | SA event triggered reporting tests without gap under non-DRX |
| A.14.5.1.2 | SA event triggered reporting tests without gap under DRX |
| A.14.5.1.3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading |
| A.14.5.1.4 | SA event triggered reporting tests with single measurement gap under non-DRX for satellite access |
| A.14.5.1.5 | SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access |
| A.14.5.1.6 | SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access |
| A.14.5.2.1 | Event triggered reporting test without gap under non-DRX |
| A.14.5.2.2 | Event triggered reporting tests without gap under DRX |
| A.14.6.3.1 | SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell |
| A.14.6.3.2 | SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell |
| A.14.6.4.1 | SSB based L1-RSRP measurement |
| A.14.6.4.2 | CSI-RS based L1-RSRP measurement on resource set with repetition off |

### A.3.36.3	Principle of testing different ephemeris formats

Satellite access RRM test cases are defined such that satellite ephemeris information is sent to UE in each test case, according to tables 14.0.3-1 and 14.0.3-2.

Table A.3.36.3-1: Test cases configuring EphemerisInfo as PositionVelocity

| Functional Area | Test Case |
| --- | --- |
| RRC_IDLE state mobility | A.14.1.3 |
|  | A.14.1.4 |
|  | A.14.1.5 |
|  | A.14.1.6 |
|  | A.14.1.9 |
|  | A.14.1.10 |
| Handover | A.14.2.1.3 |
|  | A.14.2.1.4 |
|  | A.14.2.1.5 |
|  | A.14.2.1.6 |
| Timing | A.14.3.1 |
| Radio Link Monitoring | A.14.4.1.1 |
|  | A.14.4.1.2 |
|  | A.14.4.1.5 |
|  | A.14.4.1.6 |
| BFD and LR procedures | A.14.4.2.1 |
|  | A.14.4.2.3 |
|  | A.14.4.2.5 |
| Active BWP switch | A.14.4.3.1 |
| UE specific CBW change | A.14.4.4.1 |
| PL-RS switching delay | A.14.4.5.1 |
| Intra-frequency measurements | A.14.5.1.1 |
|  | A.14.5.1.3 |
|  | A.14.5.1.5 |
| Inter-frequency measurements | A.14.5.2.1 |
|  | A.14.5.2.3 |
|  | A.14.5.2.7 |
| L1-RSRP measurements | A.14.5.3.1 |
|  | A.14.5.3.3 |
| SS-RSRP accuracy | A.14.6.1.1 |
| SS-RSRQ accuracy | A.14.6.2.1 |
| SS-SINR accuracy | A.14.6.3.1 |
| L1-RSRP accuracy | A.14.6.4.1 |

Table A.3.36.3-2: Test cases configuring EphemerisInfo as Orbital

| Functional Area | Test Case |
| --- | --- |
| RRC_IDLE state mobility | A.14.1.1 |
|  | A.14.1.2 |
|  | A.14.1.5 |
|  | A.14.1.6 |
| Handover | A.14.2.1.1 |
|  | A.14.2.1.2 |
| RRC Connection Mobility Control | A.14.2.2.1 |
|  | A.14.2.2.2 |
|  | A.14.2.2.3 |
| Timing | A.14.3.2 |
| Radio Link Monitoring | A.14.4.1.3 |
|  | A.14.4.1.4 |
|  | A.14.4.1.7 |
|  | A.14.4.1.8 |
| BFD and LR procedures | A.14.4.2.2 |
|  | A.14.4.2.4 |
|  | A.14.4.2.6 |
| Active BWP switch | A.14.4.3.2 |
| Intra-frequency measurements | A.14.5.1.2 |
|  | A.14.5.1.4 |
|  | A.14.5.1.6 |
| Inter-frequency measurements | A.14.5.2.2 |
|  | A.14.5.2.4 |
|  | A.14.5.2.6 |
|  | A.14.5.2.8 |
| L1-RSRP measurements | A.14.5.3.2 |
|  | A.14.5.3.4 |
| SS-RSRP accuracy | A.14.6.1.2 |
| SS-RSRQ accuracy | A.14.6.2.2 |
| SS-SINR accuracy | A.14.6.3.2 |
| L1-RSRP accuracy | A.14.6.4.2 |