3GPP TSG RAN WG4 Meeting #116	R4-2509247

Bengaluru, India, August 25th – 29th , 2025

Title:	Replied LS on Low NR band carrier aggregation via switching

Response to:	R1-2504869

Release:	Release-19

Work Item:	NR_LBCA_Sw

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact person:

Name:		Peng Zhang

E-mail Address:	zhangpeng16@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

Overall Description

RAN4 thanks RAN1 for the LS on Low NR band carrier aggregation via switching and sharing RAN1’s agreements about this topic.

RAN4 noticed that the maximum values of LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 and LBCA-SwitchinggGap-SCelltoPCell-r19 are still FFS in RAN1’s feature list R1-2503243.

Based on RAN4’s evaluation about the duration of switching gap considering different UE switching period capability (35us, 70us and 140us), the following observations are made.

For LBCA-SwitchinggGap-Duration-PCelltoSCell-r19, the maximum value should be equal to or larger than three OFDM symbols with 15kHz SCS, no matter what the TTA is .

For LBCA-SwitchinggGap-SCelltoPCell-r19, the maximum value should be equal to or larger than the following values based on the different maximum cell coverage to be supported.

| Maximum Cell Coverage (km) | The maximum value of LBCA-SwitchinggGap-SCelltoPCell-r19 |
| --- | --- |
| 100km | 12 OFDM symbols with 15kHz SCS |
| 21km | 5 OFDM symbols with 15kHz SCS |
| 14km | 4 OFDM symbols with 15kHz SCS |
| 10km | 4 OFDM symbols with 15kHz SCS |
| 0.5km~5km | 3 OFDM symbols with 15kHz SCS |

Actions

To RAN1/RAN2:

Action: RAN4 respectfully asks RAN1/RAN2 to take the above information into account.

Dates of Next TSG-RAN1 Meeting

RAN4 #116bis			13 October – 17 October 2025		Prague, Czech Republic

RAN4 #117			17 November – 21 November 2025		The U.S., Dallas