3GPP TSG-RAN WG4 Meeting #116                                                                                                         R4-2509346

Bengaluru, India, 25th –29th August, 2025

Source: 	CATT

Title: 	Further discussion on LP-WUS demodulation requirements

Agenda item:	7.24.7

Document for:	Approval

- Introduction

In RAN4#115 meeting, a WF for [115][332] NR_LPWUS_Demod was agreed[1], and the remaining issues on LP-WUS demodulation requirements in WF require further discussion.

In this contribution, we provide our views on LP-WUS demodulation requirements.

- Discussion

Whether to introduce FAR test

As per the WF [1], the remaining open issue concerning whether to introduce FAR test is shown as below:

Issue 3-1-3: Whether to introduce FAR test - WF - Option 1: Yes. - Option 2: No.

As WF, RAN4 agrees to introduce MDR test for LP-WUS, since the MDR test is dependent on FAR test, like PRACH FAR, It is necessary to introduce FAR test.

Proposal 1: Adopt Option 1 for whether to introduce FAR test as below:

  - Option 1: Yes.

Test metric for FAR test (if introduced)

As per the WF [1], the remaining open issue concerning test metric for FAR test is shown as below:

Issue 3-1-5: Test metric for FAR test (if introduced) - WF - Option 1: 1% FAR - Option 2: 0.1% FAR

For PRACH, its FAR was defined 0.1%, since it is for establishing Uplink Synchronization, so we think that the 0.1% FAR for PRACH is reasonable. Referring to PRACH, LP-WUS is mainly used for terminal devices to receive network signals in a low-power mode when they are not connected, thereby achieving energy saving and fast wake-up functions. LP WUS does not require smaller FAR, we think that 1% FAR is sufficient.

Proposal 2: Define Option 1 for test metric for FAR test as below:

  - Option 1: 1% FAR

LP-WUS waveform

As per the WF [1], the remaining open issue concerning LP-WUS waveform is shown as below:

Issue 3-1-6: LP-WUS waveform - WF Option 1: Consider both OOK-1 and OOK-4

It is sufficient to consider both OOK-1 and OOK-4 for LP-WUS demodulation, we propose Option 1.

Proposal 3: Adopt Option 1 for LP-WUS waveform as below:

Option 1: Consider both OOK-1 and OOK-4

Receiver type

As per the WF [1], the remaining open issue concerning receiver type for LP-WUS is shown as below:

Issue 3-1-7: Receiver type - WF - Option 1: Consider both OOK and OFDM receivers

Since the OOK and OFDM receiver are supported by LP-WUS, and they are different receiver type for LP-WUS, so it is reasonable to consider both OOK and OFDM receivers. So we propose,

Proposal 4: Adopt Option1 for receiver type as below:

  - Option 1: Consider both OOK and OFDM receivers

Frequency range

As per the WF [1], the remaining open issue concerning frequency range for LP-WUS is shown as below:

Issue 3-1-8: Frequency range - WF - Option 1: Consider both FR1 and FR2. - Option 2: Prioritize FR1; FFS FR2.

We support Option 2, since the FR2 LP-WUS was not widely discussed in RAN1.

Proposal 5: Adopt Option 2 for frequency range as below:

  - Option 2: Prioritize FR1; FFS FR2.

Duplex mode

As per the WF [1], the remaining open issue concerning duplex mode for LP-WUS is shown as below:

Issue 3-1-9: Duplex mode - WF - Option 1: Consider both TDD and FDD.

Since both TDD and FDD are supported for NR, so it is reasonable to consider both TDD and FDD for LP-WUS for NR. So we propose,

Proposal 6: Adopt Option 1 for duplex mode as below:

  - Option 1: Consider both TDD and FDD.

SCS

As per the WF [1], the remaining open issue concerning SCS for LP-WUS is shown as below:

Issue 3-1-10: SCS - WF - Option 1: 15kHz for FDD FR1 and 30kHz for TDD FR1. - Option 2: 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for FR2

Since we think that FR2 frequency range is necessary to be defined for LP-WUS, we propose option 1: 15kHz for FDD FR1 and 30kHz for TDD FR1 as SCS for LP-WUS demodulation.

Proposal 7: Adopt Option 1 for SCS as below:

- Option 1: 15kHz for FDD FR1 and 30kHz for TDD FR1.

Bandwidth

As per the WF [1], the remaining open issue concerning bandwidth for LP-WUS is shown as below:

Issue 3-1-11: Bandwidth - WF - Option 1: For the channel bandwidth and SCS, take below assumptions for the starting points - 10MHz CHBW with SCS 15KHz for FR1 FDD - 40MHz CHBW with SCS 30KHz for FR1 TDD - 100MHz CHBW with SCS 120KHz for FR2

As above, we propose to choose 15kHz SCS for FDD FR1 and 30kHz SCS for TDD FR1, so it is reasonable to adopt 10MHz CHBW with SCS 15KHz for FR1 FDD and 40MHz CHBW with SCS 30KHz for FR1 TDD for LP-WUS demodulation.

Proposal 8: Adopt following CBW for LP-WUS demodulation:

    - 10MHz CHBW with SCS 15KHz for FR1 FDD

    - 40MHz CHBW with SCS 30KHz for FR1 TDD

Number of RBs

As per the WF [1], the remaining open issue concerning number of RBs is shown as below:

Issue 3-1-12: Number of RBs - WF - Option 1: 11RB

We support option 1, 11RB is sufficient for LP-WUS demodulation.

Proposal 9: Adopt Option 1for number of RBs as below:

  - Option 1: 11RB

Antenna configuration

As per the WF [1], the remaining open issue concerning antenna configuration is shown as below:

Issue 3-1-13: Antenna configuration - WF - Option 1: 1T1R.

We think that 1T1R is sufficient for LP-WUS demodulation. We propose,

Proposal 10: Adopt Option 1 for antenna configuration as below:

  - Option 1: 1T1R.

Channel model

As per the WF [1], the remaining open issue concerning channel model is shown as below:

Issue 3-1-14: Channel model - WF - Option 1: TDLA30-10 only - Option 2: TDLC300-100 only - Option 3: Both TDLC300-100 and TDLA30-10

For PUSCH demodulation, TDLC300-100 is used for 16QAM, and TDLA30-10 is used for 64QAM, for LP-WUS, the OOK modulation is used, TDLC300-100 is sufficient. So we propose Option 2.

Proposal 11: Adopt Option 2 for channel model as below:

  - Option 2: TDLC300-100 only

Time offsets

As per the WF [1], the remaining open issue concerning time offsets is shown as below:

Issue 3-1-15: Time offsets - WF - Option 1: T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

Since the T= 6.4us for OOK-1 in RAN1, so we support the Option 1 for time offsets for LP-WUS demodulation.

Proposal 12: Adopt Option 1 for time offsets as below:

  - Option 1: T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

Frequency offsets

As per the WF [1], the remaining open issue concerning frequency offsets is shown as below:

Issue 3-1-16: Frequency offsets - WF - Option 1: For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm - Option 1: up to 20 ppm

In RAN1, 20ppm was used for simulation. So the Option 1 and Option 2 are reasonable. We propose option 1.

Proposal 13: Adopt Option 1 for frequency offsets as below:

  - Option 1: For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm

Whether to define performance requirements for LP-SS

As per the WF [1], the remaining open issue concerning whether to define performance requirements for LP-SS is shown as below:

Issue 3-1-17: Whether to define performance requirements for LP-SS - WF - Option 1: No - Option 2: Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

Since the performance requirements for LP-SS will be same as that for LP-WUS.so we think it is not necessary to define performance requirement for LP-SS, so we propose Option 1(No).

Proposal 14: Adopt Option 1 for whether to define performance requirements for LP-SS as below:

  - Option 1: No

Whether to define single requirement for Type 1 and Type 2 receivers

As per the WF [1], the remaining open issue concerning whether to define single requirement for Type 1 and Type 2 receivers is shown as below:

Issue 3-1-18: Whether to define single requirement for Type 1 and Type 2 receivers - WF - Option 1: Further discuss based on simulation results if one or two sets of requirements are to be defined

Since one shot OFDM receiver and OOK receiver have similar performance, so it is sufficient to define a set of requirements for Type 1 and Type 2 receivers.

Proposal 15: Define a set of requirements for Type 1 and Type 2 receivers.

Coding/decoding algorithms for LP-WUS

As per the WF [1], the remaining open issue concerning coding/decoding algorithms for LP-WUS is shown as below:

Issue 3-1-19: Coding/decoding algorithms for LP-WUS - WF - Option 1: 1 or 2 information bits for small block coding and 3 or more information bits for RM coding

From our simulation result in RAN1, there is not obvious difference in performance between 1 and 2 information bits for small blocking coding. So, we propose 1 information bits for small block coding. Since 5 information bits for RM coding was agreed for sensitivity [2], so we propose 5 information bits for RM coding.

Proposal 16: Adopt 1 information bits for small block coding and 5 information bits for RM coding.

- Conclusion

This contribution provides analysis on LP-WUS demodulation requirements. The following proposal is concluded as follows:

Proposal 1: Adopt Option 1 for whether to introduce FAR test as below:

  - Option 1: Yes.

Proposal 2: Define Option 1 for test metric for FAR test as below:

  - Option 1: 1% FAR

Proposal 3: Adopt Option 1 for LP-WUS waveform as below:

Option 1: Consider both OOK-1 and OOK-4

Proposal 4: Adopt Option1 for receiver type as below:

  - Option 1: Consider both OOK and OFDM receivers

Proposal 5: Adopt Option 2 for frequency range as below:

  - Option 2: Prioritize FR1; FFS FR2.

Proposal 6: Adopt Option 1 for duplex mode as below:

  - Option 1: Consider both TDD and FDD.

Proposal 7: Adopt Option 1 for SCS as below:

- Option 1: 15kHz for FDD FR1 and 30kHz for TDD FR1.

Proposal 8: Adopt following CBW for LP-WUS demodulation:

    - 10MHz CHBW with SCS 15KHz for FR1 FDD

    - 40MHz CHBW with SCS 30KHz for FR1 TDD

Proposal 9: Adopt Option 1for number of RBs as below:

  - Option 1: 11RB

Proposal 10: Adopt Option 1 for antenna configuration as below:

  - Option 1: 1T1R.

Proposal 11: Adopt Option 2 for channel model as below:

  - Option 2: TDLC300-100 only

Proposal 12: Adopt Option 1 for time offsets as below:

  - Option 1: T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

Proposal 13: Adopt Option 1 for frequency offsets as below:

  - Option 1: For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm

Proposal 14: Adopt Option 1 for whether to define performance requirements for LP-SS as below:

  - Option 1: No

Proposal 15: Define a set of requirements for Type 1 and Type 2 receivers..

Proposal 16: Adopt 1 information bits for small block coding and 5 information bits for RM coding.

- References

- R4-2508626, WF for [115][332] NR_LPWUS_Demod, Moderator (Qualcomm), RAN4#115

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, vivo, RAN4#115