3GPP TSG-RAN WG4 Meeting #116            	R4-2509402

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.20.6

Source:	Samsung

Title:	View on BS demodulation requirement for MIMO with 3Tx

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WI was endorsed to support NR MIMO phase 5. One of objective related with uplink as following in WID [1]

- Objective of Core part - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource - Note: UL full power transmission mode 1 and mode 2 are not supported - Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching

In the last meeting, the test scope of performance part of NR MIMO phase 5 was under discussed, the related agreement was captured into WF in [2] as following

- General scope - Clarify if demodulation requirements are needed for 3-antenna-port codebook-based UL transmissions - Option 1: RAN4 introduce PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmission - Option 2: RAN4 do not introduce requirements for 3-antenna-port codebook-based UL transmission

In this contribution, the view on the test scope and test setup of BS demodulation requirement was provided.  Meanwhile, the initial simulation results are provided for feasibility study.

# 2	Discussion

## 2.1	General scope

NR supports 1-port, 2-port, 4-port and 8-port PUSCH. Nonetheless, current commercial mobile UEs are generally equipped with only 1 or 2 Tx antennas. To enhance the UL performance and anticipating advancements in hardware and design technology, the emergence of UEs equipped with 3 Tx antennas is a foreseeable development in the near future. Consequently, it is advantageous to augment the NR standards to support 3-port PUSCH supporting up to 3 layers.

In Rel-19, RAN1 has introduced the non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmission with following matrix

Table 6.3.1.5-48: Precoding matrix W for single-layer transmission using three antenna ports with 4portSRS_3TX configured. TPMI index; Precoder matrix W (ordered from left to right in increasing order of TPMI index). 0 – 2; 13100; 13010; 13001; -; -; -; -; -. Table 6.3.1.5-49: Precoding matrix W for two-layer transmission using three antenna ports with 4portSRS_3TX configured. TPMI index; Precoder matrix W (ordered from left to right in increasing order of TPMI index). 0 – 2; 13100100; 13100001; 13001001; -. Table 6.3.1.5-50: Precoding matrix W for three-layer transmission using three antenna ports with 4portSRS_3TX configured. TPMI index; Precoder matrix W (ordered from left to right in increasing order of TPMI index). 0; 13100010001; -; -; -.

Compared with the non-coherent codebook for 3 layers with 4Tx transmission as following, the power scaling factor is different.

Table 6.3.1.5-6: Precoding matrix W for three-layer transmission using four antenna ports with transform precoding disabled. TPMI index; (ordered from left to right in increasing order of TPMI index). 0 – 3. 4 – 6; -.

Observation 1: Compared with non-coherent codebook of 3 layers with 4Tx transmission, different power scaling factor is used for the non-coherent codebook of 3 layers with 3 Tx transmission

Meanwhile, the PUSCH requirement with supporting 4Tx scenario targeting for FWA or CPE devices was introduced in Rel-18 NR FR1, only 4-layer requirement was specified for 4Tx transmission, there is no specific 3-layer requirement for 4Tx transmission.

Observation 2: PUSCH requirement with supporting 4Tx transmission scenario targeting for FWA or CPE devices, only 4 layers requirement was specified for 4Tx transmission in Rel-18. There is no specific 3-layer requirement for 4Tx transmission.

Therefore, considering the difference of non-coherent codebook for 3-layer for 3Tx and 4Tx transmission, and there is also no requirement for 3-layer transmission, we think it is necessary to introduce PUSCH demodulation performance requirements for UE which support 3Tx with 3- layer transmission for guaranteeing the test coverage

Proposal 1: RAN4 should specify PUSCH requirement with 3Tx with 3-layer transmission with non-coherent codebook.

## 2.2	Test Setup

In Rel-15, RAN4 has introduced PUSCH requirements with UE supporting 1Tx and 2Tx scenarios. And PUSCH requirement with supporting 4Tx in Rel-18 NR FR1 WI. In general, the existing test configurations can be considered as reference for specifying PUSCH requirements with 3Tx transmission.

Duplex operation

From BS demodulation requirement aspect, there is no minor different between FDD and TDD. As agreed, the same requirements are applicable to FDD and TDD with different UL-DL patterns.

Proposal 2: RAN4 specifies PUSCH requirement for both TDD and FDD. The same requirements can be applied for FDD and TDD with different UL-DL patterns

SCS and BW

In Rel-15, RAN4 introduced PUSCH with specifying most of SCS and CBW combination for FR1, where large effort for simulation results and results alignment are suffered. For 4Tx-4layer transmission, the typical channel bandwidth, including the minimum and maximum channel bandwidth for each SCS were selected as

- 15kHz SCS, 5MHz, 20MHz, 50MHz

- 30kHz SCS, 10MHz, 40MHz, 100MHz

Therefore, the 3Tx-3-layer transmission, we prefer to select the same SCS and BW configuration for specifying PUSCH requirement with 3Tx transmission

Proposal 3: Consider the following SCS and channel bandwidth for specifying PUSCH requirement with 3Tx with 3 layers transmission

- 15kHz SCS, 5MHz, 20MHz, 50MHz

- 30kHz SCS, 10MHz, 40MHz, 100MHz

Antenna configuration

For UL-2-layer transmission, different Rx configuration as 2Rx/4Rx/8Rx are considered with test applicable rule. And for UL-4-layer transmission, the PUSCH requirement was specified with 4Rx and 8Rx. For UL-3-layer transmission, it is straightforward to introduce PUSCH demodulation performance requirements with 4Rx and 8Rx.

Proposal 4: Considering 4Rx and 8Rx for specifying PUSCH requirement with 3Tx with 3 layers transmission

Number of layers

According to RAN1 agreement, to support non-coherent-codebook-based PUSCH transmission by a 3Tx UE, UE reports a capability of up to a maximum of 3	layers.  Although low layer can be supported by 3Tx transmission, how to select the proper non-coherent codebook index is up to BS scheduling. In existing 4Tx transmission, only 4 layers transmission was considering when specifying the PUSCH requirement with 4Tx. Similar as 4Tx, only 3layers transmission is considered for requirement definition, and only one TMPI index is available for 3Tx 3 Layers, the related TPMI index is configured as 0

Proposal 5: RAN4 only specifies PUSCH requirement with UL 3 Layers transmission with 3Tx UE, and the TPMI index is configured as 0

Number of DMRS

Since the 3Tx UE is not targeting for high-speed condition, similar as normal PUSCH demodulation requirement for 1Tx/2Tx/4Tx, only configured 2 DMRS should be enough.

Proposal 6: RAN4 specifies PUSCH requirement only with 2 DMRS symbols

DMRS type

In Rel-15, only DMRS type 1 was considered. To facilitate the 3Tx transmission, both Rel-15 DMRS pattern and Rel-18 DMRS pattern can be supported. Rel-18 DMRS pattern is mainly targeting to support more UE Multiplexing for MU-MIMO scenarios. Since this WI is not targeting for MU-MIMO, we prefer to only consider Rel-15 DMRS pattern for requirement, the related DM-RS port is configured as {0,1,2}

Proposal 7: RAN4 specifies PUSCH requirement only with Rel-15 DMRS pattern, the DM-RS port index  is {0,1,2}

Channel model and MCS

Similar as existing 4Tx requirement, we prefer to consider TDLB100-400, TDLA30-10 and TDLC300-100 for specifying the PUSCH requirement with 3Tx. For MCS, MCS 2, MCS 12 and MCS 17 were considered when specifying the requirements with 4Tx-4-layer transmission. They can be considered as starting point,

In the last meeting, companies also propose with same MCSs as 2Tx transmission, in this paper, we provide the initial results to check the feasibility of using the higher MCS values for each modulation level

Mapping type

For PUSCH mapping type, both mapping type A and mapping type B are supported. Both requirements are defined with test applicability for 1Rx/2Rx and 4Rx. Similar, we prefer to consider both mapping type A and mapping type B for specifying PUSCH requirement with 3Tx.

Proposal 8: RAN4 specifies PUSCH requirement with both PUSCH resource mapping type A and type B

Other

For other parameters, the existing simulation assumption for 4Tx requirement can be reused as reference.

Proposal 9: The existing simulation assumption for 4Tx requirement can be reused as reference for specifying PUSCH requirements with 3Tx transmission

# 3	Simulation Results

In this section, the initial simulation results are provided to check the feasibility of using higher MCS values with 3 Tx 3-layer transmission

Table 1: Initial simulation results of PUSCH with 3Tx- 3layer transmission, 15KHz SCS

| Case | Tx/Rx | SCS | BW | Channel | Mapping Type | DMRS | MCS | SNR@70% TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3Tx/4Rx | 15KHz | 5MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.432 |
| 2 | 3Tx/4Rx | 15KHz | 5MHz | TDLC300-100 | A | (2,11) | MCS12 | 8.981 |
| 3 | 3Tx/4Rx | 15KHz | 5MHz | TDLA30-10 | A | (2,11) | MCS17 | 12.455 |
| 4 | 3Tx/4Rx | 15KHz | 5MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.594 |
| 5 | 3Tx/4Rx | 15KHz | 5MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.016 |
| 6 | 3Tx/4Rx | 15KHz | 20MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.413 |
| 7 | 3Tx/4Rx | 15KHz | 20MHz | TDLC300-100 | A | (2,11) | MCS12 | 9.205 |
| 8 | 3Tx/4Rx | 15KHz | 20MHz | TDLA30-10 | A | (2,11) | MCS17 | 12.779 |
| 9 | 3Tx/4Rx | 15KHz | 20MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.835 |
| 10 | 3Tx/4Rx | 15KHz | 20MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.679 |
| 11 | 3Tx/4Rx | 15KHz | 50MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.521 |
| 12 | 3Tx/4Rx | 15KHz | 50MHz | TDLC300-100 | A | (2,11) | MCS12 | 9.325 |
| 13 | 3Tx/4Rx | 15KHz | 50MHz | TDLA30-10 | A | (2,11) | MCS17 | 14.134 |
| 14 | 3Tx/4Rx | 15KHz | 50MHz | TDLC300-100 | A | (2,11) | MCS16 | 14.7 |
| 15 | 3Tx/4Rx | 15KHz | 50MHz | TDLA30-10 | A | (2,11) | MCS20 | 17.252 |

Table 2: Initial simulation results of PUSCH with 3Tx- 3layer transmission, 30KHz SCS

| Case | Tx/Rx | SCS | BW | Channel | Mapping Type | DMRS | MCS | SNR@70% TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3Tx/4Rx | 30KHz | 10MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.669 |
| 2 | 3Tx/4Rx | 30KHz | 10MHz | TDLC300-100 | A | (2,11) | MCS12 | 9.070 |
| 3 | 3Tx/4Rx | 30KHz | 10MHz | TDLA30-10 | A | (2,11) | MCS17 | 12.390 |
| 4 | 3Tx/4Rx | 30KHz | 10MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.712 |
| 5 | 3Tx/4Rx | 30KHz | 10MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.090 |
| 6 | 3Tx/4Rx | 30KHz | 40MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.645 |
| 7 | 3Tx/4Rx | 30KHz | 40MHz | TDLC300-100 | A | (2,11) | MCS12 | 9.342 |
| 8 | 3Tx/4Rx | 30KHz | 40MHz | TDLA30-10 | A | (2,11) | MCS17 | 12.977 |
| 9 | 3Tx/4Rx | 30KHz | 40MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.773 |
| 10 | 3Tx/4Rx | 30KHz | 40MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.820 |
| 11 | 3Tx/4Rx | 30KHz | 100MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.724 |
| 12 | 3Tx/4Rx | 30KHz | 100MHz | TDLC300-100 | A | (2,11) | MCS12 | 9.529 |
| 13 | 3Tx/4Rx | 30KHz | 100MHz | TDLA30-10 | A | (2,11) | MCS17 | 13.657 |
| 14 | 3Tx/4Rx | 30KHz | 100MHz | TDLC300-100 | A | (2,11) | MCS16 | 14.371 |
| 15 | 3Tx/4Rx | 30KHz | 100MHz | TDLA30-10 | A | (2,11) | MCS20 | 16.900 |

Observation 3: Compared with 5MHz and 20MHz results for 15KHz SCS, there is about 1 dB performance degradation in 50MHz, especially for high MCSs, such as MCS 16 and MCS 20. Similar as 100MHz for 30KHz

Compared with different MCSs, it is feasible to consider the high MCS for each modulation order, which means the same MCS for 2Tx requirement can be used for specifying the PUSCH requirement with 3Tx

Observation 4: Compared with different MCSs, it is feasible to consider the high MCS for specifying the PUSCH requirement with 3Tx with 3-layer transmission

Proposal 10:  The following MCS can be considered for specifying PUSCH requirement with 3Tx with 3-layer transmission

- MCS 2: TDLB100-400

- MCS 16: TDLC300-100

- MCS 20: TDLA30-10

# 4	Conclusion

In this contribution, the view on the test scope and test setup of PUSCH requirement with 3Tx transmission was provided. Meanwhile, the initial simulation results are provided for feasibility checking with different MCS and channel bandwidth.

Observation 1: Compared with non-coherent codebook of 3 layers with 4Tx transmission, different power scaling factor is used for the non-coherent codebook of 3 layers with 3 Tx transmission

Observation 2: PUSCH requirement with supporting 4Tx transmission scenario targeting for FWA or CPE devices, only 4 layers requirement was specified for 4Tx transmission in Rel-18. There is no specific 3-layer requirement for 4Tx transmission.

Proposal 1: RAN4 should specify PUSCH requirement with 3Tx with 3-layer transmission with non-coherent codebook.

Duplex operation

Proposal 2: RAN4 specifies PUSCH requirement for both TDD and FDD. The same requirements can be applied for FDD and TDD with different UL-DL patterns

SCS and BW

Proposal 3: Consider the following SCS and channel bandwidth for specifying PUSCH requirement with 3Tx with 3 layers transmission

- 15kHz SCS, 5MHz, 20MHz, 50MHz

- 30kHz SCS, 10MHz, 40MHz, 100MHz

Antenna configuration

Proposal 4: Considering 4Rx and 8Rx for specifying PUSCH requirement with 3Tx with 3 layers transmission

Number of layers

Proposal 5: RAN4 only specifies PUSCH requirement with UL 3 Layers transmission with 3Tx UE, and the TPMI index is configured as 0

Number of DMRS

Proposal 6: RAN4 specifies PUSCH requirement only with 2 DMRS symbols

DMRS type

Proposal 7: RAN4 specifies PUSCH requirement only with Rel-15 DMRS pattern, the DM-RS port index is {0,1,2}

Mapping type

Proposal 8: RAN4 specifies PUSCH requirement with both PUSCH resource mapping type A and type B

Other

Proposal 9: The existing simulation assumption for 4Tx requirement can be reused as reference for specifying PUSCH requirements with 3Tx transmission

Observation 3: Compared with 5MHz and 20MHz results for 15KHz SCS, there is about 1 dB performance degradation in 50MHz, especially for high MCSs, such as MCS 16 and MCS 20. Similar as 100MHz for 30KHz

Observation 4: Compared with different MCSs, it is feasible to consider the high MCS for specifying the PUSCH requirement with 3Tx with 3-layer transmission

Proposal 10:  The following MCS can be considered for specifying PUSCH requirement with 3Tx with 3-layer transmission

- MCS 2: TDLB100-400

- MCS 16: TDLC300-100

- MCS 20: TDLA30-10

# References

RP-242394, WID revision: NR MIMO Phase 5, Samsung

R4-2508627, Way Forward for [115][329] NR_MIMO_demod , Samsung