3GPP TSG-RAN WG4 Meeting #116	                              R4-2509428

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.7

Source: 	Apple

Title: 			 Demodulation and/or CSI reporting requirements for CSI prediction

Document for:	Discussion

# Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved. This contribution provides discussion on demodulation requirements for CSI prediction.

# Discussion

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

According to the revised WID RP-243244, CSI prediction (one-sided model) is one of the objectives for WI, and RAN4 need to specify necessary RAN4 requirements. In previous meeting, it was agreed to use relative throughput as requirement metric for CSI prediction. Reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook.

For CSI prediction, it was agreed that AI/ML based performance requirement should not be worse than the legacy, and FFS which legacy requirement/feature are relevant.

Agreements in RAN4#112bis Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

During RAN4#115, the following agreement was reached:

Issue 1-1: Requirement considerations

Agreement:

The performance of AI based CSI prediction should not be worse than R16 etypeII.

- The exact test procedure and configurations are FFS

- The impact of different timeline between AI and R16 etype II should be also carefully analysed

Issue 1-3: Doppler handling

Agreement:

  - De-prioritize 100Hz Doppler in R19

    - Consider 100Hz Doppler in the future release if needed

In the context of AI/ML-based CSI prediction, defining meaningful and fair performance requirements is fundamentally more complex than in legacy PMI testing. Unlike traditional PMI reporting, where the algorithm behavior is deterministic and independent of deployment scenarios, the performance of AI/ML models is scenario-dependent, tightly linked to the training dataset and inference conditions. Factors such as antenna configuration, UE speed (Doppler), observation window length, and prediction window length directly affect prediction accuracy and generalization. This poses a challenge for RAN4, as the current legacy testing framework assumes a single PMI requirement is sufficient to represent conformance across all conditions.

While non-static channel conditions (e.g., changing SNR or fading profiles over time) offer a more realistic and comprehensive test of model generalization, they introduce significant complexity and ambiguity, especially for a group like RAN4 that has not previously handled time-varying test conditions. Defining the appropriate dynamics, reference models, and reproducible baselines would require extensive new work and agreement, which is not feasible within the limited timeline of Rel-19.

Therefore, to ensure AI/ML model robustness and performance while remaining within practical constraints, it is essential that RAN4 focuses on well-defined static test conditions for Rel-19. However, unlike legacy PMI tests, multiple static test cases must be defined, each reflecting a different scenario dimension. This allows the test framework to reflect real-world variability in AI/ML model behavior without requiring non-static (time-varying) setups, thereby ensuring a balance between test coverage and feasibility.

RAN4 is invited to focus on static test conditions for CSI prediction performance evaluation in Rel-19, while explicitly defining multiple test cases to reflect the dependency of AI/ML model performance on deployment scenarios. These test cases should vary in terms of key parameters such as antenna configurations, UE speed, observation window length, and prediction window length.

AI/ML-based CSI prediction introduces a dynamic behavior that diverges from the deterministic operation of legacy PMI reporting. Unlike traditional methods, the performance of an AI model can vary significantly over time due to changes in deployment environments, UE behavior, and data drift. This creates a need for a runtime performance monitoring mechanism to detect and react to model degradation or misalignment. Without such monitoring, a malfunctioning or outdated model could lead to persistent suboptimal performance, such as inaccurate precoder selection, reduced throughput, or even link failures, without the network being aware.

To address this, RAN1 has introduced the concept of Type 3 UE-assisted performance monitoring, which allows the UE to internally assess the quality of the AI/ML model's predictions. A promising metric identified for this purpose is the Squared Generalized Cosine Similarity (SGCS) between the predicted CSI and either the actual measured CSI or the CSI-RS-implied channel. However, there is currently no standardized procedure in RAN4 for how this metric should be computed, aggregated (e.g., as a CDF), or reported. Additionally, there is no guidance on how the network should interpret these values, nor how such reports should interact with fallback or reconfiguration mechanisms.

Without a clearly defined performance monitoring and reporting framework, operators cannot reliably detect or diagnose AI model underperformance. This would undermine the trust in deploying AI-based solutions and limits their scalability. Furthermore, the lack of agreed thresholds or reporting granularity hinders lifecycle management (LCM), as it becomes difficult to determine when a model needs retraining, replacement, or deactivation. A standardized Type 3 monitoring procedure would provide the necessary transparency and robustness to safely operationalize AI/ML-based CSI prediction in real-world

RAN4 is invited to define a Type 3 performance monitoring framework for AI/ML-based CSI prediction, using SGCS or a similar metric to assess model prediction accuracy at runtime. The framework should specify how the metric is computed, the format of the reported results (e.g., CDF-based thresholds or per-instance values), the reporting granularity (such as wideband or subband level), and the conditions under which monitoring results should trigger model fallback or AI/ML model reconfiguration.

In last meeting, the baseline simulation assumptions for AIML based CSI prediction was approved [11]. We provide simulation results for step 1 to check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE).

# Conclusion

This contribution provides discussion on CSI prediction. The proposals are:

- RAN4 is invited to focus on static test conditions for CSI prediction performance evaluation in Rel-19, while explicitly defining multiple test cases to reflect the dependency of AI/ML model performance on deployment scenarios. These test cases should vary in terms of key parameters such as antenna configurations, UE speed, observation window length, and prediction window length.

- RAN4 is invited to define a Type 3 performance monitoring framework for AI/ML-based CSI prediction, using SGCS or a similar metric to assess model prediction accuracy at runtime. The framework should specify how the metric is computed, the format of the reported results (e.g., CDF-based thresholds or per-instance values), the reporting granularity (such as wideband or subband level), and the conditions under which monitoring results should trigger model fallback or AI/ML model reconfiguration.

# Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface