3GPP RAN WG4 Meeting #116		R4-2509543

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.1.2.2.1

Source:	Apple

Title:	On Rel-19 power domain enhancement for FR1

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase when starting the Rel-19 work. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

##### 2.2 On extension for 5MHz channel

RAN4 discussed various extension options and agreed that the baseline extension shall be 0.5 of CBW/NRB. A second extension factor is 0.25. These factors result in most cases into extension aligning with regular channel sizes. E.g. 20MHz CBW extended by 10MHz (factor 0.5) and 5MHz (factor 0.25). However, there exists cases where irregular extension sizes will be present. E.g. 15MHz resulting into 7.5MHz and 3.75MHz. RAN4 needs to explicitly specify the extension for each UE channel.

In case of 5MHz channel the 0.5 single-sided extension results into an extended bandwidth of 7.5MHz. There is nothing inherently wrong with that extension, but it might be preferable to limit the extension to 7MHz as it falls into the newly defined CBW. It would allow operators who have 7MHz spectrum available to deploy the extension if required. Furthermore, there exist spectrum block sizes which are 7MHz wide and this tweak would allow to make use of the feature Similar decision could be made for the 0.25 extension. Instead of specifying single sided extension of 1.25MHz it might be limited to 1MHz so that the extended channel results into 6MHz which is highly requested channel size for Rel-20.

While the single sided extension for 5MHz is limited to 2MHz and 1MHz, the double-sided extension should not be limited. Which means that a 5MHz channel would still be extended to 10MHz which allows the conversion of all outer allocations into inner allocations.

Table 1: Extension for 5MHz channel with 15kHz SCS

| UE channel | Extension Type | Extension Size | Extension to each side in MHz | Extension to each side in RBs |
| --- | --- | --- | --- | --- |
| 5MHz | Single-Sided | Size 1 | 2 | 12 |
|  | Single-Sided | Size 2 | 1 | 6 |
|  | Double-Sided | Size 1 | 2.5 | 15 |
|  | Double-Sided | Size 2 | 1.25 | 8 |

Observation: In case of 5MHz channel the 0.5 single sided extension results into an extended bandwidth of 7.5MHz. There is nothing inherently wrong with that extension, but it might be preferable to limit the extension to 7MHz as it falls into the newly defined CBW. It would allow operators who have 7MHz spectrum available to deploy the extension if required. Furthermore, there exist spectrum block sizes which are 7MHz wide and this tweak would allow to make use of the feature. Similar decision could be made for the 0.25 extension. Instead of specifying single sided extension of 1.25MHz it might be limited to 1MHz so that the extended channel results into 6MHz which is highly requested channel size for Rel-20.

Proposal 1: For 5MHz channel bandwidth the extension should be differentiated between single-sided and double-sided. This would allow to define single-sided extension resulting into 6MHz and 7MHz.

##### 2.3 On signalling

The table outlines the initial approach on the signalling for CBW extension-based MPR reduction in. It includes five entries.

The first one indicates support for a fixed CBW extension of 0.5 CBW/NRB, which can be either symmetric or asymmetric. It is applicable only to FR1 and is optional with capability signalling. If not supported by the UE, the feature cannot be supported. The type is per band, and there is no need for FDD/TDD differentiation.

The second one indicates support for a flexible CBW extension based on a limited number of fixed values, which can also be symmetric or asymmetric. The specific values for the CBW extension size are to be determined. Like the previous entry, it is applicable only to FR1, optional with capability signalling, and the type is per band. There is no need for FDD/TDD differentiation.

Table 2: Signalling for CBW extension-based MPR reduction

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | Support of CBW extension based MPR reduction | Indicates support of CBW extension - Extension in terms of BW/RB is fixed and defined by 38.101-1 - Extension can be symmetric or asymmetric |  | Yes |  | The feature cannot be supported and legacy requirements of single CC apply. | Per Band | No | FR1 only |  |  | Optional with capability signaling |
| 46. NR_ENDC_RF_Ph4 | 46-2 | Support of CBW extension based MPR reduction | Indicates support of CBW extension - Extension in terms of BW/RB is more flexible and defined by 38.101-1 - Extension can be symmetric or asymmetric |  | Yes |  | The feature cannot be supported and legacy requirements of single CC apply. | Per Band | No | FR1 only |  | TBD values of CBW extension size | Optional with capability signaling |
| 46. NR_ENDC_RF_Ph4 | 46-3 | MPR enhancement for carrier aggregation | Indicates support for single CC MPR if only one CC is actively transmitting in an intra-band UL contiguous CA configuration. |  | Yes |  | The feature cannot be supported and legacy requirements of intra-band UL contiguous CA apply. | Per BC | No | FR1 only |  |  | Optional with capability signaling |
| 46. NR_ENDC_RF_Ph4 | 46-4 | MPR enhancement for FR2 intra-band CA based on CABW | Indicates support of FR2 MPR improvement for intra-band CA based on CABW |  | Yes |  | The feature cannot be supported and legacy requirements of FR2 intra-band UL CA apply. | Per BC | No | FR2 only |  |  | Optional with capability signaling |
| 46. NR_ENDC_RF_Ph4 | 46-5 | MPR enhancement for FR2 intra-band CA based on activated carrier | Indicates support of FR2 MPR improvement for intra-band CA based on activated carrier |  | Yes |  | The feature cannot be supported and legacy requirements of intra-band UL CA apply. | Per BC | No | FR2 only |  |  | Optional with capability signaling |

Proposal 2: Consider Table 2 when discussing the signalling for CBW extension-based MPR reduction.

# 4	Conclusions

This contribution provides certain considerations to the discussion on the MPR reduction which his part of the power domain enhancement WID. The following observations and proposals are made:

Observation: In case of 5MHz channel the 0.5 single sided extension results into an extended bandwidth of 7.5MHz. There is nothing inherently wrong with that extension, but it might be preferable to limit the extension to 7MHz as it falls into the newly defined CBW. It would allow operators who have 7MHz spectrum available to deploy the extension if required. Furthermore, there exist spectrum block sizes which are 7MHz wide and this tweak would allow to make use of the feature. Similar decision could be made for the 0.25 extension. Instead of specifying single sided extension of 1.25MHz it might be limited to 1MHz so that the extended channel results into 6MHz which is highly requested channel size for Rel-20.

Proposal 1: For 5MHz channel bandwidth the extension should be differentiated between single-sided and double-sided. This would allow to define single-sided extension resulting into 6MHz and 7MHz.

Proposal 2: Consider Table 2 when discussing the signalling for CBW extension-based MPR reduction.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103

- R4-2507930, “WF on Power domain enhancement”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #115

- R4-2418170, “Discussion on UE RF power enhancement for NR single carrier”, vivo, 3GPP TSG-RAN4 Meeting #113