3GPP TSG-RAN WG4 Meeting # 116		R4-2509567

Bengaluru , IN, August 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.101-1 introducing 6Rx RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RF requirements for 6Rx are not yet defined in the latest version of the TS 38.101-1 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To implement 6Rx REFSENS and SRS insertion loss requirements into the specifications based on the following previously agreed WFs: - R4-2414285, REFSENS and SRS IL for t1r6, t2r6 and t1r6-t2r6 - R4-2417209 - R4-2503014, SRS IL for t3r6, t1r6-t3r6, t2r6-t3r6, t1r6-t2r6-t3r6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF requirements for 6Rx feature will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.2.4, 7.2, 7.3.1, 7.3.2, 7.3A.1, 7.3A.5, 7.3G, 7.6A.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< begin change 1 >>

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIBC	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band contiguous CA operation

ΔRIBNC	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band non-contiguous CA operation

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔRIB,6R	Reference sensitivity adjustment due to support for 6 antenna ports

ΔRIB,8R	Reference sensitivity adjustment due to support for 8 antenna ports

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

ΔRXR,2R	Reference sensitivity adjustment for two antenna ports XR UEs on bands defined in Table 7.3.2-2b

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

<< end change 1 >>

<< begin change 2 >>

### 6.2.4	Configured transmitted power

<< unchanged content omitted >>

ΔPPowerClass =

-	3 dB for a power class 2 UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3 dB for a power class 1.5 UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1 on the band where UE indicates power class 2;

-	3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 or txDiversity2Tx-r18 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r6’ or ‘t1r8’;

-	0 dB otherwise;

NOTE:	UE reports ∆PPowerClass when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the ue-PowerClass after the duty cycle exceedance.

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for V2X operating bands for concurrent operation, CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

ΔPPowerBoost is defined as 1dB for power class 3 and 0.5dB for power class 2, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, and if IE powerBoostPi2BPSK-r18 and/or powerBoostQPSK-r18 is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2 in a TDD band or power class 3

-	IfΔPPowerClass is 0dB

-	If scheduled UL transmission is DFT-s-OFDM with either PI/2 BPSK modulation (with either Rel-15 DMRS or with pi/2 BPSK DMRS) or QPSK modulation

-	If the RB allocation belongs to the inner region defined in clause 6.2.2.

-	If UE indicates power class 3, the percentage of uplink symbols transmitted in a certain evaluation period is less than 80%

-	If UE indicates power class 2, when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame)

-	0 dB otherwise;

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

MPRc and A-MPRc for serving cell c are specified in clause 6.2.2 and clause 6.2.3, respectively and in clause 6.2F.2 and clause 6.2F.3 respectively for shared spectrum access operation;

∆MPRc for serving cell c is specified in clause 6.2.2 and in clause 6.2F.2 for shared spectrum access operation.

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)	UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2' or 't1r1-t1r2'

b)	UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4' or 't1r1-t1r2-t1r4' or, 't1r1-t1r2-t2r2-t1r4-t2r4'

c)	UE transmits SRS from the SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', or 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4', or

d)	UE transmits SRS to a DL-only carrier

e)	UE transmits SRS on the second, third, fourth, fifth, sixth, seventh and eighth SRS resources of the total 8 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't1r8', or

f)	UE transmits SRS from the SRS port pair on the second, third and fourth SRS resource in every configured SRS resource set consisting of two SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't2r8', or

g) 	UE transmits SRS from the set of SRS ports on the second SRS resource in every configured SRS resource set consisting of four SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't4r8',

h)	UE transmits SRS on the second, third, fourth, fifth, and sixth SRS resources of the total 6 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't1r6', or

i)	UE transmits SRS from the SRS port pair on the second, third SRS resource in every configured SRS resource set consisting of two SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't2r6', or

[j) 	UE transmits SRS from the set of SRS ports on the second SRS resource in every configured SRS resource set consisting of three SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't3r6'.

The following ∆TRxSRS applies according to the indicated SRS-TxSwitch or srs-AntennaSwitchingBeyond4RX-r17 capabilities:

if  't1r8' and 't4r8' are indicated:

-	The value of ∆TRxSRS is 7.3 dB for bands whose FUL_high is higher than the FUL_low of n79 and 5.8 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 10.3 dB for bands whose FUL_high is higher than the FUL_low of n79 and 8.8 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't1r8' and 't2r8' are indicated:

-	The value of ∆TRxSRS is 6.0 dB for bands whose FUL_high is higher than the FUL_low of n79 and 4.5 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

-	The value of ∆TRxSRS is 9.0 dB for bands whose FUL_high is higher than the FUL_low of n79 and 7.5 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't1r8' or 't2r8', but not both is indicated:

-	The value of ∆TRxSRS is 5.5 dB for bands whose FUL_high is higher than the FUL_low of n79 and 4.0 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 8.5 dB for bands whose FUL_high is higher than the FUL_low of n79 and 7.0 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't1r2', 't1r1-t1r2', 't1r4', 't1r4-t2r4', 't1r1-t1r2-t1r4', 't2r4', 't1r1-t1r2-t2r2-t2r4', 't1r1-t1r2-t2r2-t1r4-t2r4' or 't4r8' is indicated:

-	The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

For other SRS transmissions ∆TRxSRS is zero;

else, if 't1r6' is indicated:

-	The value of ∆TRxSRS is 6.5dB for band n104, 5.5dB for band n79 and 4.0dB for band n41, n77 and n78, when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 9.5dB for band n104, 8.5dB for band n79 and 7.0dB for band n41, n77 and n78 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't2r6' is indicated:

-	The value of ∆TRxSRS is 5.5dB for band n104, 5.0dB for band n79 and 3.5dB for band n41, n77 and n78, when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 8.5dB for band n104, 8.0dB for band n79 and 6.5dB for band n41, n77 and n78 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't1r6' and ‘t2r6’ are indicated or 't1r6' and ‘t3r6’ are indicated:

-	The value of ∆TRxSRS is 7.0dB for band n104, 6.0dB for band n79 and 4.5dB for band n41, n77 and n78, when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 10.0dB for band n104, 9.0dB for band n79 and 7.5dB for band n41, n77 and n78 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't3r6' is indicated:

-	The value of ∆TRxSRS is 5.0dB for band n104, 4.5dB for band n79 and 3.0dB for band n41, n77 and n78, when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 8.0dB for band n104, 7.5dB for band n79 and 6.0dB for band n41, n77 and n78 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't2r6' and ‘t3r6’ are indicated:

-	The value of ∆TRxSRS is 6.0dB for band n104, 5.5dB for band n79 and 4.0dB for band n41, n77 and n78, when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 9.0dB for band n104, 8.5dB for band n79 and 7.0dB for band n41, n77 and n78 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if ‘t1r6’, 't2r6' and ‘t3r6’ are indicated:

-	The value of ∆TRxSRS is 8.0dB for band n104, 7.0dB for band n79 and 5.5dB for band n41, n77 and n78, when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 11.0dB for band n104, 10.0dB for band n79 and 8.5dB for band n41, n77 and n78 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

For other SRS transmissions ∆TRxSRS is zero;

<< end change 2 >>

<< begin change 3 >>

## 7.2	Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination.

Unless otherwise stated, the following applicability rules apply,

-	For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, additional requirements for four and six Rx ports shall be verified in operating bands where the UE is equipped with six Rx antenna ports, and additional requirements for four and eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports.

-	For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with six Rx antenna ports, the UE shall be verified with eight Rx antenna ports and skip two, four, and six antenna ports requirements in operating bands where the UE is equipped with eight Rx antenna ports unless the UE does not support eight Rx ports for band(s) in a band combination in which case those band(s) shall be verified with four Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.

-	The above rules apply for all clauses except for clause 7.9.

<< end change 3 >>

<< begin change 4 >>

## 7.3	Reference sensitivity

### 7.3.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

-	when the UE is verified with 2 Rx antenna ports, it shall be verified against those requirements by applying the REFSENS value in Table 7.3.2-1a, Table 7.3.2-1b and Table 7.3.2-1c or Table 7.3.2-1d with 2 Rx antenna ports tested;

-	when the UE is verified with 4 Rx antenna ports, it shall be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2 with 4 Rx antenna ports tested.

-    when the UE is verified with 6 Rx antenna ports, it shall be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2c with 6 Rx antenna ports tested.

-	when the UE is verified with 8 Rx antenna ports, it shall be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2a with 8 Rx antenna ports tested.

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1c, Table 7.3.2-1d , Table 7.3.2-2, Table 7.3.2-2a, and 7.3.2-2c.

<< unchanged content omitted >>

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n13, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

For UE(s) equipped with 8 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2a for the applicable operating bands.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n1, n3, n7, n25 | -4.5 |
| n34, n39, n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

For two Rx antenna port XR UE(s) indicating UE capability supportOf2RxXR-r18, reference sensitivity for two Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRXR,2R in Table 7.3.2-2b for the applicable operating bands.

Table 7.3.2-2b: Two antenna port XR UE reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41, n48, n77, n78, n79 | -1.0 |

For UE(s) equipped with 6 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,6R in Table 7.3.2-2c for the applicable operating bands.

Table 7.3.2-2c: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.41, -3.72 |
| n77, n78, n79, n104 | -3.01, -3.32 |
| NOTE 1: When 6 Rx operation is supported by handheld UE. NOTE 2: When 6 Rx operation is supported by FWA form factor. |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1c, Table 7.3.2-1d, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-2c , and Table 7.3.2-2b shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

<< end change 4 >>

<< begin change 5 >>

## 7.3A	Reference sensitivity for CA

### 7.3A.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For operations with 4 Rx, 6Rx, or 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.2-2,  ΔRIB,6R in Table 7.3.2-2c, or ΔRIB,8R in Table 7.3.2-2a when MSD > 0.

For reference sensitivity exception test points where the specified carrier frequency does not correspond to a valid NR-ARFCN, the closest NR-ARFCN as specified in clause 5.4.2 applies.

For reference sensitivity level tests or reference sensitivity exception tests specified in clause 7.3A, SCS=15kHz based UL test configuration can be replaced by SCS=30kHz based UL test configuration. The equivalent substitution relationship between different SCS UL test configuration is shown in Table 7.3A.1-1 for the operating bands above 2.2GHz.

<< end change 5 >>

<< begin change 6>>

### 7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two CCs from up to two UL NR bands and three CCs from two UL NR bands given in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-1b, Table 7.3A.5-2, Table 7.3A.5-2a and Table 7.3A.5-2b the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-1b, Table 7.3A.5-, Table 7.3A.5-2a and Table 7.3A.5-2b. For these test points the reference sensitivity requirement specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2 and Table 7.3.2-2a, and Table 7.3.2-2c   are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-1b, Table 7.3A.5-2, Table 7.3A.5-2a and Table 7.3A.5-2b.

<< end change 6 >>

<< begin change 7>>

## 7.3G	Reference sensitivity for Tx Diversity

For UE supporting Tx diversity, the minimum requirements specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1d, Table 7.3.2-2 and Table 7.3.2-2a, and Table 7.3.2-2c shall be met with Tx diversity configuration described in clause 6.2G.1. For Tx diversity, the parameter PUMAX is defined in clause 6.2G.4 with the sum of the output power from all UE antenna connectors.

<< end change 7>>

<< begin change 8>>

### 7.6A.4	Narrow band blocking for CA

#### 7.6A.4.1	Narrow band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6A.4.1-1 with the uplink configuration. For UE(s) supporting one uplink, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.6A.4.1-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6A.4.1-1.

Table 7.6A.4.1-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |  |
| --- | --- | --- | --- | --- |
|  |  |  | B | C |
| n1, n2, n3, n25, n38, n41, n66, n71, n48, n40 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA Bandwidth Class specific value below |  |
|  |  | dB | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 |
|  | Fuw (offset forf = 15 kHz, 30 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2c Table 7.3.2-2 and Table 7.3.2-2a for two, four, six, and eight antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

<< end change 8>>