# 3GPP TSG-RAN WG4 Meeting # 116	R4-2509647

Bengaluru, India, August 25th – 29th, 2025

Source: 	Huawei, HiSilicon

Title: 	Delta Ppowerclass and Delta TRxSRS for 4Tx

Agenda Item:	5.6.1

Document for:	Approval

# Introduction

Still RAN4 has not been able to reach a consensus on how to handle ΔPPowerClass and ΔTRxSRS for 4Tx during SRS antenna switching (SRS AS), even though the discussion is about R18 WI. In RAN#115, there was a proposal [1, 2] to introduce UE capability to resolve the issue. This contribution discusses the proposal.

# Discussion

As mentioned in our previous contributions [3-5], RAN4 has struggled with a dilemma that if ΔPPowerClass = 3 dB, the specification leaves room for UE to perform antenna virtualization (AV), but alternatively, if ΔPPowerClass = 6 dB, UEs with 26 dBm capable PA is not allowed to deliver 26 dBm, but rather allowed to transmit up to 23 dBm per SRS transmission, e.g., when the UE is configured with e.g., SRS resources corresponding to t1ry.

A proposal in [1, 2] is the introduction of UE capability for UE to clarify ΔPPowerClass value per SRS antenna switching configuration is in order to address the above dilemma. The below is excerpt from [2].

In our view, unfortunately, the proposal doesn’t address the fundamental issue, since even if UE indicates ΔPPowerClass = 3dB for t1ry, the UE can do AV to achieve 26 dBm. Thus, capability itself cannot prevent UE from performing AV.

Observation 1: Reporting capability of ΔPPowerClass per SRS AS resource(s), e.g., t1ry, t2ry or t4ry still doesn’t solve the original problem, e.g., it doesn’t preclude UE from performing antenna virtualization, e.g., when ΔPPowerClass is 3 dB for PC1.5 4Tx.

Moreover, the capability is 4Tx specific and it’s not a generic solution meaning that if 3Tx must have a similar issue, e.g., if another than fullpower mode 0 is introduced and/or PC1.5 is introduced in the future.

Observation 2: The solution of reporting capability of ΔPPowerClass per SRS AS resource(s) cannot apply to some other similar cases, e.g., PC1.5 3Tx.

Finally, the capability itself wouldn’t be useful from network operation perspective.

Observation 3: The capability is only for test purpose and wouldn’t be useful from network operation perspective.

As summary, we don’t think that the introduction of the capability is justified.

Proposal 1: Capability to convey ΔPPowerClass shouldn’t be introduced.

Alternatively, as proposed in [5], it must be enough to make the specification to have a text to avoid AV. According to TS38.101-1, the max power for TxD is defined as follows.

As can be seen from the above, power from all UE transmit antenna connectors shall be summed for output power Tx Diversity. In an opposite way, if the specification explicitly doesn’t allow to sum the power, we can avoid AV.

In case of 1TyR, the highlighted texts are the delta from our CR of [R4-2500378] in RAN4#114.

- 3 or 6dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC1.5 UE with four transmit antenna connectors further indicates SRS-TxSwitch capability ‘t1r2’, ‘t1r4’, ‘t1r1-t1r2’, ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r2’, ‘t1r4’ or ‘t1r8’, where the configured power shall be verified per antenna connector per SRS transmission occasion;

In case of 2TyR.

-	0 or 3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of two SRS ports when PC1.5 UE with four transmit antenna connectors further indicates SRS-TxSwitch capability 't2r2', 't2r4', 't1r4-t2r4', 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t2r2’, ‘t2r4’ or ‘t2r8’, where the configured power shall be verified per a pair of antenna connectors per SRS transmission occasion;

In the above, 3 dB for 1TyR and 0 dB for 2TyR correspond to 26 dBm x 4 and 6 dB for 1TyR and 3 dB for 2TyR correspond to 23 dBm x 4 implementations, respectively. Since the specification explicitly says that UE’s configured output power shall be verified per or a pair of antenna connector(s), unnecessary AV must be avoided.

It is noted that what ΔPPowerClass is can be declared by UE vendors, if needed. It is noted that such kind of declaration is already used in TS38.101-1, e.g., post antenna connector gain Gpost connector for V2X.

# Summary

As a result, we obtained following observations and two proposals. A companion CR based on this contribution is also submitted [6].

Observation 1: Reporting capability of ΔPPowerClass per SRS AS resource(s), e.g., t1ry, t2ry or t4ry still doesn’t solve the original problem, e.g., it doesn’t preclude UE from performing antenna virtualization, e.g., when ΔPPowerClass is 3 dB for PC1.5 4Tx.

Observation 2: The solution of reporting capability of ΔPPowerClass per SRS AS resource(s) cannot apply to some other similar cases, e.g., PC1.5 3Tx.

Observation 3: The capability is only for test purpose and wouldn’t be useful from network operation perspective.

Proposal 1: Capability to convey ΔPPowerClass shouldn’t be introduced.

Proposal 2: Following texts should be incorporated into TS38.101-1 to accommodate 26 dBm x 4 and 23 dBm x 4 implementation as well as the request to avoid Antenna Virtualization.

In case of 1TyR, the highlighted texts are the delta from our CR of [R4-2500378] in RAN4#114.

- 3 or 6dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC1.5 UE with four transmit antenna connectors further indicates SRS-TxSwitch capability ‘t1r2’, ‘t1r4’, ‘t1r1-t1r2’, ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r2’, ‘t1r4’ or ‘t1r8’, where the configured power shall be verified per antenna connector per SRS transmission occasion;

In case of 2TyR.

-	0 or 3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of two SRS ports when PC1.5 UE with four transmit antenna connectors further indicates SRS-TxSwitch capability 't2r2', 't2r4', 't1r4-t2r4', 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t2r2’, ‘t2r4’ or ‘t2r8’, where the configured power shall be verified per a pair of antenna connectors per SRS transmission occasion;

# References

[1] R4-2507736, 4 Tx Delta PPowerClass for SRS antenna switching, Qualcomm Technologies, RAN4#115

[2] R4-2508057, Capabilities for Delta PPowerClass for 4Tx SRS transmission, Qualcomm Technologies, RAN4#115

[3] R4-2417517, More on Delta Ppowerclass and Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#113

[4] R4-2500377, More on Delta Ppowerclass and Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#114

[5] R4-2506829, More on Delta Ppowerclass and Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#115

[6] R4-2509648, Introduction of Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#116