3GPP TSG-RAN WG4 Meeting #116	R4-2509952

Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance requirements for network energy saving

Agenda item:	7.23.4

Document for:	Discussion

- 1. 	Introduction

Performance aspects for Rel-19 network energy saving [1] were discussed in last meeting. In this contribution we continue discussing the test cases for the respective features based on latest agreements and recommended WF [2]. Further discussion details can be found in the topic summary [3].

- OD-SSB SCell operation

- 2.1 Test scenarios

In last meeting, the following test scenarios are listed for discussion [2]. We will discuss the proposals which we have concerns.

Sub-issue 1: Test scope Issue 3-1-1: Scenario RAN4 to discuss whether the following test scenarios can be agreed. - RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. - RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a. - For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B. - For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell. - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

Test cases in scenario #3B

For OD-SSB SCell operation, RAN1 has identified different cases and scenarios as below, and made further down scoping or restriction on the details. Up to now, we understood the following cases and scenarios are agreed by RAN1 in this release:

- Case #1: No always-on SSB on the cell

  - Scenario #2: OD-SSB is activated “before UE receives SCell activation command”

  - Scenario #2A: OD-SSB is activated “when UE receives SCell activation command”

  - Scenario #3B: OD-SSB is activated “when SCell activation is completed and SCell is activated” or “after SCell activation is completed and SCell is activated”

- Case #2: Always-on SSB is periodically transmitted on the cell

  - Scenario #2: OD-SSB is activated “before UE receives SCell activation command”

  - Scenario #2A: OD-SSB is activated “when UE receives SCell activation command”

  - Scenario #3B: OD-SSB is activated “when SCell activation is completed and SCell is activated” or“After SCell activation is completed and SCell is activated”

In Case #1, since there is no always-on SSB on the cell, OD-SSB is at least needed in order to activate the SCell. Hence the first OD-SSB activation shall happen no later than when UE receives SCell activation command. Based on the latest RAN1 agreements, scenario #3B was also agreed but we understood this does not refer to the first OD-SSB activation as RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated. This means we can assume OD-SSB is always activated after UE receives SCell activation command. In summary, for Case 1 test cases design, we should consider scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command. There is no need to define test cases dedicated for scenario #3B.

Observation 1: In Case 1, RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated hence in Scenario #3B OD-SSB shall be always activated.

Proposal #1: For Case 1, RAN4 should define the test cases considering scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command.

In Case #2, there is always-on SSB on the cell hence OD-SSB is not a must for activating the SCell. Then all the scenarios listed above shall be considered.

Proposal #2: For Case 2, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B regarding to when the OD-SSB is activated.

Test cases for T2

The UE behaviour for OD-SSB reactivation has been discussed for long time. Pending on if the OD-SSB activation command comes within T2 after it was deactivated, the UE is expected to perform OD-SSB based measurements in different ways. Hence it would be valuable to verify the UE measurement behaviour based on when the OD-SSB is reactivated relevant to T2. This will also provide reference information to network implementation about OD-SSB activation/deactivation.

Proposal #3: For Case 1, RAN4 to define the test cases to verify the UE measurement behaviour based on when the OD-SSB is reactivated relevant to T2.

Scenarios in Case 2

RAN1 has agreed on the different scenarios based on the time/frequency relation between AO-SSB and OD-SSB for Case 2, including Alt. Time C1, Alt. Time C2 and scenario 3. If RAN4 would agree to define the RRM requirements for all these scenarios, we should define the test cases according to verify the UE behaviour.

Proposal #4: For Case 2, RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if we would agree to define the RRM requirements for all these scenarios.

- 2.2 Test case list

The following test cases are being considered in last meeting.

Sub-issue 2: Test case list Issue 3-1-3: Test case list RAN4 to discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 1 - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement(deactivated SCell) in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC

In our view, as RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated, the difference from legacy SCell activation is only OD-SSB periodicity is used instead of SMTC. Hence it is not necessary to verify all the SCell activation scenarios. In addition, multiple SCell scenarios are being discussed in [3]. Based on current agreement, we don’t see how activating multiple SCells could happen. We prefer to define the test cases only for single SCell activation.

Proposal #5: RAN4 to reduce the number of test cases for SCell activation and define the TCs only for single SCell activation.

OD-SIB1

RAN4#115 has discussed performance requirements for OD-SIB1 [2].

### Sub-topic 3-2: OD-SIB1 performance part Issue 3-2-1: Test case - Proposals - Option 1: RAN4 not to define OD-SIB1 RRM performance test case. - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

At RAN4#115, the following has been agreed on RRM requirements for On-demand SIB1 [4].

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

Thus, a test case for verifying that the paging interruption time is not exceeded in case of OD-SIB1, is desirable to be defined.

In particular, a longer TSI-NR is appropriate since the SI acquisition takes longer in case of OD-SIB1. Additional components for the acquisition time versus legacy SIB1 are:

- Duration of UL WUS transmission to NES cell,

- Duration of RAR reception and message decoding,

- Duration of OD-SIB1 window.

The discussion related to component 3 is still ongoing in RAN1 (5-5120ms, multiple of 160ms, etc. vs. 160ms periodicity of legacy SIB1). Thus, the additional time for OD-SIB1 acquisition allowed in the test should take into account those three components. It can be defined once RAN1 has concluded on component 3.

Proposal #6:  RAN4 to define a test case for verifying longer interruption in case of OD-SIB1 support in NES cell, taking into account additional delay from following components:

- Duration of UL WUS transmission to NES cell,

- Duration of RAR reception and message decoding,

- Duration of OD-SIB1 window.

However, no paging interruption tests exists in TS 38.133, neither for idle nor inactive mode. The only related test is the cell reselection test in idle mode. In fact, this test verifies the cell reselection delay which is highly correlated to the paging interruption time. Thus, we propose to modify this test for UEs supporting OD-SIB1 and specify a higher value for TSI_NR in all applicable cell reselection tests in idle mode.

Observation 2: No paging interruption test for idle or inactive mode exists in TS 38.133.

These test cases are specified in TS 38.133 Annex A.6.1.1 for FR1 and Annex A.7.1.1 for FR2 to verify the UE’s cell reselection behaviour. In the test cases, the cell reselection delay is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

TS 38.133: A.6.1.1.1.3 Test Requirements The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1, i.e. the 3 components depicted above. Hence the required time for cell reselection in test requirement needs to be extended. A modified test case is needed to verify the cell reselection behaviour to an OD-SIB1 cell.

Observation 3: If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1.

Proposal #7: RAN4 to specify a new test case with longer TSI-NR to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell, rather than specifying a test case for paging interruption.

SSB adaptation

Test case scope

The recommended test case list [2] from RAN4#115 is shown below.

Issue 3-3-1: Test case list - Recommended WF - Moderator suggests the group to check the initial test case list as follows. - L1-RSRP - Support: Nokia, LG, China Telecom, CMCC, Samsung, ZTE, Ericsson - Object: OPPO - L1-SINR - Support: Nokia, LG, China Telecom, CMCC, Samsung, ZTE, Ericsson - Object: OPPO - CBD - Support: Nokia, LG, China Telecom, CMCC, Samsung, ZTE, Ericsson - Object: OPPO - L3 measurement - Support: Nokia, LG, China Telecom, CMCC, Samsung, ZTE, Ericsson - Deactivated SCell measurement - Object: Nokia (if no new UE behaviour) - normal SCell activation - Support: Nokia, LG, China Telecom, CMCC, Samsung, ZTE, Ericsson, Huawei - direct SCell activation - Support: Nokia, Huawei - PUCCH SCell activation - Support: Huawei

We support all of the listed test cases (also PUCCH SCell activation, which was earlier agreed) except the TC for deactivated SCell measurement, which should reuse the legacy UE behaviour based on measCycleScell. In addition, we support the TC for Rel-18 SCell activation with L3 reporting.

Proposal #8: 	Test cases for SSB adaptation should be introduced for L1-RSRP, L1-SINR, CBD, L3 measurements, Normal SCell activation, direct SCell activation, PUCCH SCell activation and Rel-18 SCell activation with L3 reporting.

General testing principle

The proposals and the recommended general testing principle [2] from RAN4#115 are shown below.

Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection

We agree with Proposals 1, 3 and 4. Regarding proposal 3, we think there is some relevance of these test cases, as they are inherent to SSB periodicity adaptation, but we should limit the testing effort.

Proposal #9: 	For general testing principle, test both F1 and FR2, test only non-DRX case and test without SSB time index detection. For test cases related to L1/L3 transition requirements, the testing effort should be limited.

SMTC and SSB configurations

The proposals and the recommended WF [2] from RAN4#115 are shown below.

Issue 3-3-3: SMTC and SSB configuration - Proposals - Option 1: - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation. - Option 2: - SSB periodicity: 40 ms to 20 ms - SMTC periodicity: 40ms - Option 3: - SSB periodicity from 80ms to 20ms - SMTC periodicity from 80ms to 20ms

We agree with the intention of option 1. Option 2 in that sense is not suitable. Thereagainst option 3 is appropriate, since it better covers the SSB periodicity range up to 160ms.

Proposal #10: 	RAN4 to use SMTC periodicity change from 80ms to 20ms and SSB periodicity change from 80ms to 20ms, and vice versa, for test case design.

NCD-SSB configuration

The proposals and the recommended WF [3] from RAN4#115 are shown below.

Issue 3-3-4: NCD-SSB configuration - Proposals - Proposal 1(Ericsson): RAN4 to introduce new NCD-SSB configuration with 20ms and 80ms. - Recommended WF - Further discussion.

It is noted, that this issue is related to RRM core requirements rather than RRM performance requirements. The NCD-SSB configuration is specified in TS 38.331, an extract being depicted below.

#### – NonCellDefiningSSB The IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an RedCap-specific initial BWP that does not contain the CD-SSB or a dedicated BWP that does not contain the CD-SSB. NonCellDefiningSSB information element -- ASN1START -- TAG-NONCELLDEFININGSSB-START NonCellDefiningSSB-r17 ::= SEQUENCE { absoluteFrequencySSB-r17 ARFCN-ValueNR, ssb-Periodicity-r17 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S ssb-TimeOffset-r17 ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 } OPTIONAL, -- Need S ... } -- TAG-NONCELLDEFININGSSB-STOP -- ASN1STOP NonCellDefiningSSB field descriptions. absoluteFrequencySSB Frequency of the NCD-SSB. The network configures this field so that the SSB is within the bandwidth of the BWP configured in BWP-DownlinkCommon.. ssb-Periodicity The periodicity of this NCD-SSB. The network configures only periodicities that are larger than the periodicity of serving cell's CD-SSB. If the field is absent, the UE applies the SSB periodicity of the CD-SSB (ssb-periodicityServingCell configured in ServingCellConfigCommon or ServingCellConfigCommonSIB).. ssb-TimeOffset The time offset between CD-SSB of the serving cell and this NCD-SSB. Value ms5 means the first burst of NCD-SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value ms10 means the first burst of NCD-SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero. For UEs in TDD cells, the network configures this time offset to be an integer multiple of the periodicity of the serving cell's CD-SSB..

When NCD-SSB is used for SSB adaptation, then the NCD-SSB configuration has to be updated to include the combination of SSB periodicities which are used before and after SSB adaptation via DCI command 2_9. There are different options:

Option 1: 	Extend the Rel-17 IE by adding a Rel-19 IE with the parameter ssb-PeriodicityAdditional-r19 to define the additional SSB periodicity value.  If the ssb-Periodicity-r17 specifies the default (denser) value, then ssb-PeriodicityAdditional-r19 specifies the sparser value, e.g. ranging from 20 ms to 160 ms with some spare values. The ssb-TimeOffset parameter is then shared between both SSB periodicities (no different time offset allowed according RAN1 design). This option is flexible allowing to configure different SSB periodicities as part of the NCD-SSB IE and is future-proof in extending the range of the additional SSB periodicity. The extension of the NCD-SSB IE is shown below.

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

ssb-PeriodicityAdditional-r19   ENUMERATED { ms20, ms40, ms80, ms160, spare4, spare3, spare2, spare1 }   OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

Option 2: 	The spare bits (2) for ssb-Periodicity-r17 are used to accommodate two combinations of SSB periodicities, i.e. spare 1 := SSB periodicity combination with 20ms and 80ms and spare 2 := SSB periodicity combination with 20ms and 40ms. This only allows a limited set (=2) of SSB periodicity combinations and hence is not future proof.

Thus, we propose to proceed along option 1 and ask RAN2 to do these modifications.

Proposal #11: 	RAN4 to ask RAN2 to extend the NCD-SSB IE in TS 38.331 by adding the Rel-19 field for the additional SSB periodicity based on the proposal below.

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

ssb-PeriodicityAdditional-r19   ENUMERATED { ms20, ms40, ms80, ms160, spare4, spare3, spare2, spare1 }   OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

Detail design

The proposals for and the recommended WF [3] from RAN4#115 are shown below.

Issue 3-3-5: Detail design - Proposals - Proposal 1(Qualcomm, Samsung): - TSSB and TSMTC periodicity updates should be tested based on DCI 2-9. - Proposal 2(Samsung): - FFS Which configured event should be considered/how to trigger the measurement report - Recommended WF - Further discussion.

We agree with proposal 1 which is in line with the RAN1 design for SSB adaptation.

Regarding proposal 2, SSB adaptation is used at time of SCell (de-)activation and for (de-)activated SCell. Thus, the legacy trigger events for measurement report can be reused.

Proposal #12: 	RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

Proposal #13: 	RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

Conclusion

In this contribution, we discussed the RRM performance requirements for Rel-19 network energy saving for the features OD-SSB SCell operation, OD-SIB1 and SSB adaptation.

The following observations and proposals are made.

OD-SSB SCell operation

Observation 1: In Case 1, RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated hence in Scenario #3B OD-SSB shall be always activated.

Proposal #1: For Case 1, RAN4 should define the test cases considering scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command.

Proposal #2: For Case 2, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B regarding to when the OD-SSB is activated.

Proposal #3: For Case 1, RAN4 to define the test cases to verify the UE measurement behaviour based on when the OD-SSB is reactivated relevant to T2.

Proposal #4: For Case 2, RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if we would agree to define the RRM requirements for all these scenarios.

Proposal #5: RAN4 to reduce the number of test cases for SCell activation and define the TCs only for single SCell activation.

OD-SIB1

Proposal #6:  RAN4 to define a test case for verifying longer interruption in case of OD-SIB1 support in NES cell, taking into account additional delay from following components:

- Duration of UL WUS transmission to NES cell,

- Duration of RAR reception and message decoding,

- Duration of OD-SIB1 window.

Observation 2: No paging interruption test for idle or inactive mode exists in TS 38.133.

Observation 3: If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1.

Proposal #7: RAN4 to specify a new test case with longer TSI-NR to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell, rather than specifying a test case for paging interruption.

SSB adaptation

Proposal #8: 	Test cases for SSB adaptation should be introduced for L1-RSRP, L1-SINR, CBD, L3 measurements, Normal SCell activation, direct SCell activation, PUCCH SCell activation and Rel-18 SCell activation with L3 reporting.

Proposal #9: 	For general testing principle, test both F1 and FR2, test only non-DRX case and test without SSB time index detection. For test cases related to L1/L3 transition requirements, the testing effort should be limited.

Proposal #10: 	RAN4 to use SMTC periodicity change from 80ms to 20ms and SSB periodicity change from 80ms to 20ms, and vice versa, for test case design.

Proposal #11: 	RAN4 to ask RAN2 to extend the NCD-SSB IE in TS 38.331 by adding the Rel-19 field for the additional SSB periodicity based on the proposal below.

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

ssb-PeriodicityAdditional-r19   ENUMERATED { ms20, ms40, ms80, ms160, spare4, spare3, spare2, spare1 }   OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

Proposal #12: 	RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

Proposal #13: 	RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- References

[1] 	RP-251678, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, Apple, RAN#108

[2]	R4-2508277, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson, RAN4#115

[3]	R4-2505570, “Topic summary for [115][221] Netw_Energy_NR_enh_Part1”, Moderator (Ericsson), RAN4#115

[4] 	R4-2508441, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei, RAN4#115