3GPP TSG-RAN-WG4 Meeting #116 	R4-2509985

# Bangalore, India, 25th – 29th August, 2025

# Title:	4.9 GHz band Coexistence

# Source:	T-Mobile USA

# Agenda Item:	5.29.1

# Document for:	Approval

# Introduction

At RAN#108 a new Work Item on Introduction of NR TDD 4.9GHz Band for US Operation was approved [1]. The first objective is a one quarter study phase to identify and document coexistence requirements for the new band. This document provides some input for the coexistence study.

# Discussion

The 4.9 GHz Work Item contains the following objective:

A one quarter study phase to identify and document coexistence requirements for the new band:

- Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band.

    - Identify adjacent spectrum protection requirements

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

    - Identify other technologies operating in the 4940-4990 MHz band

      - Identify potential coexistence mechanism(s)

        - Frequency division sharing

        - Geographic separation

        - TDD synchronization

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

## 4.9 GHz Waiver requests

According to Zenji Nakazawa, the Chief of the FCC’s Public Safety and Homeland Security Bureau speaking at the APCO conference in July, “As a critical initial step in implementing this framework, the Bureau has completed its collection of granular technical data from 4.9 GHz incumbent licensees. To date, we received roughly 7,650 new 4.9 GHz apps, and staff has issued almost 2,700 new licenses. We are currently in the process of reviewing applications and waivers to ensure that incumbent public safety operations are protected from harmful interference.”

Observation 1: To date, The FCC has received roughly 7,650 new 4.9 GHz apps, and staff has issued almost 2,700 new licenses.

## NYC MTA waiver request

On January 10, 2025 the New York City Transit Authority on behalf of the Metropolitan Transit Authority (MTA) proposed to license fixed base stations and mobile units throughout numerous counties in New York on channels in the 4940-4990 MHz (4.9 GHz) band to facilitate deployment of a 5G communications system to replace outdated signaling technology used for train safety. As a result, the FCC issued a request for comments on the waiver request, with comments due Sept 8, 2025 [2]. If granted, this would formalize and potentially expand the geographic and operational footprint (multiple NY counties) of protected public-safety use. Potential impacts of this waiver are:

  Less white-space to use. Any additional NYCTA sites/areas approved by waiver become incumbent operations that FirstNet must protect. That reduces the time-frequency-location “gaps” available for a 5G layer. [3]

  Tighter coordination/engineering. FirstNet (via the eventual Band Manager) would need more detailed coordination—frequency blocks, EIRP limits, antenna patterns/tilts, guard zones and timing—to avoid harmful interference with CBTC, cameras, call boxes, etc. Used spectrum becomes effectively off-limits or power-constrained for FirstNet near MTA facilities and rights-of-way.

  Possible deployment delays/sequencing. Until the Band Manager is selected and a sharing agreement is executed, large-scale FirstNet 4.9 GHz in NYC is procedurally gated. A granted waiver in parallel could prioritize/lock in MTA uses first, forcing FirstNet to design around them.

  Data-driven protection zones. The FCC is requiring incumbents to submit granular site data (deadline originally set for June 9, 2025, later extended). As those records are populated—with any new NYCTA sites from the waiver—FirstNet’s permissible operating areas in NYC get algorithmically smaller/more carved-up.

Observation 2: The NYC MTA has requested a waiver which could impact FirstNet deployment of 5G.

## 4.9 GHz Incumbents

According to the FCC, as of 2023:

The 4.9 GHz band supports a wide variety of broadband applications. The following the types of uses have primary status in the 4.9 GHz band (however, all licensees will make every practical effort to protect radio astronomy operations identified in the Table of Frequency Allocations)

- Wireless LANs for incident scene management (ad hoc mobile networks)

- Mesh networks

- Wi-fi hotspots

- Voice over IP (VoIP)

- Temporary fixed communications

- Permanent fixed point-to-point/multipoint links that deliver broadband service

- Permanent fixed point-to-point video surveillance

- Permanent fixed point-to-point/multipoint backhaul of broadband traffic originating from 700 MHz public safety broadband networks

The following types of uses have secondary status to primary uses of the band:

- Permanent fixed point-to-point/multipoint links that deliver narrowband traffic

- Permanent fixed point-to-point/multipoint backhaul of traffic originating from public safety bands not designated for broadband (i.e. public safety VHF, UHF, narrowband 700 MHz and 800 MHz) to other networks

Observation 3: There are a variety of public safety applications currently operating in the 4.9 GHz band.

## Examples of products operating in 4.9 GHz

Example of radio products that operate in the 4.9 GHz band in the USA:

- Cambium PTP 450i (4.9–5.9 GHz) – fixed PTP/PTMP backhaul

Supports 4.9 GHz operation with 5/10/20 MHz channels (and higher in 5 GHz), ruggedized IP66/67, high TX power and integrated 23 dBi option—commonly used for video backhaul and site links.

- Cambium PTP 650 (4.9–6.05 GHz) – high-reliability PTP backhaul

Multi-band radio covering 4.9 GHz with user-selectable channel widths down to 5 MHz; widely deployed for municipal/public-safety camera and facility links.

- Ubiquiti airFiber AF-4X (licensed 4.9 GHz variant) – PTP backhaul

Purpose-built for the public-safety band; TDD, OFDM, up to ~687 Mbps real throughput, marketed explicitly for police/fire/EMS municipal backhaul.

- Rajant BreadCrumb (e.g., LX5, Hawk) – 4.9 GHz kinetic mesh nodes

Rajant nodes support 4.9 GHz among other bands; used for incident-scene mesh, mobile command, yards/depots, and transit operations where multi-hop Wi-Fi-derived mesh is needed.

These platforms are Wi-Fi/OFDM-derived (802.11-class PHYs with proprietary MAC and management), optimized for fixed backhaul, point-to-multipoint, or portable mesh rather than 3GPP mobility.

Observation 4: Given many of the systems operating in the 4.9 GHz band are IEEE 802.11, one question will be if LBT will be required to coexist with legacy technologies in the band.

## 4.9 GHz spectrum channelization

### Historical Operation – Narrow Channel Segments

Under the original Part 90 Subpart Y rules (pre-2021), the FCC’s band plan divided 4.9 GHz into ten 5 MHz channels (plus some 10/15/20 MHz combinations). Most Wi-Fi/OFDM-based radios were configured to one channel or an adjacent pair (e.g., 10 MHz or 20 MHz total) to minimize interference and match available hardware. Public safety agencies would often assign different channels to different applications — for example:

- Channel 1: video surveillance backhaul

- Channel 5: incident scene mesh

- Channel 9: point-to-point backhaul between stations

### Modern Rules – Wider Channel Options

In October 2021, the FCC relaxed the rules, allowing up to the full 50 MHz contiguous block to be used for wideband operations if needed, provided emission mask and power limits are met. This was partly to accommodate high-capacity backhaul or potential future LTE/5G overlays. The reality today is that most incumbent deployments are still 5–20 MHz wide because:

- Their radios (Wi-Fi derived) don’t support the full 50 MHz natively.

- Wider channels can cause more mutual interference in dense metro deployments.

- Agencies tend to segment the spectrum to serve multiple functions/users.

### Exceptions – Large-Channel Backhaul

Some Cambium PTP 650/700, Ubiquiti AF-4X, and licensed microwave systems in the band can run 30–40 MHz-wide channels (sometimes just shy of the full 50 MHz). These are typically exclusive, coordinated links (e.g., a transit control center to a yard) where no other users need the spectrum in that path.

### Coordination Implications

In shared urban environments like NYC, agencies coordinate to avoid full-band occupation unless the link is:

- Point-to-point and narrow beam

- Isolated from other users

This allows multiple incumbent systems (police, fire, EMS, transit) to co-exist in the same geography without harmful interference.

Bottom line: The vast majority of incumbent 4.9 GHz systems today use part of the band — typically 5 to 20 MHz channels — with only specialized, coordinated backhaul links ever approaching full-band use. Full 50 MHz occupation is rare in current public safety deployments.

Observation 5: The vast majority of incumbent 4.9 GHz systems today use part of the band — typically 5 to 20 MHz channels — with only specialized, coordinated backhaul links ever approaching full-band use. Full 50 MHz occupation is rare in current public safety deployments.

Observation 6: For TDD systems, 3GPP RAN4 assumes UL/DL alignment and synchronization for networks using adjacent spectrum. If 5G NR is going to operate in 4.9 GHz spectrum in channels that are adjacent to Wi-Fi based technologies, UL/DL alignment and synchronization will be challenging, if not impossible.

## Coexistence Parameters

In order to establish the coexistence criterion, RAN4 should agree on coexistence parameters. Here is a potential coexistence parameter table:

Proposal: RAN 4 should agree on coexistence parameters to be used to determine the requirements for 4.9 GHz NR to coexist with legacy incumbent technologies that will persist in the 4.9 GHz band.

# Conclusion

Observation 1: To date, The FCC has received roughly 7,650 new 4.9 GHz apps, and staff has issued almost 2,700 new licenses.

Observation 2: The NYC MTA has requested a waiver which could impact FirstNet deployment of 5G.

Observation 3: There are a variety of public safety applications currently operating in the 4.9 GHz band.

Observation 4: Given many of the systems operating in the 4.9 GHz band are IEEE 802.11, one question will be if LBT will be required to coexist with legacy technologies in the band.

Observation 5: The vast majority of incumbent 4.9 GHz systems today use part of the band — typically 5 to 20 MHz channels — with only specialized, coordinated backhaul links ever approaching full-band use. Full 50 MHz occupation is rare in current public safety deployments.

Observation 6: For TDD systems, 3GPP RAN4 assumes UL/DL alignment and synchronization for networks using adjacent spectrum. If 5G NR is going to operate in 4.9 GHz spectrum in channels that are adjacent to Wi-Fi based technologies, UL/DL alignment and synchronization will be challenging, if not impossible.

Proposal: RAN 4 should agree on coexistence parameters to be used to determine the requirements for 4.9 GHz NR to coexist with legacy incumbent technologies that will persist in the 4.9 GHz band.

# Reference

[1] RP-251887	“New WID on Introduction of NR TDD 4.9GHz Band for US Operation,”	Fujitsu Limited

[2] https://docs.fcc.gov/public/attachments/DA-25-651A1.pdf

[3] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y/section-90.1217

[4] https://responsecoalition.com/new-york-4-9-usage-case-study-mta/

[5] https://www.iafc.org/blogs/blog/iafc/2024/10/23/the-fcc-announces-new-rules-for-the-4.9-ghz-band

[6] https://www.fcc.gov/document/deadline-49-ghz-granular-data-submission-extended-30-days

[7] Remarks of Zenji Nakazawa, Chief, Public Safety and Homeland Security Bureau, APCO Conference

Baltimore, Maryland, July 29, 2025  https://docs.fcc.gov/public/attachments/DOC-413356A1.pdf

[8] https://www.fcc.gov/49-ghz-public-safety-spectrum