3GPP TSG-RAN WG4 Meeting #116 	R4-2510250

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Discussion on the AIoT test method

Agenda Item:	7.22.5

Document for:	Approval

- Introduction

In this contribution, we provide our views on the OTA test method of device.

- Discussion

- Combine results from polarization combination

In [1], the polarization issue was pointed out:

Agreement in RAN4#115:

- The CW signal shall be sent in two polarized sequentially, FFS on how to combine two polarization test results.

Since the device 1 can only backscatter the power based on the incident CW, the polarization mismatch could happen in two aspects: 1) mismatch between incident CW and device antenna 2) mismatch between backscatter D2R and measurement antenna, so there will be total 4 polarization combination in EIRP

| Polarization combination | EIRPθ | EIRPϕ |
| --- | --- | --- |
| CWθ | EIRPθθ,ϕ¦cwθθ,ϕ | EIRPϕθ,ϕ¦cwθθ,ϕ |
| CWϕ | EIRPθθ,ϕ¦cwϕθ,ϕ | EIRPϕθ,ϕ¦cwϕθ,ϕ |

Observation 1: For backscatter power of device, there are total 4 polarization combinations for measurement antenna and incident CW.

Since normally the device is equipped with dipole antenna which is linearly polarized, it is better to test all possible polarization combination above and combine them together, to avoid the unexpected large backscatter loss due to the polarization mismatch.

Proposal 1: The D2R backscatter power is combine from all polarization combinations:

EIRPθ,ϕ=EIRPθθ,ϕ|cwθθ,ϕ+EIRPϕθ,ϕ|cwθθ,ϕ+EIRPθθ,ϕ|cwϕθ,ϕ+EIRPϕθ,ϕ|cwϕθ,ϕ

The polarization combination in EIS is much easier, but how to combine them there are different ways in 3GPP. For FR1 TRS, the formula is listed below:

TRS=4πθ=0πϕ=02π1EISθθ,ϕ+1EISϕθ,ϕsinθdϕdθ

This means that for each test point, the EIS is combined as:

EISθ,ϕ=1(1EISθθ,ϕ+1EISϕθ,ϕ)

Meanwhile for FR2, the EIS is combined as average value:

EISθ,ϕ=2(1EISθθ,ϕ+1EISϕθ,ϕ)

The formula of FR2 had a long discussion in R15 and due to the reason from implementation perspective, e.g., lack of dedicated antenna connector for each polarization, beam selection could change under different polarized DL signal, the formula changes to use average value between two polarizations. For AIoT device, such implementation concern doesn’t exist, we can use normal FR1 way for combining EIS from different polarization.

Proposal 2: The EIS is combined from θ and ϕ polarization by following equation:

EISθ,ϕ=1(1EISθθ,ϕ+1EISϕθ,ϕ)

- Power difference

Agreement in RAN4#115:

- Performance metric: EIRP

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

- Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna.

  - FFS for BPSK

- The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

For backscatter loss in core part, the following agreement was achieved:

There are two information related the test:

- CW transmission and D2R measurement are in the same direction

- CW will be removed in D2R power measurement.

Agreement in RAN4#115:

- Power difference between received backscatter signal and the CW signal leakage is at least [15dB]

  - FFS on reference point of power difference

In the last meeting, we also had the following agreement for test:

The agreement above originally come from the concern of the CW power leakage will impact on the measurement accuracy of D2R power if the D2R power includes all power within D2R channel bandwidth. Since the D2R power does not include the CW now, it is not needed to further discuss such limitation and leave how to extract the D2R power to TE implementation, e.g., using circulator.

Proposal 3: No need to define the power difference between backscatter signal and CW leakage.

- Additional test point for sensitivity

During offline discussion, companies prefer to test the minimum EIS over a partial sphere centered on peak gain. Typically, the device equipped with dipole antenna, as shown in table below:

Figure I Example dipole antenna

Within a partial sphere centered on peak gain, the minimum antenna gain of dipole degradation always appears at edge of sphere and at the direction that perpendicular to the long side of the antenna. To simplify the test, we propose only test the point at the edge of partial sphere with 90° degree separation.

Figure I Illustration of additional test point for sensitivity

Proposal 4: For min EIS over partial sphere, only test the points with 90° separation at edge of partial sphere.

- CW configuration for Rx test

For Tx requirements of device, the CW configuration need to be discussed with core requirement, but for Rx requirements, the purpose of CW configuration is only to ensure the backscatter signal can be received correctly. As we analyzed in previous part, polarization combination of incident CW signal and measurement antenna has total 4 types, but it is unnecessary to iterate all combinations for Rx requirements, and the easiest way is that we use same CW configuration as backscatter loss, the only use the polarization combination that can get maximum D2R power.

Observation 2: The CW configuration in Rx requirement is only to ensure the D2R can be correctly received by TE.

Proposal 5: For Rx requirements verification, the CW configuration is same as backscatter loss with the polarization combination that can get maximum EIRP.

- Conclusion

Observation 1: For backscatter power of device, there are total 4 polarization combinations for measurement antenna and incident CW.

Proposal 1: The D2R backscatter power is combine from all polarization combinations:

EIRPθ,ϕ=EIRPθθ,ϕ|cwθθ,ϕ+EIRPϕθ,ϕ|cwθθ,ϕ+EIRPθθ,ϕ|cwϕθ,ϕ+EIRPϕθ,ϕ|cwϕθ,ϕ

Proposal 2: The EIS is combined from θ and ϕ polarization by following equation:

EISθ,ϕ=1(1EISθθ,ϕ+1EISϕθ,ϕ)

Proposal 3: No need to define the power difference between backscatter signal and CW leakage.

Proposal 4: For min EIS over partial sphere, only test the points with 90° separation at edge of partial sphere.

Observation 2: The CW configuration in Rx requirement is only to ensure the D2R can be correctly received by TE.

Proposal 5: For Rx requirements verification, the CW configuration is same as backscatter loss with the polarization combination that can get maximum EIRP.

- References

- R4-2505230	WF on A-IOT device requirements	CMCC

- R4-2506841_Discussion on the RF requirement of AIoT BS