3GPP TSG-RAN4 Meeting #116	R4-2510298

# Bangalore, India, 25 Aug - 29 Aug, 2025

Agenda item:	 7.24.2

Source:	 vivo

Title:	Discussions on LP-WUS UE RF feature list

Document for:	 Approval

- Introduction

RAN4 made good progress on LP-WUS last meeting, including target SNR, system parameters, ASCS requirements, ACS guard RB, Max input level requirement, and RMC. Given this is the last meeting to finalize all the core part work, there are still some open issues should be concluded.

- ASCS test parameters

- ACS requirements and test parameters

- REFSENS requirements

- Max input level test parameters

- Blockings – in-band, out of band, narrow band

- Spurious response

- Wide band intermodulation

- LP-WUS UE RF feature list

Among all the open issues, the UE RF capability is a general issue related how to specify the requirements which should be concluded first. In this contribution, we focus on whether to specify a capability for LP-WUS RF.

- LP-WUS UE RF Capability

In RAN1, the UE feature for LP-WUS is specified in [2]:

| Features | Index | Feature group | skip | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | skip | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | skip | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | skip | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | skip | Optional with capability signalling |
| 62. NR_LPWUS | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | skip | Optional with capability signalling |

OOK-based receiver and OFDM-based receiver are optional features, one or both can be supported by a UE. Besides, RAN2 assumes the thresholds defined for LP-WUS monitoring and RRM relaxation/offloading are based on OOK-based WUR or OFDM-based WUR (measuring SSB or measuring SSB&LP-SS). They are not define based on Type-1 or Type-2 in RAN4.

In addition, RAN4 specify type-1 and type-2 RF requirement differently based on the agreed two sets of NF+IM value, if the threshold and UE behavior is not directly linked to UE RF requirements type-1 and type-2, then there is no necessity to define a dedicate capability for UE RF in RAN4. Whether type-1 or type-2 RF requirement is applied, can be based on UE declaration, only one type should be supported by a UE.

Proposal 1: RAN4 do not specify a dedicate UE RF capability for type-1 or type-2 LP-WUS.

- Conclusion

In this contribution, we share our views on LP-WUR UE RF capability.

Proposal 1: RAN4 do not specify a dedicate UE RF capability for type-1 or type-2 LP-WUS.

# References

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, vivo, RAN4#115.

- R1-2504673, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121, AT&T, NTT DOCOMO