3GPP TSG RAN WG4 Meeting #116	R4-2510324

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.3.5

Source:	vivo

Title:	On RRM performance requirements for LB CA via switching

Document for:	Discussion

# Introduction

In the past RAN4 meetings, RAN4 discussed RRM core requirements for Rel-19 LB CA via switching. A semi-static switching pattern is configured to UE by RRC reconfiguration. UE will follow the switching pattern to switch between PCell and SDL SCell. RRM requirements for SCell activation, L3 measurements for activated/deactivated SCell, L1 measurements for SCell considering switching pattern will be specified. RRM requirements impact to PCell operation due to switching is under discussion.

In this contribution, we provide our views on RRM performance requirements for Rel-19 LB CA via switching.

# Discussion

## 2.1 SCell operation

In the RAN4#115 meeting, following agreements on necessary RRM requirements for SDL SCell operation was agreed as captured in the WF [1].

Table 1. Necessity of requirement on SDL CC

| Requirement | Whether or not RAN4 will have requirement | Note |
| --- | --- | --- |
| interruption requirement for CC switching | No (can be revisited if not aligned with RAN1 further agreement.) | Issue 1-2-1, Agreed on Tuesday online session |
| Activated SDL SCell L3 measurement | Yes | Issue 1-3-1, Agreed on Tuesday online session |
| Deactivated SDL SCell measurement [and corresponding interruption] | Yes | Issue 1-3-3, Agreed on Tuesday online session |
| SDL SCell activation/deactivation and corresponding interruption | Yes |  |
| [BFD/CBD] | Yes | Related with issue 1-3-10 |
| TCI state switch delay | Yes | Related with issue 1-3-11 |
| PL-RS switch delay | No | Related with issue 1-3-12 |
| L1-RSRP/SINR | Yes | Related with issue 1-3-13 |

In addition, which features related to SCell activation is supported or not supported for LB CA via switching were also agreed.

- The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA.

The purpose of the test is to verify UE can perform L3/L1 measurements on SDL SCell and activate/deactivate the SCell correctly when switching pattern is configured and corresponding requirements should be met.

For SCell activation, a test should be introduced to very FR1 inter-band SCell activation delay requirements where UE activates the SCell without following the configured switching pattern. Whether interruption requirements should also be tested can be FFS.

Proposal 1: Introduce a test to verify SCell activation delay requirements when switching pattern is configured. FFS test for interruption requirements for SCell activation.

For activated SDL SCell L3/L1 measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window/L1-RS (SSB or CSI-RS) and SDL SCell reception duration indicated by switching pattern. It is new UE measurement behaviour and requirements are different from legacy. Tests should be introduced to verify requirements for the L3/L1 measurements on SDL SCell.

Proposal 2: Introduce tests to verify L3/L1 measurements on activated SCell when switching pattern is configured.

Proposal 3: FFS which L3/L1 measurements to be tested after core part is completed.

Since the change on TCI state switch is the L1-RSRP measurement, there is no need to test TCI state switch delay requirements if L1-RSRP measurement requirements is tested.

Proposal 4: Introduce test to verify L1-RSRP measurements and no test for verifying TCI state switch delay requirements when switching pattern is configured.

For deactivated SCell measurements, legacy delay requirements are reused. There is no need to define new tests. However, the interruption on PCell could be different from legacy one. It is suggested to define a test to verify interruption requirements for deactivated SCell measurements.

Proposal 5: Introduce test to verify interruption requirements due to L3 measurements on deactivated SCell when switching pattern is configured.

For PCell operation under LB-CA via switching, it was agreed that legacy timing requirements are reused. RRM impacts on other measurements are under discussion in core part. Tests for potential new requirements for PCell operation can be FFS after core part is completed.

Proposal 6: FFS tests for PCell operation with LB-CA via switching after core part is completed.

# Summary

In this contribution, we provided views on RRM performance requirements for Rel-19 LB-CA via switching. Following proposals and observations are present.

Proposal 1: Introduce a test to verify SCell activation delay requirements when switching pattern is configured. FFS test for interruption requirements for SCell activation.

Proposal 2: Introduce tests to verify L3/L1 measurements on activated SCell when switching pattern is configured.

Proposal 3: FFS which L3/L1 measurements to be tested after core part is completed.

Proposal 4: Introduce test to verify L1-RSRP measurements and no test for verifying TCI state switch delay requirements when switching pattern is configured.

Proposal 5: Introduce test to verify interruption requirements due to L3 measurements on deactivated SCell when switching pattern is configured.

Proposal 6: FFS tests for PCell operation with LB-CA via switching after core part is completed.

- References

- R4-2508259	WF on RRM requirements for NR_LBCA_Sw, Apple