3GPP TSG RAN WG4 Meeting#116                                    R4-2510342

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.18.3

Source:	vivo

Title:	Discussion on testing issues for CSI compression two-sided models

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In Rel-19, the study on interoperability and testability for CSI compression continues in the WI [2].

In last meeting, we have the following agreements for CSI compression.

Simulation result observations Agreements: During the ad-hoc, it was agreed that the moderator should propose some general observation that the study has focussed on SGCS convergence, not absolute SGCS. The following text is provided: The focus in this study has been on convergence between SGCS reported for different company UE encoders when operating with test decoders, in order to understand whether it is feasible to specify a test decoder (or, for option 4, information used to create a test decoder) that can be used for consistent performance testing across multiple UE vendor implementations. Absolute SGCS has not been considered, since the assumed model structure is somewhat arbitrary and the purpose of the study has been to consider testing.. For option 3 track 1, the endorsed observations are as follows: - It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional - The performance of the encoder is very dependent on the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testset - If the encoder is trained using a dataset of the UE vendor, the UE vendor dataset should contain the [features] captured in the dataset used to develop the test decoder. - The variation in the performance depending on the training datasets and test datasets can be as much as 30-50% in SGCS. Further details are available in the results spreadsheet. - For option 3, track 1 to be useful, more careful attention would be needed to the alignment of datasets - Only one encoder structure was considered for option 3 track 1. Other lower complexity encoder structures were not examined.. For option 3 track 2, the endorsed observations are as follows: - There is reasonable SGCS convergence when companies jointly train an encoder-decoder pair using the mixed dataset and test using the mixed testset - Around +-2% variation in SGCS; see results spreadsheet for more details - It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional - When training “own” encoder with the frozen decoder, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results - Around - 2% to +1.4% variation in SGCS from the average - When training “own” encoder with the frozen decoder, with training of the frozen decoder using the mixed dataset, own encoder using own dataset and testing using the mixed dataset, there is convergence in average SGCS results from results presented so far - Around -+-4% variation in SGCS from the average (+-1.5% with outlier performance excluded) - When training “own” encoder with the frozen decoder, with training of the frozen decoder using the mixed dataset, own encoder using own dataset and testing using the own dataset, there is much less convergence in average SGCS results from results presented so far - Around -40% to 13% variation in SGCS from the average (+-8% to 5% with outlier performance excluded) - The results considering “own” datasets demonstrate that there is divergence in the datasets, and attention must be paid to alignment of the training datasets. - Note: Alignment of datasets does not refer to aligning UE algorithms such as channel estimation, CSI estimation etc.. For option 4, the endorsed observations are as follows: - For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used - For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b) - If the frozen encoder is created using the mixed dataset, TE/UE vendors create decoders using the mixed dataset, UE vendors create an encoder using the mixed dataset and testing uses the mixed testset then good convergence is seen in SGCS - Around +- 1.5% from average SGCS - UE encoders were shown to be interoperable with all companies own decoders - This represents the situation where there is a well defined dataset, and UE vendors use the same dataset to train their encoder models - Some companies observed that - if the frozen encoder is created using the mixed dataset, - TE/UE vendors create decoders using the mixed dataset, - UE vendors create an encoder using their own dataset and - testing uses the mixed testset then [good] convergence is seen in SGCS, but there is more variation than when mixed dataset is used for training the encoder - Around [+- 3%] from average SGCS - [Further SGCS results may be provided in the next meeting] - [UE encoders were shown to be interoperable with all companies own decoders] - This represents the situation where there is a well defined dataset that is used for all 3GPP responsibilities (Creating the frozen decoder, providing information for the TE vendor to train the decoder, providing test data), but the UE vendor uses their own dataset to create the encoder - Some companies observed that if the frozen encoder is created using the mixed dataset, - TE/UE vendors create decoders using the mixed dataset, - UE vendors create an encoder using their own dataset and - testing uses the own testset then [convergence is seen in SGCS,] but there is more variation than when mixed dataset is used for training the encoder. The variation is similar between using the mixed dataset and own dataset for testing. - Around [+- 1.6 to +4.7%] from average SGCS - [Further SGCS results may be provided in the next meeting] - [UE encoders were shown to be interoperable with all companies own decoders] - This represents the situation where there is a well defined dataset that is used for all 3GPP responsibilities (Creating the frozen decoder, providing information for the TE vendor to train the decoder, providing test data), but the UE vendor uses their own simulation to create the encoder - Two lower complexity encoders were considered and tested based on the frozen decoder. - A transformer with 3.63 MFlops - A CNN encoder with 4.14 MFlops - When the lower complexity encoders were examined using the mixed dataset for all training and the mixed test set for testing, SGCS convergence (- 1 to 3.5%) and interoperability with all companies “own” decoders was observed for the considered structures.. Further simulations (not agreement): In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP What should be captured or specified for each of the options Agreements: The following agreements were reached for option 3. For clarity, the agreement is added alongside the agreement from RAN4#114bis. Agreements from RAN4#114bis - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) New agreement from RAN4#115: The training dataset used to train the test decoder in 3GPP should be captured - FFS whether channel, Eigenvectors or something else. The following agreements were reached for option 4a. For clarity, the agreement is added alongside the agreement from RAN4#114bis. Agreements from RAN4#114bis - Dataset is specified for option 4a. - Assume reference decoder is captured. New agreements from RAN4#115: - Reference encoder should be captured - FFS whether the labelled dataset is based on Eigenvector, channel estimation or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. Interoperability in the case that the structure of the TE decoder is not specified has not been investigated. The following agreements were reached for option 4b. For clarity, the agreement is added alongside the agreement from RAN4#114bis. Agreements from RAN4#114bis - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. New agreements from RAN4#115: - Specify dataset to train frozen encoder and decoder - TE vendor shall use the dataset to train the test decoder - This is the same dataset as used to train the frozen encoder - FFS whether the datasets consist of Eigenvector, channel or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. Interoperability in the case that the structure of the TE decoder is not specified has not been investigated. Other agreements Agreements: If RAN1 direction C agreed, RAN4 and RAN1 should co-ordinate on model structure etc. for direction C due to the similarities between option 3/4 and direction C - Duplicated work is to be avoided At the current moment in time, neither option 3 track 2, nor option 4a or 4b should be ruled out for a WI. Option 3 track 1 can be ruled out. Option 3 and 4 are similar: - An encoder, decoder and training dataset are always captured - The difference is that for option 3, the TE vendor is mandated to use the standardized test decoder. In option 4 they could choose to use the reference test decoder or to create their own based on the standardized information.

In this contribution, we further provide our views on testing issues for CSI compression two-sided models.

- Discussion

  - Further simulation results for Option 3 track 2

In this section, further simulation results of Option 3 track 2 will be provided. In previous meetings, there are sufficient simulation results of Option 3 and Option 4. Only some further simulations for low complexity encoder are suggested in this meeting. Detailed simulation assumptions are suggested in [3].

Further simulations (not agreement): In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP

According to the suggestion, we provide the further simulation results of Option 3 for track 2. For SGCS-4, we have tried different combinations of (n1, n2), and found that n1=0.9 and n2=0.1 can provide best performance. In the below table, the results of SGCS-1, SGCS-2, SGCS-3 and SGCS-4 for Encoder-1 (CNN), Encoder-2 (Transformer), Encoder-3 (CNN) and Encoder-4 (MLP) are provided. Among the 4 encoders, it is seen that Encoder-2 (Transformer) can achieve best performance, while with low complexity. Encoder-1 (CNN) and Encoder-3 (CNN) have similar performance while Encoder-3 is much simpler than Encoder-1. Encoder-4 (MLP) is very simple and has worst performance, especially for the case of new encoder training with (Samsung’s) frozen decoder (SGCS-2). It looks likes that the Samsung’s frozen decoder trained with CNN does not work well with this simple MLP. Considering SGCS-4, it shows that one decoder can work with different encoder structures, when the training of decoder considers these different encoder structures.

Table 2.1-1. Further SGCS results of low complexity encoders in Option 3 for track 2, where n1= 0.9 and n2 = 0.1 in SGCS-4.

| Encoder structure | Company | (1-to-1) Joint training (no fixed decoder) | New encoder training with (Samsung’s) frozen decoder | (N-to-1) Joint training SGCS-4 Step 1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| --- | --- | --- | --- | --- | --- |
| Encoder-1 (CNN) | vivo | 0.696 | 0.689 | 0.697 | 0.697 |
| Encoder-2 (Transformer) | vivo | 0.709 | 0.688 | 0.701 | 0.702 |
| Encoder-3 (CNN) | vivo | 0.691 | 0.694 | 0.692 | 0.693 |
| Encoder-4 (MLP) | vivo | 0.671 | 0.634 | 0.674 | 0.674 |

Thus, we have the following observations.

Observation 1: Further SGCS results of low complexity encoders in Option 3 for track 2 is shown in Table 2.1-1.

Observation 2: Encoder 2 (Transformer) can achieve best performance, while with low complexity.

Observation 3: SGCS-4 (2-step joint training) shows that one decoder can work with different encoder structures, when the training of decoder considers these different encoder structures.

  - Considerations on Option 3 and Option 4 of test decoder

In this subsection, details of Option 3 and Option 4 of test decoder will be discussed. In previous meetings, what to be specified have achieved some agreements, which will be shown in the following.

The following agreements were reached for option 3.

Agreements from RAN4#114bis - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) New agreement from RAN4#115: The training dataset used to train the test decoder in 3GPP should be captured - FFS whether channel, Eigenvectors or something else

The following agreements were reached for option 4a.

Agreements from RAN4#114bis - Dataset is specified for option 4a. - Assume reference decoder is captured. New agreements from RAN4#115: - Reference encoder should be captured - FFS whether the labelled dataset is based on Eigenvector, channel estimation or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

The following agreements were reached for option 4b.

Agreements from RAN4#114bis - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. New agreements from RAN4#115: - Specify dataset to train frozen encoder and decoder - TE vendor shall use the dataset to train the test decoder - This is the same dataset as used to train the frozen encoder - FFS whether the datasets consist of Eigenvector, channel or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

The main remaining issue would be the definition/specification of reference encoder for Option 3/4a. The specification of reference encoder is needed, due to following aspects.

- Reference encoder is needed for inter-vendor training collaboration. In Rel-19, RAN1 and RAN4 both work on reference model. RAN4 reference model is designed to derive test decoder, and achieve no worse performance than eType II codebook at least on pre-defined test condition. RAN1 reference model is used to generate encoder and decoder in field, and achieve acceptable performance gain than eType II in field. At least reference encoder is considered in RAN1 now. Then, reference encoder is needed to facilitate inter-vendor training collaboration, including fully defined/specified reference model (“Direction C”) and specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing). The following is the reference/test model related aspects for R20 WID.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Per Rel-20, at least specification of standardized encoder model structure plus parameter exchange should be done, which can leverage reference model or defined model of ‘direction C’. Since RAN4 is the lead working group for this objective, it is highly expected to also leverage RAN4 outcome of reference encoder.

- Reference encoder is needed for TE test decoder verification. For both Option 3 and Option 4, since TE vendors will need to implement the test decoder based on the encoder, decoder or dataset in the spec, a verification procedure would be needed to verify that the decoder is correctly implemented and has the right level of performance. A verification encoder will be needed for the verification vendor to verify TE test decoder. TE may also need a verification encoder to test the performance of its test decoder. The simplest and most efficient way is to specify the reference encoder, which is used as verification encoder. The vendor, who is responsible for the TE test decoder verification, could just use the specified reference encoder to test the TE test decoder.

  - No extra training procedure to train the verification encoder will be needed. If there is no specified reference encoder, the verification vendor will need considerable training time of hours or days and considerable computation resources, to train the verification encoder.

  - No performance degradation from imperfectly matched verification encoder will be caused using different training method and/or train dataset. Currently, there is no aligned training method. RAN4 has aligned training data generation method, but does not have aligned dataset. If there is no specified reference encoder, the verification vendor will need to train the verification encoder using its own training method and/or train dataset. Then, different verification vendor would have different verification encoder, which may imperfectly match with TE test decoder. When the inference performance of test decoder and verification encoder is not so good, it may not be the problem caused by TE vendor.

- Reference encoder is needed for better field performance. Also, as discussed in Subsection 2.4, using reference encoder could achieve better field performance, since only finetuning decoder is better than only finetuning encoder.

Based on above aspects, especially considering Rel-20 WID objectives, we have the following proposal for reference encoder.

Proposal 1: Reference encoder is to be specified in TS for Option 3/4a from Rel-19 SI perspective.

Regarding how the reference encoder is developed and its relationship with performance requirements and/or test decoders, it can be further discussed in Rel-20 WI. It is also under discussion how the reference encoder is specified/defined and in which TS. All these parts can be further discussed. From Rel-19 SI outcome perspective, it would be enough to conclude that reference encoder will be specified by RAN4 by taking Rel-20 WID objectives into account.

Proposal 2: Reference encoder will be developed and specified by RAN4.

  - Relation between RAN4 test decoder Option 3/4a/4b and RAN1 inter-vendor collaboration direction C

After rounds of discussion, RAN1 has achieved significant progress on model structure standardization. As seen in Appendix C, RAN1 agreed the full-pictured model structure for studying the standardized model structure, and some options on the scalable model structure specification. RAN4 also has good progress on reference model standardization.

In last meeting, the following agreement in RAN4 is achieved.

Agreements: If RAN1 direction C agreed, RAN4 and RAN1 should co-ordinate on model structure etc. for direction C due to the similarities between option 3/4 and direction C - Duplicated work is to be avoided

Furthermore, the following is the reference/test model related aspects for R20 WID. RAN4 will work on test/reference models, and RAN1 will use these models for inter-vendor training collaboration for two-sided AI/ML models.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Model specification for R20 WID

In Direction C in inter-vendor training collaboration of RAN1, the reference model is fully defined/specified. Note that, reference model is not the actual implemented model, but could be used for NW/UE to develop real decoder/encoder. For Interoperability and RRM requirement of RAN4, model specification at least includes the fully specified test decoder.

Observation 4: Model specification for R20 WID includes:

- Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C.

  - Reference model is not the actual implemented model, but could be used for NW/UE to develop real decoder/encoder.

- Fully specified test decoder.

RAN4 job for check-point

For inter-vendor training collaboration Direction C and Direction A, sub-option 3a-1, there is check-point in RAN#110. There are only 2 or 3 meetings left before the check-point. RAN4 does not need to rush to agree detailed model structure and scalability solutions. The main concern is that whether RAN4 can work on R19 RAN1/4 agreements and fulfill the requirements from WID on reference model/structure, and then whether the reference model from RAN4 can be used in field for RAN1. From these perspectives, RAN4 is suggested to agree on steps to align the model structure and scalability, based on R19 RAN1/4 agreements, to show that RAN4 can fulfill the requirements from WID on reference model/structure.

Proposal 3: RAN4 job for check-point of inter-vendor training collaboration Direction C and Direction A, sub-option 3a-1:

- Agree on steps to align the model structure and scalability, based on R19 RAN1/4 agreements, to show that RAN4 can fulfill the requirements from WID on reference model/structure.

Backbones of reference model

As seen in Appendix C, RAN1 agreed Transformer as the backbone for reference model during the study. All RAN1 scalability results, observation and conclusions are based on Transformer. The observations agreed in RAN1 shows that the scalability issue can be solved by the listed solutions.

In current RAN4 study on test decoder, CNN, Transformer and MLP are evaluated. As seen in below table, for either encoder or decoder, Transformer has better performance compared to CNN and MLP, with similar complexity.

Table 2.3-1. SGCS results for different encoder structure with 1 to 1 joint training.

| Different encoder backbones | SGCS | Flops |
| --- | --- | --- |
| Encoder-1 (CNN) | 0.696 | 47M |
| Encoder-2 (Transformer) | 0.709 | 3.63M |
| Encoder-3 (CNN) | 0.689 | 4.14M |
| Encoder-4 (MLP) | 0.671 | 0.22M |

Table 2.3-2. SGCS results for different decoder structure with 1 to 1 joint training.

| Different decoder backbones | SGCS | Flops |
| --- | --- | --- |
| Aligned CNN decoder | 0.709 | 740M |
| Proposed transformer decoder: dmodel = 192, Nhead = 8, dhead = 24, NTF = 4 | 0.729 | 31.3M |

Thus, it is suggested that at least Transformer is considered as the backbone for reference encoder for 3a-1 and direction C. More backbones can be considered.

Proposal 4: At least Transformer is considered as the backbone for reference encoder for 3a-1 and direction C. More backbones can be considered.

Model structure scalability

As seen in Appendix C, the following alternatives for scalability were agreed in RAN1 during the study, where the number of sources and performance loss of common structure compared to dedicated structure are added in each alternative. RAN1 confirms the scalability of model structure and parameters for direction A 3a-1 and direction C.

- For the choice of token dimension and feature dimension,

  - Alt 1: Use subband as the token dimension and Tx port as a feature dimension (8 sources, median -1.10%).

  - Alt 2: Use Tx port as the token dimension and subband as a feature dimension.

  - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. (1 source, median -0.3%).

- For scalability over the feature dimension,

  - Alt1: specific embedding layer for each feature size (4 sources, median -2.08%).

  - Alt2: a common embedding layer with padding (7 sources, median -0.85%).

- For scalability over the token dimension,

  - Alt1: positional embedding specific to each token index (2 sources, median -1.5%).

  - Alt2: Padding at the input (3 sources, median -0.75%).

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration (4 sources, median -2.08%).

  - Alt2: truncation/masking of the output linear layer output (3 sources, median -0.85%).

  - Alt3: by varying quantization parameters.

Below is the illustration for some potential scalability solutions.

Figure 2.3-1. Illustration for some potential scalability solutions.

The observations agreed in RAN1 shows that the scalability issue can be solved by the listed solutions. RAN4 does not need repeat RAN1’s work again in R20. Thus, for scalability down-selection, RAN4 to directly consider RAN1 scalability observations.

Below is one suggested scalability solution, which is straight forward and future-proof. Use subband as the token dimension and Tx port as a feature dimension. A common embedding layer with padding is used for scalability over the feature dimension. Padding before quantization is used for scalability over the token dimension. Truncation/masking of the output linear layer output is used for scalability over the payload configurations. Below table is the simulation results for 16/32 Tx, 9/13 subbands and PC1/3/8, which shows that this proposed scalability solution can provide good performance for different configurations of antenna ports, subbands and payload configurations.

Figure 2.3-2. Illustration for the proposed scalability solution.

Table 2.3-3. Simulation results for proposed scalability solution, for scalability over 16/32 Tx, 9/13 subbands and PC1/3/8.

| Dedicated structure with dedicated parameters | Common structure with dedicated parameters | Common structure with common parameters |
| --- | --- | --- |
| Baseline | Up to 0.1% SGCS loss | Average 0.8~1.0% SGCS loss |

Thus, we have the following proposal.

Proposal 5: For model structure scalability,

- RAN1 confirms the scalability of model structure and parameters for direction A 3a-1 and direction C.

- RAN4 to consider RAN1 scalability observations, and these simulations would not need to be repeated in RAN4.

- RAN4 to pick the most straight forward and future-proof one among the alternatives.

Steps to align the model structure and scalability

During Rel-19 study, RAN4 concludes the feasibility of all testing model options. Similar methodology of RAN4 R19 study for test model options, can be carried on to Rel-20 WI to align on the reference model structure and scalability.

- Step 0: Evaluation assumptions and mixed dataset based on R19 can be reused. In R19, RAN4 reuses the RAN1 simulation assumptions. For the requirement definition, extra dataset may be needed for scalability solutions.

- Step 1: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 1-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

  - Step 1-2: Check on performance alignment -> see simulation results from contributing companies.

  - Step 1-3: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed. If companies can achieve good performance alignment in above step, this step may be not needed.

  - Step 1-4: Select one or more model structures, which includes both model backbone and model hyperparameters.

  - Step 1-5: Select one or more scalability solutions. This step can be done in parallel with Step 1-4 (selection of model structures).

- Step 2: Specify the model for the following work in WID. Where and how to specify may need further discussion.

  - Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C, and

  - Fully specified test decoder.

Thus, we have the following proposal.

Proposal 6: Similar methodology of RAN4 R19 study for test model options, can be carried on to Rel-20 WI to align on the reference model structure and scalability.

- Step 0: Evaluation assumptions and mixed dataset based on Rel-19 can be reused.

- Step 1: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 1-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

  - Step 1-2: Check on performance alignment -> see simulation results from contributing companies.

  - Step 1-3: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed.

  - Step 1-4: Select one or more model structures.

  - Step 1-5: Select one or more scalability solutions. This step can be done in parallel with Step 1-4.

- Step 2: Specify the model for the following work in WID. Where and how to specify may need further discussion.

  - Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C, and

  - Fully specified test decoder.

Datasets for training and testing

In RAN4 test, the channel model is LLS, which is TDL or CDL. However, SLS is used in RAN1 and RAN4 evaluations in R19 for reference/test model. Also, as shown in Sub-section 2.4, UMa trained model can achieve good performance in field, while CDL trained model works bad in field.

Below table presents the baseline link level simulation assumptions of CDL channel for reference model derivation, where some unsuitable information is removed.

Table 2.3-4. Baseline link level simulation assumptions of CDL channel for reference model derivation.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD OFDM |
| Carrier frequency | 2GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C |
| UE speed | 3kmhr |
| Delay spread | 30ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |

Based on our proposed LLS assumptions for CDL-C channel model and CNN model structures proposed in our contributions [4] in the previous meeting, below table shows the SGCS results for different model structures under the different tests and training datasets. Note that the UMa dataset is generated based on the SLS assumption from RAN1 and related parameters are same as our proposed LLS assumptions for CDL-C. The mixed dataset contains the partial CDL-C dataset and UMa dataset, which are mixed together in one dataset randomly for model training.

Table 2.3-5. SGCS under different channel model for the proposed CNN and Transformer.

| Model structure |  | CNN |  | Transformer |  |
| --- | --- | --- | --- | --- | --- |
| CSI feedback bits | Training dataset | Test dataset |  | Test dataset |  |
|  |  | CDL-C | UMa | CDL-C | UMa |
| 116 bits | CDL-C | 0.994 | 0.336 | 0.996 | 0.253 |
|  | UMa | 0.791 | 0.787 | 0.831 | 0.821 |
|  | Mixed | 0.982 | 0.773 | 0.988 | 0.801 |
| CNN encoder: FLOPS 126M, size 0.1M. CNN decoder: FLOPS 1974M, size 2.4M. Transformer encoder: FLOPS139M, size 5.6M. Transformer decoder: FLOPS 139M, size 5.6M Note: Mixed training dataset includes CDL-C and UMa channel model. |  |  |  |  |  |

It can be observed that, the models trained under CDL-C dataset could provide a very high SGCS results under the CDL-C test datasets, while the performance degrades severely under the UMa test datasets. However, the models trained under UMa could provide a medium good performance of SGCS for UMa test dataset, and provide a higher SGCS under the CDL-C test dataset compared with the model training under CDL-C but tested in UMa. For the model trained under mixed dataset, it could provide good SGCS results under CDL-C and UMa test datasets and only degrade a little compared with the results of <CDL-C trained, CDL-C tested> and <UMa trained, UMa tested>. Thus, the best option is to use the mixed dataset for training and one of types of the dataset for testing. Since the TDL channel is a simpler channel compare with CDL and UMa, it can also be used for mixing and RAN4 test case. Similar to the previous test case, RAN4 tests could only use TDL channel. Based on the following analysis, we could have the following proposal.

Proposal 7: Using the mixed dataset for reference model training, including the mixing of SLS and LLS, while using the LLS dataset for RAN4 tests. Other mixing rules are not precluded.

  - Initial field test result of reference model for CSI compression

Initial field test of reference model for CSI compression is conducted as following. The data is collected from actual 5G network and the whole collecting area including Cell 1 and Cell 2 is about 250m * 150m. The detailed parameters are provided in the below table.

Table 2.4-1. Parameters of field test of CSI compression.

| Parameters | Value |
| --- | --- |
| Scenario | Actual 5G network |
| Carrier frequency | 3.45GHz |
| Subcarrier spacing | 30KHz |
| Frequency resources | 52 RBs, 13 subbands for 4RBs per subband |
| Rank | Max rank 1 |
| gNB antenna | 8 antenna ports |
| UE antenna | 4 antenna ports |
| CSI payload | 55 bits payload |

The performance of different physical cells is analysed in the following. We have tested the coverage of different cells in the industrial park, according to the measured RSRP, RSRQ and SINR of mobile phones. The channel samples are collected by our prototype. The coverage areas of two typical cells in the industrial park are shown in the below figure. Note that in the coverage areas of each cell, the signal of target cell is acceptable in most places but would be not always the strongest among all cells. This means that the map of actual cell section would be different from the following figure.

Figure 2.4-1. The map of data collecting cells.

For Cell 1, we have collected about 144k outdoor samples. For Cell 2, we have collected about 65k outdoor samples. We choose 10% of them as validation samples.

We use the almost the same simulation assumptions of UMa and CDL in Subsections 2.1 and 2.2, except changing gNB 32Tx to 8Tx. Also, it is important to point out that there would be plenty of parameter mismatches between simulation data and field data.

The CNN encoder and the Transformer encoder trained by UMa and CDL are tested on Cell 1 and Cell 2 in field. Transformer decoder is used for both CNN encoder and Transformer encoder. The SGCS results are provided in the below table. It is seen that compared with eType II codebook, UMa transformer has 2.2% SGCS loss in Cell 1, but 2.7% SGCS gain in Cell 2. However, considerable SGCS loss is obtained by using AI/ML model trained by CDL. For UMa training data, Transformer would provide about 4.2%~5.1% SGCS gain compared with CNN. Note that the CNN/Transformer encoder structures used in this section are referred to the proposed model structures in our contributions [4] in the last meeting.

Table 2.4-2. The SGCS results of multiple AI/ML models trained by simulation data and tested on field data.

| 55 feedback bits | SGCS on Cell 1 | SGCS on Cell 2 |
| --- | --- | --- |
| eTypeII | 0.677 | 0.839 |
| Field Transformer encoder (Cell 1 + Cell 2) | 0.779 | 0.915 |
| Field CNN encoder (Cell 1 + Cell 2) | 0.765 | 0.910 |
| UMa CNN encoder | 0.630 | 0.827 |
| UMa Transformer encoder | 0.662 | 0.862 |
| CDL CNN encoder | 0.202 | 0.512 |
| CDL Transformer encoder | 0.309 | 0.557 |

Observation 5: From initial results for field test, the generalization performance of AI/ML model trained by UMa simulation data on field data seems acceptable, which has similar performance as eType II codebook. The generalization performance of AI/ML model trained by CDL simulation data on field data is worse than AI/ML model trained by UMa simulation data.

Based on the results of above table, we further study the case that NW-side and/or UE-side may separately train new decoder and/or encoder compatible to the reference AI/ML model.

The simulation results of different simulation cases are provided in the below table. Since transformer has better performance than CNN, transformer is used in the below table. Note that different from the above table, the results of AI/ML model trained by field data (Case 1) are obtained by using the field data of current cell, that is, the results of cell 1 is obtained by using Cell 1 data.

Case 2A-1 and Case 2A-2 are the cases that only one side finetunes their part based on reference encoder and decoder pair. Here the reference encoder and decoder pair is obtained by training with UMa data, which is generated by RAN4 aligned simulation assumptions. In Case 2A-1, the encoder is finetuned from reference encoder using field data of current cell, while the decoder is fixed to be reference decoder. In Case 2A-2, the decoder is finetuned from reference decoder using field data of current cell, while the encoder is fixed to be reference encoder. Case 2A-3 and Case 2A-4 is the case that both two sides finetune their part based on reference encoder and decoder pair, separately. Since UE and NW would not use the same training data in practical, mismatch on training data between UE and NW is considered in Case 2A-3 and Case 2A-4. In both Case 2A-3 and Case 2A-4, decoder is finetuning from reference decoder using field data of current cell. In Case 2A-3, small mismatch is considered, where encoder is finetuned from reference encoder using field data of Cell 1 and Cell 2. In Case 2A-4, large mismatch is considered, where encoder is finetuned from reference encoder using field data of the other cell.

Compared to directly use reference model (Case 2), it is seen that only finetuning encoder (Case 2A-1) has up to 8.6% SGCS gain and only finetuning decoder (Case 2A-2) has up to 11.0% SGCS gain. Compared to fully trained by field data (Case1), it is seen that Case 2A-1 has up to 9.7% SGCS loss and Case 2A-2 has up to 7.7% SGCS loss.

Compared to Case 2A-2, considering mismatch on training data between UE and NW, encoder and decoder separate training (Case 2A-3, Case 2A-4) has performance loss, and sometimes the performance is terrible (Case 2A-4 on Cell 1). Then, it seems that it is benefit for NW to finetune the decoder from reference decoder using field data, while UE just implements the reference encoder without finetuning.

Table 2.4-3. The SGCS results of AI/ML models trained by simulation data, and then finetuned on field data.

| Index | Simulation case | Performance on Cell 1 |  | Performance on Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SGCS | Gain to eType II | SGCS | Gain to eType II |
| Case 0 | eType II | 0.677 | 0.0% | 0.839 | 0.0% |
| Case 1 | upper bound | 0.796 | 17.6% | 0.929 | 10.7% |
| Case 2 | Reference model (Trained by UMa) | 0.662 | -2.2% | 0.862 | 2.7% |
| Case 2A-1 | Encoder trained by field data, while using reference decoder | 0.719 | 6.2% | 0.866 | 3.2% |
| Case 2A-2 | Decoder trained by field data, while using reference encoder | 0.735 | 8.6% | 0.886 | 5.6% |
| Case 2A-3 | Encoder and decoder separate training considering small mismatch on training data between UE and NW | 0.734 | 8.4% | 0.885 | 5.5% |
| Case 2A-4 | Encoder and decoder separate training considering large mismatch on training data between UE and NW | 0.439 | -35.2% | 0.850 | 1.3% |

Observation 6: From initial results of field test, it is observed that

- Directly use reference model in field (Case 2): similar performance on field data as eType II for one cell and performance loss compared to eType II is observed for another cell.

- Finetuned encoder or decoder using field data against reference decoder/encoder (Case 2A-1, Case 2A-2): performance improved compared to directly use reference model (Case 2), but still has performance loss compared to fully trained by field data (Case1). Finetuned decoder (Case 2A-2) is better than finetuned encoder (Case 2A-1).

- Pairing of finetuned encoder and finetuned decoder (Case 2A-3, Case 2A-4): performance loss in some cases considering mismatch on training data between UE and NW.

Then from the comparison between Case 2A-1, Case 2A-2, Case 2A-3 and Case 2A-4, it is seen that only finetuning decoder is better than only finetuning encoder. Encoder and decoder separate training may be the worst or better than only finetuning encoder, depending of the level of training data mismatch. Thus, reference encoder seems necessary in reference model based inter-vendor training collaboration, and then the reference model is at least the encoder.

Observation 7: In field performance, the reference encoder performs better than reference decoder.

- Summary

Based on above analysis, following proposals and observations are present.

Observation 1: Further SGCS results of low complexity encoders in Option 3 for track 2 is shown in Table 2.1-1.

Table 2.1-1. Further SGCS results of low complexity encoders in Option 3 for track 2, where n1= 0.9 and n2 = 0.1 in SGCS-4.

| Encoder structure | Company | (1-to-1) Joint training (no fixed decoder) | New encoder training with (Samsung’s) frozen decoder | (N-to-1) Joint training SGCS-4 Step 1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| --- | --- | --- | --- | --- | --- |
| Encoder-1 (CNN) | vivo | 0.696 | 0.689 | 0.697 | 0.697 |
| Encoder-2 (Transformer) | vivo | 0.709 | 0.688 | 0.701 | 0.702 |
| Encoder-3 (CNN) | vivo | 0.691 | 0.694 | 0.692 | 0.693 |
| Encoder-4 (MLP) | vivo | 0.671 | 0.634 | 0.674 | 0.674 |

Observation 2: Encoder 2 (Transformer) can achieve best performance, while with low complexity.

Observation 3: SGCS-4 (2-step joint training) shows that one decoder can work with different encoder structures, when the training of decoder considers these different encoder structures.

Proposal 1: Reference encoder is to be specified in TS for Option 3/4a from Rel-19 SI perspective.

Proposal 2: Reference encoder will be developed and specified by RAN4.

Observation 4: Model specification for R20 WID includes:

- Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C.

  - Reference model is not the actual implemented model, but could be used for NW/UE to develop real decoder/encoder.

- Fully specified test decoder.

Proposal 3: RAN4 job for check-point of inter-vendor training collaboration Direction C and Direction A, sub-option 3a-1:

- Agree on steps to align the model structure and scalability, based on R19 RAN1/4 agreements, to show that RAN4 can fulfill the requirements from WID on reference model/structure.

Proposal 4: At least Transformer is considered as the backbone for reference encoder for 3a-1 and direction C. More backbones can be considered.

Proposal 5: For model structure scalability,

- RAN1 confirms the scalability of model structure and parameters for direction A 3a-1 and direction C.

- RAN4 to consider RAN1 scalability observations, and these simulations would not need to be repeated in RAN4.

- RAN4 to pick the most straight forward and future-proof one among the alternatives.

Proposal 6: Similar methodology of RAN4 R19 study for test model options, can be carried on to Rel-20 WI to align on the reference model structure and scalability.

- Step 0: Evaluation assumptions and mixed dataset based on Rel-19 can be reused.

- Step 1: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 1-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

  - Step 1-2: Check on performance alignment -> see simulation results from contributing companies.

  - Step 1-3: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed.

  - Step 1-4: Select one or more model structures.

  - Step 1-5: Select one or more scalability solutions. This step can be done in parallel with Step 1-4.

- Step 2: Specify the model for the following work in WID. Where and how to specify may need further discussion.

  - Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C, and

  - Fully specified test decoder.

Proposal 7: Using the mixed dataset for reference model training, including the mixing of SLS and LLS, while using the LLS dataset for RAN4 tests. Other mixing rules are not precluded.

Observation 5: From initial results for field test, the generalization performance of AI/ML model trained by UMa simulation data on field data seems acceptable, which has similar performance as eType II codebook. The generalization performance of AI/ML model trained by CDL simulation data on field data is worse than AI/ML model trained by UMa simulation data.

Observation 6: From initial results of field test, it is observed that

- Directly use reference model in field (Case 2): similar performance on field data as eType II for one cell and performance loss compared to eType II is observed for another cell.

- Finetuned encoder or decoder using field data against reference decoder/encoder (Case 2A-1, Case 2A-2): performance improved compared to directly use reference model (Case 2), but still has performance loss compared to fully trained by field data (Case1). Finetuned decoder (Case 2A-2) is better than finetuned encoder (Case 2A-1).

- Pairing of finetuned encoder and finetuned decoder (Case 2A-3, Case 2A-4): performance loss in some cases considering mismatch on training data between UE and NW.

Observation 7: In field performance, the reference encoder performs better than reference decoder.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.

- R4-2508084, “Simulation assumptions on AIML CSI compression”, RAN4#115, May 19-23, 2025.

- R4-2408294, vivo, “Discussion on testability and interoperability issues for CSI compression and CSI prediction”, RAN4#111, May 20 – 24, 2024.

- Appendix A: Comparison of AI model performance for CSI compression

In order to better understand the performance difference for different AI models, initial evaluation was conducted under the CDL-A channel model with 30ns delay. Fig. A-1 shows the simulation procedure of the CSI compression.

Fig. A-1. Illustration of the simulation procedure of CSI compression

The simulation results are provided in Table A-1 with transformer encoder and Table A-2 with CNN encoder, respectively.

Table A-1. Performance comparison for different decoder structure and complexity (Transformer encoder)

| Encoder | Decoder |  |  |  | SGCS (R16 CB: 0.8375) |
| --- | --- | --- | --- | --- | --- |
|  | Back-bone | Model parameter | Number of model parameter | FLOPS |  |
| Back-bone: Transformer Number of model parameter: 4.1M FLOPS: 5×107 Depth: 10 transformer blocks Width: embedding=140 | Transformer | Depth: 18 transformer blocks Width: embedding=148 | 8.0M | 2×108 | 0.80 |
|  |  | Depth: 10 transformer blocks Width: embedding=198 | 8.0M | 2×108 | 0.92 |
|  |  | Depth: 20 transformer blocks Width: embedding=140 | 7.9M | 2×108 | 0.62 |
|  | CNN | Depth: conv layers Width: 165 feature maps | 0.2M | 2×108 | 0.88 |
|  |  | Depth: 17 conv layers Width: 41 feature maps | 0.2M | 2×108 | 0.91 |
|  |  | Depth: 150 conv layers Width: 20 feature maps | 0.2M | 2×108 | 0.92 |
|  | MLP | Depth: 1 FC layers Width: 9500 neurons | 98.7M | 2×108 | 0.91 |
|  |  | Depth: 10 FC layers Width: 3200 neurons | 109.6M | 2×108 | 0.89 |
|  |  | Depth: 15 FC layers Width: 2600 neurons | 106.6M | 2×108 | 0.83 |

In Table A-1, transformer encoder is used in the evaluation for verifying performance of different decoders. It can be seen that there could be large performance variance if model structure, including back-bone, parameters of decoder are different even if the complexity (FLOPs) are similar. For same type of decoder, if parameters are different, especially for transformer and MLP type of decoders, performance variance can be observed. The performance of CNN decoder seems not very sensitive to some parameters as listed in the table.

Table A-2. Performance comparison for different decoder structure and complexity (CNN encoder)

| Encoder | Decoder |  |  |  | SGCS (R16 CB: 0.8375) |
| --- | --- | --- | --- | --- | --- |
|  | Back-bone | Model parameter | Number of model parameter | FLOPS |  |
| Back-bone: CNN Number of model parameter: 4.1M FLOPS: 5×107 Depth: 17 conv layers Width: 32 feature maps | CNN | Depth: 17 conv layers Width: 16 feature maps | 0.08M | 4×107 | 0.88 |
|  | Transformer | Depth: 4 transformer blocks Width: embedding =110 | 1.4M | 4×107 | 0.84 |
|  | MLP | Depth: 3 FC layers Width: 2200 neurons | 16.5M | 4×107 | 0.89 |

In table A-2, CNN encoder is used in the evaluation for verifying performance of different decoders. Similar observations can be made.

- Appendix B: Agreement of assumptions for aligned reference encoder and reference decoder

In previous RAN4 meeting, the simulation assumptions have been agreed for feasibility study of aligning model among companies.

System-level simulation parameters (from TR): Parameter; Value. Duplex, Waveform; FDD, OFDM. Multiple access; OFDMA. Scenario; Dense Urban (Macro only). Frequency Range; FR1 only, [2GHz, 4GHz]. Inter-BS distance; 200m. Channel model; According to TR 38.901. Antenna setup and port layouts at gNB; Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ. Antenna setup and port layouts at UE; 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4). BS Tx power; 44dBm for 20MHz. BS antenna height; 25m. UE antenna height & gain; Follow TR36.873. UE receiver noise figure; 9dB. Numerology; Slot/non-slot; 14 OFDM symbol slot. SCS; Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz. Simulation bandwidth; Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz. Frame structure; Slot Format 0 (all downlink) for all slots. MIMO scheme; SU-MIMO. MIMO layers; Baseline: 1 Optional: 2. CSI feedback; Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms. Traffic load (Resource utilization); Baseline: 50% Optional: 20/70%. UE distribution; CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h). UE receiver; MMSE-IRC as the baseline receiver. Feedback assumption; Realistic. Channel estimation; Realistic or ideal channel estimation.

In the e-mail discussion after RAN4#111 meeting, a CNN based model structure pair for both encoder and decoder has been aligned for feasibility study.

Based on the two rounds of discussions, the following parameters are proposed: Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Table 1. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 47M (vs. 1.6M for eTypeII). Model size; 448Kbyte. Num of Resnet blocks; 6. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,32). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 740M. Model size; 7.2Mbyte. Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Other model related parameters: Activation function; ReLU. Latent message size; 32 float values. Payload size, bits; 64 per layer.. Quantization; 2 bits. System parameters: Frequency range; 2GHz (4GHz optional). MIMO Layers; 1 layer (2 layers optional, companies to. Sub-bands; 13. Channel estimation; ideal. Metrics; SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI.. Companies are also invited to provide the training parameters used.

In RAN4#112 meeting, more details of data generation and training method have also been aligned to further reduce the mismatch between companies

Parameter/Configurations; Options/Values; Company comments. Channel/data-set related. UE antenna element assumptions; - Option 1: Omni-directional antenna elements (Ericsson) - Option 2: Directional antenna elements - Option 3: Other; [Apple] We provided 2 simulation results: Directional (both horizontal/vertical) Omni (both horizontal/vertical) [QC] Option 1 (both horizontal and vertical) [Intel] Option 1 [OPPO]:Option1 [CMCC] option 1. Configuration for alignment: - Ideal omni-directional antenna elements following RAN1 assumptions. Sub-band channel averaging; - [Nokia, QC, Apple, vivo, Ericsson,OPPO] Average of covariance channel matrix per RBs in the sub-band before SVD - [ZTE]: Per sub-band - [CMCC]: Average of raw channel matrix per RBs in the sub-band before SVD; CATT: Prefer not to do the averaging before SVD.. Configuration for alignment: - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected. Any other pre-processing; - To be listed if used - [QC, Ericsson] Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - [OPPO] Normalize the phase of all entries of the eigenvector based on the phase of the last entry of the eigenvector - [Apple]: Normalize the phase of all entries of the eigenvector based on the phase of that entry that has the maximum amplitude averaged over the sub-bands; [Intel] prefer no other pre-processing. Configuration for alignment: - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1.. Data collection assumptions; - [Nokia, CATT, ZTE, Ericsson, CMCC] Each channel realization is collected from independent UE drop - Multiple channel realization can be collected from the same UE - Other - [QC] Multiple channels realizations are collected from multiple UEs and multiple drops of same UE - [Apple, Intel, OPPO]: Multiple drops. Each drop collect data from multiple UEs. Additionally collect samples from channel realization from each UE spaced at de-correlated interval.. Configuration for alignment: - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs.. Training dataset size; - [Nokia]: 630K - [QC]: 450K - [Samsung]: 25K - [R&S]: 614,440 - [MTK]: 700K - [Ericsson]: 800K - [Intel]: 200K - [CATT]: 102,600 - [ZTE]: 600k - [Apple]: 600K - [vivo]: 243000 - [OPPO]：600k - [CMCC]: 100K. Configuration for alignment: - Aim to use the dataset around 600K. - At least 100K sample to be used.. Dataset quantization; - Float32. Other data-set characteristics/statistics; - Companies to propose. - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII.. Model-related. Quantization of encoder output; - [Nokia]: Non-trainable quantization (bypassed during training) - [QC, CMCC] Quantization aware training - [Apple]: Quantization in forward and approximation in backward to enable learning. (avoid zero gradient of round function) - [vivo] VQ with quantization aware training - [Intel] Quantization aware training. Scalar quantization with non-trainable quantization grid. - [Ericsson] 2 bit scalar quantization, quantizer aware training; CATT: Consider to align the activation function and quantization method.. - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization. Normalization before quantization in the encoder; - [Nokia, Ericsson]: Normalization with tanh function after FC layer - [CATT, ZTE, vivo, CMCC]: Sigmoid function used after FC layer - [QC] Normalization not done due to the use of quantization aware training - [Apple]: Normalization with tanh function after FC layer. We don’t apply inverse of that in de-quantization - [Intel] Sigmoid after FC layer. Inverse sigmoid after de-quantization stage in decoder.. Configuration for alignment: - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional.. Use and configuration of drop-out; - [Nokia]: Use of on single drop-out layer after FC layer in the encoder and decoder, Drop-out rate = 0.4. - [CATT, QC, ZTE, Apple, vivo, Intel, Ericsson, CMCC]: No drop-out used.. Configuration for alignment: - Not to use drop-out layers. Training loss function, use of regularization; - [Nokia]: - SGCS with L2 regularization with weight decay, λ = 10-2 - [QC]: SGCS + additional terms for quantization aware training - [Apple, CATT, CMCC]: 1- SGCS - [R&S, MTK, Intel]: SGCS - [Ericsson]: NMSE - [ZTE]: MSE - [vivo]: 1-SGCS plus VQ loss for encoder output (MSE between original and quantized encoder output) - [Intel]: SGCS, no L1/L2 regularization. Configuration for alignment: - Use (1-SGCS) as loss function as reference. Optimizer; - [Samsung, QCM, Apple, MTK, Intel, CATT, Ericsson, CMCC]: Adam - [Nokia]: RMSProp (Root Mean Square Propagation) - [ZTE, vivo]: AdamW. Configuration for alignment: - Use Adam. Learning rate; - [Nokia]: - [QC, R&S, CATT, Ericsson]: 10^-4 - [R&S]: Learning rate adaptation: Reduce on plateau by factor 0.5 with patience of 50 epochs. - [Apple]: 0.0025 - [Samsung, Intel, MTK, CMCC]: 0.001 - [ZTE]:1.5e-3 - [vivo]: Warm-up to initial lr 1e-3 and then linearly decay to final lr 8e-4, update after each batch. Configuration for alignment: - Fixed learning rate of 10^-4. Batch size; - [Nokia]:1024 - [QC, MTK, CMCC]: 128 - [Apple]: 256 - [Samsung]: 200 - [R&S, CATT, vivo, Ericsson]: 512 - [Ericsson]: 32 - [Intel]: 10 - [ZTE]:400. Configuration for alignment: - 256. (Max) Number of epochs; - [QC]: 150 - [Apple, vivo, Intel]: 100 - [R&S, Nokia, CATT]: 200 - [MTK, ZTE]: 300 - [CMCC]: 500. Configuration for alignment: - 150 epochs. Stopping criteria; - [Nokia, Apple, CATT, QC, ZTE, Intel, CMCC]: No change in metric over a given number of epochs - Validation patience: [Intel, Nokia] 10, [QC] 50 - + Ericsson with constraint on max Epochs - [vivo] Stop when reaching the configured number of epochs. Configuration for alignment: - No improvement over 25 epochs.. Weight initialization; - [Apple, CATT, QC, vivo, Intel, CMCC]: Default - Ericsson: Variance scaling. Configuration for alignment: - Default. Kernel Parameters: - Dilation - Padding Type The value of groups (VG); - [CMCC, Samsung, Vivo, CATT]: Dilation:1, Padding type=0, VG= 1 - [QC, ZTE, Apple, Intel]: Default. Configuration for alignment: - Default.

Appendix C: Agreements of standardization of model structure in RAN1

RAN1#120 agreed the full-pictured model structure for studying the standardized model structure as following

Agreement For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose, Encoder description: Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment.

And RAN1 #120 agreed some options on the scalable model structure specification.

A Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

RAN1 #120-bis agreed some observations on the scalable model structure solutions.

Observation For choice of token/feature dimension in the scalable model design, companies observe that there is small loss in SGCS for Case1 over Case2 under tokenization (Alt1, Alt3) For layer1, under tokenization (Alt1), - 8 sources [Qualcomm, CATT, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe that average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.33% with median value of -1.10% - 2 sources [CATT, Xiaomi] observe average gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 2.04% to 7.84% with median value of 6.61% - 3 sources [CATT, Xiaomi, Ericsson] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under tokenization (Alt3), - 1 source [vivo] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.7% to 0.4% with median value of -0.3% - 1 source [Ericsson] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.4% to 6% For scalability over the feature dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2, Alt1+Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Fujitsu ] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.34% to -0.4% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 7 sources [Qualcomm, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.1% with median value of -0.85% - 1 source [Xiaomi] observes gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For layer1, under (Alt1+Alt2), - 1 source [Qualcomm] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.3% to -2.9% with median value of -3.1%. For scalability over the token dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 2 sources [Qualcomm, vivo] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.9% with median value of -1.5%. For layer1, under (Alt2), - 3 sources [CATT, vivo, ZTE ] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.4% with median value of -0.75%. - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51% to 7.84% with median value of 6.87% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12% to 10.2% with median value of 10.1% For scalability over the payload configurations, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Huawei] observe average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.73% to -3.34% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 3 sources [Xiaomi, vivo, ZTE] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.4% with median value of -0.85% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For scalability over feature dimension, token dimension and payload configuration jointly, For tokenization Alt1, feature scalability Alt1, token scalability Alt2 and payload scalability Alt1, - 1 source [CATT] observes average gain in SGCS of Case 1 over case 2 is in the range of -3.34%~2.03% with median value of -2.93% - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51%~7.84% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12%~10.23% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, payload configuration Alt2, - 2 sources [ZTE, vivo] observe average gain in SGCS of Case 1 over case 2 is in the range of -2.8%~1.2% with median value of -1.2%, under same parameter set. - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.1% ~ 0.8% with median value of 0.1%, under different parameter set For tokenization Alt1, feature scalability Alt2, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -1.7% ~ 5.4% with median value of -2.6%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.6% ~ 0% with median value of -0.9%, under different parameter set For tokenization Alt1, feature scalability Alt1, payload configuration Alt1, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -4.76%~-0.82% with median value of -2.08%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 3% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, payload configuration Alt2, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -5.52%~-1.59% with median value of -2.96%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, - 1 source [ZTE] observes average gain in SGCS of Case 1 over case 2 is in the range of -0.4%~0.9% with median value of -0.23%. For tokenization Alt3, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.1% ~ 1.6% with median value of -0.5%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.8% ~ 0% with median value of -0.4%, under different parameter set