# 3GPP TSG-RAN WG4 Meeting #116                                R4-2510404

# Bengaluru, India, 25th-29th Aug., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on testability of LP-WUR

# Agenda Item:   7.24.3.4

# Document for:  Approval

- Introduction

In last meeting, it was agreed that for idle-mode only or connected-mode only LP-WUR, the test should be done accordingly, but if UE support LP-WUS operation in both IDLE mode and CONNECTED mode, further study is needed[1].

Issue 4-1-1: How to verify LP-WUR Agreement: Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) - The ultimate test method including the potential test mode will be decided by RAN5 Issue 4-1-2: FR2 skipping rule WF: - Further check whether FR2 LP-WUR spherical coverage can be skipped. Issue 4-1-3: skipping rule for LP-WUR different mode Agreements: - For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly. - FFS For LP-WUR support both.

- Discussion

According to the WF, RAN4 has determined the general framework for the test of LP-WUR. The LP-WUR should be tested based on the response from UE to the TE upon successfully detecting the LP-WUS, including ACK/NACK in RRC_CONNECTED mode or MSG1/3 in RRC_IDLE state or other methods. The operating process and test duration of LP-WUS operation are different in RRC_IDLE/INACTIVE mode and RRC_CONNECTED mode. Then if UE declares that it supports LP-WUS operation in both modes, then both modes should be tested, because test in only one mode can not guarantee that the LP-WUR can work in the other mode.

Proposal 1: For LP-WUR supports both RRC_IDLE/INACTIVE mode and RRC_CONNECTED mode, test under both mode is needed.

However, we understand that testing both modes may cause a lot more cost. Considering that power saving of LP-WUS mainly make sense in RRC_IDLE/INACTIVE mode, if Proposal 1 is not agreed, we prefer to test the LP-WUR in RRC_IDLE/INACTIVE mode for LP-WUR support both RRC modes.

- Conclusion

Proposal 1: For LP-WUR supports both RRC_IDLE/INACTIVE mode and RRC_CONNECTED mode, test under both mode is needed.

- Reference

- R4-2507936 WF for [115][136] NR_LPWUS_UERF, Vivo, RAN4#115, St. Julian’s, Malta, May 2025.