3GPP TSG-RAN WG4 Meeting #116	R4-2510425

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance and CSI reporting requirements for MIMO_Ph5

Agenda item:	7.20.6

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made:

Do NOT introduce UE requirements for the following:

- UE demodulation/CSI reporting requirements for UE-initiated/event-driven beam management

- UE demodulation/CSI reporting requirements for asymmetric DL sTRP/UL mTRP

- PDSCH/PDCCH/PBCH/SDR demodulation requirements for CSI enhancement up to 128 CSI-RS ports.

- CQI/RI reporting requirements for CSI enhancement up to 128 CSI-RS ports.

- CRI reporting requirements for extension of CRI-based CSI reporting.

Introduce UE requirements for the following:

- PMI reporting requirements for typeI-SinglePanel-r19.

Use the following test assumptions for typeI-SinglePanel-r19:

| Test Metrics | Reuse the test metric for Type-I PMI reporting requirements as the starting point, throughput ratio between the following PMI and random Type-I PMI, where the same Type-I codebook scheme is used for both follow PMI and random PMI throughput measurements. |
| --- | --- |
| Duplex Mode | Both FDD and TDD |
| Number of Rx antennas | Both 2Rx and 4Rx |
| Propagation channel | TDLA30-5 as the starting point |
| Correlation configuration | Extend the antenna correlation matrix models for X-pol arrays in TS 38.101-4 to support greater than 32 ports in the following manner, capture it in NR_MIMO_demod related CR. For the first dimension (N1), define the vector r1: r₁(k) = αk - 1N1- 12, for k = 1, 2, ..., N₁ RgNBDim,1 = Toeplitz(r₁) where Toeplitz(r₁) constructs an N1×N1 Toeplitz matrix from the vector r1. For the second dimension (N2), define the vector r2: r₂(k) = αk - 1N2- 12, for k = 1, 2, ..., N₂ RgNBDim,2 = Toeplitzr2 If N2=1, RgNBDim,2=1. Then the correlation matrix at the gNB is RgNB=RgNBDim,1⨂ RgNBDim,2 The correlation matrix at the UE side assumes a 1D cross-polar array: rue(k) = βk - 1N- 12, for k = 1, 2, ..., N RUE = Toeplitz(rue) - The calculation of Rspat, Rhigh, and Rmedium remains the same as currently specified in TS 38.101-4. - The values of α, β, and the scaling factor a for specific correlation levels and antenna configurations are FFS.. |
| typeI-SinglePanel-ri-restriction-r19 | Rank 2 |
| Number of CSI-RS ports | 64, [128] |
| N1 and N2 configuration for 128 CSI-RS ports (if introduced) | (16, 4) |
| (Number of aggregated NZP CSI-RS resources) K | K=2 for 64 CSI-RS ports K=4 for 128 CSI-RS ports |
| typeI-codebookMode-r19 | Scheme A |
| portMappingMethod | Select one for requirements. FFS on which one next meeting. |
| Number of slots for CSI-RS transmission | 64 ports: one slot 128 ports: TBD |
| MCS level | MCS20 (Table 1) as starting point. |
| Other parameters | Reuse the setting for Single PMI with 32TX TypeI-SinglePanel Codebook as the starting point. |

In the following we will discuss the remaining issues and provide observations and proposals where needed.

Discussion: UE demodulation/CSI reporting performance requirements

General

Clarify if PMI reporting requirements are needed for eTypeII-r19 and eTypeII-Doppler-r19

In RAN4#115 it was agreed to introduce PMI reporting requirements for typeI-SinglePanel-r19, however it was still open if requirements are needed for eTypeII-r19 and eTypeII-Doppler-r19 (see [1]):

Issue 1-1-7: Clarify if PMI reporting requirements are needed for eTypeII-r19 Way forward: - Option 1: RAN4 introduce PMI reporting requirements for eTypeII-r19 - Option 2: RAN4 do not introduce PMI requirements for eTypeII-r19

Issue 1-1-8: Clarify if PMI reporting requirements are needed for typeII-Doppler-r19 Way forward: - Option 1: RAN4 introduce PMI reporting requirements for typeII-Doppler-r19 - Option 2: RAN4 do not introduce PMI requirements for typeII-Doppler-r19

As we discussed in our previous contribution [2], the codebook structures for the extended eTypeII codebook and the eTypeII Doppler codebook have not been modified.  The extensions to these codebooks, as required by the work item objective, have been implemented by adding support for larger port counts without structural changes to the codebooks.  Therefore, the methods used by UEs to implement the extended Type II codebook search will likely be extended versions of algorithms used for the legacy codebooks.  Also, legacy codebooks are prerequisite features for the extended codebooks in the UE capabilities being developed for Rel-19.  Introducing additional requirements for the extended Type II codebooks would test the extended search algorithms as well as extended prediction algorithms for the eTypeII Doppler codebook.  In addition, requirements would test the increased computational complexity due to the additional antenna ports or areas where the implementation is modified, rather than simply extended, to control the computational complexity.

The extended eTypeII and eTypeII Doppler codebooks have been implemented by adding support for larger port counts without structural changes to the codebooks.  UE implementations of these codebooks are likely to be extensions of the legacy codebook implementations.

The structures of the eTypeII and eTypeII Doppler codebooks are similar especially when N4=1 in the Doppler codebook and N4=1 is the only case with requirements in the legacy codebook.  Assuming that N4=1 would be the only case considered for the Rel-19 Doppler codebook, requirements for both Rel-19 codebooks would be testing similar mechanisms for deriving the codebook element, except for the prediction algorithm and the additional CSI-RS overhead required for the Doppler codebook.  Satisfying a legacy Doppler requirement and a Rel-19 eTypeII requirement should be sufficient for the eTypeII Doppler codebook.  Therefore, we propose to only introduce requirements for the eTypeII codebook.

For Rel-19 CSI reporting extensions, introduce PMI reporting requirements for the extended eTypeII codebook.

For Rel-19 CSI reporting extensions, do not introduce PMI reporting requirements for the extended eTypeII Doppler codebook.

Clarify if UE demodulation requirements are needed for UE reporting enhancement for CJT

In the RAN4#115 meeting, it was discussed whether it should be introduced demodulation requirements for the UE reporting associated with the CJT offset features introduced for RAN1 (see [1]):

Issue 1-1-10: Clarify if UE demodulation requirements are needed for UE reporting enhancement for CJT Way forward: - Option 1: RAN4 do not introduce demodulation requirements for reporting enhancements for CJT deployments - Option 2: RAN4 further discussion on whether to introduce new PDSCH demodulation performance requirements for PDSCH transmission Scheme C/Scheme D/Scheme E.

Our view is that the scheme C, D and E with associated new proposed PDSCH demodulation requirements are not required to be introduced as an additional test. These QCL assumptions should be included implicitly as part of the necessary parameters for the new CJT calibration offset reporting requirements proposed to be introduced in TS38 101-4 [3].

Do not introduce demodulation requirements for reporting enhancements for CJT deployments. The QCL assumptions related to Scheme C/Scheme D/Scheme E shall be implicitly part of the CJT calibration offset reporting requirements if introduced.

Clarify if CSI reporting requirements are needed for UE reporting enhancements for CJT

In RAN4#115 it was discussed if CSI reporting requirements are to be introduced for the UE reporting enhancements for CJT (see [1]):

Issue 1-1-11: Clarify if CSI reporting requirements are needed for UE reporting enhancements for CJT Way forward: - Option 1: RAN4 do not introduce CSI reporting requirements for UE reporting enhancements for CJT - Option 2: RAN4 introduce new requirements for CJT calibration offset reporting for the time offset (TO), the carrier frequency offset (CFO) and the phase offset (PO) scenarios. Further study exact implementation of the new requirements. - RAN4 do not introduce new CQI/RI and dedicated PMI reporting requirements but consider PMI calculation as part of the CJT calibration reporting requirements. - Introduce a new subsection in TS 138 101-4 to cover introduction of CJT calibration offset reporting requirements.

Based on what it was discussed in the RAN4#115 meeting we agree with the Option 2 of this issue and all the listed items herein. First of all, a new type of test is needed, and this implies the need for a new subsection in TS38 101-4 [1] to cover the introduction of CJT calibration offset reporting requirements. This type of test has special characteristics because it requires not only measuring, reporting and compensating one of the CJT offsets (e.g., TO, CFO, PO) but it also may need channel acquisition knowledge at the testing equipment. Therefore, PMI or in general Channel State Information (CSI) are also important pieces needed to be considered for this potential test. The testing equipment requires knowledge of CSI and the offset values to properly pre-code and compensate PDSCH. This is the right way to obtain adequate and realistic levels of throughput at the UE.

On the other hand, alternatively we may create a test by using a random PMI with the expectation that the compensation provided by correcting the offset impairment is enough for having a clear gain with the case with compensated offset vs. the case with non-compensated offset for PDSCH with random precoding. The latter needs to be verified by simulations. We provide some starting point simulations of this scheme in this contribution.

Having said that, we express that our first preference is using the actual PMI, i.e., follow PMI, and having realistic CSI acquired from the UE for the PDSCH precoding. However, we are also open to the possibility of using random PMI to simplify the eventual test framework. Later in this contribution, we provide our full view of a possible implementation for the new requirements test to be applied for CJT calibration of TO, CFO and PO compensation cases.

RAN4 to further discuss the necessary details for a new test framework of performance requirements for the CJT calibration offsets, i.e., TO, CFO and PO. Part of the work would be to determine whether random PMI can be used as viable/reliable option, or the follow PMI is required during the CJT test.

Test assumptions for typeI-SinglePanel-r19

Number of CSI-RS ports

In RAN4#115 it was agreed to introduce requirements for the Type I single panel codebooks for 64 ports, but requirements for 128 CSI-RS ports are FFS (see [1]):

Issue 1-2-7: Number of CSI-RS ports Agreement: - Agree on to define requirement for 64 ports. Not to define requirements for 48 ports. FFS on 128 ports.

For Type I single panel codebooks in Rel-19, UE support for 64 ports is the baseline feature and UE support for 128 ports is in addition to support for 64 ports.  An additional UE capability is the ability to aggregate CSI-RS resources from two slots.  This is relevant for setting requirements for 128 ports because the required four 32-port CSI-RS resources cannot be configured in a single slot with density 1.  This is illustrated in the left side of Figure 1 which shows two PRBs adjacent in frequency.

Figure 1:  Multiple CSI-RS resource configurations (the vertical axis is frequency and the horizontal axis is time)

In this figure, three 32-port CSI-RS resources have been configured using the configuration parameters in Table 1, where the row number indicates the row in Table 7.4.1.5.3-1 of TS 38.211 used for the configuration.

Table 1:  CSI-RS Configuration for Density 1 Resources

| Resource | Row | Density | CDM Type | l0,l1 | k0,k1,k2,k3 |
| --- | --- | --- | --- | --- | --- |
| 1 | 17 | 1 | FD2/TD2 | (2,4) | (0,2,4,6) |
| 2 | 17 | 1 | FD2/TD2 | (6,8) | (0,2,4,6) |
| 3 | 17 | 1 | FD2/TD2 | (10,12) | (0,2,4,6) |

Row 17 has been used for the examples, but the discussion applies equally to the other 32 port configurations (rows 16 and18) because there is always an FDM of two across adjacent REs and the CDM group configurations always utilize OFDM symbols in (at least) pairs.  Thus, due to the configuration constraints of the 32-port CSI-RS resources, a single 32-port resource will always occupy 8 RE’s across 4 OFDM symbols in s single PRB (though not necessarily contiguous).  As can be seen in the figure, once three 32-port resources have been configured (96 ports total), there is no longer room for a fourth resource in the same PRB.

The four 32-port CSI-RS resources required for 128 ports cannot be configured in a single slot using resources with density 1.

Therefore, two alternatives exist for configuring 128 CSI-RS ports:

- Configure the CSI-RS resources with density 1/2 in a single slot

- Configure the CSI-RS resources with density 1 over two slots, if the UE supports this configuration

The first option is illustrated in the right side of Figure 1, where the resource configurations are shown in Table 2.

Table 2:  CSI-RS Configuration for Density 1/2 Resources

| Resource | Row | Density | CDM Type | l0,l1 | k0,k1,k2,k3 |
| --- | --- | --- | --- | --- | --- |
| 1 | 17 | 1/2 (even) | FD2/TD2 | (2,4) | (0,2,4,6) |
| 2 | 17 | 1/2 (even) | FD2/TD2 | (6,8) | (0,2,4,6) |
| 3 | 17 | 1/2 (odd) | FD2/TD2 | (2,4) | (0,2,4,6) |
| 4 | 17 | 1/2 (odd) | FD2/TD2 | (6,8) | (0,2,4,6) |

In this example, two resources have been configured in even-numbered slots (R1 and R2) and two in odd-numbered slots (R3 and R4).  For the second option, a UE must not only support Type I CSI feedback with 128 ports, but it must also support the UE capability to aggregate CSI-RS resources from two slots.  A third option exists similar to the second option where the CSI-RS resources can be configured with density 1/2 over two slots.  However, since it is possible to configure with density 1 providing channel response sampling in every PRB, the density 1 option is preferred when the UE supports aggregation over two slots.

Two alternatives for supporting PMI reporting requirements for 128 CSI-RS ports:
- Configure 128 CSI-RS ports in a single slot using four CSI-RS resources with density ½
- For UEs which support 128 ports and aggregation of CSI-RS resources from two slots, configure 128 CSI-RS ports with four CSI-RS resources with density 1 over two slots.

Due to the increased computational complexity for calculating the PMI for 128 ports (compared to 64 ports), introducing a requirement for 128 ports is desirable to ensure that a UE which supports 128 ports can meet the desired performance target.  The increased complexity is expected to be greater when the UE supports aggregation over two slots (assuming a CSI-RS density of 1) since there are twice as many REs involved in the calculations and the span of time over which the CSI-RS resources are received is longer.  For this reason, we propose to introduce requirements for 128 ports using the second option.

For Rel-19 typeI-SinglePanel-r19, introduce PMI reporting requirements for 128 CSI-RS ports for UEs which support aggregation of CSI-RS resources from two slots.  Configure the 128 CSI-RS ports using four CSI-RS resources with density 1 over two slots.

Correlation configuration

In RAN4#115 it was agreed to extend the correlation matrix definitions in TS 38.101-4 to 128 ports (see [1]):

Issue 1-2-5: Correlation configuration Agreement: - Extend the antenna correlation matrix models for X-pol arrays in TS 38.101-4 to support greater than 32 ports in the following manner, capture it in NR_MIMO_demod related CR. For the first dimension (N1), define the vector r1: r₁(k) = αk - 1N1- 12, for k = 1, 2, ..., N₁ RgNBDim,1 = Toeplitz(r₁) where Toeplitz(r₁) constructs an N1×N1 Toeplitz matrix from the vector r1. For the second dimension (N2), define the vector r2: r₂(k) = αk - 1N2- 12, for k = 1, 2, ..., N₂ RgNBDim,2 = Toeplitzr2 If N2=1, RgNBDim,2=1. Then the correlation matrix at the gNB is RgNB=RgNBDim,1⨂ RgNBDim,2 The correlation matrix at the UE side assumes a 1D cross-polar array: rue(k) = βk - 1N- 12, for k = 1, 2, ..., N RUE = Toeplitz(rue) - The calculation of Rspat, Rhigh, and Rmedium remains the same as currently specified in TS 38.101-4. - The values of α, β, and the scaling factor a for specific correlation levels and antenna configurations are FFS.. - FFS on the correlation configuration.

Two issues remain open after RAN4#115.  First of all, the values of α, β, and the scaling factor a for specific correlation levels and antenna configurations remain to be decided.  Second, the specific correlation configuration for the single panel codebook requirements remains to be determined.

However, we want to note here that the previous agreement to further extend the adhoc RAN4 antenna correlation framework, will severely overestimate the ill-conditionedness of the channel. RAN4 historically chose to use an exponential correlation model with an adhoc squaring of the exponent to match a single measurement paper, which was intended to be applied up to 3 antennas. Thus, adding more antennas (above 4) to this model barely adds spatial diversity. In the future it may be beneficial to consider spatial channel models that are more aligned with the RAN1 expectations for core feature development, i.e., that are more aligned with CDL channel models.

The previous agreement to further extend the adhoc RAN4 antenna correlation framework results in severe underestimation of channel spatial diversity increase from increasing CSI-RS ports.

Based on the existing specifications in TS 38.101-4 [1], we propose to adopt the existing values of α and β for the currently supported correlation conditions for both ULA arrays and cross-polarized arrays.  The existing values for ULA arrays are currently specified in Table B.2.3.1.2-1 in TS 38.101-4 and the existing values for cross-polarized arrays are currently specified in Table B.2.3.2.2-1.  For convenience, copies of these two tables are shown below:

In TS 38.101-4, extend the application of the parameters in Table B.2.3.1.2-1 to ULA MIMO correlation matrices for more than 32 ports.  Apply the parameters in Table B.2.3.2.2-1 to cross-polarized antennas for more than 32.

In addition, we have conducted calculations of the specified correlation matrices using these values and analyzed their eigen structures to determine values of the scaling factor which will ensure that the matrices are positive definite.

To ensure that the spatial correlation matrices (Rspat, as defined in TS 38.101-4 [3]) remain positive semi-definite (PSD) after rounding to four decimal digits, we introduce a small diagonal scaling term a such that:

Rhigh=[Rspat+aIn]/(1+a) or Rmedium=[Rspat+aIn]/(1+a).

The value of a is the minimal scaling factor required to maintain PSD, while remaining consistent with the 3GPP specification style of small, five-decimal values.

Results are shown below for the cross-polarized and Uniform Linear array types. These results were obtained with the value of ‘a’ set to zero and incremented in small steps (10-5) till the matrix condition number was less than the threshold value (50,000; 100,000; or 150,000). The correlation matrix condition threshold values were deduced by using the array configurations whose scaling factors were already specified in 38.101.  The values of α, β, and γ proposed above were used in these calculations. The table below shows the results at the three thresholds.

Table 3:  Eigen structures analysis for Antenna Correlation Matrices

| Configuration (total ports) | XPOL / ULA | Correlation | CondThreshold = 50,000 |  |  | CondThreshold = 100,000 |  |  | CondThreshold = 150,000 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | a threshold | Min(eig(R_spat)) | Condition Number | a threshold | Min(eig(R_spat)) | Condition Number | a threshold | Min(eig(R_spat)) | Condition Number |
| 8 (4,1,2) × 2 | XPOL | high | 0.00010 | 0.00010 | 49223.43 | 0.00005 | 0.00005 | 95953.93 | 0.00004 | 0.00004 | 118443.1 |
| 16 (4,2,2) × 2 | XPOL | high | 0.00020 | 0.00020 | 47945.20 | 0.00010 | 0.00010 | 95761.91 | 0.00007 | 0.00007 | 136646.6 |
| 16 (4,2,2) × 4 | XPOL | high | 0.00037 | 0.00037 | 49303.08 | 0.00019 | 0.00019 | 96003.77 | 0.00013 | 0.00013 | 140303.7 |
| 32 (8,2,2) × 2 | XPOL | high | 0.00039 | 0.00039 | 49558.36 | 0.00020 | 0.00020 | 96637.85 | 0.00013 | 0.00013 | 148673.1 |
| 32 (8,2,2) × 4 | XPOL | high | 0.00074 | 0.00074 | 49625.33 | 0.00037 | 0.00037 | 99249.66 | 0.00025 | 0.00025 | 146889.0 |
| 32 (4,4,2) × 2 | XPOL | high | 0.00040 | 0.00040 | 49112.01 | 0.00020 | 0.00020 | 98223.01 | 0.00014 | 0.00014 | 140318.2 |
| 32 (4,4,2) × 4 | XPOL | high | 0.00075 | 0.00075 | 49766.82 | 0.00038 | 0.00038 | 98223.02 | 0.00025 | 0.00025 | 149298.5 |
| 48 (8,3,2) × 2 | XPOL | high | 0.00059 | 0.00059 | 49984.78 | 0.00030 | 0.00030 | 98302.43 | 0.00020 | 0.00020 | 147453.2 |
| 48 (8,3,2) × 4 | XPOL | high | 0.00113 | 0.00113 | 49586.68 | 0.00057 | 0.00057 | 98302.43 | 0.00038 | 0.00038 | 147453.2 |
| 48 (6,4,2) × 2 | XPOL | high | 0.00060 | 0.00060 | 49333.90 | 0.00030 | 0.00030 | 98666.81 | 0.00020 | 0.00020 | 147999.7 |
| 48 (6,4,2) × 4 | XPOL | high | 0.00113 | 0.00113 | 49770.48 | 0.00057 | 0.00057 | 98666.81 | 0.00038 | 0.00038 | 147999.7 |
| 64 (8,4,2) × 2 | XPOL | high | 0.00080 | 0.00080 | 49429.44 | 0.00040 | 0.00040 | 98857.89 | 0.00027 | 0.00027 | 146455.6 |
| 64 (8,4,2) × 4 | XPOL | high | 0.00151 | 0.00151 | 49756.78 | 0.00076 | 0.00076 | 98857.89 | 0.00051 | 0.00051 | 147317.1 |
| 64 (16,2,2) × 2 | XPOL | high | 0.00078 | 0.00078 | 49686.58 | 0.00039 | 0.00039 | 99372.16 | 0.00026 | 0.00026 | 149057.7 |
| 64 (16,2,2) × 4 | XPOL | high | 0.00148 | 0.00148 | 49753.72 | 0.00074 | 0.00074 | 99506.44 | 0.00050 | 0.00050 | 147269.1 |
| 128 (16,4,2) × 2 | XPOL | high | 0.00159 | 0.00159 | 49869.00 | 0.00080 | 0.00080 | 99113.66 | 0.00053 | 0.00053 | 149605.0 |
| 128 (16,4,2) × 4 | XPOL | high | 0.00302 | 0.00301 | 49885.52 | 0.00151 | 0.00151 | 99770.03 | 0.00101 | 0.00101 | 149160.6 |
| 128 (8,8,2) × 2 | XPOL | high | 0.00160 | 0.00160 | 49748.93 | 0.00080 | 0.00080 | 99496.86 | 0.00054 | 0.00054 | 147402.3 |
| 128 (8,8,2) × 4 | XPOL | high | 0.00303 | 0.00302 | 49913.11 | 0.00152 | 0.00152 | 99496.86 | 0.00101 | 0.00101 | 149737.3 |
| 8 (4,1,2) × 2 | XPOL | Medium | 0.00000 | 0.00344 | 1163.754 | 0.00000 | 0.00344 | 1163.754 | 0.00000 | 0.00344 | 1163.754 |
| 16 (4,2,2) × 2 | XPOL | Medium | 0.00000 | 0.00241 | 2161.257 | 0.00000 | 0.00241 | 2161.257 | 0.00000 | 0.00241 | 2161.257 |
| 16 (4,2,2) × 4 | XPOL | Medium | 0.00000 | 0.00024 | 41063.88 | 0.00000 | 0.00024 | 41063.88 | 0.00000 | 0.00024 | 41063.88 |
| 32 (8,2,2) × 2 | XPOL | Medium | 0.00022 | 0.00022 | 49767.16 | 0.00011 | 0.00011 | 99533.10 | 0.00008 | 0.00008 | 136857.4 |
| 32 (8,2,2) × 4 | XPOL | Medium | 0.00042 | 0.00042 | 49530.27 | 0.00021 | 0.00021 | 99059.54 | 0.00014 | 0.00014 | 148588.8 |
| 32 (4,4,2) × 2 | XPOL | Medium | 0.00023 | 0.00025 | 49820.58 | 0.00011 | 0.00013 | 96944.34 | 0.00007 | 0.00009 | 141585.3 |
| 32 (4,4,2) × 4 | XPOL | Medium | 0.00047 | 0.00047 | 49541.47 | 0.00024 | 0.00024 | 96686.46 | 0.00016 | 0.00016 | 144525.0 |
| 48 (8,3,2) × 2 | XPOL | Medium | 0.00038 | 0.00038 | 48922.28 | 0.00019 | 0.00019 | 97843.54 | 0.00013 | 0.00013 | 143001.6 |
| 48 (8,3,2) × 4 | XPOL | Medium | 0.00071 | 0.00071 | 49749.12 | 0.00036 | 0.00036 | 98115.36 | 0.00024 | 0.00024 | 147172.5 |
| 48 (6,4,2) × 2 | XPOL | Medium | 0.00039 | 0.00039 | 49037.24 | 0.00020 | 0.00020 | 95618.77 | 0.00013 | 0.00013 | 147100.3 |
| 48 (6,4,2) × 4 | XPOL | Medium | 0.00073 | 0.00073 | 49777.65 | 0.00037 | 0.00037 | 98208.83 | 0.00025 | 0.00025 | 145348.3 |
| 64 (8,4,2) × 2 | XPOL | Medium | 0.00052 | 0.00052 | 49817.05 | 0.00026 | 0.00026 | 99633.11 | 0.00018 | 0.00018 | 143914.0 |
| 64 (8,4,2) × 4 | XPOL | Medium | 0.00099 | 0.00099 | 49716.41 | 0.00050 | 0.00050 | 98437.52 | 0.00033 | 0.00033 | 149147.2 |
| 64 (16,2,2) × 2 | XPOL | Medium | 0.00045 | 0.00045 | 49712.67 | 0.00023 | 0.00023 | 97262.96 | 0.00015 | 0.00015 | 149136.0 |
| 64 (16,2,2) × 4 | XPOL | Medium | 0.00086 | 0.00086 | 49423.65 | 0.00043 | 0.00043 | 98846.30 | 0.00029 | 0.00029 | 146564.7 |
| 128 (16,4,2) × 2 | XPOL | Medium | 0.00106 | 0.00106 | 49933.12 | 0.00053 | 0.00053 | 99865.23 | 0.00036 | 0.00036 | 147023.3 |
| 128 (16,4,2) × 4 | XPOL | Medium | 0.00202 | 0.00202 | 49784.80 | 0.00101 | 0.00101 | 99568.6 | 0.00068 | 0.00068 | 147888.2 |
| 128 (8,8,2) × 2 | XPOL | Medium | 0.00110 | 0.00110 | 49602.25 | 0.00055 | 0.00055 | 99203.51 | 0.00037 | 0.00037 | 147464.2 |
| 128 (8,8,2) × 4 | XPOL | Medium | 0.00208 | 0.00208 | 49840.72 | 0.00104 | 0.00104 | 99680.44 | 0.00070 | 0.00070 | 148096.2 |
| 4 (4,1,1) × 1 | ULA | High | 0.00008 | 0.00008 | 46383.79 | 0.00004 | 0.00004 | 88733.82 | 0.00003 | 0.00003 | 114979.2 |
| 4 (4,1,1) × 2 | ULA | High | 0.00015 | 0.00015 | 49115.38 | 0.00008 | 0.00008 | 91886.82 | 0.00005 | 0.00005 | 146601.5 |
| 4 (4,1,1) × 4 | ULA | High | 0.00031 | 0.00031 | 48746.42 | 0.00016 | 0.00016 | 94445.24 | 0.00011 | 0.00011 | 137374.4 |
| 48 (48,1,1) × 2 | ULA | High | 0.00180 | 0.00180 | 49763.27 | 0.00090 | 0.00090 | 99525.54 | 0.00060 | 0.00060 | 149287.8 |
| 48 (48,1,1) × 4 | ULA | High | 0.00367 | 0.00366 | 49935.51 | 0.00184 | 0.00184 | 99598.63 | 0.00123 | 0.00123 | 148992.6 |
| 64 (64,1,1) × 2 | ULA | High | 0.00239 | 0.00238 | 49980.67 | 0.00120 | 0.00120 | 99543.84 | 0.00080 | 0.00080 | 149315.3 |
| 64 (64,1,1) × 4 | ULA | High | 0.00489 | 0.00487 | 49978.73 | 0.00245 | 0.00244 | 99752.47 | 0.00163 | 0.00163 | 149934.2 |
| 128 (128,1,1) × 2 | ULA | High | 0.00478 | 0.00476 | 49994.27 | 0.00239 | 0.00238 | 99987.54 | 0.00160 | 0.00160 | 149355.9 |
| 128 (128,1,1) × 4 | ULA | High | 0.00978 | 0.00969 | 49992.33 | 0.00489 | 0.00487 | 99983.67 | 0.00326 | 0.00325 | 149975.0 |
| 2 (2,1,1) × 4 | ULA | Medium | 0.00010 | 0.00010 | 49223.43 | 0.00005 | 0.00005 | 95953.93 | 0.00004 | 0.00004 | 118443.1 |
| 4 (4,1,1) × 4 | ULA | Medium | 0.00024 | 0.00024 | 49816.10 | 0.00012 | 0.00012 | 99623.44 | 0.00008 | 0.00008 | 149423.0 |
| 48 (48,1,1) × 2 | ULA | Medium | 0.00153 | 0.00153 | 49959.08 | 0.00077 | 0.00077 | 99268.35 | 0.00051 | 0.00051 | 149875.2 |
| 48 (48,1,1) × 4 | ULA | Medium | 0.00313 | 0.00312 | 49963.82 | 0.00157 | 0.00157 | 99608.41 | 0.00105 | 0.00105 | 148937.8 |
| 64 (64,1,1) × 2 | ULA | Medium | 0.00205 | 0.00205 | 49792.97 | 0.00103 | 0.00103 | 99101.53 | 0.00069 | 0.00069 | 147933.7 |
| 64 (64,1,1) × 4 | ULA | Medium | 0.00418 | 0.00416 | 49961.99 | 0.00209 | 0.00209 | 99922.98 | 0.00140 | 0.00140 | 149170.2 |
| 128 (128,1,1) × 2 | ULA | Medium | 0.00410 | 0.00408 | 49908.34 | 0.00205 | 0.00205 | 99815.68 | 0.00137 | 0.00137 | 149358.7 |
| 128 (128,1,1) × 4 | ULA | Medium | 0.00838 | 0.00831 | 49958.23 | 0.00419 | 0.00417 | 99915.47 | 0.00280 | 0.00279 | 149515.9 |

The scaling factor a is determined based on the minimum eigenvalue (λmin) of the spatial correlation matrices (Table 3). The objective is to ensure that the adjusted correlation matrix remains positive semi-definite (PSD) after rounding to four decimal digits, consistent with the style of the 3GPP specifications. It should be noted that for cross-polarized arrays with medium correlation, there is no mention of any sample scaling factors in the specification 38.101-4 [3].

The proposed scaling factor ‘a’ is picked from the results corresponding to the spatial correlation matrix condition number threshold of 100,000. This logic yields a small a value satisfying the PSD requirement while maintaining consistency with the style of existing specification values.

From the above analysis, the scaling factors in Table 4 are proposed.

In TS 38.101-4, adopt the following table of scaling factors for MIMO correlation matrices for more than 32 transmit ports, covering both ULA and cross-polarized arrays.

Table 4:  Proposed Scaling Factors for Antenna Correlation Matrices for More than 32 Tx Ports

| Array Type | Antenna Configuration Tx (N1, N2, P) × Rx | Correlation Level | Scaling Factor a |
| --- | --- | --- | --- |
| Cross-polarized | 48 (8,3,2) × 2 | High | 0.00030 |
| Cross-polarized | 48 (8,3,2) × 4 | High | 0.00057 |
| Cross-polarized | 48 (6,4,2) × 2 | High | 0.00030 |
| Cross-polarized | 48 (6,4,2) × 4 | High | 0.00057 |
| Cross-polarized | 64 (8,4,2) × 2 | High | 0.00040 |
| Cross-polarized | 64 (8,4,2) × 4 | High | 0.00076 |
| Cross-polarized | 64 (16,2,2) × 2 | High | 0.00039 |
| Cross-polarized | 64 (16,2,2) × 4 | High | 0.00074 |
| Cross-polarized | 128 (16,4,2) × 2 | High | 0.00080 |
| Cross-polarized | 128 (16,4,2) × 4 | High | 0.00151 |
| Cross-polarized | 128 (8,8,2) × 2 | High | 0.00080 |
| Cross-polarized | 128 (8,8,2) × 4 | High | 0.00152 |
| Cross-polarized | 48 (8,3,2) × 2 | Medium | 0.00019 |
| Cross-polarized | 48 (8,3,2) × 4 | Medium | 0.00036 |
| Cross-polarized | 48 (6,4,2) × 2 | Medium | 0.00020 |
| Cross-polarized | 48 (6,4,2) × 4 | Medium | 0.00037 |
| Cross-polarized | 64 (8,4,2) × 2 | Medium | 0.00026 |
| Cross-polarized | 64 (8,4,2) × 4 | Medium | 0.00050 |
| Cross-polarized | 64 (16,2,2) × 2 | Medium | 0.00023 |
| Cross-polarized | 64 (16,2,2) × 4 | Medium | 0.00043 |
| Cross-polarized | 128 (16,4,2) × 2 | Medium | 0.00053 |
| Cross-polarized | 128 (16,4,2) × 4 | Medium | 0.00101 |
| Cross-polarized | 128 (8,8,2) × 2 | Medium | 0.00055 |
| Cross-polarized | 128 (8,8,2) × 4 | Medium | 0.00104 |
| ULA | 48 (48,1,1) × 2 | High | 0.00090 |
| ULA | 48 (48,1,1) × 4 | High | 0.00184 |
| ULA | 64 (64,1,1) × 2 | High | 0.00120 |
| ULA | 64 (64,1,1) × 4 | High | 0.00245 |
| ULA | 128 (128,1,1) × 2 | High | 0.00239 |
| ULA | 128 (128,1,1) × 4 | High | 0.00489 |
| ULA | 48 (48,1,1) × 2 | Medium | 0.00077 |
| ULA | 48 (48,1,1) × 4 | Medium | 0.00157 |
| ULA | 64 (64,1,1) × 2 | Medium | 0.00103 |
| ULA | 64 (64,1,1) × 4 | Medium | 0.00209 |
| ULA | 128 (128,1,1) × 2 | Medium | 0.00205 |
| ULA | 128 (128,1,1) × 4 | Medium | 0.00419 |

In all legacy requirements for the Type I Single Panel codebooks, the antenna correlation configuration is cross-polarized with high correlation.  We propose to keep the cross-polarized configuration for the extended codebooks since we expect most implementations to use cross-polarized arrays.  Whether to select high or medium correlation for these requirements is not clear until simulation results are evaluated.  We propose to use high correlation as a starting point and investigate medium correlation if necessary.

For Rel-19 typeI-SinglePanel-r19, adopt a cross-polarized antenna configuration with high correlation as the starting point.

N1 and N2 configuration for 64 CSI-RS ports

In RAN4#115 two options were identified for the port configuration of the 64-port PMI requirement for Type I single panel codebooks (see [1]):

Issue 1-2-8: N1 and N2 configuration for 64 CSI-RS ports Way forward: - Select single one from below options - Option 1: (16,2) - Option 2: (8,4)

In many respects, we don’t see significant differences between the two array configuration options.  Both configurations have the same number of ports and search over the same number of beams.  Unless there are differences in the codebook search algorithm at the UE which cause a computational complexity difference, we expect similar computational complexity for both cases.  In addition, it is not clear whether there are significant differences in the ability of the UE to satisfy a requirement for one array or the other.  However, we are inclined to expect that a (8,4) array is more likely to be implemented than a (16,2) array since the 8 azimuth ports provide sufficient azimuth beam control with additional elevation control due to the 4 elevation ports.  For this reason, we tend to prefer Option 2, but are open to further discussion of the relative merits of Option 1.

For Rel-19 typeI-SinglePanel-r19 requirements with 64 CSI-RS ports, use an N1, N2 configuration of 8,4 (Option 2).

portMappingMethod

In RAN4#115 it was agreed to select only one port mapping method for requirement definitions. The actual selection was postponed to RAN4#116 (see [1]):

Issue 1-2-13: portMappingMethod Agreement: - Select one for requirements. FFS on which one next meeting.

For each method, it is important to ensure at the transmitter that the CSI-RS ports are assigned in such a way that the order is compatible with the expected port ordering in the codebook.  This will ensure better performance of the codebook.  If only one port mapping method is used over all of the Type I single panel codebook requirements, we tend to prefer mapping method 1.  However, both methods may be tested by using different methods in different configurations (such as number of CSI-RS ports, TDD vs. FDD, etc.).  The advantage of this approach is that both methods can be verified without adding additional tests.

For Rel-19 typeI-SinglePanel-r19 requirements, use port mapping method 1 as a starting point if a single method is to be used for all requirements.

Number of slots for CSI-RS transmission

In RAN4#115 it was agreed to use one slot for 64 ports, however it required further analysis to decide on the number of slots for 128 ports (see [1]):

Issue 1-2-14: Number of slots for CSI-RS transmission Agreement: - One slot for 64 ports. FFS for 128 ports.

As already described in section 2.2.1 it will be required to use two slots with 128 CSI-RS ports to sustain a density of 1, hence we prefer using 2 slots for 128 ports.

Use 2 slots for 128 ports if requirements for 128 ports are introduced.

CQI/RI/PMI delay

In RAN4#115 the issue of CQI/RI/PMI delay was brought up (see [1]):

Issue 1-2-15: CQI/RI/PMI delay Way forward: - Option 1: RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing capabilities. - Option 2: Re-discuss the CSI-RS scheduling and CSI reporting pattern to match the capability 2 of timeline to support up to 128 CSI-RS ports

As the requirements needs to be supported by UEs independent on which capability is reported, we see option 1 as the best solution since we see it as it covering both capabilities.

RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing capabilities.

Test assumptions for eTypeII-r19 (if introduced)

In RAN4#115 several test assumptions were proposed if requirements are to be introduced for etypeII-r19 (see [1]):

Issue 1-3-1: Test metric Way forward: - Option 1: reuse the test metric for eType-II PMI reporting requirements, throughput ratio between the following PMI and random Rel-19 refined Type-I PMI Mode A. - Other options are not precluded Issue 1-3-2: Duplex mode Way forward: - Option 1: Both FDD and TDD - Other options are not precluded Issue 1-3-3: Number of Rx antennas Way forward: - Option 1: Both 2Rx and 4Rx - Other options are not precluded Issue 1-3-4: Propagation channel Way forward: - Option 1: TDLA30-5 - Other options are not precluded Issue 1-3-5: Correlation configuration Way forward: - Option 1: XP-Medium - Other options are not precluded Issue 1-3-6: typeII-ri-restriction-r19 Way forward: - Option 1: Rank2 - Other options are not precluded Issue 1-3-7: Number of CSI-RS ports Way forward: - Option 1: 64 ports - Other options are not precluded Issue 1-3-8: N1 and N2 configuration for 64 CSI-RS ports Way forward: - Option 1: (8,4) - Other options are not precluded Issue 1-3-9: (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports Way forward: - Option 1: K=2 - Other options are not precluded Issue 1-3-10: MCS level Way forward: - Option 1: MCS20, and RAN4 may consider higher MCS (code rate) for PDSCH FRC, if the required SNR value is too low to achieve 90% of the peak rate with the follow PMI. - Other options are not precluded Issue 1-3-11: numberOfPMI-SubbandsPerCQI-subband-r19 Way forward: - Option 1: set as 1 - Other options are not precluded Issue 1-3-12: paramCombination-r19 Way forward: - Option 1: set paramCombination-r19 as 6 (L =4, pν =1/2, β=1/2) as the starting point. - Other options are not precluded Issue 1-3-13: Other parameters Way forward: - Option 1: Reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline. - Other options are not precluded

We see the proposed configuration to be acceptable as a starting point if requirements are considered to be introduced for eTypeII-r19.

If requirements for eTypeII-r19 are decided to be introduced, use the following configuration as starting point:

| Configuration | Value |
| --- | --- |
| Test metric | Reuse the test metric for eType-II PMI reporting requirements, throughput ratio between the following PMI and random Rel-19 refined Type-I PMI Mode A |
| Duplex mode | Both FDD and TDD |
| Number of Rx antennas | Both 2Rx and 4Rx |
| Propagation channel | TDLA30-5 |
| Correlation configuration | XP-Medium |
| typeII-ri-restriction-r19 | Rank2 |
| Number of CSI-RS ports | 64 ports |
| N1 and N2 configuration for 64 CSI-RS ports | (8,4) |
| (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports | K=2 |
| MCS level | MCS20, and RAN4 may consider higher MCS (code rate) for PDSCH FRC, if the required SNR value is too low to achieve 90% of the peak rate with the follow PMI. |
| numberOfPMI-SubbandsPerCQI-subband-r19 | 1 |
| paramCombination-r19 | 6 |
| Other parameters | Reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline |

Test assumptions for typeII-Doppler-r19 (if introduced)

In RAN4#115 a number of test assumptions were proposed if requirements are to be introduced for typeII-Doppler-r19 (see [1]):

Issue 1-4-1: Duplex mode Way forward: - Option 1: Both FDD and TDD - Other options are not precluded Issue 1-4-2: Number of Rx antennas Way forward: - Option 1: Both 2Rx and 4Rx - Other options are not precluded Issue 1-4-3: Propagation channel Way forward: - Option 1: TDLA30-20 - Other options are not precluded Issue 1-4-4: Correlation configuration Way forward: - Option 1: XP-Medium - Other options are not precluded Issue 1-4-5: Number of CSI-RS ports Way forward: - Option 1: 64 ports - Other options are not precluded Issue 1-4-6: N1 and N2 configuration for 64 CSI-RS ports Way forward: - Option 1: (8,4) - Other options are not precluded Issue 1-4-7: (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports Way forward: - Option 1: K=2 - Other options are not precluded Issue 1-4-8: typeII-ri-restriction-r19 Way forward: - Option 1: Rank2 - Other options are not precluded Issue 1-4-9: numberOfPMI-SubbandsPerCQI-subband-r19 Way forward: - Option 1: set as 1 - Other options are not precluded Issue 1-4-10: number of configured resource groups Kdopp Way forward: - Option 1: The number of configured resource groups Kdopp set as 4 as the starting point. - Other options are not precluded

We see the proposed configuration to be acceptable as a starting point if requirements are considered to be introduced for typeII-Doppler-r19.  In addition, we propose to use the existing Rel-18 eType-II Doppler PMI reporting requirements as the baseline for other parameters.

If requirements for typeII-Doppler-r19 are decided to be introduced, use the following configuration as starting point:

| Configuration | Value |
| --- | --- |
| Duplex mode | Both FDD and TDD |
| Number of Rx antennas | Both 2Rx and 4Rx |
| Propagation channel | TDLA30-20 |
| Correlation configuration | XP-Medium |
| Number of CSI-RS ports | 64 ports |
| N1 and N2 configuration for 64 CSI-RS ports | (8,4) |
| (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports | K=2 |
| typeII-ri-restriction-r19 | Rank2 |
| numberOfPMI-SubbandsPerCQI-subband-r19 | 1 |
| number of configured resource groups Kdopp | 4 |

If requirements for typeII-Doppler-r19 are decided to be introduced, reuse the test parameters of the existing Rel-18 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline.

Test assumptions for CJT calibration (if introduced)

How to define CJT calibration offset reporting requirements

In RAN4#115 it was highlighted that further study would be needed on how to define CJT calibration offset reporting requirements (see [1]):

Issue 1-5-1: How to define CJT calibration offset reporting requirements Way forward: - Option 1: Further study how to define CJT calibration offset reporting requirements with focus on specific methodologies for FDD and TDD cases. Starting point shall be: - TDD channel acquisition (for TO, CFO and PO) - FDD CSI based channel acquisition (for TO and CFO). - Other options are not precluded

Basic Comparison TDD vs. FDD:

We have different type of alternatives regarding the tests that could carried out for CJT offset calibration when TDD and FDD schemes are considered in the design of a possible test:

- Case FDD: create a test for performance requirements only for FDD. We need firstly to determine whether the PMI is necessary in the sequence of steps.

- Case TDD: create a test for performance requirements only for TDD. We need to clearly determine the technical requirements for the uplink channel acquisition (i.e., SRS) at the testing equipment.

- Both cases FDD/TDD covered: both cases could be covered if there are no technical issues on the way how the CSI may be acquired either in DL or UL. However, a question is whether both type of tests are needed.

In the case with FDD scheme, the main question is whether we want to calculate a PMI with CJT eType II mode 2 accompanying the CJT offset calculation and the PDSCH compensation. Also as mentioned before, random PMI accompanying the CJT offset may be also a possibility.

A disadvantage of doing the test with FDD is that we would only be able to cover TO and CFO. PO needs necessarily to be implemented with the TDD scheme.
One advantage of the FDD case from the reference signal transmission point of view is that FDD requires only DL reference signals, more in particular TRS (for each TRP).
TDD on the other hand would be a bit more cumbersome as it would require uplink signals also.

Both using random PMI as well as follow PMI could be viable options and should be further analysed and justified. Simulations can be done by with both cases by evaluating the throughput gain of PDSCH compensated vs. PDSCH non-compensated.

A disadvantage with FDD case only is that we could only create requirements for CJT calibration offset with TO and CFO. TDD is required for CJT calibration offset with PO.
An advantage with the FDD case only is that only DL reference signals are required, i.e., TRS.

On the other hand, with TDD the question is about the technical issues related to the CSI obtained with uplink acquisition. Technical requirements to acquire the CSI by uplink (with SRS) must be discussed and defined. For that, it is necessary to evaluate by simulation the acceptable values of the transmitting power and SNR for uplink. The quality of the received signal is very important not only to acquire the CSI but also for the PO estimation. Noisy measurements are the main bottleneck for TDD and the CJT scenario is not the exception. As mentioned above, one of the main advantages of creating a test with TDD is that we may test all cases, i.e., TO, CFO and PO.

Finally, if the final goal is to test the offset compensation and the mechanism of CSI acquisition is not what it is being evaluated, a good approach may be to narrow down the test only to TDD case. Thus, we would not need to worry about using PMI or not in the test. However, also as discussed, the way how the CSI is obtained from UL needs to be studied in detail to determine the required parametrization. Moreover, as mentioned before in the TDD case we would need in addition to the UL reference signals (mostly for CSI acquisition), also DL reference signals to estimate the TO, CFO and PO offsets (i.e., single port CSI-RS for PO or TRS for TO and CFO). This makes the TDD test a bit more cumbersome, however in principle feasible.

The key part to be determined for the study for CJT calibration offsets with a TDD scheme is the mechanism and parameters (e.g., transmission power, SNR, etc) to implement the CSI based on uplink acquisition. Measurement cannot be noisy to avoid compromising not only the CSI acquisition but also other measurements like the PO estimation which depends on uplink also.

An advantage of creating a test with TDD is that we may test TO, CFO and PO. This may open the question whether we need also a test of requirements with FDD if the final goal is not studying the CSI acquisition but the effect of the offset compensation on the attainable throughput performance with PDSCH transmission.
A disadvantage of the case with TDD is that we need both DL and UL reference signals which makes the test a bit more cumbersome from the implementation point of view, however in principle feasible.

In the table below we provide a comparative of the things needed and/or the advantages/disadvantages by implementing a FDD or TDD based performance requirements test. Based on this comparison, we can support our discussion on the way forward, whether we want both tests or do we want to focus only in one, i.e., FDD or TDD.

Table 5. Comparison between FDD and TDD setups for a CJT calibration offsets performance requirement test

| Characteristics of the performance requirements test. | FDD | TDD |
| --- | --- | --- |
| Does it require a PMI calculation? | In principle, yes. | No. |
| Is it relevant to test Random PMI? | Yes. | No. |
| May it cover TO, CFO and PO? | Only TO, CFO. | TO, CFO, PO. |
| Does it require what type of reference signals? | DL reference signals only. | DL and UL reference signals. |
| Is it relevant to deeply study the characteristics of the UL reference signals? | No. | Yes, noisy UL reference signals are a major issue. |

Possible test implementation (or simulations) for TDD vs. FDD cases

Stimuli generator: the testing equipment counts with a stimuli generator. This is common for our proposal regarding FDD and TDD cases. This stimuli generator is used to update the offset to be added on top of the reference signals in a time occasion basis. If the test is done assuming 2 TRPs, only one of the TRPs has this offset introduced. Each time a new TRS/CSI-RS transmission that is triggered, it would correspond to a new time index. For the type of stimuli, we  propose using an offset function with input parameters including the time index. Based on the time index we have a certain offset input which could be a sequence, for instance, a ramp of a scalar (a single-phase value) or a sequence of phase ramp vectors (e.g. DFT vector of a codebook of phase ramps). The ramp is not necessarily soft like a straight line, it could have a staircase shape based on the number of measurements to test with a given offset with index i and same offset value could be measured during a time window i with one or more time indexes associated. The latter is illustrated in Figure 2.

- for instance, if we speak of CFO the stimuli added should correspond to a time variant complex exponential scalar value e.g.,  e-j∆ft  added in one of the TRPs (and assuming the other TRP is the anchor with reference carrier frequency, i.e. no offset).

-∆f correspond to the CFO and t is the transmission time occasion or time index. As noticed the phase value increases following a time ramp behaviour. This scalar value multiplies the reference signal r(t,n,p) at a time occasion t, a frequency component n (e.g. representing a subcarrier or PRB) and an antenna port p associated with the testing equipment interface that represents one of the TRPs carrying the frequency drift. The corresponding expression is then rt,n,p=r(t,n,p)e-j∆ft.
-This complex exponential multiplies equally the signal across full bandwidth for all N frequency components. To detect the variation due to e-j∆ft, it would require a set of TRS samples assuming fixed offset ∆f. A set of measurements during a time window makes it possible to detect the phase ramp due to e-j∆ft. The latter implies defining a number of TRS samples for each estimation window i with a fixed offset ∆fi to obtain enough measurements of the channel to detect the phase ramp produce by the given e-j∆fi t. Then in the next round of measurements we have a new window for new estimation i+1 with a new set of TRS samples with a new ∆fi+1 and in consequence a new measured ramp.

- For the case of TO, the delay associated to a given TRP with synchronization mismatch with respect to an anchor TRP, it is represented by the associated effect of the ith delay shift in the frequency domain, which is represented by a phase ramp vector vi= 1,  e-j2πiN,…,e-j2πi(N-1)N T. This phase ramp vector multiplies the reference signal for a given port  p in each transmission time occasion t across the whole bandwidth with N frequency components, i.e., rt=rt*vi , with  rt∈C 1×N. In order to carry out a sweeping of different delay shifts a codebook V with vi ∈ V is used to generate the ramp sequence.

- For the case of PO, it would be similarly to CFO but with a phase gap between the UL and DL reference signals corresponding to e-jϕ, with 0<ϕ≤2π applied in the downlink reference signal associated to one of the downlink ports p of one of the TRPs   rt,p=r(t,p)e-jϕ. This phase offset is not time variant like with CFO and we may assume it quite stable for the frequency bandwidth (i.e. wideband) as starting point. The sequence should be in the form of a phase ramp sweeping from 0 to 2π across a set of time windows.

Figure 2. Description of the ramp proposed to generate the stimuli on the reference signals from the test equipment.

Signalling diagram: We provide our views on the test for FDD but also for TDD more in detail to understand the differences on signalling. Descriptions are based on the flowcharts provided in Figure 3 and Figure 4.

FDD Case:

The sequence of steps is explained in the following based on the Figure 3.We assume two TRPs are used to test the UE.

Step 1:	The stimuli generator is initialized with input conditions. For example, the range in which offsets are going to be measured and the way values are going to be swept, for instance, using a ramp sequence.

Step 2:	First set of DL reference signals (e.g., TRS) is used to measure the offset. For that, the DL reference signals of one of the TRPs (testing equipment interface) are modified by adding a giving offset value on the top. The sum (+) symbol in the example should not be understood as a mathematical operator, but as representing the offset being added onto the reference signal. The offset could be represented generically by a term Oat that multiplies the reference signal rt to get as a result  rt=rt*Oat.

Step 3:	UE measures the offset and reports the offset based on a defined quantization step and quantization range (initial configuration steps for the UE reporting are omitted). We may call the reported offset as Oqt.

Step 4:	A second set of DL reference signals (i.e., CSI-RS) are used to calculate the CSI for a plurality of CSI-RS ports of the TRP set. Notice that this second set of reference signals, for one of the TRPs that is not the anchor or reference TRP (same one from step 2), must carry the same offset as in step 2.

Step 5:	UE reports the CSI with CJT eType II to the testing equipment for the two measured TRPs.

Step 6:	Based on the received CSI report (from step 5) and the reported offset (from step 3), the testing equipment may compensate the offset and apply the precoding obtained from the CSI report on PDSCH. Compensate the offset means removing the offset by multiplying by its own conjugate. Very generically speaking, if the actual transmitted signal (without compensation) in PDSCH is rnct=rt*Oa(t) for a time index t, the compensation consists of generating a compensated signal by multiplying by the conjugate of the reported offset, i.e., Oqt*, i.e. rct=rt*Oat*Oqt*. If Oat=Oq(t) there is  a perfect compensation of the offset. However, the compensation is not perfect due to quantization and measurement issues at the UE side, which may lead to some compensation error.

Step 7:	The baseline reference signal for comparison is transmitted to compare with the previous case (step 6) in which PDSCH had the offset compensated based on the UE report. The baseline reference signal could be perfectly compensated (no offset) or randomly compensated or no compensated at all (with an offset). In our view, these 3 alternatives are possible to create the test.

Step 8:	UE reports the CSI based on the baseline reference signal received in step 7.

Step 9:	With the received CSI report for the baseline reference signal, the testing equipment precodes the PDSCH based only on this CSI report without the compensation from the offset estimation (from step 3) made by the UE.

Step 10:	Finally, the comparison consists of comparing the PDSCH performance from the offset compensated case vs. the PDSCH as configured for the baseline.

Figure 3. Flowchart for the CJT offset performance test with FDD scheme (and PMI calculation).

TDD Case:

The sequence of steps is explained in the following based on the Figure 4. We assume again two TRPs are used to test the UE.

Step 1:	It is just the same as Step 1 in the FDD case.

Step 2:	Uplink signals (i.e., SRS) are sent from the UE toward the testing equipment. The UE reference signals may be used for CSI acquisition (or dedicated signals can be used for this in step 5) but mainly to support the offset estimation for the PO case. For TO and CFO there is no impact from these UL signals to estimate and/or compensate the offsets.

Step 3:	First set of DL reference signals (e.g., TRS) is used to measure the offset. For that, the DL reference signals of one of the TRPs (testing equipment interface) are modified by adding a giving offset value on the top. The signal processing is the same as in the step 2 from the FDD case.

Step 4:	UE measures the offset and reports the offset Oqt based on a defined quantization step and quantization range. This step is equivalent to the step 3 from the FDD case.

Step 5:	A second set of UL signals can be used dedicated for obtaining CSI or the same signals from step 2 can be used for the channel acquisition step.

Step 6:	Based on the acquired CSI with UL signals (from step 2 or 5) and the reported offset (from step 4), the testing equipment may compensate the offset and apply the precoding obtained from the CSI report on PDSCH in the same way as explained for the FDD case.

Step 7:	Based on the received CSI acquisition from the UL signals (from step 2 or 5), the testing equipment generates the baseline by precoding the PDSCH without the compensation from the offset estimation made by the UE but only uses the acquired CSI for precoding. Then, the testing equipment by itself could apply the offset (non-compensated case), apply the PDSCH without any offset (perfectly compensated case) or generate a random compensation at PDSCH in order to generate the baseline case.

Step 8:	Finally, the comparison consists of comparing the PDSCH performance from the offset compensated case vs. the PDSCH as configured for the baseline.

Figure 4. Flowchart for the CJT offset performance test with TDD scheme.

As seen from the Figure 3 and Figure 4, FDD case contains more steps than the TDD case (due to the CSI acquisition), and in that sense TDD may sound more attractive for a potential test. We are interested only in studying the offset compensation, not the CSI acquisition; the CSI acquisition at best is only an enabler for having reasonable throughput gains. On the other hand, the need of UL and DL signals in TDD may include more complexity to the test as UL signals should be taken into account in order to design the test as CSI acquisition depends on it. In our view, the FDD case with follow PMI sounds like the best option due to the capability to have realistic throughput values. Alternatively random PMI could be considered as well in order to further simplify the framework. TDD case shows some advantages as mentioned above, however, the fact that the characteristics of UL signal are needed to be considered in the test design may be a difficulty, so for that reason we have it as the lower priority case. Nonetheless, we are open to any of the described options.

For FDD there is no need to deal with UL signal acquisition issues, hence it is the preferred option given the time available.

We see using follow PMI to provide the most realistic throughput values during the test but are open to consider random PMI in case follow PMI is found not feasible.

As a starting point consider testcases with FDD. Consider follow PMI as first priority and include random PMI in the analysis as second priority.

Whether the PMI calculation is necessary for the CJT calibration performance requirements

In RAN4#115 it was brought up, if PMI calculation is necessary for the CJT calibration performance requirements (see [1])

Issue 1-5-2: Whether the PMI calculation is necessary for the CJT calibration performance requirements Way forward: - Option 1: Further study whether the PMI calculation is necessary for the CJT calibration performance requirements studies in the FDD case by simulating the following cases: 1) CJT with CJT PMI and with CJT calibration 2) CJT with CJT PMI and without CJT calibration 3) CJT with random PMI and with CJT calibration 4) CJT with random PMI and without CJT calibration 5) sTRP eType-II PMI. - Other options are not precluded

As starting point we provide some initial simulations by comparing the follow and random PMI precoding with different cases for TO, CFO with offset compensated/not-compensated PDSCH. We use MCS level 13, 4 Rx antenna at the UE with transmission limited to single rank (rank 1), 2 TRP transmission each TRP with 8 transmission ports. For CSI we use Rel. 18 eType II mode 2. Other parameters correspond in this simulation to the Rel. 18 assumptions used for PMI requirements.

Regarding the CJT calibration parameters they were reused from the current studies carried out in RRM core. For CFO we consider a central carrier frequency at 3.5GHz with a fixed CFO corresponding to 0.01ppm (350Hz). CFO is quantized with maximum quantization range AFO=0.02ppm and quantization resolution equal to MFO= 256 steps (8 bits). On the other hand, for TO the delay is fixed to 0.5CP = 2.345μs (microseconds). The maximum quantization range is  ATO=CP and quantization resolution equal to MTO= 256 steps.

Follow and Random PMI precoding case with separate and joint TO and CFO calibration offsets reporting:

For TO and CFO, we analysed both offsets simulated and measured separately (Figure 5 with TO and Figure 6 with CFO, respectively) and also the case for joint with TO/CFO (Figure 7) was considered. We notice that both random and follow PMI when the TO and/or CFO are compensated, they have little difference with the baseline, i.e., upper bound with no offset(s) or ideal compensation. This result may lead apriori to think that using the ideal case as baseline is not a good option for the designed test, as the gain gap is quite reduced. In contrast, the gain is quite clear and noticeable when then comparison is made against the non-compensated case. At first glimpse, the non-compensated case could be a good candidate for being the actual baseline.

On the other hand, when follow PMI is compared vs. random PMI in terms of feasibility as mechanism for CSI acquisition, we may notice that both provides gains with respect to the TO, CFO and TO/CFO non-compensated cases, then both may fit well as part of the test. In our view, we would prefer to use the follow PMI because it provides realistic throughput results, however we also see random PMI as a valid option based on our first evaluation.

| (a) | (b) |
| --- | --- |

Figure 5. Follow (a) and Random (b) PMI precoding case with TO calibration offsets reporting and with/without TO compensation (TOC)

| (a) | (b) |
| --- | --- |

Figure 6. Follow (a) and Random (b) PMI precoding case with CFO calibration offsets reporting and with/without FO compensation (FOC)

Figure 7. Follow (a) and Random (b) PMI precoding case with joint TO/CFO calibration offsets reporting and with/without TO/FO compensation (TOC/FOC)

Based on our first results the upper bound baseline (no offset or ideal compensation) does not look as a good candidate for being baseline of the CJT offset calibration reporting test. The gain gap is quite narrow with respect to the compensated case under test, for both cases with follow and random PMI.

Comparison of the compensated case vs. non-compensated for both random and follow PMI provides a good gain gap and is a good baseline for the CJT offset calibration reporting test.

The simulations presented in this section are only a starting point. Further simulations with follow PMI and random PMI should be carried out and evaluated. We suggest using same simulation parameters for Rel. 18 CJT PMI requirements as starting point. For the moment, MCS level 13 could be used but other values could be considered also. In addition, we suggest simulating for 2Rx and 4Rx cases. CJT calibration setup may be reused from the studies done in RRM core (with fixed offset) and further improved, e.g. including a ramp to sweep the offset for the stimuli generation.

Companies to provide simulations to study the behaviour of random and follow PMI. As starting point use 2 TRPs with 8 transmission ports each, MCS level 13 (other values may be also discussed), 2Rx/4Rx antennas at the UE. Remaining PMI parameters shall be reused from the PMI requirement for Rel. 18 eType II CJT. CJT calibration parameters shall be reused from the accuracy studies carried out in RRM core.

Test metric for CJT

In RAN4#115 the test metric for CTJ was brought up (see [1]):

Issue 1-5-3: Test metric for CJT Way forward: - Option 1: Further study the metrics to be used in a CJT calibration offsets reporting requirements. Starting point shall be one of the following options: A) Comparison of the compensated PDSCH performance based on the reported offsets measured by UE vs. random compensation of PDSCH. B) Comparison of the PDSCH performance based on the reported offsets measured by UE vs. the non-compensated PDSCH. C) Comparison of the PDSCH performance based on the reported offsets measured by UE vs. ideally compensated PDSCH. - Other options are not precluded

In order to evaluate the quality of the reports from the UE we need to evaluate the throughput metric based on the compensation of PDSCH when the testing equipment uses the estimated and reported offsets provided by the UE. Different baselines could be considered to determine the resulting γ metrics. The necessary evaluations include the following comparisons vs. a baseline:

- Comparison of the compensated PDSCH performance based on the reported offsets measured by UE vs. random compensation of PDSCH.

- Comparison of the compensated PDSCH performance based on the reported offsets measured by UE vs. the non-compensated PDSCH.

- Comparison of the compensated PDSCH performance based on the reported offsets measured by UE vs. ideally compensated PDSCH.

There is not any apparent issue of selecting among cases A, B or C. Based on our first simulations the ideally compensated case is not best for a baseline as the gain gap is quite narrow. In contrast, the non-compensated case seems to be more reasonable for the moment. In any case, the most urgent issue before determining the metric is determining the mechanism of channel acquisition that we should introduce to make the test feasible, i.e., follow PMI, random PMI only or UL CSI (TDD case). In principle, any case is likely to work if we use follow PMI or UL CSI as the testing equipment have all necessary information. By doing this we may have a PDSCH which was both compensated and precoded with realistic information (i.e., which is consistent with the channel conditions) from the UE and/or based on signals transmitted by the UE. The random PMI seems at the same time a good and simple enough option.

Decide the type of CSI acquisition (follow PMI, random PMI, UL CSI acquisition) before deciding the type of baseline used for the test metric.

Discussion: BS demodulation performance requirements

General Scope

Clarify if demodulation requirements are needed for 3-antenna-port codebook-based UL transmissions

Issue 1-1-1: Clarify if demodulation requirements are needed for 3-antenna-port codebook-based UL transmissions Way forward: - Option 1: RAN4 introduce PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions - Option 2: RAN4 do not introduce requirements for 3-antenna-port codebook-based UL transmissions

As the introduction of new 3Tx codebooks has been part of the MIMO work item in RAN1, Nokia firmly believes that it would be worthwhile to introduce performance requirements for these in RAN4.

RAN4 shall introduce PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions

Test assumptions for PUSCH demodulation requirements (if introduced)

Duplex mode

Issue 2-2-1: Duplex mode Way forward: - Option 1: both FDD and TDD - Other options are not precluded

As per the codebook may be used in bands that support FDD and TDD, Nokia believes that requirements could be defined for both FDD and TDD; however noting that we may want to limit test cases, this may also be a fruitful discussion to have regarding which is preferential over another.

RAN4 may define requirements for both FDD and TDD

Number of Rx antennas

Issue 2-2-2: Number of Rx antennas Way forward: - Option 1: both 4Rx and 8Rx - Other options are not precluded

As a 3Tx UL codebook may be received on a Base Station supporting both 4 and 8 Rx requirements should be defined for both however noting that we may want to limit test cases, this may also be a fruitful discussion to have regarding which is preferential over another.

RAN4 may define requirements for both 4Rx and 8Rx

SCS and Channel BW for FDD

Issue 2-2-3: SCS and CHBW for FDD Way forward: - Option 1: 15kHz SCS, 5MHz, 50MHz - Option 2: 15kHz SCS, 5MHz, 20MHz If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage - Other options are not precluded

Nokia believes that for FDD, we should aim for min and max channel BW assessment; such that 5 MHz and 50 MHz channels should have requirements defined.

In reality we expect performance to be broadly similar for all channel BWs.

RAN4 shall define requirements for 15kHz SCS, 5MHz, 50MHz for TDD PUSCH

SCS and Channel BW for TDD

Issue 2-2-4: SCS and CHBW for TDD Way forward: - Option 1: 30kHz SCS, 10MHz, 100MHz - Other options are not precluded

Nokia believes that for TDD, we should aim for min and max channel BW assessment; such that 5 MHz and 100 MHz channels should have requirements defined.

In reality we expect performance to be broadly similar for all channel BWs.

RAN4 shall define requirements for 30kHz SCS, 5MHz, 100MHz for FDD PUSCH

Number of layers

Issue 2-2-5: Number of layers Way forward: - Option 1: only 3 layers with TPMI index 0 - Other options are not precluded

For the number of layers, the maximum number should be used to test the codebook, in this case 3. As a correlation channel isn’t being used the TPMI is arbitrary, so index 0 is a sensible choice.

RAN4 Shall use 3 layers with TPMI index 0 for PUSCH requirements

Number of DMRS symbols

Issue 2-2-6: Number of DMRS symbols Way forward: - Option 1: 1+1 - Other options are not precluded

Regarding DMRS symbols, there is little need to vary from 1+1, so Nokia believes this should be used for definition of requirements

RAN4 shall use DMRS symbols 1+1

DMRS ports

Issue 2-2-7: DMRS ports Way forward: - Option 1: Rel-15 DM-RS port as {0,1,2} - Other options are not precluded

Regarding DMRS ports, as 3Tx will require 3 ports it seems a reasonable approach to use {0,1,2}.

RAN4 shall use DMRS ports {0,1,2}

Propagation channel and MCS level

Issue 2-2-8: Propagation channel and MCS level Way forward: - Option 1: - TDLB100-400, MCS 2 - TDLA30-10, MCS 17 - TDLC300-100, MCS 12 - Option 2: - TDLB100-400, MCS 2 - TDLA30-10, MCS 20 - TDLC300-100, MCS 16 Companies are also encouraged to assess the feasibility of using higher MCS values for each modulation level. - Other options are not precluded

Nokia would prefer to take a slightly lower MCS to provide a reasonable SNR operating point, however following simulation alignment this may be left open if a higher MCS can also be accommodated.

Thus, initially we would support:

-	TDLB100-400, MCS 2

-	TDLA30-10, MCS 17

-	TDLC300-100, MCS 12

RAN4 shall use the following Propagation and MCS levels (TDLB100-400, MCS2; TDLA30-10, MCS 17; TDLC300-100, MCS 12)

PUSCH mapping type

Issue 2-2-9: PUSCH mapping type Way forward: - Option 1: both mapping type A and type B - Option 2: only mapping type A - Other options are not precluded

For PUSCH mapping type, it would be preferential to leave both mapping types open until further discussion and simulation alignment.

RAN4 shall use both mapping type A and type B

Conclusion

The following Observations and Proposals were made:

Discussion: UE demodulation/CSI reporting performance requirements

General

Clarify if PMI reporting requirements are needed for eTypeII-r19 and eTypeII-Doppler-r19

- The extended eTypeII and eTypeII Doppler codebooks have been implemented by adding support for larger port counts without structural changes to the codebooks.  UE implementations of these codebooks are likely to be extensions of the legacy codebook implementations.

- For Rel-19 CSI reporting extensions, introduce PMI reporting requirements for the extended eTypeII codebook.

For Rel-19 CSI reporting extensions, do not introduce PMI reporting requirements for the extended eTypeII Doppler codebook.

Clarify if UE demodulation requirements are needed for UE reporting enhancement for CJT

Do not introduce demodulation requirements for reporting enhancements for CJT deployments. The QCL assumptions related to Scheme C/Scheme D/Scheme E shall be implicitly part of the CJT calibration offset reporting requirements if introduced.

Clarify if CSI reporting requirements are needed for UE reporting enhancements for CJT

RAN4 to further discuss the necessary details for a new test framework of performance requirements for the CJT calibration offsets, i.e., TO, CFO and PO. Part of the work would be to determine whether random PMI can be used as viable/reliable option, or the follow PMI is required during the CJT test.

Test assumptions for typeI-SinglePanel-r19

Number of CSI-RS ports

The four 32-port CSI-RS resources required for 128 ports cannot be configured in a single slot using resources with density 1.

Two alternatives for supporting PMI reporting requirements for 128 CSI-RS ports:
- Configure 128 CSI-RS ports in a single slot using four CSI-RS resources with density ½
- For UEs which support 128 ports and aggregation of CSI-RS resources from two slots, configure 128 CSI-RS ports with four CSI-RS resources with density 1 over two slots.

For Rel-19 typeI-SinglePanel-r19, introduce PMI reporting requirements for 128 CSI-RS ports for UEs which support aggregation of CSI-RS resources from two slots.  Configure the 128 CSI-RS ports using four CSI-RS resources with density 1 over two slots.

Correlation configuration

The previous agreement to further extend the adhoc RAN4 antenna correlation framework results in severe underestimation of channel spatial diversity increase from increasing CSI-RS ports.

In TS 38.101-4, extend the application of the parameters in Table B.2.3.1.2-1 to ULA MIMO correlation matrices for more than 32 ports.  Apply the parameters in Table B.2.3.2.2-1 to cross-polarized antennas for more than 32.

In TS 38.101-4, adopt the following table of scaling factors for MIMO correlation matrices for more than 32 transmit ports, covering both ULA and cross-polarized arrays.

| Array Type | Antenna Configuration Tx (N1, N2, P) × Rx | Correlation Level | Scaling Factor a |
| --- | --- | --- | --- |
| Cross-polarized | 48 (8,3,2) × 2 | High | 0.00030 |
| Cross-polarized | 48 (8,3,2) × 4 | High | 0.00057 |
| Cross-polarized | 48 (6,4,2) × 2 | High | 0.00030 |
| Cross-polarized | 48 (6,4,2) × 4 | High | 0.00057 |
| Cross-polarized | 64 (8,4,2) × 2 | High | 0.00040 |
| Cross-polarized | 64 (8,4,2) × 4 | High | 0.00076 |
| Cross-polarized | 64 (16,2,2) × 2 | High | 0.00039 |
| Cross-polarized | 64 (16,2,2) × 4 | High | 0.00074 |
| Cross-polarized | 128 (16,4,2) × 2 | High | 0.00080 |
| Cross-polarized | 128 (16,4,2) × 4 | High | 0.00151 |
| Cross-polarized | 128 (8,8,2) × 2 | High | 0.00080 |
| Cross-polarized | 128 (8,8,2) × 4 | High | 0.00152 |
| Cross-polarized | 48 (8,3,2) × 2 | Medium | 0.00019 |
| Cross-polarized | 48 (8,3,2) × 4 | Medium | 0.00036 |
| Cross-polarized | 48 (6,4,2) × 2 | Medium | 0.00020 |
| Cross-polarized | 48 (6,4,2) × 4 | Medium | 0.00037 |
| Cross-polarized | 64 (8,4,2) × 2 | Medium | 0.00026 |
| Cross-polarized | 64 (8,4,2) × 4 | Medium | 0.00050 |
| Cross-polarized | 64 (16,2,2) × 2 | Medium | 0.00023 |
| Cross-polarized | 64 (16,2,2) × 4 | Medium | 0.00043 |
| Cross-polarized | 128 (16,4,2) × 2 | Medium | 0.00053 |
| Cross-polarized | 128 (16,4,2) × 4 | Medium | 0.00101 |
| Cross-polarized | 128 (8,8,2) × 2 | Medium | 0.00055 |
| Cross-polarized | 128 (8,8,2) × 4 | Medium | 0.00104 |
| ULA | 48 (48,1,1) × 2 | High | 0.00090 |
| ULA | 48 (48,1,1) × 4 | High | 0.00184 |
| ULA | 64 (64,1,1) × 2 | High | 0.00120 |
| ULA | 64 (64,1,1) × 4 | High | 0.00245 |
| ULA | 128 (128,1,1) × 2 | High | 0.00239 |
| ULA | 128 (128,1,1) × 4 | High | 0.00489 |
| ULA | 48 (48,1,1) × 2 | Medium | 0.00077 |
| ULA | 48 (48,1,1) × 4 | Medium | 0.00157 |
| ULA | 64 (64,1,1) × 2 | Medium | 0.00103 |
| ULA | 64 (64,1,1) × 4 | Medium | 0.00209 |
| ULA | 128 (128,1,1) × 2 | Medium | 0.00205 |
| ULA | 128 (128,1,1) × 4 | Medium | 0.00419 |

For Rel-19 typeI-SinglePanel-r19, adopt a cross-polarized antenna configuration with high correlation as the starting point.

N1 and N2 configuration for 64 CSI-RS ports

For Rel-19 typeI-SinglePanel-r19 requirements with 64 CSI-RS ports, use an N1, N2 configuration of 8,4 (Option 2).

portMappingMethod

For Rel-19 typeI-SinglePanel-r19 requirements, use port mapping method 1 as a starting point if a single method is to be used for all requirements.

Number of slots for CSI-RS transmission

Use 2 slots for 128 ports if requirements for 128 ports are introduced.

CQI/RI/PMI delay

RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing capabilities.

Test assumptions for eTypeII-r19 (if introduced)

If requirements for eTypeII-r19 are decided to be introduced, use the following configuration as starting point:

| Configuration | Value |
| --- | --- |
| Test metric | Reuse the test metric for eType-II PMI reporting requirements, throughput ratio between the following PMI and random Rel-19 refined Type-I PMI Mode A |
| Duplex mode | Both FDD and TDD |
| Number of Rx antennas | Both 2Rx and 4Rx |
| Propagation channel | TDLA30-5 |
| Correlation configuration | XP-Medium |
| typeII-ri-restriction-r19 | Rank2 |
| Number of CSI-RS ports | 64 ports |
| N1 and N2 configuration for 64 CSI-RS ports | (8,4) |
| (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports | K=2 |
| MCS level | MCS20, and RAN4 may consider higher MCS (code rate) for PDSCH FRC, if the required SNR value is too low to achieve 90% of the peak rate with the follow PMI. |
| numberOfPMI-SubbandsPerCQI-subband-r19 | 1 |
| paramCombination-r19 | 6 |
| Other parameters | Reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline |

Test assumptions for typeII-Doppler-r19 (if introduced)

If requirements for typeII-Doppler-r19 are decided to be introduced, use the following configuration as starting point:

| Configuration | Value |
| --- | --- |
| Duplex mode | Both FDD and TDD |
| Number of Rx antennas | Both 2Rx and 4Rx |
| Propagation channel | TDLA30-20 |
| Correlation configuration | XP-Medium |
| Number of CSI-RS ports | 64 ports |
| N1 and N2 configuration for 64 CSI-RS ports | (8,4) |
| (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports | K=2 |
| typeII-ri-restriction-r19 | Rank2 |
| numberOfPMI-SubbandsPerCQI-subband-r19 | 1 |
| number of configured resource groups Kdopp | 4 |

If requirements for typeII-Doppler-r19 are decided to be introduced, reuse the test parameters of the existing Rel-18 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline.

Test assumptions for CJT calibration (if introduced)

How to define CJT calibration offset reporting requirements

Both using random PMI as well as follow PMI could be viable options and should be further analysed and justified. Simulations can be done by with both cases by evaluating the throughput gain of PDSCH compensated vs. PDSCH non-compensated.

A disadvantage with FDD case only is that we could only create requirements for CJT calibration offset with TO and CFO. TDD is required for CJT calibration offset with PO.
An advantage with the FDD case only is that only DL reference signals are required, i.e., TRS.

The key part to be determined for the study for CJT calibration offsets with a TDD scheme is the mechanism and parameters (e.g., transmission power, SNR, etc) to implement the CSI based on uplink acquisition. Measurement cannot be noisy to avoid compromising not only the CSI acquisition but also other measurements like the PO estimation which depends on uplink also.

An advantage of creating a test with TDD is that we may test TO, CFO and PO. This may open the question whether we need also a test of requirements with FDD if the final goal is not studying the CSI acquisition but the effect of the offset compensation on the attainable throughput performance with PDSCH transmission.
A disadvantage of the case with TDD is that we need both DL and UL reference signals which makes the test a bit more cumbersome from the implementation point of view, however in principle feasible.

For FDD there is no need to deal with UL signal acquisition issues, hence it is the preferred option given the time available.

We see using follow PMI to provide the most realistic throughput values during the test but are open to consider random PMI in case follow PMI is found not feasible.

As a starting point consider testcases with FDD. Consider follow PMI as first priority and include random PMI in the analysis as second priority.

Whether the PMI calculation is necessary for the CJT calibration performance requirements

Based on our first results the upper bound baseline (no offset or ideal compensation) does not look as a good candidate for being baseline of the CJT offset calibration reporting test. The gain gap is quite narrow with respect to the compensated case under test, for both cases with follow and random PMI.

Comparison of the compensated case vs. non-compensated for both random and follow PMI provides a good gain gap and is a good baseline for the CJT offset calibration reporting test.

Companies to provide simulations to study the behaviour of random and follow PMI. As starting point use 2 TRPs with 8 transmission ports each, MCS level 13 (other values may be also discussed), 2Rx/4Rx antennas at the UE. Remaining PMI parameters shall be reused from the PMI requirement for Rel. 18 eType II CJT. CJT calibration parameters shall be reused from the accuracy studies carried out in RRM core.

Test metric for CJT

Decide the type of CSI acquisition (follow PMI, random PMI, UL CSI acquisition) before deciding the type of baseline used for the test metric.

Discussion: BS demodulation performance requirements

General Scope

Clarify if demodulation requirements are needed for 3-antenna-port codebook-based UL transmissions

RAN4 shall introduce PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions

Test assumptions for PUSCH demodulation requirements (if introduced)

Duplex mode

RAN4 may define requirements for both FDD and TDD

Number of Rx antennas

RAN4 may define requirements for both 4Rx and 8Rx

SCS and Channel BW for FDD

RAN4 shall define requirements for 15kHz SCS, 5MHz, 50MHz for TDD PUSCH

SCS and Channel BW for TDD

RAN4 shall define requirements for 30kHz SCS, 5MHz, 100MHz for FDD PUSCH

Number of layers

RAN4 Shall use 3 layers with TPMI index 0 for PUSCH requirements

Number of DMRS symbols

RAN4 shall use DMRS symbols 1+1

DMRS ports

RAN4 shall use DMRS ports {0,1,2}

Propagation channel and MCS level

RAN4 shall use the following Propagation and MCS levels (TDLB100-400, MCS2; TDLA30-10, MCS 17; TDLC300-100, MCS 12)

PUSCH mapping type

RAN4 shall use both mapping type A and type B

- References

- R4-2508627-Way Forward for [115][329] NR_MIMO_demod.

- R4-2506442, “Discussion on UE demodulation performance and CSI reporting requirements for MIMO_Ph5,” Nokia, 3GPP TSG RAN4#115, St. Julian’s, Malta, 19-23 May 2025.

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)