3GPP TSG-RAN WG4 Meeting #116	R4-2510654

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on performance requirements for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	7.21.5

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#115, and the outcomes are captured in WF [1]. Based on the work plan, RAN4 should continue the performance part of the WI in this meeting.

In this paper, we will provide our views on RRM performance requirements for SBFD.

Discussion

Accuracy requirements

Based on the core part discussion, RAN4 needs to define accuracy requirements for L1 LCI measurement including L1-SRS-RSRP and L1-CLI-RSSI. For L1 and L3 measurements, existing accuracy requirements should apply except for one case, i.e. L1-RSRP and L1-SINR for which RAN4 will define relaxed accuracy for DUD Case 3.

L1-SRS-RSRP

In RAN4#114 the following is agreed.

- Agreements - Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition. - For SRS Es/Iot, further discuss the following options - Option 1: 1dB - Option 2: > 1dB - For SRS BW, further discuss the following options - Option 1: 24RB - Option 2: 48RB

For the timing offset, the open issue is whether to account for the 3us cell phase error or not. Below are our main consideration factors:

- Cell phase error is considered for inter-cell scenario. Based on RAN1 LS, inter-cell scenario is supported for L1 CLI measurement, but intra-cell scenario can be prioritized for defining the requirements.

- It is preferred to define a single set of accuracy requirements based on a single set of side conditions.

- The measurement performance is worse with cell phase error, and the degradation depends on SCS.

Based on those considerations, we support the view from operator that different timing offset can be used for different SCS, i.e.

- when the performance loss with cell phase error is small, the timing offset accounts the cell phase error so that the requirements can fully cover inter-cell scenario, and

- when the performance loss with cell phase error is large, the timing offset does not account the cell phase error so that the requirements are reasonable for prioritized intra-cell scenario.

In Table 1 we show the performance difference between Case 1 (with cell phase error) and Case 2 (without cell phase error) for all SCS-es based on our simulation results, for 1dB Es/Iot and 24RB.

Table 1: Comparison between Case 1 (with cell phase error) and Case 2 (without cell phase error)

|  | AWGN |  | TDL-C |  |
| --- | --- | --- | --- | --- |
| SCS | Case 1 | Case 2 | Case 1 | Case 2 |
| 15 | 1.16 | 0.68 | 1.68 | 1.05 |
| 30 | 1.99 | 0.96 | 2.37 | 1.33 |
| 60 (FR1) | 3.67 | 1.49 | 4.15 | 1.88 |
| 60 (FR2) | 2.67 | 0.9 | 3.3 | 1.35 |
| 120 | 6.52 | 1.24 | 6.65 | 1.84 |

It can be seen that the performance loss is around 0.5dB for 15kHz and 1dB for 30kHz, and above 2dB for 60kHz and 120kHz. Therefore, we suggest to define the timing offset with cell phase error for 15kHz and 30kHz, and without cell phase error for 60kHz and 120kHz.

It is noted that regardless of value for the timing offset condition, the accuracy requirements apply for both intra-cell and inter-cell scenarios, as long as the side condition is met. For example, in inter-cell scenario, if the offset between received timing and SRS arrival timing is smaller than 0.67us (Case 2) in FR2, the accuracy requirement should also apply.

On SRS RB number, we suggest to use 24 RB for the side condition based on discussions in RAN4#114. The reason is that the measurement is performed within UL subband which is only part of CBW, and in some gNB configurations the BW of UL subband may be smaller than 48 RB especially for DUD case.

On Es/Iot, we suggest to re-use the R16 side condition of 1dB. Based on our results, the gain of 3dB over 1dB Es/Iot is not much, so it is meaningful to keep using 1dB to ensure the requirement coverage.

Proposal 1: Define L1-SRS-RSRP accuracy requirements based on the following side conditions

- Timing offset

  - 15kHz and 30kHz: with cell phase error

  - 60kHz and 120kHz: without cell phase error

  - The requirements apply for both intra-cell and inter-cell scenarios

- SRS Es/Iot >= 1dB

- SRS BW >= 24 RB

The exact accuracy numbers should be defined based on simulation results for the agreed side conditions. The sample number should be 1 based on RAN4#114 agreement.

Proposal 2: Define L1-SRS-RSRP accuracy requirements based on simulation results for the agreed side conditions.

L1-CLI-RSSI

For RSSI measurements, there is no signal power estimation at baseband, so the accuracy is mainly based on RF calibration error. This has been the assumption for all RSSI measurements defined so far. In this sense, it is straightforward to re-use R16 accuracy requirements for L3 CLI-RSSI.

Proposal 3: R16 accuracy requirements for L3 CLI-RSSI are re-used for L1-CLI-RSSI.

CSI-RS based L1-RSRP/L1-SINR

In RAN4#115 following is agreed for L1 CSI-RS measurement.

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability.

In [1] the simulation assumption for evaluating L1-RSRP and L1-SINR accuracy performance with 24 RB is approved. In [2] we provide our simulation results, and we observe that the performance difference between 24 RB and 48 RB is around 0.5dB, and for some configuration of L1-SINR there is even no degradation. Therefore, we suggest to define the accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB).

Proposal 4: Define the accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB).

Report mapping

In RAN4#114-bis following is agreed for UE L1 CLI report mapping.

- Agreement (online session on Monday) - Confirm that the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI, including both absolute and differential reporting, given by RAN1 LS R1-2501620, are reasonable. - Note: In RAN4 understanding, the “Infinite” in Table 10.1.22.1.2-1, TS38.133 is also reused for L1 SRS-RSRP. - RAN4 to define measurement report mapping for L1-SRS-RSRP and L1-CLI-RSSI, including both absolute reporting and differential reporting.

For absolute reporting, it is straightforward to re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for L1-SRS-RSRP and L1-CLI-RSSI, respectively.

For differential reporting, RAN1 WA is as follows.

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

As RAN1 WA which confirmed by RAN4, it is straightforward to re-use Table 10.1.6.1-2 and change the label to “DIFF_SRS-RSRP” and “DIFF_CLI-RSSI”.

Table 10.1.6.1-2: Differential SS-RSRP and CSI-RSRP measurement (for L1 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFFRSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_15 | -30≥ΔRSRP | dB |

Proposal 5: Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 6: Use Table 2 and Table 3 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Table 2: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 3: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

Test cases

General setup

So far, most of the RRM tests in FR1 are defined with following 3 test configurations. They are however not suitable for SBFD as the BW is too small.

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

The maximum CBW for 15kHz SCS is 50MHz, and in real deployment it is usually smaller, so we suggest not to test 15kHz SCS for SBFD. For 30kHz SCS, we suggest to use 100MHz CBW.

So far, most of the RRM tests in FR2 are defined with following test configuration. In our view, it is fine to re-use it for SBFD tests.

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Proposal 7: Define SBFD RRM tests for the following test configurations.

| Config | Description |
| --- | --- |
| FR1 | NR 30 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| FR2 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

For FR1 we have the following TDD configurations for 30kHz. In our view, they are not suitable for SBFD due to small DL ratio. SBFD is more suitable for DL heavy TDD configuration.

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 | TDDConf.2.2 | TDDConf.2.3 |
| referenceSubcarrierSpacing | kHz | 30 | 30 | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ | ‘1D1S2U’ S=’11DL: 1GP:2UL’ | ‘3D1S4U’ S=’4DL:4GP:6UL’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 2 | 4 |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ | Not configured | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 1 | Not configured | 1 |

Therefore, we suggest to introduce a new TDD configuration for 30kHz as below. It is same as existing TDDConf.3.1 for 120kHz with scaling.

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TDDConf.2.4 |
| referenceSubcarrierSpacing | kHz | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S1U’ S=’10DL:2GP:2UL’ |
| dl-UL-TransmissionPeriodicity | ms | 2.5 |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |
| dl-UL-TransmissionPeriodicity | ms | Not configured |

For 120kHz, the existing TDDConf.3.1 can be re-used.

Proposal 8: Introduce TDDConf.2.4 (DDDSU) for 30kHz for SBFD RRM tests.

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TDDConf.2.4 |
| referenceSubcarrierSpacing | kHz | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S1U’ S=’10DL:2GP:2UL’ |
| dl-UL-TransmissionPeriodicity | ms | 2.5 |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |
| dl-UL-TransmissionPeriodicity | ms | Not configured |

RAN4 needs to define the time frequency configuration for SBFD RRM tests.

For frequency domain, one issue is whether DU or DUD should be used. Our view is that both needs to be tested, and for requirements that are applicable for both setups, we can use DU to simply the test; for DUD specific requirements, we should use DUD. In our view, DUD specific requirements include 1) L1-CLI-RSSI measurement Method 1 and resource type 2, and 2) L1-RSRP/SINR measurement accuracy based on 24 RB per DL subband.

Proposal 9: Introduce 2 SBFD configurations for DU and DUD

- For DUD specific requirements, use DUD configuration

- For other requirements, use DU configuration

For time domain, we suggest to use DXXXU, i.e. in TDD configuration of DDDSU, the first DL slot is kept as DL slot, and the other DL and special slot are configured as SBFD slot, i.e. all D and F symbols in those slots are configured as SBFD symbol.

Proposal 10: Use DXXXU as the SBFD configuration for RRM tests, where X denotes SBFD slot with all D and F symbols in the slots configured as SBFD symbol.

Test case list

The impacted core requirements by the WI mainly include

- L1-SRS-RSRP measurement

- L1-CLI-RSSI measurement

- CSI-RS based RLM

- CSI-RS based BFR

- CSI-RS based L1-RSRP measurement

- CSI-RS based L1-SINR measurement

- CSI-RS based L3 measurement

We suggest to define test cases for all except 6) and 7). For 6) the procedure itself and the impact of SBFD for L1-SINR are very similar to L1-RSRP, so it is sufficient to test only 5). For 7) our understanding is that UE behavior and requirements are same as legacy, but it is up to NW to ensure proper resource configuration so there is no need to test UE performance. In addition, RAN4 would define accuracy requirements for L1-SRS-RSRP, L1-CLI-RSSI and L1-RSRP, which also need to be tested.

For L1-CLI-RSSI, there are multiple configurations including method 3, method 1 resource type 1 and method 1 resource type 2. We suggest to test both method 3 and method 1 resource type 2 as they require different UE implementations.

Proposal 11: RAN4 to define the following list of test cases for SBFD RRM.

TC1: L1-SRS-RSRP measurement delay FR1

TC2: L1-SRS-RSRP measurement delay FR2

TC3: L1-CLI-RSSI measurement delay (method 1 resource type 2) FR1

TC4: L1-CLI-RSSI measurement delay (method 1 resource type 2) FR2

TC5: L1-CLI-RSSI measurement delay (method 3) FR1

TC6: L1-CLI-RSSI measurement delay (method 3) FR2

TC7: L1-SRS-RSRP measurement accuracy FR1

TC8: L1-SRS-RSRP measurement accuracy FR2

TC9: L1-CLI-RSSI measurement accuracy (method 1 resource type 2) FR1

TC10: L1-CLI-RSSI measurement accuracy (method 1 resource type 2) FR2

TC11: L1-CLI-RSSI measurement accuracy (method 3) FR1

TC12: L1-CLI-RSSI measurement accuracy (method 3) FR2

TC13: CSI-RS based RLM OOS FR1

TC14: CSI-RS based RLM OOS FR2

TC15: CSI-RS based RLM IS FR1

TC16: CSI-RS based RLM IS FR2

TC17: CSI-RS based BFR FR1

TC18: CSI-RS based BFR FR2

TC19: CSI-RS based L1-RSRP measurement delay FR1

TC20: CSI-RS based L1-RSRP measurement delay FR2

TC21: CSI-RS based L1-RSRP measurement accuracy (DUD) FR1

TC22: CSI-RS based L1-RSRP measurement accuracy (DUD) FR2

Conclusions

In this paper we provided our views on RRM performance requirements for SBFD.

Proposal 1: Define L1-SRS-RSRP accuracy requirements based on the following side conditions

- Timing offset

  - 15kHz and 30kHz: with cell phase error

  - 60kHz and 120kHz: without cell phase error

  - The requirements apply for both intra-cell and inter-cell scenarios

- SRS Es/Iot >= 1dB

- SRS BW >= 24 RB

Proposal 2: Define L1-SRS-RSRP accuracy requirements based on simulation results for the agreed side conditions.

Proposal 3: R16 accuracy requirements for L3 CLI-RSSI are re-used for L1-CLI-RSSI.

Proposal 4: Define the accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB).

Proposal 5: Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 6: Use Table 2 and Table 3 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Table 2: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 3: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

Proposal 7: Define SBFD RRM tests for the following test configurations.

| Config | Description |
| --- | --- |
| FR1 | NR 30 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| FR2 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Proposal 8: Introduce TDDConf.2.4 (DDDSU) for 30kHz for SBFD RRM tests.

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TDDConf.2.4 |
| referenceSubcarrierSpacing | kHz | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S1U’ S=’10DL:2GP:2UL’ |
| dl-UL-TransmissionPeriodicity | ms | 2.5 |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |
| dl-UL-TransmissionPeriodicity | ms | Not configured |

Proposal 9: Introduce 2 SBFD configurations for DU and DUD

- For DUD specific requirements, use DUD configuration

- For other requirements, use DU configuration

Proposal 10: Use DXXXU as the SBFD configuration for RRM tests, where X denotes SBFD slot with all D and F symbols in the slots configured as SBFD symbol.

Proposal 11: RAN4 to define the following list of test cases for SBFD RRM.

TC1: L1-SRS-RSRP measurement delay FR1

TC2: L1-SRS-RSRP measurement delay FR2

TC3: L1-CLI-RSSI measurement delay (method 1 resource type 2) FR1

TC4: L1-CLI-RSSI measurement delay (method 1 resource type 2) FR2

TC5: L1-CLI-RSSI measurement delay (method 3) FR1

TC6: L1-CLI-RSSI measurement delay (method 3) FR2

TC7: L1-SRS-RSRP measurement accuracy FR1

TC8: L1-SRS-RSRP measurement accuracy FR2

TC9: L1-CLI-RSSI measurement accuracy (method 1 resource type 2) FR1

TC10: L1-CLI-RSSI measurement accuracy (method 1 resource type 2) FR2

TC11: L1-CLI-RSSI measurement accuracy (method 3) FR1

TC12: L1-CLI-RSSI measurement accuracy (method 3) FR2

TC13: CSI-RS based RLM OOS FR1

TC14: CSI-RS based RLM OOS FR2

TC15: CSI-RS based RLM IS FR1

TC16: CSI-RS based RLM IS FR2

TC17: CSI-RS based BFR FR1

TC18: CSI-RS based BFR FR2

TC19: CSI-RS based L1-RSRP measurement delay FR1

TC20: CSI-RS based L1-RSRP measurement delay FR2

TC21: CSI-RS based L1-RSRP measurement accuracy (DUD) FR1

TC22: CSI-RS based L1-RSRP measurement accuracy (DUD) FR2

Reference

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-25xxxxx, Simulation results for L1-RSRP and L1-SINR accuracy, Huawei, HiSilicon