3GPP TSG- WG4 Meeting # 116	R4-2510788

# Bengaluru, India 25-29 Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | Draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to 38.101-1: Time mask for LB-LB switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Core |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To complete the Rel-19 time mask requirements for NR_LBCA_Sw-Core |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Rel-19 time mask requirements for low band CA via switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE RF requirements of Rel-19 NR_LBCA_Sw-Core WI are absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

>> Start of Changes <<

6.3A.3.3.7	Time mask for FDD-SDL carrier aggregation via low band switching

For a downlink inter-band CA configuration provided by LBCA-SwitchingPattern, the ON/OFF time masks for NR UL transmissions in FR1 at the start of an UL slot succeeding an SDL to FDD switch and at the end of an UL slot preceding an FDD to SDL switch are specified in Figure 6.3A.3.3.7-1 for a timing advance NTA = 0 µs.

The switching period of a duration X µs indicated by [UE capability] is located between the start/end of the switching gap and the end/start of the transient period.

For scheduled or triggered PUSCH, or PUCCH, or SRS, or PRACH transmission, the ON/OFF time mask at the start of an UL slot succeeding an SDL to FDD switch is specified in Figure 6.3A.3.3.7-1 assuming a timing advance NTA = 0 µs, the set of symbols of these transmissions can be in the preceding switching gap that covers both the timing advance and the switching period.

The ON/OFF time mask at the end of an UL slot succeeding an FDD to SDL switch is specified in Figure 6.3A.3.3.7-2. At the end of an UL slot, the uplink transmission symbols that partially or fully overlap with the switching gap shall be handled in accordance with TS 38.213, Clause 24.

Figure 6.3A.3.3.7-1: Time mask for low band CA via switching from SDL carrier to FDD carrier, where the switching period is located in SDL carrier

Figure 6.3A.3.3.7-2 Time mask for low band CA via switching from FDD carrier to SDL carrier, where the switching period is located in FDD carrier

>> End of Changes <<