3GPP TSG-RAN WG4 Meeting # 116	R4-2510889

# Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on remaining issues and simulation results for BS MMSE-IRC with inter cell interference

Source: 	Huawei, HiSilicon

Agenda item:	7.13.4

Document for:	Discussion

Background

This contribution provides our views on remaining issues on BS MMSE-IRC with interference.

- Discussion

One issue is applicability for different BS classes and test scenarios, the options are:

Whether to consider additional applicability rule on how the test is performed - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

We prefer option 1, we don’t see any need to require BS pass two cases with different interference level since interference power will not change baseband processing behaviour.

Proposal 1: Prefer option 1, if a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

- Simulation results

In this clause we provide our initial simulation results, the simulation assumptions are captured in Table 3-1:

Table 3-1: Simulation assumptions

| Assumptions |  | Serving cell | Interference cell 1 | Interference cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | 5MHz/15kHz, 20MHz/15kHz, 10MHz/30kHz 100MHz/30kHz |  |  |
| MCS |  | 16 | 16QAM random symbols | 16QAM random symbols |
| Propagation conditions: |  | TDLC300-100 for DIP set1 TDLA30-10 for DIP set2 | TDLC300-100 Low | TDLC300-100 Low |
| Antenna configuration |  | 1T2R, 1T4R,1T8R |  |  |
| Interference model |  |  | DIP set1: DIP1=-1.11dB DIP set2: DIP1=-0.43Db (Present for 2Rx) | DIP set1: DIP2=-10.91dB DIP set2: DIP2=-13.78dB Not present for 2Rx |
| DM-RS | DM-RS configuration type | 1 | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB | -3 dB |
|  | DM-RS port | {0} | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Receiver assumption |  | MMSE-IRC | N/A | N/A |

The simulation results are captured in Table 3-2:

Table 3-2: Simulation results

| Bandwidth/SCS | DIP Set | Propagation conditions | MCS | Antenna configuration | Ideal SNR(dB)@70% of maxTP | Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 5MHz/15kHz | DIP Set1 | TDLC300-100 | MCS16 | 1T2R | 12.6 | 14.6 |
|  |  |  |  | 1T4R | 8.0 | 10.0 |
|  |  |  |  | 1T8R | 4.6 | 6.6 |
|  | DIP Set2 | TDLA30-10 | MCS16 | 1T2R | 13.4 | 15.4 |
|  |  |  |  | 1T4R | 8.4 | 10.4 |
|  |  |  |  | 1T8R | 4.4 | 6.4 |
| 20MHz/15kHz | DIP Set1 | TDLC300-100 | MCS16 | 1T2R | 12.7 | 14.7 |
|  |  |  |  | 1T4R | 8.2 | 10.2 |
|  |  |  |  | 1T8R | 4.5 | 6.5 |
|  | DIP Set2 | TDLA30-10 | MCS16 | 1T2R | 13.9 | 15.9 |
|  |  |  |  | 1T4R | 8.8 | 10.8 |
|  |  |  |  | 1T8R | 4.5 | 6.5 |
| 10MHz/30kHz | DIP Set1 | TDLC300-100 | MCS16 | 1T2R | 11.8 | 13.8 |
|  |  |  |  | 1T4R | 7.6 | 9.6 |
|  |  |  |  | 1T8R | 4.3 | 6.3 |
|  | DIP Set2 | TDLA30-10 | MCS16 | 1T2R | 12.7 | 14.7 |
|  |  |  |  | 1T4R | 8.1 | 10.1 |
|  |  |  |  | 1T8R | 4.3 | 6.3 |
| 100MHz/30kHz | DIP Set1 | TDLC300-100 | MCS16 | 1T2R | 12.6 | 14.6 |
|  |  |  |  | 1T4R | 8.2 | 10.2 |
|  |  |  |  | 1T8R | 4.6 | 6.6 |
|  | DIP Set2 | TDLA30-10 | MCS16 | 1T2R | 13.9 | 15.9 |
|  |  |  |  | 1T4R | 9.0 | 11.0 |
|  |  |  |  | 1T8R | 4.7 | 6.7 |

- Conclusion

This contribution provides our views on remaining issues and simulation results. The proposal is:

Proposal 1: Prefer option 1, if a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

- Reference

[1]   R4-2508618 Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS