3GPP TSG-RAN WG4 Meeting #116	R4-2510968

# Bengaluru, India, August 25th – 29th, 2025

Title:	LB CA Switching

Source:	Qualcomm Incorporated

Agenda item:	7.3.3

Release:	Rel-19

WI: 	NR_LBCA_Sw-Core

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

For WI LB DL CA via switching, WF [10] was approved in RAN4#115. This paper discusses some aspect of the agreements in the WF.

# 2. 	Discussion

## 2.1	Switching diagram aspects

We discussed in [11] different requirements that the switching time mask can include. Here we briefly provide our view on the issues under discussion.

UL TA can be included but it is not necessary. The RF Requirement and UE capability can be tested without UL TA but some time difference can be included to verify the UE behaviour with Non Zero TA. For this, the UE behaviour should be clear. Since TA has impact to DL and WF states that the DL is discussed in RRM session, then before agreeing these aspects in RF Requirements, RRM session should conclude their discussion for this part.

Observation 1: UL TA in switching times masks must accommodate RRM discussion conclusions.

Transient periods location should be outside the switching time for UL as per the agreement in [12].

RMCs should be discussed and agreed for this feature. Specically UL RMC is needed since how to test FDD when UL is not available for 100 % of the time is not clear. DL RMCs are needed if REFSENS or related parameter is tested for this feature.

Observation 2: UL RMCs and DL FRCs are needed for FDD and SDL for this feature.

The patters and RRC parameters were discussed in RAN1. One good reference with proper patters is provided in [13] from Ericsson. Care should be taken to place all the reference symbols correctly and these should be reflected in the FRCs and RMCs.

Proposal : UL RMCs and Dl FRS are defined for this with needed RRC parameters that is needed in the configuration of the pattern for the UE and placed in RAN4 specifications

A picture of possible patters with different periodicity is in Figure 1.

Figure 1: Exemplary switching patterns configured for roughly 50-50 time sharing between FDD and SDL carriers for each group of users. Three pattern periodicities are shown, 10 ms, 20 ms, and 40 ms, where the latter is equal to the 40 ms TRS periodicity. (Source [13])

# Conclusion

Observation 1: UL TA in switching times masks must accommodate RRM discussion conclusions.

Observation 2: UL RMCs and DL FRCs are needed for FDD and SDL for this feature.

Proposal : UL RMCs and Dl FRS are defined for this with needed RRC parameters that is needed in the configuration of the pattern for the UE and placed in RAN4 specifications

# Reference

[1]	RP-243317, “New WID on low band carrier aggregation via switching”, Apple, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[3]	R4-2503016, “WF on low-low band switching”, Apple, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[4]	R4-2502877, “LS on switching periods for low-low band switching”, AT&T, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[5]	R4-2500492, “Initial aspects of RX Switching”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[6]	R4-2501572, “Time masks for LB-LB CA switching”, Ericsson, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[7]	R4-2500405, “NR_LBCA_Sw Considerations”, Nokia, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[8]	R1-2502860, “Low band carrier aggregation via switching”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[9]	R4-2505223, “WF on UE RF requirements for LB CA via switching”, Apple, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[10]	R4-2508118, “WF on NR_LBCA_Sw”, Apple, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[11]	R4-2506166, “Switching time diagram”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[12]	R4-2507932, “TP for 38.768: switching period location and time masks”, Ericsson, Xiaomi, MediaTek, Apple, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[13]	R1-2502426, “ Low band carrier aggregation through switching “, Ericsson, 3GPP TSG RAN WG1 #120bis	Wuhan, China, April 7th – 11th, 2025 ()