# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2510975

# Bangalore, India, 25th – 29th August, 2025

# Title: 	Discussion on BS demodulation performance requirements MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:    7.13.4

# Document for:	Discussion

# 1 Introduction

In RAN #104 meeting, the revised WID of NR demodulation performance requirements for NR BS with MMSE-IRC receiver was approved in [1], wherein the objectives related to NR BS MMSE-IRC receiver are listed as below:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we give some initial simulation results for NR BS MMSE-IRC receiver.

- Discussion

In RAN4 #115 meeting, some simulation assumptions are reached with some parameters pended.

Base on the conclusion in RAN4 # 115 meeting[2], we provide our initial simulation results with the simulation assumptions listed in Table 2-1.

Table 2-1 Simulation assumption for BS MMSE-IRC

| Parameter |  | Tested signal | Interferer 1 (Note 1) | Interferer 2 (Note 1) |
| --- | --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | N/A | N/A |
| RV sequence |  | 0, 2, 3, 1 | N/A | N/A |
| DIP (Note 2) | Set 1 | N/A | -1.11 | -10.91 |
|  | Set 2 | N/A | -0.43 | -13.78 |
| Interference model |  | N/A | 16QAM randomly modulated data | 16QAM randomly modulated data |
| Cyclic Prefix |  | Normal |  |  |
| Duplex mode |  | FDD |  |  |
| Network type |  | Synchronous scenario |  |  |
| Antenna configuration |  | 1 Tx 2/4/8 Rx |  |  |
| Number of interferers |  | 1 interferer for 2 Rx 2 interferers case for 4/8 Rx |  |  |
| SCS and BW |  | 15kHz SCS, 5MHz, 20MHz 30kHz SCS, 10MHz,100MHz |  |  |
| MCS |  | MCS 16 |  |  |
| Waveform |  | CP-OFDM |  |  |
| Receiver type |  | MMSE-IRC |  |  |
| PUSCH mapping type |  | Type A |  |  |
| Test metric |  | 70% of maximum throughput |  |  |
| Propagation condition |  | TDLA30-10: serving cell TDLC300-100:serving cell |  |  |
| DMRS | DM-RS configuration type | 1 |  |  |
|  | DM-RS duration | single-symbol DM-RS |  |  |
|  | Additional DM-RS position | pos1 |  |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |  |
|  | DM-RS port(s) | 0 |  |  |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |  |  |

Table 2-2 summarizes the demodulation performance with 70% max throughput operating SNR.

Table 2-2 SNR operating point for BS MMSE-IRC under FDD duplex model

| DIP set | channel | MCS for target cell | SCS, BW | Mapping type | Rx | SNR(dB) MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- |
| HomNet set1: (-1.11. N/A) | TDLC300-100 | 16 | 15kHz,5MHz | Type A | 2 | 11.5 |
| HomNet set1: (-1.11. N/A) | TDLC300-100 | 16 | 15kHz,5MHz | Type B | 2 | 11.6 |
| HetNet set2: (-0.43. N/A) | TDLA30-10 | 16 | 15kHz,5MHz | Type A | 2 | 12.0 |
| HetNet set2: (-0.43. N/A) | TDLA30-10 | 16 | 15kHz,5MHz | Type B | 2 | 12.1 |
| HomNet set1: (-1.11. N/A) | TDLC300-100 | 16 | 15kHz,20MHz | Type A | 2 | 11.6 |
| HomNet set1: (-1.11. N/A) | TDLC300-100 | 16 | 15kHz,20MHz | Type B | 2 | 11.7 |
| HetNet set2: (-0.43. N/A) | TDLA30-10 | 16 | 15kHz,20MHz | Type A | 2 | 12.2 |
| HetNet set2: (-0.43. N/A) | TDLA30-10 | 16 | 15kHz,20MHz | Type B | 2 | 12.3 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 15kHz,5MHz | Type A | 4 | 7.4 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 15kHz,5MHz | Type B | 4 | 7.3 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 15kHz,5MHz | Type A | 4 | 7.0 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 15kHz,5MHz | Type B | 4 | 7.1 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 15kHz,20MHz | Type A | 4 | 8.2 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 15kHz,20MHz | Type B | 4 | 8.3 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 15kHz,20MHz | Type A | 4 | 7.5 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 15kHz,20MHz | Type B | 4 | 7.6 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 15kHz,5MHz | Type A | 8 | 4.4 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 15kHz,5MHz | Type B | 8 | 4.6 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 15kHz,5MHz | Type A | 8 | 3.4 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 15kHz,5MHz | Type B | 8 | 3.3 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 15kHz,20MHz | Type A | 8 | 4.8 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 15kHz,20MHz | Type B | 8 | 4.9 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 15kHz,20MHz | Type A | 8 | 3.6 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 15kHz,20MHz | Type B | 8 | 3.7 |

Table 2-3 SNR operating point for BS MMSE-IRC under TDD duplex model

| DIP set | channel | MCS for target cell | SCS, BW | Mapping type | Rx | SNR(dB) MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- |
| HomNet set1: (-1.11. N/A) | TDLC300-100 | 16 | 30kHz,10MHz | Type A | 2 | 11.6 |
| HomNet set1: (-1.11. N/A) | TDLC300-100 | 16 | 30kHz,10MHz | Type B | 2 | 11.6 |
| HetNet set2: (-0.43. N/A) | TDLA30-10 | 16 | 30kHz,10MHz | Type A | 2 | 12.8 |
| HetNet set2: (-0.43. N/A) | TDLA30-10 | 16 | 30kHz,10MHz | Type B | 2 | 12.9 |
| HomNet set1: (-1.11. N/A) | TDLC300-100 | 16 | 30kHz,100MHz | Type A | 2 | 11.9 |
| HomNet set1: (-1.11. N/A) | TDLC300-100 | 16 | 30kHz,100MHz | Type B | 2 | 11.9 |
| HetNet set2: (-0.43. N/A) | TDLA30-10 | 16 | 30kHz,100MHz | Type A | 2 | 13.1 |
| HetNet set2: (-0.43. N/A) | TDLA30-10 | 16 | 30kHz,100MHz | Type B | 2 | 13.2 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 30kHz,10MHz | Type A | 4 | 8.1 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 30kHz,10MHz | Type B | 4 | 8.2 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 30kHz,10MHz | Type A | 4 | 7.7 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 30kHz,10MHz | Type B | 4 | 7.8 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 30kHz,100MHz | Type A | 4 | 8.0 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 30kHz,100MHz | Type B | 4 | 8.1 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 30kHz,100MHz | Type A | 4 | 8.1 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 30kHz,100MHz | Type B | 4 | 8.1 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 30kHz,10MHz | Type A | 8 | 4.7 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 30kHz,10MHz | Type B | 8 | 4.8 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 30kHz,10MHz | Type A | 8 | 4.3 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 30kHz,10MHz | Type B | 8 | 4.4 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 30kHz,100MHz | Type A | 8 | 4.2 |
| HomNet set1: (-1.11. -10.91) | TDLC300-100 | 16 | 30kHz,100MHz | Type B | 8 | 4.2 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 30kHz,100MHz | Type A | 8 | 4.5 |
| HetNet set2: (-0.43. -13.78) | TDLA30-10 | 16 | 30kHz,100MHz | Type B | 8 | 4.5 |

# 3	Reference

- RP-241297, Revised WID on NR demodulation performance: Phase 5.

- R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS.