3GPP TSG-RAN WG4 Meeting #116	R4-2510993

Bengaluru, India, August 25 – 29, 2025

Source:	Nokia

Title:	On Enhancements for Air-to-Ground Network for NR

Agenda item:	7.8.7

Document for:	Discussion

Introduction

The Way Forward (WF) from RAN4#115 discussions on Rel-19 Enhancements for Air-to-ground network for NR is documented in [1]. The discussions covered 3 topics, namely, DL CA, DL MIMO and UL MIMO, with agreements as summarized below:

<Topic 1> DL CA Issue 1-1-1: CA Duplex mode Agreement: - TDD 30 kHz + TDD 30 kHz - FDD 15 kHz + TDD 30 kHz Issue 1-1-2-4: Bandwidth Agreement: - all bandwidth configuration for FDD 15kHz and TDD 30kHz - Single carrier requirements for each of the considered bandwidth sizes in the bandwidth combination sets shall then be specified. <Topic 2> DL MIMO Issue 2-1-1: Antenna configuration Agreement: - XP-low model Issue 2-2-2: RI Agreement: - Don’t introduce RI requirement for ATG MIMO <Topic 3> UL MIMO Issue 3-1-1: Whether to reuse existing requirements for ATG UL MIMO Agreement: - Only the applicable PUSCH test cases in Clause 8.2.1 with 2Tx 2 layer will be applied for ATG Issue 3-1-2: Whether to introduce new PUSCH test cases for ATG UL MIMO Agreement: - Limited new test cases could be considered for ATG 2Tx PUSCH demodulation requirements Issue 3-1-3: Whether to introduce new PUCCH and PRACH test cases for ATG UL MIMO Agreement: - No PUCCH and PRACH requirement for UL MIMO with 2Tx Issue 3-1-5: Propagation condition for incremental test Agreement: - For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference - Capture the static channel matrix in spec Issue 3-1-6: Bandwidth & SCS for incremental test Agreement: - 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS Issue 3-1-7: Mapping type for incremental test Agreement: - Mapping type A Issue 3-1-8: DMRS configuration for incremental test Agreement: - Rel-15 DMRS type Issue 3-1-9: Rank for incremental test Agreement: - Rank 2 Issue 3-1-10: MCS for incremental test Agreement: - 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and [256QAM Table 2 MCS 20] for initial simulation - Only one MCS will be used Issue 3-1-11: TPMI for incremental test Agreement: - TPMI index 0 Issue 3-1-12: Others Agreement: - Existing configuration as R15 BS PUSCH demodulation requirement with 2Tx

In [1], the open issues on the above-mentioned 3 topics are also documented.

In this contribution, we share our views on the open issues to be discussed in RAN4#116.

Discussion

DL CA

The issues going to be addressed here are:

Issue 1-1-2: Test setups Issue 1-1-2-1: TDD pattern - Option 1: Use the legacy TDD pattern as the starting point - Option 2: Define new TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirement Issue 1-1-2-2: Rank - Option 1: For CA requirement, use Rank 2 as the starting point - Option 2: Only consider Rank 1 for DL CA demodulation requirements Issue 1-1-2-3: MCS - Option 1: Consider 64QAM MCS22 for ATG downlink CA requirements - Other Options are not precluded

Referring to Table A.1.2-2c: TDD UL-DL configuration for SCS 30 kHz for ATG in 38.101-4 [2], the TDD pattern of 30D4S6U is mentioned. As DL CA TDD 30 kHz + TDD 30 KHz is already agreed in RAN4#115, 30D4S6U could be considered for the requirements.

For DL CA, RAN4 to consider 30D4S6U, Rank 1 and 64QAM MCS 22.

DL MIMO

The open issue to be addressed in this topic is:

Issue 2-1-2: TDD pattern - Option 1: Use legacy TDD pattern 7DS2U as the starting point. Issue 2-1-3: MCS - Option 1: consider MCS 13 and MCS 22 - Option 2: Consider 64QAM - Other Options are not precluded

Concerning the TDD pattern, the legacy TDD pattern 7DS2U can be considered as a starting point. While both MCS 13 and MCS 22 are used in the requirements for Rel-18 ATG in 38.101-4 [2], for DL MIMO RAN4 to prioritize MCS 22 to show the benefit of having MIMO.

For DL MIMO, RAN4 to consider the legacy TDD pattern 7DS2U and prioritize MCS 22.

UL MIMO

The open issues to be addressed here are:

Issue 3-1-4: Antenna configuration for incremental test - Option 1: 2T2R - Option 2: 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point - Option 3: 2T2R with low correlation

As the aim for Rel-19 UL is only for incremental test, only 2T2R with low correlation is sufficient.

For incremental test in UL MIMO, RAN4 to consider 2T2R with low correlation.

Conclusion

In this contribution, we provide our views on Rel-19 Enhancements for ATG network for NR. The following Observations and Proposals were made:

Proposal 1: For DL CA, RAN4 to consider 30D4S6U, Rank 1 and 64QAM MCS 22.

Proposal 2: For DL MIMO, RAN4 to consider the legacy TDD pattern 7DS2U and prioritize MCS 22.

Proposal 3: For incremental test in UL MIMO, RAN4 to consider 2T2R with low correlation.

- References

- R4-2508724, Way Forward for [115][328] NR_ATG_enh_demod, 3GPP RAN4#115, CMCC, Malta, May 19-23, 2025

- 3GPP TS 38.101-4, NR - User Equipment (UE) radio transmission and reception, Part 4: Performance requirements, V18.7.0, 03-2025