3GPP TSG-RAN WG4 Meeting #116                                           R4-2511000

Bangalore, India, 25th – 29th August, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on BS RF for Rel-19 LP-WUS

Agenda Item:	7.24.4

Document for:	Approval

Introduction

BS RF requirements have been further discussed for LP-WUS and some aspects to be further analysed are captured in WF [1].

This contribution provides further analysis for the issues identified in the WF. As August meeting is the last meeting for the Rel-19 RAN1 led WI, we have to complete all the remaining issues for LP-WUS BS RF in this meeting.

Discussion

Other requirements other than dynamic range

Besides the dynamic range requirement, the main discussion in RAN4 is focused on LP-WUS EVM. In last meeting, we have the following WF:

Issue: Whether to define EVM requirements for LP-WUS

- Option 1: No BS LP-WUS EVM requirement is specified.

•	The background and consideration of LP-WUS transmitted signal quality are captured in TR 38.774.

- Option 2: BS LP-WUS EVM requirement as follows, no conformance test for this requirement.

•	For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK-4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2.

Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for LP-WUS | Required EVM |
| --- | --- |
| OOK-4 | [FFS] % |

Several companies have conducted simulations on the PAPR for the combined signal of LP-WUS and NR. The results indicate that LP-WUS has minimal impact on the PAPR of the composite signal. Consequently, these companies propose omitting the EVM requirement specification for LP-WUS. Additionally, the TP on EVM consideration for LP-WUS is already captured in the TR as a background information to reflect the corresponding discussion in RAN4. Therefore, we see no need to have the EVM requirement in the TS.

Proposal 1: No need to define EVM requirements for LP-WUS.

Apart from the requirement, we did some further evaluation relevant to EVM. Table 1 presents comparative simulation results analyzing both PAPR and EVM characteristics for a pure NR signal versus an NR+LP-WUS composite signal, using 256QAM modulation for the NR component. The evaluation includes both average EVM and per-symbol peak EVM measurements across multiple channel bandwidths. While the PAPR values remain comparable between the two signal configurations, the results demonstrate measurable EVM degradation in specific 256QAM cases when LP-WUS is introduced. This suggests a non-negligible impact on NR signal integrity at higher modulation orders. Notably, our assessment reveals this effect to be significantly less pronounced for 64QAM modulation scenarios.

Table 4: PAPR & EVM comparison between pure NR CP-OFDM vs NR+LP-WUS

| Channel Bandwidth | Signal type | PAPR 0.01% | EVMavg (%) | EVMmax (%) |
| --- | --- | --- | --- | --- |
| 10MHz | NR only | 9.725 | 3.14 | 4.51 |
|  | NR+LP-WUS | 9.564 | 3.23 | 4.45 |
| 20MHz | NR only | 9.767 | 2.91 | 4.67 |
|  | NR+LP-WUS | 9.788 | 3.01 | 4.95 |
| 50MHz | NR only | 9.754 | 2.82 | 3.40 |
|  | NR+LP-WUS | 9.972 | 3.22 | 4.52 |

Observation: LP-WUS could introduce noticeable EVM degradation in certain cases for 256QAM.

Proposal 2: It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance.

Conclusion

This contribution provides further consideration for BS RF of LP-WUS. With above analysis, we have the following observations and proposals:

Proposal 1: No need to define EVM requirements for LP-WUS.

Observation: LP-WUS could introduce noticeable EVM degradation in certain cases for 256QAM.

Proposal 2: It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance.

Companion draft CRs on clarification of the applicable requirements with LP-WUS operation for TS 38.1014 and TS 38.141 are provided in [3]-[5].

- References

[1] R4-2508668, WF on NR LP-WUS BS RF, Huawei, HiSilicon

[2] R4-2508770, TP to TR38.774 BS RF requirement overview for LP-WUS, Ericsson

[3] R4-25xxxxx, draft CR to TS 38.104 on clarification of transmitted signal quality for LP-WUS operation, Huawei

[4] R4-25xxxxx, draft CR to TS 38.141-1 on clarification of transmitted signal quality for LP-WUS operation, Huawei

[5] R4-25xxxxx, draft CR to TS 38.141-2 on clarification of transmitted signal quality for LP-WUS operation, Huawei