3GPP TSG-RAN WG4 Meeting#116                                R4-2511137

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on system parameters for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. During the last RAN4 meeting, we have some initial discussions on work plan and general aspects for SBFD BS, however there are still lots of open issues not addressed yet. In this contribution, we would like to share further views on these remaining issues.

- Sub-topic 1-1: Declaration and flexibility for SBFD operation - Issue 1-1-1: Declaration and flexibility for SBFD operation Agreement: - For all FR2 BS classes, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - X value will be selected in next meeting. - Sub-topic 1-2: DL/UL subband configurations support for SBFD Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not. - Issue 1-2-2: Restriction on the number of DL subband sizes supported Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not. - Issue 1-2-3: Restriction on guardband size for SBFD - No agreement in this meeting and the way forward from RAN4#114bis can still be followed. - Issue 1-2-4: Restriction on DL subband size Agreement: - If the SSB symbol is overlapped with SBFD symbol, - there is no RAN4 impact at least on RAN4 core requirement for expected regarding DL subband size configuration - FFS the test configuration during performance part discussion. .

Discussion

## Issue 1-1-1: Declaration and flexibility for SBFD operation

Agreement:

- For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

  - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

- FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. Regarding the declaration and flexibility, most controversial part is about the necessity of having the analog filtering after the LNA at the receiver side, if there are no such kind of analog filter for the protection of receiver’s saturation, then the RAN4 standard should be flexible to have different DL-UL sub-band configuration and only test the worst case for the conformance testing. For FR1 WA SBFD BS, it is clear that some analog filtering are needed at the receiver, this is part of reasoning why BS could only operate under the tested sub-band configuration. However for FR1 MR SBFD BS, from our understanding, it is also fine to assume the analog filter implementation as compromise solution.

- Proposal 1: for FR1 MR SBFD BS, propose to use the 33dBm as TRP or total conducted power as the power threshold.

- Topic 4: HPUE related updates due to the SBFD configuration

- For SBFD was agreed to be specified in FR1/FR2 TDD bands only and there are lots of HPUE related duty cycle specified for the single TDD band and also EN-DC or NR-CA. For the legacy dutycycle evaluation, the calculation is based on the legacy uplink symbols, however considering the SBFD feature would allow the DL/flexible symbols to be configured as SBFD symbols with both DL sub-band and also UL sub-band, this will increase the potential symbols for the dutycycle evaluation at the UE side. During the discussion in last RAN4 meeting, to simplify the spec changes, we propose to add the following two sentence in the general section 6.1.

6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

The applicability of transmitter requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the minimum requirements for Band n41.

- For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

- For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission..

Proposal 2: in order to simplify the spec change, propose to add the following two sentence in the general section 6.1.

- For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

- For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission..

Conclusions

In this contribution, we want to share some further views on general aspects for SBFD BS and proposals/observations are made as following:

- Proposal 1: for FR1 MR SBFD BS, propose to use the 33dBm as TRP or total conducted power as the power threshold.

Proposal 2: in order to simplify the spec change, propose to add the following two sentence in the general section 6.1.

- For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

- For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission..

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2406107, Way Forward for [110bis][313] NR_duplex_evo

- R4-2409958, Way forward for [110bis][313] NR_duplex_evo, Approved

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General, Approved.

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General

- R4-1801031, WF on NR BS RF FRCs, approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.

- R4-2501967	Further discussion on general aspects for SBFD system, Samsung

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.

- R4-2504751	Way Forward for [114bis][306] NR_duplex_evo_General

- R4-2508707 Way Forward for [115][306] NR_duplex_evo_General, approved