3GPP TSG-RAN WG4 Meeting #116                                                               R4-2511248

Bengaluru, India, 25th – 29th August, 2025

Source:	Samsung

Title:	Study on remaining issues for CSI compression two-sided models

Agenda item:			7.18.3

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two use cases (i.e., beam management and positioning accuracy enhancements) [2].

Furthermore, the study objectives with corresponding checkpoints in RAN#105 (Sep ‘24) particularly on the two sub-use cases of CSI feedback enhancements are provided [2]. From RAN4 perspective, it is tasked to further analyze the various testing options for two-sided models, i.e., particularly related to the use case of CSI compression among all (sub-)use cases discussed in Rel-18 study item.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

As per the discussions in RAN#105, the exploration of testability and interoperability challenges related to AI-driven CSI compression is set to continue within the remaining timeframe of Release 19, while the relevant objectives are included in a newly set-up study item [RP-250308]. In this contribution, we aim to further the discussion on testing issues for CSI compression two-sided models.

# 2. Remaining simulation for low-complexity encoder training

As identified as one remaining task [23], interested companies are encouraged to bring further simulation results with lower-complexity encoder structures and with other encoder backbones. Based on some companies’ observations, the agreed encoder is regarded as being impractical due to its extremely high complexity, and different UE vendors may employ encoders with different backbones and with different complexity levels.

It is identified that verifying whether it is feasible to achieve a common decoder (which is compatible with encoders having different backbones and complexity levels) can make the feasibility verification more substantial. The simulation results may also provide guidance for the design of the encoder/decoder in the Release-20 WI stage.

### 2.1 Performance baseline (SGCS-0 and SGCS-1)

RAN4 has completed the encoder/decoder training procedure practice for test decoder options. For Option 3 track 2, the good convergence is already observed that:

RAN4#115 Ad hoc minutes on AI/ML air interface study When training “own” encoder with the frozen decoder, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results Around - 2% to +1.4% variation in SGCS from the average

Therefore, the results with the agreed CNN-based backbones (i.e., no backbone mismatch between the encoder and decoder, and a common encoder structure for pair-wise encoder/decoder training and the separate encoder retraining) should be considered as the baseline performance.

Observation 1: To evaluate the feasibility of the common decoder compatible with different encoder backbones/complexities, the performance achieved for CNN-based backbones should be considered as the baseline performance (SGCS-0 for joint training and SGCS-1 for separate training).

### 2.2 Joint and separate training for low complexity Encoder-2/3/4 (SGCS-3 and SGCS-2)

Furthermore, by following the different low complexity encoder structures as below (details provided in [23]),

- Encoder-2: Transformer based lower-complexity encoder

- Encoder-3: CNN based lower-complexity encoder

- Encoder-4: MLP based lower-complexity encoder

and also applying the separate training (using agreed Samsung decoder as frozen decoder) and joint training (using agreed CNN-based structure for decoder), the SGCS-2 and SGCS-3 can be obtained for comparison:

Straightforwardly, by comparing SGCS-1 and SGCS-2 (i.e., separate training for different encoder backbones with the same decoder), we can conclude that whether or not the agreed Samsung’s decoder can be a common future-proof decoder that is compatible to encoders with different backbones and with different complexities.

Observation 2: By comparing the separate training for encoder-2/3/4 (SGCS-2) and encoder-1 (SGCS-1), the following evaluation purposes should be focused:

- Purpose-(1): the feasibility of future-proof common decoder, i.e., whether it is feasible to derive a common decoder which can be compatible with encoders with different/new backbones which has not been considered in the training. In other words, Purpose-(1) guarantee the RAN4-specified common decoder is future-proof for unseen AI/ML model backbone.

- Purpose-(2): the feasibility of low-complexity encoder, i.e., whether it is feasible to have a separate training to derive low-complexity encoder(s) which is compatible with known common decoder with more complex backbone structure. In other words, Purpose-(2) guarantee the UE vendors can derive models with low complexity for implementation flexibility.

Furthermore, by comparing SGCS-0 and SGCS-3 (i.e., joint training for different encoder backbones with the same decoder backbone), we can understand the performance loss of separate training from joint training. Based on the above analysis, the following observation can be made:

Observation 3: For a given new encoder-x with different backbone from encoder/decoder-1, by evaluating the performances for separate training for encoder-x (SGCS-2), separate training for encoder-1 (SGCS-1), joint training for encoder-x (SGCS-3) and joint training for encoder-1 (SGCS-0), the following conclusions can be made:

| What we observed | What we can conclude | What we observed | What we can conclude |
| --- | --- | --- | --- |
| SGCS-3 ≈ SGCS-0 | Encoder-x backbone is compatible with the common decoder backbone | SGCS-2 ≈ SGCS-1 | (1) Separate training is feasible for encoder-x backbone; (2) Specified common decoder is feasible and future-proof for unseen low-complexity encoder. |
|  |  | SGCS-2 < SGCS-1 | (1) Separate training is NOT feasible for encoder-x backbone, while it is to be study the encoder-structure-aware training is feasible for this case. (2) Specified common decoder is NOT feasible/future-proof if similar observation for different encoder backbones. |
| SGCS-3 < SGCS-0 | Encoder-x backbone is NOT compatible with the common decoder backbone | N/A |  |

### 2.3 N-to-1 Multi-encoder-structure-aware training (SGCS-4)

As we identified in Observation 3, if SGCS-3 ≈ SGCS-0 and SGCS-2 < SGCS-1, then we can conclude “Separate training is NOT feasible for encoder-x backbone”. However, RAN4 group has discussed the more complex training procedure, named as “Multi-encoder-structure-aware training” in this paper, which is proposed to further study whether or not the problem comes from the normal separate training or not.

Proposal 1: Use the term “N-to-1 Multi-encoder-structure-aware training” to represent the procedure for SGCS-4.

The agreed Samsung’s decoder follows the CNN-based structure (which is not aligned with the Encoder-2/3/4 structures) and the decoder is trained without considering any low-complexity structures. Therefore, the more complex encoder-structure-aware training is proposed to guarantee the decoder generalization performance with different encoder structures.

SGCS-4 (Step-1)

ste

As illustrated in the above figure, the Multi-encoder-structure-aware training procedure is divided into two steps:

- Step-1: Joint training for Encoder-1/2/3/4 and agreed CNN-based decoder structure, with the following outputs obtained:

  - The common decoder trained from step-1

  - The Encoder-1/2/3/4 trained from step-1

- Step-2: Separate training for Encoder-x (x can be 1, 2, 3, 4) with the frozen common decoder from Step-1

It should be noted that two SGCS-4 values can be provided for this Multi-encoder-structure-aware training, in which

- SGCS-4 (step-1): is the achievable performance for encoder-x from step-1, over mixed dataset for testing

- SGCS-4 (step-2): is the achievable performance for encoder-x from step-2, over mixed dataset for testing

For the detailed training procedure in the above step-1, we followed the procedure provided in [23]

The detailed training procedure of SGCS-4 Step-1 is described as follows. - For each loop, use the Loss to update parameters of the decoder, in addition, - update parameters of Encoder-1 - update parameters of Encoder-2 - update parameters of Encoder-3 - update parameters of Encoder-4 Note: In each loop, decoder parameters are updated for one time based on the total loss, each Encoder (e.g., Encoder-1,2,3,4) is also updated for one time based on the backpropagation results from the decoder. - One of the candidate methods to calculate the Loss: Loss=n1* Loss1+ n2* Loss2, n1+n2=1 Note: n1> n2, using n1=0.7, n2=0.3 as a starting point Loss1=average (lossV,V1+lossV,V2+lossV,V3+lossV,V4) Loss2=average (lossV1, V2+lossV1, V3+lossV1, V4+lossV2, V3+lossV2, V4+lossV3, V4) - Note: Introducing Loss2 is to ensure that comparable SGCSs can be achieved for each encoder. Otherwise, after N-to-1 joint training, SGCS of some encoders (SGCS-4 in Step-1) may be much lower than that of others. - Note: Each lossX,Y can be the reconstruction loss function used in training of SGCS-0/1/2/3, e.g. SGCS loss. - Note: Other methods of calculating the Loss are not precluded. Companies are encouraged to report other methods if they use.

Observation 4: On top of Observation 3, the evaluation of multi-encoder-structure-aware training (SGCS-4, Step-2), the highlighted further conclusions can be made:

| What we observed | What we can conclude | What we observed | What we can conclude | What we observed | What we can conclude |
| --- | --- | --- | --- | --- | --- |
| SGCS-3 ≈ SGCS-0 | Encoder-x backbone is compatible with the common decoder backbone | SGCS-2 ≈ SGCS-1 | (1) Separate training is feasible for encoder-x backbone; (2) Specified common decoder is feasible and future-proof for unseen low-complexity encoder. | N/A |  |
|  |  | SGCS-2 < SGCS-1 | (1) Separate training is NOT feasible for encoder-x backbone, while it is to be study the multi-encoder-structure-aware training is feasible for this case. (2) Specified common decoder is NOT feasible/future-proof if similar observation for different encoder backbones. | SGCS-4 (step-2) ≈ SGCS-1 | Multi-encoder-structure-aware training is required to consider encoder-x structure |
|  |  |  |  | SGCS-4 (step-2) < SGCS-1 | Multi-encoder-structure-aware training methodology is NOT feasible for this case |
| SGCS-3 < SGCS-0 | Encoder-x backbone NOT compatible with the common decoder backbone | N/A |  |  |  |

### 2.4 Result summary and analysis

The simulation results are summarized in the following table including the relative gains over different baselines.

|  |  | （1-to-1）Joint training [gain over SGCS-0] | New encoder training with (Samsung’s) frozen decoder [gain over 1-to-1 joint training, gain over SGCS-1] | (N-to-1 Multi-encoder-structure-aware training) Joint training SGCS-4 Step-1 n2=0.05 [gain over 1-to-1 training] | New encoder training with frozen decoder from Step-1: SGCS-4 Step-2 [gain over Step-1, gain over SGCS-1] |
| --- | --- | --- | --- | --- | --- |
| Encoder-1 (47MFLOPs) | Samsung | SGCS-0 0.7018 [0%] | SGCS-1 0.7016 [-0.0285%, 0%] | SGCS-4 (step-1) 0.6992 [-0.3705%] | SGCS-4 (step-2) 0.7011 [0.2717%, -0.0713%] |
| Encoder-2 (3.63MFLOPs) Dropout=0.1 | Samsung | SGCS-3 0.7163 [2.0661%] | SGCS-2 0.7049 [-1.5915%, 0.4704%] | SGCS-4 (step-1) 0.7107 [-0.7818%] | SGCS-4 (step-2) 0.7139 [0.4503%, 1.7531%] |
| Encoder-3 (4.14MFLOPs) | Samsung | SGCS-3 0.6854 [-2.3368%] | SGCS-2 0.6826 [-0.4085%, -2.7081%] | SGCS-4 (step-1) 0.6887 [0.4815%] | SGCS-4 (step-2) 0.6893 [0.0871%, -1.7531%] |
| Encoder-4 (0.22MFLOPs) | Samsung | SGCS-3 0.6741 [-3.9470%] | SGCS-2 0.6352 [-5.7707%, -9.4641%] | SGCS-4 (step-1) 0.6754 [0.1928%] | SGCS-4 (step-2) 0.6760 [0.0888%, -3.6488%] |

By analyzing the above testing SGCSs and the relative gains over different baselines, several observations are obtained. It should be noted that the relative loss within +/-5% will be treated as “having similar performance”.

Joint training for low-complexity Encoder-2/3/4 (SGCS-3)

By following the logic in Observation 4, we can firstly compare SGCS-3 to SGCS-0, i.e., joint training performance.

Observation 5: For joint-training of Encoder-1/2/3/4 with the agreed decoder backbone (by comparing SGCS-3 for Encoder-2/3/4 to SGCS-0 for Encoder-1),

- The losses for Encoder-2, Encoder-3 and Encoder-4 are -2.0661%, 2.3368% and 3.9470%, all being regarded as similar performance as SGCS-0.

It should also be noted that SGCS-3 for encoder-4 is obviously worse than other low complexity encoder backbone, which should be noted when RAN4 perform the reference encoder backbone selection.

Observation 6: For the evaluated very low-complexity Encoder-4 (MLP-based low-complexity encoder), the SGCS from joint-trained encoder-4 has worse performance compared to other evaluated encoders.

Separate training for low-complexity Encoder-2/3/4 (SGCS-2)

Secondly, we compare SGCS-2 to SGCS-1, i.e., separate training performance with Samsung’s frozen decoder.

Observation 7: For separate training for Encoder-1/2/3/4 with Samsung’s frozen decoder, i.e., SGCS-1 for Encoder-1 and SGCS-2 for Encoder-2/3/4, by comparing to the counterpart from joint training, i.e., SGCS-0 for Encoder-1 and SGCS-3 for Encoder-1:

- The losses for separate trained Encoder-1, Encoder-2 and Encoder-3 are 0.0285%, 1.5915% and 0.4085% respectively, all being degraded at a low level and showing similar performances as joint trained ones respectively;

- The loss for Encoder-4 is 5.7707% which show obvious performance degradation.

We can further compare the performance obtained by low-complexity Encoder-2/3/4 (SGCS-2) to Encoder-1 (SGCS-1),

Observation 8: For separate training for Encoder-2/3/4 with Samsung’s frozen decoder, i.e., SGCS-2 for Encoder-2/3/4, by comparing the performance from the separate training for Encoder-1, i.e., SGCS-1:

- The losses for Encoder-2 and Encoder-3 are -0.4704% and 2.7081% respectively, being degraded at a low level and showing similar performances as Encoder-1.

- The loss for Encoder-4 is 9.4641% which show obvious performance degradation compared to other encoders.

Based on the above two observations, and by following the logic in observation 4, we can have:

Observation 9: The evaluated very low-complexity Encoder-4 (MLP-based low-complexity encoder) lack enough model capacity to compress the input features into the latent representation that the common decoder requires.

Proposal 2: For the evaluated very low-complexity Encoder-4 (MLP-based low-complexity encoder), it should not be recommended as reference encoder selection if the multi-encoder-structure-aware training is not considered:

- The low performance reason can be further studied for (1) MLP backbone; (2) very low complexity; or both (1) and (2).

For the low-complexity Encoder-2/3, the following conclusion can be made:

Proposal 3: For the evaluated low-complexity Encoder-2/3 (transformer-based and CNN-based):

- Separate training with Samsung’s frozen common decoder is feasible for encoder-2/3 backbone

And considering both Encoder-2/3 works well with the common decoder, we can see

Proposal 4: The frozen Samsung’s decoder is feasible and future-proof for unseen low-complexity encoder, which also demonstrate the methodology of separate training.

N-to-1 Multi-encoder-structure-aware training for Encoder-1/2/3/4 (SGCS-4, step-2)

Next, we perform the evaluation of multi-encoder-structure-aware training for Encoder-1/2/3/4 (i.e., SGCS-4, Step-2). When conducting multi-encoder-structure-aware training, simulations associated with n2=0.3, 0.2, 0.1 and 0.05 are compared. The best average testing SGCS of the four encoders is derived when n2 adopts 0.05.

Observation 10: When conducting multi-encoder-structure-aware training, it is demonstrated that n2 = 0.05 achieves the best average testing SGCS performance.

For multi-encoder-structure-aware training for Encoder-1/2/3/4 (i.e., SGCS-4, Step-2), by comparing to Encoder-1’s separate training SGCS-1, we can have the following observations for multi-encoder-structure-aware training:

Observation 11: For multi-encoder-structure-aware training for Encoder-1/2/3/4 (i.e., SGCS-4, Step-2), by comparing to Encoder-1’s separate training (i.e., SGCS-1),

- The losses for Encoder-1, Encoder-2, Encoder-3 and Encoder-4 are 0.0713%, -1.7531%, 1.7531% and 3.6488% respectively, all being degraded at a low level and showing similar performances as Encoder-1’s separate training.

This time for Encoder-4, big complexity difference doesn’t lead to large performance degradation, because the joint trained decoder from N-to-1 multi-encoder-structure-aware training can deliver a new latent representation that is compatible to the small model capacity of Encoder-4.

Observation 12: With the N-to-1 multi-encoder-structure-aware training, the joint trained decoder can deliver a new latent representation that is compatible to the small model capacity of Encoder-4.

Base on all the above observations, we can have the following conclusion.

Proposal 5: The feasibility of a common decoder for future-proof compatible to encoders with different backbones and with different complexities can be confirmed, by considering:

- Minimum complexities of the different encoder backbones should be guaranteed for the specified common decoder, if 1-to-1 separate training is used.

- N-to-1 multi-encoder-structure-aware training can help to improve particular low-complexity encoder backbone.

# 3. Remaining issues for non-simulation

In this section, we provided our viewpoints on the non-simulation issues for CSI compression. Since Option 3 is considered in the WID for Rel-20, the corresponding analysis is prioritized.

## 3.1 What needs to be captured or specified for Option 3

For the content to be captured or specified for Option 3, the following agreement is achieved:

Agreements: The following agreements were reached for option 3. For clarity, the agreement is added alongside the agreement from RAN4#114bis. Agreements from RAN4#114bis - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) New agreement from RAN4#115: The training dataset used to train the test decoder in 3GPP should be captured - FFS whether channel, Eigenvectors or something else

Based on our understanding, RAN4 reach the agreement to store the reference encoder since it could facilitate UE encoder development, e.g., to use the reference encoder directly or finetuning on top of the reference encoder. However, we expect one reference encoder is generated at most corresponding to one specified decoder, if it is 1-to-1 joint training. If it is encoder-only training based on the frozen test decoder, based on different model structure, we expect multiple reference encoders can be obtained.

Proposal 6: For Option 3, the encoder that corresponds to the standardized test decoder is stored within 3GPP:

- If this encoder is not provided as RAN1 reference encoder, it could be stored in TR as standardization intermediate outcome, which can further refer to a certain 3GPP database location storing the reference encoder;

- If this encoder and the corresponding standardized test decoder are 1-to-1 joint-trained:

-- Only one reference encoder is expected;

- If this encoder is obtained by encoder-only training with frozen test decoder:

-- More than one reference encoders can be generated, but the value of storing them is low.

Based on our experience, in most cases, the joint-training of the encoder and test decoder can perform better than encoder-only training, therefore, we expect the reference encoder stored here should be obtained by joint-training.

Proposal 7: For Option 3, the encoder that corresponds to the standardized test decoder is stored within 3GPP:

- It should be based on 1-to-1 joint-training of this encoder and corresponding standardized test decoder.

For the corresponding datasets for training and test, based on the experience till now, the mixed dataset is better when generating the test decoder with reasonable generalization, therefore, it is reasonable to store the training dataset and corresponding test dataset, which could be used for UE encoder development and TE environment development. For the detailed content to be stored, “whether channel, Eigenvectors or something else” should depends on the standardized use case in Rel-20.

Observation 13: For Option 3, for dataset used for training and testing, the detailed content to be stored, “whether channel, Eigenvectors or something else” should depends on the standardized use case in Rel-20.

## 3.2 What needs to be captured or specified for Option 4

Since Test decoder Option 4 is not considered in the WID for Rel-20, no more analysis on Option 4 is required.

# 4. Conclusion

In this contribution, we provided our viewpoints on the on the interoperability and testability aspects for AI-CSI compression use case, accordingly the following observations and proposals are obtained:

Remaining simulation for low-complexity encoder training

Observation 1: To evaluate the feasibility of the common decoder compatible with different encoder backbones/complexities, the performance achieved for CNN-based backbones should be considered as the baseline performance (SGCS-0 for joint training and SGCS-1 for separate training).

Observation 2: By comparing the separate training for encoder-2/3/4 (SGCS-2) and encoder-1 (SGCS-1), the following evaluation purposes should be focused:

- Purpose-(1): the feasibility of future-proof common decoder, i.e., whether it is feasible to derive a common decoder which can be compatible with encoders with different/new backbones which has not been considered in the training. In other words, Purpose-(1) guarantee the RAN4-specified common decoder is future-proof for unseen AI/ML model backbone.

- Purpose-(2): the feasibility of low-complexity encoder, i.e., whether it is feasible to have a separate training to derive low-complexity encoder(s) which is compatible with known common decoder with more complex backbone structure. In other words, Purpose-(2) guarantee the UE vendors can derive models with low complexity for implementation flexibility.

Observation 3: For a given new encoder-x with different backbone from encoder/decoder-1, by evaluating the performances for separate training for encoder-x (SGCS-2), separate training for encoder-1 (SGCS-1), joint training for encoder-x (SGCS-3) and joint training for encoder-1 (SGCS-0), the following conclusions can be made:

| What we observed | What we can conclude | What we observed | What we can conclude |
| --- | --- | --- | --- |
| SGCS-3 ≈ SGCS-0 | Encoder-x backbone is compatible with the common decoder backbone | SGCS-2 ≈ SGCS-1 | (1) Separate training is feasible for encoder-x backbone; (2) Specified common decoder is feasible and future-proof for unseen low-complexity encoder. |
|  |  | SGCS-2 < SGCS-1 | (1) Separate training is NOT feasible for encoder-x backbone, while it is to be study the encoder-structure-aware training is feasible for this case. (2) Specified common decoder is NOT feasible/future-proof if similar observation for different encoder backbones. |
| SGCS-3 < SGCS-0 | Encoder-x backbone is NOT compatible with the common decoder backbone | N/A |  |

Proposal 1: Use the term “N-to-1 Multi-encoder-structure-aware training” to represent the procedure for SGCS-4.

Observation 4: On top of Observation 3, the evaluation of multi-encoder-structure-aware training (SGCS-4, Step-2), the highlighted further conclusions can be made:

| What we observed | What we can conclude | What we observed | What we can conclude | What we observed | What we can conclude |
| --- | --- | --- | --- | --- | --- |
| SGCS-3 ≈ SGCS-0 | Encoder-x backbone is compatible with the common decoder backbone | SGCS-2 ≈ SGCS-1 | (1) Separate training is feasible for encoder-x backbone; (2) Specified common decoder is feasible and future-proof for unseen low-complexity encoder. | N/A |  |
|  |  | SGCS-2 < SGCS-1 | (1) Separate training is NOT feasible for encoder-x backbone, while it is to be study the multi-encoder-structure-aware training is feasible for this case. (2) Specified common decoder is NOT feasible/future-proof if similar observation for different encoder backbones. | SGCS-4 (step-2) ≈ SGCS-1 | Multi-encoder-structure-aware training is required to consider encoder-x structure |
|  |  |  |  | SGCS-4 (step-2) < SGCS-1 | Multi-encoder-structure-aware training methodology is NOT feasible for this case |
| SGCS-3 < SGCS-0 | Encoder-x backbone NOT compatible with the common decoder backbone | N/A |  |  |  |

Observation 5: For joint-training of Encoder-1/2/3/4 with the agreed decoder backbone (by comparing SGCS-3 for Encoder-2/3/4 to SGCS-0 for Encoder-1),

- The losses for Encoder-2, Encoder-3 and Encoder-4 are -2.0661%, 2.3368% and 3.9470%, all being regarded as similar performance as SGCS-0.

Observation 6: For the evaluated very low-complexity Encoder-4 (MLP-based low-complexity encoder), the SGCS from joint-trained encoder-4 has worse performance compared to other evaluated encoders.

Observation 7: For separate training for Encoder-1/2/3/4 with Samsung’s frozen decoder, i.e., SGCS-1 for Encoder-1 and SGCS-2 for Encoder-2/3/4, by comparing to the counterpart from joint training, i.e., SGCS-0 for Encoder-1 and SGCS-3 for Encoder-1:

- The losses for separate trained Encoder-1, Encoder-2 and Encoder-3 are 0.0285%, 1.5915% and 0.4085% respectively, all being degraded at a low level and showing similar performances as joint trained ones respectively;

- The loss for Encoder-4 is 5.7707% which show obvious performance degradation.

Observation 8: For separate training for Encoder-2/3/4 with Samsung’s frozen decoder, i.e., SGCS-2 for Encoder-2/3/4, by comparing the performance from the separate training for Encoder-1, i.e., SGCS-1:

- The losses for Encoder-2 and Encoder-3 are -0.4704% and 2.7081% respectively, being degraded at a low level and showing similar performances as Encoder-1.

- The loss for Encoder-4 is 9.4641% which show obvious performance degradation compared to other encoders.

Observation 9: The evaluated very low-complexity Encoder-4 (MLP-based low-complexity encoder) lack enough model capacity to compress the input features into the latent representation that the common decoder requires.

Proposal 2: For the evaluated very low-complexity Encoder-4 (MLP-based low-complexity encoder), it should not be recommended as reference encoder selection if the multi-encoder-structure-aware training is not considered:

- The low performance reason can be further studied for (1) MLP backbone; (2) very low complexity; or both (1) and (2).

Proposal 3: For the evaluated low-complexity Encoder-2/3 (transformer-based and CNN-based):

- Separate training with Samsung’s frozen common decoder is feasible for encoder-2/3 backbone

Proposal 4: The frozen Samsung’s decoder is feasible and future-proof for unseen low-complexity encoder, which also demonstrate the methodology of separate training.

Observation 10: When conducting multi-encoder-structure-aware training, it is demonstrated that n2 = 0.05 achieves the best average testing SGCS performance.

Observation 11: For multi-encoder-structure-aware training for Encoder-1/2/3/4 (i.e., SGCS-4, Step-2), by comparing to Encoder-1’s separate training (i.e., SGCS-1),

- The losses for Encoder-1, Encoder-2, Encoder-3 and Encoder-4 are 0.0713%, -1.7531%, 1.7531% and 3.6488% respectively, all being degraded at a low level and showing similar performances as Encoder-1’s separate training.

Observation 12: With the N-to-1 multi-encoder-structure-aware training, the joint trained decoder can deliver a new latent representation that is compatible to the small model capacity of Encoder-4.

Proposal 5: The feasibility of a common decoder for future-proof compatible to encoders with different backbones and with different complexities can be confirmed, by considering:

- Minimum complexities of the different encoder backbones should be guaranteed for the specified common decoder, if 1-to-1 separate training is used.

- N-to-1 multi-encoder-structure-aware training can help to improve particular low-complexity encoder backbone.

Remaining issues for non-simulation

Proposal 6: For Option 3, the encoder that corresponds to the standardized test decoder is stored within 3GPP:

Proposal 7: For Option 3, the encoder that corresponds to the standardized test decoder is stored within 3GPP:

- It should be based on 1-to-1 joint-training of this encoder and corresponding standardized test decoder.

Observation 13: For Option 3, for dataset used for training and testing, the detailed content to be stored, “whether channel, Eigenvectors or something else” should depends on the standardized use case in Rel-20.

# 5. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402390, “Discussion on testability and interoperability issues for AI-CSI”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] R4-2405653, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#110bis, April 2024.

[6] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[7] R4-2409762, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#111, May 2024.

[8] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[9] https://en.wikipedia.org/wiki/Multiply%E2%80%93accumulate_operatio

[10] R4-2411636, “Further study on testability and interoperability issues for AI-CSI”, RAN#112, Aug. 2024.

[11] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[12] R4-2416495, “Further study on testability and interoperability issues for AI-CSI compression”, Samsung, RAN4#112bis, Oct. 2024.

[13] R4-2417207, “WF on data sharing for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[15] R4-2418095, “Further study on testability and interoperability issues for AI-CSI compression”, Samsung, RAN4#113, Nov. 2024.

[16] R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, RAN4#113, Nov. 2024.

[17] R4-2501966, “Study on testing issues for CSI compression two-sided models”, Samsung, RAN4#114, Feb. 2025.

[18] R4-2502953, “WF on AI/ML air interface study”, Ericsson, RAN4#114, Feb. 2025.

[19] R4-2504217, “Study on testing issues for CSI compression two-sided models”, Samsung, RAN4#114bis, Apr. 2025.

[20] R4-2505107, “WF on AI/ML air interface study”, Ericsson, RAN4#114bis, Apr. 2025.

[21] R4-2505156, “Simulation assumptions on AI/ML CSI compression”, Huawei, RAN4#114bis, Apr. 2025.

[22] R4-2508051, “WF on NR_AIML_air_part2”, Ericsson, RAN4#115, May 2025.

[23] R4-2508084, “Simulation assumptions for CSI compression”, Huawei, RAN4#115, May 2025.