3GPP TSG-RAN WG4 Meeting #116                                                              R4-2511249

Bengaluru, India, 25th – 29th August, 2025

Source:	Samsung

Title:	Discussion on RRM core requirement and testing framework for beam management

Agenda item:		7.17.4

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two confirmed use cases (i.e., beam management and positioning accuracy enhancements) [2].

Particularly for the use case of beam management, it is required to provide specification support for DL TX beam prediction for both UE-sided and NW-sided model, and correspondingly necessary RAN4 core requirements for this use case and LCM procedure including performance monitoring [2], as follows:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In this contribution, we would like to further discuss on core requirement for AI-based beam management use cases, including the impact on both BM-Case1 and BM-Case2.

# 2. RAN4 Core Requirement for supporting UE-Sided Model

## 2.1 Performance Evaluation for AI Beam Management Model

According to the discussion in RAN#114bis, the following top-K beam prediction accuracy and the absolute RSRP accuracy have been adopted as the typical KPIs to identify the AI beam management performance.

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

In this section, we present numerical simulation result for AI beam management model based on the above two performance metrics. In the simulations, the BM-Case 1, i.e., spatial domain prediction scenario, is taken into account and the simulation in this meeting focuses on the narrow-to-narrow sub-use case.

The data used for model training, validation, and evaluation originates from the vivo shared dataset in 3GPP website, i.e., VIV0R4_115_00.zip, and the number of RSRP data entries used for training/validation/evaluation process accounts for 90%, 5%, 5% of the total RSRP data volume, respectively.

For the purpose of performance alignment, the reference AI model defined in document [21] has been adopted in the following simulations. The AI model takes a set B RSRP tensor input with size of (batch size, 8), and output the prediction beam information with the tensor size of (batch size, 32).

### 2.1.1 Simulation results for the top-K beam prediction accuracy

Based on previous discussion [21], the beam prediction accuracy performance of top-K beams (with K = 1, 2, 3, 4, 5 and X = 0, 1, 2, 3) has been evaluated.

In the simulation, the measurement error-free scenario (Case 1), the model inference input data error scenario (Case 2a), and the model inference input data error plus ground-truth data error case have been considered. In addition, we only considered RF measurement error in this simulation stage. The RF error is modeled as a truncated Gaussian noise with zero mean, and covariance of 4.5dB, which is illustrated in document [21]. The simulation results are summarized in Table-I.

Table-I. Top-K beam prediction accuracy performance for AI-based DL Tx beam prediction

| Beam Prediction Accuracy |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| K |  | Top-1 | Top-2 | Top-3 | Top-4 | Top-5 |
| Case 1 No-Error | X = 0 | 66.73% | 82.10% | 88.99% | 93.02% | 95.67% |
|  | X = 1 | 73.05% | 86.46% | 92.24% | 95.55% | 97.55% |
|  | X = 2 | 77.04% | 89.20% | 93.97% | 96.82% | 98.32% |
|  | X = 3 | 79.57% | 91.10% | 95.23% | 97.71% | 98.79% |
| Case 2a Input Error in Inference (RF Error) | X = 0 | 28.50% | 46.76% | 59.39% | 66.56% | 75.03% |
|  | X = 1 | 34.51% | 53.72% | 65.98% | 72.96% | 80.74% |
|  | X = 2 | 38.98% | 59.17% | 71.43% | 77.97% | 84.92% |
|  | X = 3 | 42.75% | 63.64% | 75.27% | 81.76% | 87.58% |
| Case 2b Input & Label Error in Inference (RF Error) | X = 0 | 24.49% | 41.27% | 53.70% | 63.57% | 69.60% |
|  | X = 1 | 29.45% | 47.50% | 60.21% | 70.33% | 75.83% |
|  | X = 2 | 34.38% | 53.69% | 65.90% | 75.80% | 81.20% |
|  | X = 3 | 38.82% | 59.14% | 70.92% | 79.87% | 84.85% |

Based on Table-I, it can be directly inferred that the beam prediction accuracy is increased with the increment of the predicted beam number K for all measurement error cases. This is reasonable because when the AI model is configured to output the larger number of beams, it is more likely that the AI model guess the best Tx beam from the statistic perspective. On the other hand, from the simulation result, it can also be concluded that the beam predicted accuracy performance is improved when the larger RSRP threshold level X is set. The phenomenon can be attribute to the relaxed prediction accuracy requirement led be the larger X.

However, the simulation results also indicates that the reference model defined in document [21] has quite poor generalization performance when facing measurement error, especially in Top-1 prediction scenarios. To combat complex measurement error in practical environment, the robust AI-BM prediction model should be explored. In addition, comparing the prediction accuracy results in Case 2a and Case 2b scenarios, it can also be found that the prediction accuracy downgrades with more dimension of error introduced, i.e., the ground-truth label error dimension.

Observation-1: The reference model agreed shows limited performance, and especially showing poor generalization performance with measurement errors added.

Observation-2: With measurement errors considered in reference, the performance for Top-1 prediction is lower than reasonable performance even with X = 3dB margin considered (i.e., 42.75% for Case 2a and 38.82% for Case 2b).

### 2.1.2 Simulation results for the absolute RSRP accuracy

In this section, we evaluate the performance of absolute RSRP accuracy for reference AI model. In the simulation, only error-free scenario (Case 1) is considered and the 90%-tile RSRP difference performance of both Top-1, Top-3, and Top-5 beams are compared. The simulation results are summarized in Table-II.

Table-II. 90%-tile RSRP difference between the predicted L1-RSRP and the ideal L1-RSRP of the same beams

| 90%-tile RSRP difference |  |  |  |  |
| --- | --- | --- | --- | --- |
| K |  | Top-1 | Top-3 | Top-5 |
| Case 1 No-Error |  | 7.7467 | 7.8858 | 8.0114 |

Note that the performance is calculated using the predicted RSRP and the ground truth RSRP of 1st/3rd/5th predicted beams, and the performance is calculated for each beam independently. Thus, it may not be possible to inference the 90%-tile RSRP of one beam from that of the other.

Observation-3: Even without measurement errors considered in reference, RSRP accuracy is worse than 7dB.

## 2.2 RRM core requirement for model Inference

Based on the existing discussion, seems the group have the common understanding to specify the requirement for model inference for UE-sided model. Specifically, we expect corresponding RRM requirement shall be introduced: to guarantee (1) the accuracy of inference results accordingly to the agreed KPI; (2) the delay of reporting inference results (containing the inference latency).

### 2.2.1 Absolute RSRP accuracy for predicted RSRP

In previous RAN4 discussion, the definition of the absolute RSRP accuracy is provided:

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In RAN4#114, RSRP accuracy is agreed to one of the KPIs if RSRP is contained in the report. Furthermore, in RAN4#114bis, it is discussed that the absolute RSRP requirement is applicable to which beam:

Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

In current RAN1 design, only the beam with the largest RSRP is reported by using the absolute RSRP reporting, which is similar to current L1-RSRP report in CSI reporting.

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

It should be noted that in the existing L1-RSRP measurement accuracy conformance testing requirement, only the “strongest SSB” within the periodic L1-RSRP report are checked for the absolute accuracy requirement, while the 2nd strongest SSB is exanimated for the diff-RSRP value. We copied the relevant testing procedure in TS38.533 for reference:

4. The UE shall start sending valid L1-RSRP reports, a valid report shall meet the absolute L1-RSRP requirement for SSB#0 and the relative L1-RSRP requirement for SSB#1. The SS shall check following requirements: - R1: The L1-RSRP value of SSB#0 reported by the UE is compared to the expected L1-RSRP value for SSB#0. If the value is outside the limits in Table 7.7.4.1.5-3, Table 5.7.4.1.5-3a or Table 7.7.4.1.5-4 (depending on the test configuration) or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one. - R2: The DIFF-RSRP value of SSB#1 reported by the UE is compared to the expected DIFF-RSRP value. If the resulting value is outside the limits in Table 5.7.1.1.5-5 or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

Accordingly, the following proposal is given for RSRP accuracy issue.

Proposal 1: For absolute RSRP accuracy requirement, the requirement shall be defined as:

- For core requirement and conformance testing, the absolute RSRP accuracy requirement shall only be applied to the strongest beam in the configured top-K beam RSRP report.

- The existing measurement accuracy requirement is used as the baseline for prediction accuracy.

### 2.2.2 Relative RSRP accuracy for predicated RSRP

Based on the discussion in RAN4#114bis, the following agreement is achieved for relative RSRP accuracy:

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Based on RAN1 agreement as below, reported L1-RSRP is the predicted RSRP:

Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

Accordingly, the definition of relative RSRP accuracy shall be defined as:

Proposal 2: For relative RSRP accuracy requirement, if defined, the requirement shall be defined as:

- Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value given by absolute RSRP reporting.

  - Reported L1-RSRP can be predicted,

  - Reported L1-RSRP shall be predicted RSRP

- Relative RSRP accuracy requirements apply for Case 1, and also for Case 2

### 2.2.3 Metric/KPIs for beam ID prediction accuracy

Based on RAN4 discussion, the following agreement is achieved:

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values

For the remaining issue, including “FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation”, since there is no time left to evaluate 2nd ranked predicted beam’s accuracy, we propose to only consider top-1 in Rel-19.

Proposal 3: For the beam ID predication accuracy in Rel-19, only top-1 (i.e., Top-N, N=1) predicted beam is considered in the successful rate evaluation/requirement.

### 2.2.4 Prediction delay requirement including measurement period

The legacy L1-RSRP measurement requirement, which is based on one-shot L1 measurement, establishes a relationship between the measurement period and accuracy. Similar to the L1 measurement requirement, the measurement period for AI-BM prediction (for AI-BM Case 1) should first be determined. It is logical to use the legacy measurement period as a baseline and then assess the achievable prediction accuracy accordingly.

Proposal 4: For AI-BM Case-1, the legacy L1 measurement period (the same M, N, P) is used for Set-B measurement to evaluate the achievable prediction accuracy.

# 3. Conclusion

In this contribution, we would like to further discuss on core requirement for AI-based beam management use cases, including the impact on both BM-Case1 and BM-Case2.

Performance Evaluation for AI Beam Management Model

Observation-1: The reference model agreed shows limited performance, and especially showing poor generalization performance with measurement errors added.

Observation-2: With measurement errors considered in reference, the performance for Top-1 prediction is lower than reasonable performance even with X = 3dB margin considered (i.e., 42.75% for Case 2a and 38.82% for Case 2b).

Observation-3: Even without measurement errors considered in reference, RSRP accuracy is worse than 7dB.

RRM core requirement for model Inference

Proposal 1: For absolute RSRP accuracy requirement, the requirement shall be defined as:

- For core requirement and conformance testing, the absolute RSRP accuracy requirement shall only be applied to the strongest beam in the configured top-K beam RSRP report.

- The existing measurement accuracy requirement is used as the baseline for prediction accuracy.

Proposal 2: For relative RSRP accuracy requirement, if defined, the requirement shall be defined as:

- Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value given by absolute RSRP reporting.

  - Reported L1-RSRP can be predicted,

  - Reported L1-RSRP shall be predicted RSRP

- Relative RSRP accuracy requirements apply for Case 1, and also for Case 2

Proposal 3: For the beam ID predication accuracy in Rel-19, only top-1 (i.e., Top-N, N=1) predicted beam is considered in the successful rate evaluation/requirement.

Proposal 4: For AI-BM Case-1, the legacy L1 measurement period (the same M, N, P) is used for Set-B measurement to evaluate the achievable prediction accuracy.

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402389, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] 3GPP TR 38.810, v16.7.0, “NR; Study on test methods”.

[6] 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[7] R4-2405652, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110bis, April 2024.

[8] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[9] R4-2409761, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#111, May 2024.

[10] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[11] R4-2411635, “Further discussion on testability and interoperability issues for beam management”, Samsung, RAN#112, Aug. 2024.

[12] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[13] R4-2416492, “Further discussion on beam management core requirements”, RAN#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[15] R4-2418093, “Further discussion on beam management core requirements”, RAN#113, Nov. 2024.

[16] R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, RAN#113, Nov. 2024.

[17] R4-2501964, “Further discussion on beam management core requirements”, RAN#114, Feb. 2025.

[18] R4-2502856, “WF on the requirements for AI/ML air interface”, Qualcomm, RAN#114, Feb. 2025.

[19] R4-2504216, “Further discussion on beam management core requirements”, RAN#114bis, Apr. 2025.

[20] R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm, RAN#114bis, Apr. 2025.

[21] R4-2508081, “Updated simulation assumption for BM”, vivo, NTU, Nokia, etc., RAN#115, May 2025.

[22] R4-2508080, “WF on NR_AIML_air_part1”, Qualcomm, RAN#115, May 2025.

# 5. Appendix – Existing Agreements on AI-BM

In RAN4#110bis, for the metrics/KPIs for Beam Management requirements/tests, the following agreement is achieved:

Issue 2-1: Metrics/KPIs for Beam Management requirements/tests Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

In RAN4#111, for reported measurements and ground truth, the following agreements are achieved:

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

And the following agreement is achieved in RAN4#112:

Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy. Issue 2-7: UE reporting for network side models Agreement: postpone discussion until other WGs define any reporting mechanisms

And the following agreement is achieved in RAN4#112bis:

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

The following agreement is achieved in RAN4#113:

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. ### Agreements in ad-hoc session (R4-2420335): Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

The following agreement is achieved in RAN4#114:

- Agreements in ad-hoc session (R4-2503012) Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options Issue 2-4: QCL Source RS for TCI states Agreement: wait for RAN1 discussion to conclude without sending any LS ### Agreements in main session Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point

The following agreement is achieved in RAN4#114bis:

## RRM core requirement and testing framework for beam management ### Agreements in main session: Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2 Issue 2-4: Measurement period for inference Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2 Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

The following agreement is achieved in RAN4#115:

## RRM core requirement and testing framework for beam management ### Agreements in main session: Issue 2-2: Measurement period for inference Agreement: Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined) Case 2: postpone discussion until RAN1 concludes Issue 2-3: Rx beam knowledge and TCI handling Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state