3GPP TSG-RAN WG4 Meeting #116	R4-2511395

# Bangalore, India, 25th – 29th August 2025

Agenda item:		X.X.X

Source: 		Qualcomm Inc.

Title:	 	PC2 A-MPR corrections for NS_48 and NS_49

Document for:		Approval

## 1	Introduction

Corrections to n1 A-MPR has been discussed for multiple meetings. In previous meeting it was observed for NS_48 and NS_49 that in addition to the ambiguous 2 dB allowance for 2 Tx implementation, also A-MPR region boundaries have errors which allow PC2 UE to transmit less power than PC3 UE. In this contribution we analyze the needed corrections.

## 2	Discussion

NS_48

For NS_48 PC3 A-MPR exists only for 25MHz and wider channel bandwidths. PC2 adds A-MPR also for 10, 15 and 20 MHz CBW. For these CBW, A-MPR cannot exceed PC3 MPR+ 3 dB

Observation 1: A-MPR needs to be reduced to be at most 3 dB higher than PC3 MPR for 10 to 20 MHz CBW, both for NS_48 and NS_49

For wider channel bandwidth A-MPR is specified both for PC3 and PC2. Comparison of region boundaries based on current specification is shown below. Black lines represent PC3 boundaries and blue boundaries are for PC2.

Figure 1: Region boundaries for PC3 (black) and PC2 (blue), 25 MHz CBW

Figure 2: Region boundaries for PC3 (black) and PC2 (blue), 30 MHz CBW

Figure 3: Region boundaries for PC3 (black) and PC2 (blue), 40 MHz CBW

Figure 4: Region boundaries for PC3 (black) and PC2 (blue), 45 MHz CBW

Figure 5: Region boundaries for PC3 (black) and PC2 (blue), 50 MHz CBW

For most cases the boundaries for PC2 have been shifted without care on where PC3 boundaries are. This leads to situations that there are allocations which get no A-MPR in PC3, but get pulled into a PC2 region which gets an increase over PC3 A-MPR. These cases should be corrected so that PC3 regions remain for PC2, and new regions are created to cover the cases where PC2 areas are wider.

As the region numbers also shift between PC3 and PC2, it is not always possible to add 0.5 dB to A-MPR. This happends even though within same region name (A3 PC3 to A3 PC2) A-MPR increase would be less than 3 dB, there are cases where same RB allocation is defined as e.g as A3 for PC3 but A4 for PC2, which already have 3 dB difference (for CP-OFDM).

Proposal 1: A-MPR regions are corrected with following principles

- PC3 regions are re-used for PC2

- New regions are created for allocations which were allowed A-MPR for PC2, but not for PC3

- Set of RB allocations which are allowed A-MPR are unchanged, just the RB regions and values change

- 0.5 dB A-MPR increase is applied where possible to offset note 7 removal

Following these principles, NS_48 A-MPR updates are summarized in Table 1 and Table 2. As A4-AMPR is reduced to fulfil max 3 dB delta for 20 MHz CBW, for wider CBW A4 was replaced by A3, as the values are nearly the same. 1 dB increase was applied to A3 to bridge the difference.

Table 1: A-MPR regions for NS_48 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A6 |
|  |  | <1.8 | ≥0 | A6 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A6 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A6 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A6 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 1.08) < 9.72 | A6 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥13.5 | A3 |
|  |  | ≥0 | ≥12.96, < 13.5 | A6 |
|  |  | ≥21.6 | <1.08 | A5 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 3.6) < 12.96 | A6 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A2 |
|  |  | ≥2.88, <17.1 | ≥ max (0, 12*SCS*RBend - 3.6) | A3 |
|  |  | ≥2.88, <17.1 | < max (0, 12*SCS*RBend - 3.6), ≥ max (0, 12*SCS*RBend – 4.68) | A6 |
|  |  | ≥17.1, <27.36 | ≥13.5 | A3 |
|  |  | ≥27.36, <34.56 | ≥13.5 | A2 |
|  |  | ≥17.1, <34.56 | ≥12.96, <13,5 | A6 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <4.86 | >0 | A2 |
|  |  | ≥4.86, <5.22 | < max (0, 12*SCS*RBend – 3.6) | A6 |
|  |  | ≥4.86, <19 | ≥max (0, 12*SCS*RBend - 3.6) | A3 |
|  |  | ≥5.22, <19 | ≥max (0, 12*SCS*RBend - 5.4), <max (0, 12*SCS*RBend - 3.6) | A6 |
|  |  | ≥19, <37.44 | ≥15.4 | A2 |
|  |  | ≥19, <37.44 | ≥14.04, <15.4 | A6 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <6.12 | >0 | A2 |
|  |  | ≥6.12, <7.2 | <max (0, 12*SCS*RBend – 3.6) | A6 |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend – 3.6) | A3 |
|  |  | ≥7.2, <20.7 | ≥max (0, 12*SCS*RBend – 5.4) < max (0, 12*SCS*RBend – 3.6) | A6 |
|  |  | ≥20.7, <41.04 | ≥17.1 | A2 |
|  |  | ≥20.7, <41.04 | ≥15.12, <17.1 | A6 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 2: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤12.5 | ≤8.5 | ≤5 | ≤ 3.5 | ≤6.5 | ≤3.0 |
|  | QPSK | ≤12.5 | ≤8.5 | ≤5 | ≤ 4 | ≤6.5 | ≤3.0 |
|  | 16 QAM | ≤12.5 | ≤8.5 | ≤5 | ≤4.5 | ≤6.5 | ≤3.0 |
|  | 64 QAM | ≤12.5 | ≤8.5 | ≤5 | ≤4.5 | ≤6.5 | ≤3.0 |
|  | 256 QAM | ≤12.5 | ≤8.5 | ≤5 | ≤4.5 | ≤6.5 | ≤3.0 |
| CP-OFDM | QPSK | ≤13 | ≤9.5 | ≤6.5 | ≤6.0 | ≤6.5 | ≤4.5 |
|  | 16 QAM | ≤13 | ≤9.5 | ≤6.5 | ≤6.0 | ≤6.5 | ≤4.5 |
|  | 64 QAM | ≤13 | ≤9.5 | ≤6.5 | ≤6.5 | ≤6.5 | ≤4.5 |
|  | 256 QAM | ≤13 | ≤9.5 | ≤6.5 | ≤6.5 | ≤6.5 | ≤4.5 |

As implementations already exists, corrections should be made to release 19.

Proposal 2: Do all PC2 A-MPR corrections to release 19.

NS_49

Following the same principle as for NS_48, A-MPR applied for 10-20 MHz CBW cannot exceed PC3 MPR, as no A-MPR is allowed for PC3 for these CBW

Comparison of NS_49 A-MPR region boundaries based on current specification are shown below

Figure 6: Region boundaries for PC3 (black) and PC2 (blue), 25 MHz CBW

Figure 7: Region boundaries for PC3 (black) and PC2 (blue), 30 MHz CBW

Figure 8: Region boundaries for PC3 (black) and PC2 (blue), 40 MHz CBW

Figure 9: Region boundaries for PC3 (black) and PC2 (blue), 45 MHz CBW

Figure 10: Region boundaries for PC3 (black) and PC2 (blue), 45 MHz CBW

Table 3: A-MPR regions for NS_49 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A6 |
|  |  | <1.8 | ≥0 | A6 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A6 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A6 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A6 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A6 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥7.92, <9.72 | A6 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤3.96 | <1.08 | A3 |
|  |  | ≤3.96 | ≥1.08 | A6 |
|  |  | ≤6.48, >3.96 | <3.6 | A6 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <6.48 | ≥0 | A5 |
|  |  | ≥6.48, <7.92 | < max (0,12*SCS* RBend - 4.32) | A6 |
|  |  | ≥6.48, <14.4 | ≥max (0,12*SCS* RBend - 3.6) | A3 |
|  |  | ≥7.92, <14.4 | > max (0,12*SCS* RBend - 3.6), ≥max (0,12*SCS* RBend - 4.32) | A6 |
|  |  | ≥14.4, <21.6 | ≥10.8 | A4 |
|  |  | ≥21.6 | ≥10.8 | A2 |
|  |  | ≥14.4, | ≥10.44, <10.8 | A6 |
|  |  | ≥21.6 | <1.08 | A5 |
|  |  | ≥21.6 | <1.8, ≥1.08 | A6 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <7.2 | ≥0 | A1 |
|  |  | ≥7.2, <10.44 | <1.08 | A5 |
|  |  | ≥7.2, <11.52 | ≥1.08, <3.06 | A6 |
|  |  | ≥10.44, <11.52 | <1.08 | A6 |
|  |  | ≥7.2, <18 | ≥max (0, 12*SCS*RBend – 3.6) | A4 |
|  |  | ≥7.2, <18 | >3.06, ≥max (3.06, 12*SCS*RBend – 6.48), < max (3.06, 12*SCS*RBend – 3.6) | A6 |
|  |  | ≥18, <34.56 | ≥14.4, <28.8 | A2 |
|  |  | ≥18, <34.56 | <14.4, ≥11.16 | A6 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥28.8 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
|  |  | ≥6.12, <12.42 | < min (1.08, max(0,12*SCS* RBend-6.12)) | A5 |
|  |  | ≥30.76, <36.72 | <1.08 | A5 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | <36.72 | ≥16.2, <max (0, 12*SCS*RBend – 6.12) | A2 |
|  |  | <36.72 | ≥12.24, <16.2 <max (0, 12*SCS*RBend – 6.12) | A6 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 6.12) | A1 |
|  |  | <36.72 | ≥1.08, <12.24 <max (0, 12*SCS*RBend – 6.12) ≥max (0, 12*SCS*RBend – 7.92) | A6 |
|  |  | ≥36.72 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥7.74, <14.4 | < min (1.08, max(0,12*SCS* RBend-7.74)) | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥18, <max (0, 12*SCS*RBend – 7.74) | A2 |
|  |  | <39.6 | ≥13.68, <18, < max (0, 12*SCS*RBend – 7.74) | A6 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 10.08) | A1 |
|  |  | <39.6 | ≥1.08, <13.68, < max (0, 12*SCS*RBend – 7.74) ≥ max (0, 12*SCS*RBend – 10.08) | A6 |
|  |  | ≥39.6 | >0 | A1 |

Table 4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  |  | A1 |  | A2 |  | A3 |  | A4 |  | A5 |  | A6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  |
| DFT-s-OFDM | PI/2 BPSK |  | ≤13 |  | ≤7.5 |  | ≤4.5 |  | ≤6 |  | ≤6.5 |  | ≤3.0 |  |
|  | QPSK |  | ≤13 |  | ≤7.5 |  | ≤4.5 |  | ≤6 |  | ≤6.5 |  | ≤3.0 |  |
|  | 16 QAM |  | ≤13 |  | ≤7.5 |  | ≤4.5 |  | ≤6 |  | ≤6.5 |  | ≤4 |  |
|  | 64 QAM |  | ≤13 |  | ≤7.5 |  | ≤4.5 |  | ≤6 |  | ≤6.5 |  | ≤4.5 |  |
|  | 256 QAM |  | ≤13 |  | ≤7.5 |  | ≤4.5 |  | ≤6 |  | ≤6.5 |  | ≤4.5 |  |
| CP-OFDM | QPSK |  | ≤14 |  | ≤8.5 |  | ≤6 |  | ≤7.5 |  | ≤6.5 |  | ≤4.5 |  |
|  | 16 QAM |  | ≤14 |  | ≤8.5 |  | ≤6 |  | ≤7.5 |  | ≤6.5 |  | ≤5 |  |
|  | 64 QAM |  | ≤14 |  | ≤8.5 |  | ≤6 |  | ≤7.5 |  | ≤6.5 |  | ≤5.5 |  |
|  | 256 QAM |  | ≤14 |  | ≤8.5 |  | ≤6 |  | ≤7.5 |  | ≤6.5 |  | ≤5.5 |  |

Companion CR can be found in [1].

## 3	Conclusion

In this contribution A-MPR for NS_48 and NS_49 was discussed, following observations and proposals were made

Observation 1: A-MPR needs to be reduced to be at most 3 dB higher than PC3 MPR for 10 to 20 MHz CBW, both for NS_48 and NS_49

Proposal 1: A-MPR regions are corrected with following principles

- PC3 regions are re-used for PC2

- New regions are created for allocations which were allowed A-MPR for PC2, but not for PC3

- Set of RB allocations which are allowed A-MPR are unchanged, just the RB regions and values change

- 0.5 dB A-MPR increase is applied where possible to offset note 7 removal

Proposal 2: Do all PC2 A-MPR corrections to release 19.

## 4	References

[1]  R4-2511394, CR to TS 38.101-1: NS_48 and NS_49 A-MPR corrections, Qualcomm Incorporated