3GPP TSG-WG4 Meeting #116	R4-2511629

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

# Title:	NS_48 A-MPR clean-up to remove HPUE note 7.

# Agenda Item:	6.5.3

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

The removal of Footnote 7 from Table 6.2.3.1-1 for Bands n1 NS_48 and NS_49 was previously discussed in [1]. However, some companies flagged that the footnote removal is more complicated than expected because for certain RB allocations regions, the difference between the PC3 A-MPR and the PC2 A-MPR far exceeds the maximum 3dB guideline. This contribution brings our views on addressing these issues. Due to time constraints, we discuss only NS_48.

- Discussion

Table 1 uses the example of DFT-s-OFDM QPSK to highlight the NS_48 key differences between the PC3 and the PC2 A-MPR requirements, where:

- The last column entitled “A-MPR” is the difference of A-MPR between PC2 and PC3 for example case of DFT-s-OFDM QPSK. We highlight in red cases where “A-MPR>3dB, ie. cases where PC2 A-MPR > PC3 A-MPR +3dB,

- Cells highlighted in yellow indicate case where A-MPR regions may need to be redefined.

Table 1: NS_48 PC3 A-MPR vs PC2 A-MPR comparison for DFT-s-OFDM QPSK.

| CBW (MHz) | Carrier Center Frequency (MHz) | PC3 A-MPR from Table 6.2.3.26-1 |  |  |  | PC2 A-MPR from Table 6.2.3.26-3 |  |  |  | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Regions |  | A-MPR Region/ DFTs QPSK A-MPR |  | Regions |  | A-MPR Region/ DFT QPSK A-MPR |  |  |
|  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |  |  |
| 10 MHz | 1925≤ FC ≤ 1975 | MPR applies. For DFT-s-OFDM QPSK: - Edge/Outer: ≤1.0 - Inner: 0 |  |  |  | ≥0 | ≥8.1 | A6 | ≤3.0 | ≤3.0 |
|  |  |  |  |  |  | <1.8 | ≥0 | A6 | ≤3.0 | ≤3.0 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 |  |  |  |  | ≥0 | ≥9 | A6 | ≤3.0 | ≤3.0 |
|  |  |  |  |  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A6 | ≤3.0 | ≤3.0 |
| 20 MHz | 1930≤ FC ≤ 1970 |  |  |  |  | ≥0 | ≥9.72 | A4 | ≤4.5 | Up to 3.5 |
|  |  |  |  |  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A6 | ≤3.0 | ≤3.0 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 | ≤3 | ≥0 | ≥9.72 | A4 | ≤4.5 | 1.5 |
|  |  | ≥18.72 | <1.08 | A3 | ≤3 | ≥18.72 | <1.08 | A3 | ≤4.0 | 1 |
|  |  | MPR applies |  |  |  | ≥0 | ≥max (0, 12*SCS*RBend - 1.08) <9.72 | A4 | ≤4.5 | Up to 3.5 for outer |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥13.5 | A3 | ≤3 | ≥0 | ≥12.96 | A4 | ≤4.5 | 1.5 |
|  |  | ≥21.6 | <1.08 | A5 | ≤5 | ≥21.6 | <1.08 | A5 | ≤6.0 | 1 |
|  |  | MPR applies |  |  |  | ≥0 | ≥max (0, 12*SCS*RBend - 3.6) <12.96 | A4 | ≤4.5 | Up to 3.5 for outer |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A2 | ≤6 | ≥0, <2.88 | ≥0 | A1 | ≤[12] | 6 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 3.6) | A3 | ≤3 | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 4.68) | A3 | ≤4.0 | 1 |
|  |  | ≥17.1, <27.36 | ≥13.5 | A4 | ≤4 | ≥17.1, <27.36 | ≥12.96 | A4 | ≤4.5 | 0.5 |
|  |  | ≥27.36, 34.56 | ≥13.5 | A2 | ≤6 | ≥27.36, <34.56 | ≥12.96 | A2 | ≤8.0 | 2 |
|  |  | ≥27.36, 34.56 | <1.08 | A3 | ≤3 | ≥27.36, <34.56 | <1.08 | A3 | ≤4.0 | 1 |
|  |  | ≥34.56 | ≥0 | A1 | ≤10 | ≥34.56 | ≥0 | A1 | ≤[12] | 2 |
| 45 MHz | 1942.5 ≤ Fc ≤ 1957.5 | ≥0, <4.86 | >0 | A2 | ≤6 | ≥0, <5.22 | >0 | A1 | ≤[12] | 6 |
|  |  | ≥4.86, <19 | ≥max (0, 12*SCS*RBend - 3.6) | A4 | ≤4 | ≥5.22, <19 | ≥max (0, 12*SCS*RBend - 5.4) | A4 | ≤4.5 | 0.5 |
|  |  | ≥19, <37.44 | ≥15.4 | A2 | ≤6 | ≥19, <37.44 | ≥14.04 | A2 | ≤8.0 | 2 |
|  |  | ≥30.96, 37.44 | <1.08 | A5 | ≤5 | ≥30.96, <37.44 | <1.08 | A5 | ≤6.0 | 1 |
|  |  | ≥37.44 | >0 | A1 | ≤10 | ≥37.44 | >0 | A1 | ≤[12] | 2 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <6.12 | >0 | A2 | ≤6 | ≥0, <7.2 | >0 | A1 | ≤[12] | 6 |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend - 3.6) | A4 | ≤4 | ≥7.2, <20.7 | ≥max (0, 12*SCS*RBend – 5.4) | A4 | ≤4.5 | 0.5 |
|  |  | ≥20.7, <41.04 | ≥17.1 | A2 | ≤6 | ≥20.7, <41.04 | ≥15.12 | A2 | ≤8.0 | 2 |
|  |  | ≥33.84, 41.04 | <1.08 | A5 | ≤5 | ≥33.84, <41.04 | <1.08 | A5 | ≤6.0 | 1 |
|  |  | ≥41.04 | >0 | A1 | ≤10 | ≥34.56 | ≥0 | A1 | ≤[12] | 2 |

We identify 3 cases on the summary Table 1:

- For CBW of 20MHz, 25MHz, 30MHz, case where A-MPR>3dB due to PC2 A4 A-MPR vs PC3 MPR,

- For CBW of 40MHz, 45MHz, 50MHz, case where up to 6dB A-MPR may occur,

- For CBW of 30MHz, 40MHz, 45MHz, 50MHz, case PC3 and PC2 have different A-MPR region contours.

  - Case 1: A-MPR>3dB for 20, 25, 30MHz CBW

For 20MHz CBW and for PC2 allocations fulfilling {RBend*12*SCS>=0; LCRB*12*SCS > 9.72MHz},

- only MPR applies for PC3, and all these allocations are outer allocations

- A4 A-MPR applies for both outer and inner for PC2.

- A-MPR may exceed 3dB for Pi.2 BPSK and DFTs QPSK – see Table 2.

Table 2: NS_48 20MHz PC2 A4 A-MPR vs. PC3 MPR comparison, {RBend*12*SCS>=0; LCRB*12*SCS > 9.72MHz}

| Modulation/Waveform |  | PC3 MPR outer (dB) | PC2 MPR outer (dB) | PC2 A4 A-MPR (dB) | PC2 max(MPR,A-MPR) (dB) | PC2-PC3 A-MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 0 | ≤ 0.5 | ≤4.5 | ≤4.5 | 4.5 |
|  | QPSK | ≤1 | ≤ 1 | ≤4.5 | ≤4.5 | 3.5 |
|  | 16 QAM | ≤2 | ≤ 2 | ≤4.5 | ≤4.5 | 2.5 |
|  | 64 QAM | ≤2.5 | ≤ 2.5 | ≤4.5 | ≤4.5 | 2 |
|  | 256 QAM | ≤4.5 | ≤4.5 | ≤4.5 | ≤4.5 | 0 |
| CP-OFDM | QPSK | ≤3 | ≤ 3 | ≤6.0 | ≤6 | 3 |
|  | 16 QAM | ≤3 | ≤ 3 | ≤6.0 | ≤6 | 3 |
|  | 64 QAM | ≤3.5 | ≤3.5 | ≤6.0 | ≤6 | 2.5 |
|  | 256 QAM | ≤6.5 | ≤6.5 | ≤6.0 | ≤6.5 | 0 |

Proposal 1: Consider reducing the PC2 A-MPR for 20MHz CBW, RB allocations {RBend*12*SCS>=0; LCRB*12*SCS > 9.72MHz} to 4.0dB for DFTs QPSK and 3.0dB for Pi.2 BPSK.

For 25MHz CBW and PC2 RB allocations fulfilling {RBend*12*SCS>=0; 9.72>LCRB ≥max (0, 12*SCS*RBend - 1.08)},

- Only MPR applies for PC3, and some of these allocations can be inner.

- A4 A-MPR applies for both outer and inner for PC2.

- Table 3 shows that A-MPR may exceed 3dB for:

  - DFT-s-OFDM Pi.2 BPSK, QPSK and 16QAM; and

  - CP OFDM QPSK and 16QAM.

Table 3: NS_48 25MHz PC2 A4 A-MPR vs. PC3 MPR comparison, {RBend*12*SCS>=0; 9.72>LCRB ≥max (0, 12*SCS*RBend - 1.08)} inner allocations

| Modulation/Waveform |  | PC3 MPR inner (dB) | PC2 MPR inner (dB) | PC2 A4 A-MPR (dB) | PC2 max(MPR,A-MPR) (dB) | PC2-PC3 A-MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 0 | 0 | ≤4.5 | ≤4.5 | 4.5 |
|  | QPSK | 0 | 0 | ≤4.5 | ≤4.5 | 4.5 |
|  | 16 QAM | ≤1 | ≤1 | ≤4.5 | ≤4.5 | 3.5 |
|  | 64 QAM | ≤2.5 | ≤2.5 | ≤4.5 | ≤4.5 | 2 |
|  | 256 QAM | ≤4.5 | ≤4.5 | ≤4.5 | ≤4.5 | 0 |
| CP-OFDM | QPSK | ≤1.5 | ≤1.5 | ≤6.0 | ≤6 | 4.5 |
|  | 16 QAM | ≤2 | ≤2 | ≤6.0 | ≤6 | 4 |
|  | 64 QAM | ≤3.5 | ≤3.5 | ≤6.0 | ≤6 | 2.5 |
|  | 256 QAM | ≤6.5 | ≤6.5 | ≤6.0 | ≤6.5 | 0 |

Observation 1: For 25MHz CBW and PC2 RB allocations fulfilling {RBend*12*SCS>=0; 9.72>LCRB ≥max (0, 12*SCS*RBend - 1.08)}, the PC2 back-off allowance may be more than 3dB greater than the PC3 allowance for DFT-s-OFDM Pi.2 BPSK, QPSK and 16QAM and CP OFDM QPSK and 16QAM.

Proposal 2: Proponents to check if the A4 A-MPR can be revisited to ensure the PC2 back-off allowance does not exceed the PC3 allowance by more than 3dB for

- 25MHz RB allocations fulfilling {RBend*12*SCS>=0; 9.72>LCRB ≥max (0, 12*SCS*RBend - 1.08)}

- 30MHz RB allocations fulfilling {RBend*12*SCS>=0; 12.96>LCRB ≥max (0, 12*SCS*RBend - 3.6)}

  - Case 2: Up to 6dB A-MPR for 40, 45, 50MHz CBW

These cases are analyzed for each modulation and waveform in Table 3.

Table 3: NS_48 PC3 A2 A-MPR vs PC2 A1 A-MPR comparison for 40MHz, 45MHz, 50MHz CBW.

| Modulation/Waveform |  | PC3 A2 A-MPR (dB) | PC2 A1 A-MPR (dB) | PC2-PC3 A-MPR (dB) |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤6 | ≤[12] | 6 |  |
|  | QPSK | ≤6 | ≤[12] | 6 |  |
|  | 16 QAM | ≤6 | ≤[12] | 6 |  |
|  | 64 QAM | ≤6 | ≤[12] | 6 |  |
|  | 256 QAM | ≤6 | ≤[12] | 6 |  |
| CP-OFDM | QPSK | ≤7 | ≤[12.5] | 5.5 |  |
|  | 16 QAM | ≤7 | ≤[12.5] | 5.5 |  |
|  | 64 QAM | ≤7 | ≤[12.5] | 5.5 |  |
|  | 256 QAM | ≤7 | ≤[12.5] | 5.5 |  |

To our understanding, the 6dB greater back-off allowance for PC2 A1 vs PC3 A2 was proposed to cover the cases where the C-IM3 affects the band n34. We used a mid-band power amplifier (PA) to verify this PC3 / PC2 difference. The PA linearity was calibrated according to RAN4 conventions, i.e. using a DFT-s QPSK 100RB0 SCS15kHz waveform to meet -30dBc/-31dBc ACLR at MPR1 for PC3 and PC2 respectively. The measured difference of output back-off “OBO” is 2.5dB for 40MHz, 2.4dB for 45MHz and 2.2dB for 50MHz CBW at SCS 15kHz. We therefore propose to correct the PC2 allowance for these allocations to PC3 A2 + 2.5 for PC2 1Tx and to PC3 A2 +2.5 +0.5 for PC2 2Tx.

Proposal 3: For the PC2 RB allocations fulfilling

- 40MHz CBW {2.88>RBend*12*SCS>=0; >LCRB ≥0}

- 45MHz CBW {5.22>RBend*12*SCS>=0; >LCRB ≥0}

- 50MHz CBW {7.2>RBend*12*SCS>=0; >LCRB ≥0}

consider changing the PC2 A-MPR to PC3 A2 + 3dB.

  - Case 3: PC3/PC2 A-MPR region contours.

Different region contours for PC2 and PC3 may lead to cases where A-MPR exceeds 3dB.

Taking the sub-example of Table 1 for 45MHz CBW SCS 15kHz, the RB allocation LCRB=80 (RBend=200) experiences a A-MPR >3 since:

- For PC3, this allocation is an inner allocation, i.e. for QPSK 0dB MPR is allowed.

- For PC2, this allocation fulfills the A2 condition, i.e. for QPSK DFTs up to 8dB A-MPR is allowed.

Table 5: Example of region contour mismatch between PC3 and PC2 for NS_48 45MHz CBW.

| CBW | Carrier Center Frequency (MHz) | PC3 A-MPR from Table 6.2.3.26-1 |  |  |  | PC2 A-MPR from Table 6.2.3.26-3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Regions |  | A-MPR Region/ DFTs QPSK A-MPR |  | Regions |  | A-MPR Region/ DFT QPSK A-MPR |  |
|  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |  |
| 45 MHz | 1942.5 ≤ Fc ≤ 1957.5 | ≥19, <37.44 | ≥15.4 | A2 | ≤6 | ≥19, <37.44 | ≥14.04 | A2 | ≤8.0 |

It is proposed to align the region contours of the PC3 A-MPR table with those agreed for PC2 for the yellow highlighted cases of Table 1 and 30, 40, 45, 50MHz CBW.

Proposal 4: Consider porting the PC2 region contours to the PC3 A-MPR table for cases where the PC2 total back-off allowance (i.e. max(MPR, A-MPR) exceeds the PC3 total back-off allowance by more than 3dB (refer to Table 1).

Proposal 5: For all other cases where the specified PC2 1Tx A-MPR < PC3 A-MPR + 3dB, consider adding a 0.5dB A-MPR relaxation for PC2 to account for 2Tx reverse IMD.

Proposal 6: Consider adopting the following guideline for future HPUE A-MPR evaluations:

The maximum allowed back-off for PC2 shall not exceed the maximum allowed back-off for PC3 by more than 3dB, where the maximum allowed back-off is defined as max(MPR, A-MPR).

- Conclusion

This contribution summarizes our views on removing Note 7 from Table 6.2.3.1-1 and focuses on the difference of total allowed back-off between PC3 and PC2 for NS_48.

Proposal 1: Consider reducing the PC2 A-MPR for 20MHz CBW, RB allocations {RBend*12*SCS>=0; LCRB*12*SCS > 9.72MHz} to 4.0dB for DFTs QPSK and 3.0dB for Pi.2 BPSK.

Proposal 2: Proponents to check if the A4 A-MPR can be revisited to ensure the PC2 back-off allowance does not exceed the PC3 allowance by more than 3dB for

- 25MHz RB allocations fulfilling {RBend*12*SCS>=0; 9.72>LCRB ≥max (0, 12*SCS*RBend - 1.08)}

- 30MHz RB allocations fulfilling {RBend*12*SCS>=0; 12.96>LCRB ≥max (0, 12*SCS*RBend - 3.6)}

Proposal 3: For the PC2 RB allocations fulfilling

- 40MHz CBW {2.88>RBend*12*SCS>=0; >LCRB ≥0}

- 45MHz CBW {5.22>RBend*12*SCS>=0; >LCRB ≥0}

- 50MHz CBW {7.2>RBend*12*SCS>=0; >LCRB ≥0}

consider changing the PC2 A-MPR to PC3 A2 + 3dB.

Proposal 4: Consider porting the PC2 region contours to the PC3 A-MPR table for cases where the PC2 total back-off allowance (i.e. max(MPR, A-MPR) exceeds the PC3 total back-off allowance by more than 3dB (refer to Table 1).

Proposal 5: For all other cases where the specified PC2 1Tx A-MPR < PC3 A-MPR + 3dB, consider adding a 0.5dB A-MPR relaxation for PC2 to account for 2Tx reverse IMD.

Proposal 6: Consider adopting the following guideline for future HPUE A-MPR evaluations:

The maximum allowed back-off for PC2 shall not exceed the maximum allowed back-off for PC3 by more than 3dB, where the maximum allowed back-off is defined as max(MPR, A-MPR).

# References

- R4-2501984 NS_05 NS_48 NS_49 PC2 A-MPR clean-up, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc.