3GPP TSG-RAN WG4 Meeting #116bis                                           R4-2513020

# Prague, Czech Republic, October 13-17, 2025

Title:	Qualcomm views on 6G System parameters

Source:	Qualcomm Incorporated

Agenda item:	8.3

Release:	Rel-20

SI: 	FS_6G_Radio

Document for:	Approval

# 1	Introduction

For 6G, it is essential to revisit and refine RAN RF system parameters to ensure both performance and scalability. LTE and 5G based precedent have served the industry well in the past, but operational experience helps identify direction of evolution.

This contribution presents Qualcomm’s views on uplink waveform enhancements, FFT sizing in context of expanded channel BW choices, and device architecture considerations. We also share our view on channel and sync rasters for 6G.

# 2	Discussion

## 2.1	Enhancements to DFT-s-OFDM UL

### 2.1.1	 Background

DFT-s-OFDM has been effectively used in LTE and 5G uplink due to its low PAPR characteristics, enabling lower MPR compared to CP-OFDM for equivalent modulation. However, uplink coverage remains a persistent challenge and is expected to continue into 6G. While DFT-s-OFDM helps address this, its applicability is limited by constraints such as rank and allocation size.

To broaden its use in 6G, several enhancements may be explored:

- Spectral Efficiency Adaptation: Introduce smoother transitions between pi/2-BPSK and QPSK (e.g., truncated pi/2-BPSK) to better support coverage-limited scenarios

- Relaxed Allocation Constraints: Remove the legacy restriction that RB allocation sizes must be composed of prime factors 2, 3, and 5. If this can be done without increasing complexity at the transmitter or receiver, it would significantly improve scheduling flexibility.

- Support for High-Rank Uplink: Enable DFT-s-OFDM for multi-rank uplink transmissions, allowing its use in more demanding conditions.

While these enhancements require system-level design from RAN1, the underlying motivation is rooted in RF performance—making them highly relevant for RAN4. A joint RAN1–RAN4 effort would ensure that waveform evolution is both technically sound and operationally viable.

### 2.1.1	 DFT-S-OFDM with Pi/2 BPSK and Truncated Frequency Domain Mapping

For 6G uplink, waveforms offering higher spectral efficiency than legacy π/2-BPSK—particularly in low-SNR, cell-edge scenarios—are of interest. A promising approach builds on DFT-s-OFDM with π/2-BPSK, introducing a tunable trade-off between PAPR and spectral efficiency.

In traditional DFT-s-OFDM from LTE and 5G, the DFT output is mapped one-to-one to frequency-domain subcarriers. The proposed enhancement introduces variable truncation in this mapping, leveraging the conjugate symmetry observed in the DFT output of pi/2-BPSK waveforms around L/4 and 3L/4 (where L is the DFT length) [5].

Conjugate symmetry is illustrated in the figure 1.

Figure 1 - Frequency-domain symmetry of DFT-S-OFDM with Pi/2 BPSK [5]

The below figure illustrates the truncation scheme:

Figure 2 – pi/2 BPSK truncation scheme

In figure 2, the L-point DFT output [S0 S1 ………. SL-1] is truncated by discarding the edge-most Q/2 elements (both upper and lower ends). The truncated output [SQ/2 …….. SL-Q/2-1] is then weighted by a suitable set of frequency-domain filter coefficients (‘FDSS’ in the above figure) and then converted to a time domain waveform using an IFFT.

Figure 3 – PAPR of DFT-S-OFDM Pi/2 BPSK with truncated mapping

In the example above, the number of DFT output symbols that are not mapped to subcarriers is Q, the number of transmitted elements is L-Q, the truncation factor p is p=Q/L and the spectral efficiency in terms of coded bits per resource element is L/(L-Q) =1/(1-p).

The spectral efficiency improves with truncation rate by virtue of packing the ‘L’ DFT output symbols into ‘L-Q’ sub carriers, but this improvement comes at the expense of an increase in PAPR (figure 3) which has MPR ramifications. Spectral efficiency (SE) is influenced by coding design, which is typically within RAN1’s scope. In contrast, RF considerations such as MPR and PA power capabilities are best addressed by RAN4. Since waveform enhancements inherently involve a trade-off between SE and RF performance, a coordinated approach between RAN1 and RAN4 would be beneficial to ensure balanced and optimized design outcomes.

Proposal 2.1-1: RAN4 to consider optimal method for close collaboration with RAN1 to determine maximum specification compliant UL power for various waveform proposals.

Initial analysis was conducted using a representative power amplifier (PA), temporarily disregarding PA drive level constraints to explore the potential for redefining PA output levels across modulation types in 6G. In the study, BPSK waveforms were filtered using a time-domain impulse response vector of length (L–Q): [0.28, 1, 0.28, 0, 0, 0, …].

Among the variants analyzed, untruncated pi/2-BPSK achieved the highest standards-compliant output power. Using the peak power in the central region of the untruncated pi/2-BPSK waveform as a reference, relative output power for other waveforms is compared in Figure 4.

Figure 4 – maximum achievable output delta(power)

Some observations regarding max. spec. compliant power capability:

- With 20% truncation the inner waveforms have an output power very similar to the untruncated Pi/2 BPSK case (inner region delta(power) is close to 0 dB for the associated plot)

- With 40% truncation the inner waveform output power reduces by ~1.0dB compared to the reference power level. While the maximum output power capability reduces due to increased PAPR, spectral efficiency increases, offsetting the effect of reduced power.

- For QPSK waveforms the inner region output power is ~1.7dB lower compared to the same for untruncated pi/2 BPSK.

Figure 5 – Net spectral efficiency vs truncation

These observations were used to generate a ‘net SE’ plot, shown in figure 5, where elevated UL power capability for a modulation scheme accrues as an SNR benefit for that scheme (i.e. curve moves left for the modulation scheme with higher output power capability).

Observation 2.1-1: Frequency-domain mapping truncation enables Pi/2 BPSK waveforms to achieve spectral efficiencies like those for QPSK while enabling higher output power than QPSK.

To fully understand the benefit of truncation, the trade-off between maximum achievable output power and spectral efficiency needs to be studied further. This conclusion further justifies the proposal for collaboration with RAN1 on new waveforms.

### 2.1.2	DFT-s-OFDM and allocation size

In 5G and LTE, DFT-s-OFDM can only be scheduled for resource block (RB) allocations that conform to sizes of the form 2x⋅3y⋅5z2x⋅3y⋅5z. This constraint is illustrated in Figure 4, where legacy (non-truncated) waveform variants show missing rows for unsupported allocation sizes.

With low truncation rates, the set of valid RB sizes becomes even more restrictive, especially when both the DFT size and RB allocation are required to follow the same prime factor rule. This is evident in the expanded unfilled regions (grey areas) in the triangle plot for 20% truncation, also in figure 4.

To improve scheduling flexibility and make these waveforms more practical, RAN1 may consider:

- Allowing arbitrary RB allocation sizes, similar to CP-OFDM.

- Defining DFT size in terms of tones rather than RBs, i.e., requiring the DFT size to be 2a3b5ctones.

This approach retains the legacy prime factor structure to manage complexity, while enabling any RB allocation size for DFT-s-OFDM. Specifically, for an allocation of MRB, the number of tones becomes Mtones=12⋅MRB​, and the DFT size Ntones is chosen so ​Ntones≥Mtonesand  Ntones=2a3b5c for some integers a, b and c.

This enhancement significantly expands the scheduling options for DFT-s-OFDM.

Figure 1 represents an example procedure to support a 7 RB allocation for DFT-S-OFDM waveform. DFT size ‘L’ of 80 is chosen (note that  80 = 24⋅5, and hence a valid DFT size) and the DFT output is cyclically extended by 2 on either side before being mapped to 84 tones.

Note that for scenarios where a truncated mapping can be used, an L that is greater than 12 ⋅ MRB, can be chosen.

Figure 6 Supporting a 7 RB allocation for DFT-S-OFDM waveform.

Observation 2.1-2: Decoupling the size of frequency-domain allocation from the size of the DFT is the basis for enabling any RB allocation size for DFT-s-OFDM without increasing DFT compute complexity (rather than being limited to {2^x 3^y 5^z} RBs).

This type of enhancement pairs very well with truncation because the scheduler no longer loses more allocation options with low truncation ratios. In graphic terms, the large grey areas in the figure for 20% truncation in figure 4 now become available for scheduling.

### 2.1.3	DFT-s-OFDM UL with rank > 1

An enhancement under discussion in RAN1 is to enable rank 2 UL with DFT-s-OFDM.

5G precedent is that for 2 port UEs, rank 2 power requirements (max. power, MPRs, etc) apply to UL transmissions with the identity precoder, or in other words, when each port (‘PA’) sees just one layer of data. This remains a good compromise between test coverage and test case proliferation and can be retained for 6G.

Now, 5G already explicitly covers both DFT-s-OFDM as well as CP-OFDM (see section 6.2D.2) in its specification of MPR allowance for basic Rel-15 behaviour with rank 2 UL, i.e no significant further work would be needed in the UE RF part of the standard to support rank 2 UL with DFT-s-OFDM.

If 5G practice of specifying MPRs for multi-port UL transmission with identity precoders is indeed retained for 6G, multi-layer DFT-s-OFDM UL represents not much work.

Observation 2.1-3: Power, MPR and A-MPR are already specified in RAN4 for multi-layer DFT-s-OFDM UL in 5G.

If, however, RAN4 specifies coherent UL MIMO-specific full rank precoders for multi-port UL transmission in 6G, more specification work may be required in RAN4. Even so, the work is expected to be minor because the MPR values are expected to be bounded between existing MPR values in the 5G standard. This ‘bounded’ expectation comes from the PAPR itself being bounded between those of DFT-s-OFDM and CP-OFDM, respectively.

Observation 2.1-4: Any RAN1 enhancement to enable DFT-s-OFDM for multi-rank UL will have minimal to low impact on the RAN4 standard.

## 2.2 	Advanced spectral confinement techniques

Efficient utilization of spectrum resources remains a central concern. While the current OFDM-based waveforms have demonstrated robustness and flexibility, their spectral containment is inherently limited by the existing pulse shaping techniques and associated sidelobe behaviour. As a result, guard bands must be provisioned to ensure compliance with emission limits and to protect adjacent carriers, which reduces the effective payload capacity within a given channel bandwidth.

Waveform enhancement techniques using waveform design tools like filtering, windowing, clipping, and shaping methods are well understood and implementable with existing architectures. These techniques can provide tighter spectrum confinement and reduced out-of-band emissions, while maintaining acceptable EVM and PAPR characteristics. From a RAN4 perspective, such enhancements could potentially enable narrower guardband requirements or improved compliance margins against existing spectrum emission masks. The corresponding benefit is more efficient use of the channel bandwidth, for example by allowing additional RBs within a given channel bandwidth, thereby improving spectrum efficiency. Inherent benefits of such advanced techniques also include but are not limited to advanced duplexing and interband cross-carrier scheduling.

Figure 7. Simulation example of advanced spectrum containment technique – improving spectrum efficiency

Figure 7 illustrates a PC2 simulation with a 20 MHz channel bandwidth, including three additional resource blocks (RBs) over 5G arrangement and transmitting 1RB at the channel edge. This example demonstrates how advanced spectrum-tightening techniques that are reasonable complexity, easy to implement and without any significant impact to the overall system performance and with low computational complexity for example, WOLA, can enhance existing waveforms, leading to improved spectrum efficiency while meeting out-of-band emission requirements. The simulation does not account for PA effects. We propose to study the performance trade-offs of such enhancement techniques for tightening spectrum containment, with respect to RAN4 requirements.

Observation 2.2-1: Advanced spectral confinement techniques enable additional RBs within a given channel bandwidth, thereby improving spectrum efficiency. But further investigations are required to assess the RF performance trade-offs with such advanced spectral confinement techniques.

Proposal 2.2-1: Further study in RAN4 the benefits of larger NRB in same channel bandwidth for improving spectrum utilization and investigate RF performance trade-offs of advanced spectral confinement techniques.

## 2.3 	Views on Channel BW and FFT size for 6G

3GPP RAN is discussing how to meet increased data rate requirements and one component of increased data rate is bandwidth of the transmissions [4]. The native maximum bandwidth for each band and frequency range is expected to grow from 5G.

For reaching new higher peak data rates with the new generation, widerband single CC operation is more efficient than aggregating multiple smaller CC due to overhead of processing and managing multiple processes instead of one. The larger bandwidth discussion equally draws its motivation from the available spectrum discussion also and we would expect spectrum holders to bring information to RAN4 on desired schemes such as RAN sharing. New spectrum such as > 6 GHz reaching up to 8.4 GHz is good grounds for introducing new wider maximum channel bandwidth for 6GR.

Currently in in RAN4 specification defines intra-band contiguous CA aggregated channel BW up to 320 MHz for shared access bands and up to 300 MHz for licensed bands. If this would be single FFT, the needed FFT does not match up to any 2^x size as shown in Table 2.3-1. Using 8k FFT size (brown) enables 300 MHz but it also enables 400 MHz for same 60 kHz SCS. Choosing 16k FFT (green) enables 30 kHz and 60 kHz use for 400 MHz.

Table 2.3-1: FFT size for new Channel BW

| FFT size x 1000 |  | SCS [kHz] |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 | 30 | 60 |
| CH BW [MHz] | 100 | 6.7 | 3.4 | 1.7 |
|  | 200 | 13.4 | 6.7 | 3.4 |
|  | 300 | 20.0 | 10.0 | 5.0 |
|  | 400 | 26.7 | 13.4 | 6.7 |

Proposal 2.3-1: RAN4 to study increasing channel BW up to 400 MHz within the new FR1 frequency range up to 8.4 GHz.

## 2.4 	UE Rx/Tx Chain count

### 2.4.1	UE Rx

There will be several use cases which support different number of RX per band. This is nothing different to 5G; RedCap is specified down to 1RX while even 8RX is specified for same bands for Non-RedCap devices. The (minimum) number of RX chains per band for different frequencies is under RAN study. The group should also consider how to make these requirements in generic way i.e. avoiding multiplication of requirements when possible.

Proposal 2.4-1: Study how to make requirements in generic way addressing different number of Rx chains per band.

### 2.4.2	UE Tx

In NR, the standard has ‘suffix-less’ baseline Tx requirements for single port UEs and multi-port UEs prior to configuration with multiple ports. For UEs configured as multi-port devices, D-suffix requirements apply. Historically, RAN4 enabled reuse of existing PAs for higher power classes by port aggregation, for example PC2 from aggregating 2 PC3 capable chains. There are also differing assumptions on antenna isolation depending on UE formfactor. Each of these variants bring potentially unique MPRs, which makes for a confusing amount of detail that ultimately is only minimally useful if at all to the network. Moreover, specification of each MPR table consumes considerable RAN4 resources.

For 6G, a network-friendly and RAN4-efficient approach would adopt a unified MPR specification that depends on TRP, regardless of port count. The burden to meet signal quality and emissions requirements with multiple smaller PAs and design-specific antenna isolation is better handled by UE implementation. In the RAN4 context of conducted domain testing, ‘TRP’ could be modified to ‘cumulative power over all antenna connectors’.

Proposal 2.4-2: For 6G, RAN4 to consider MPR specification that depends only on the cumulative power across all antenna connectors and is agnostic of Tx chain count.

In general, the Tx chain content of a UE can be maintained as a transparent detail, with requirements only specified on a per-port or per-layer operation, rather than one that depends on antenna connector count.

## 2.5 Views on Channel and Sync Rasters

### 2.5.1 Channel Raster for 6G

3GPP systems historically adopted the 100 kHz channel raster originating in 3G. With NR (New Radio), an SCS-based raster was introduced in many bands, offering several advantages over the legacy 100 kHz raster:

- Simplified intra-band carrier aggregation (CA) operation

- A unified framework to support heterogeneous channel bandwidths within a common spectrum allocation

- Fewer synchronization-raster (sync raster) entries

To facilitate LTE–NR spectrum sharing, the 100 kHz raster has been largely retained in FDD bands. Consequently, NR must support both raster types. In addition, issues identified with the 100 kHz raster motivated the enhanced channel raster with 5 kHz granularity; however, this approach is inefficient, increases the number of raster entries, and still imposes constraints on SSB (SS/PBCH block) deployment.
Given the above, the SCS-based raster should serve as the baseline channel raster for bands where it is already defined and for any new 6G bands.

Proposal 2.5-1: Adopt the SCS based raster as the baseline in all bands where it is currently specified and in all future 6G bands.

Whether continued support for the 100 kHz raster is required will depend primarily on 5G–6G spectrum-sharing methods and whether subcarrier-level alignment is necessary. Even in such cases, RAN4 should assess the feasibility of using the SCS-based raster in NR bands that today rely on the 100 kHz raster and identify the mechanisms needed to enable a future migration.

Proposal 2.5-2: RAN4 should study the adoption of the SCS-based raster in NR bands that currently use the 100 kHz raster and a migration path to the SCS based raster if continued 100 kHz support remains necessary.

### 2.5.2 Sync Raster Considerations

NR introduced the sync raster primarily to streamline initial system acquisition. A similar concept is expected for 6G. The number of sync-raster entries depends on the channel-raster design (i.e., channel-placement constraints), the minimum channel bandwidth, and the SSB bandwidth. For bands using the 100 kHz raster, the sync raster contains approximately three times as many entries as bands using the SCS-based raster. Reducing the number of entries is desirable to improve initial acquisition time and UE power consumption.

Proposal 2.5-3: RAN4 should study sync-raster optimizations targeting faster initial acquisition and reduced UE power consumption.

Because sync-raster design is tightly coupled to the SSB structure and the minimum channel bandwidth, a detailed study should proceed once further progress on these topics is available.

# 3 	Conclusions

Section 2.1 – Enhancements to DFT-s-OFDM UL

Proposal:

- 2.1-1: RAN4 to collaborate closely with RAN1 to determine maximum specification-compliant UL power for various waveform proposals.

Observations:

- 2.1-1: Frequency-domain mapping truncation enables Pi/2 BPSK waveforms to achieve spectral efficiencies similar to QPSK while allowing higher output power.

- 2.1-2: Decoupling frequency-domain allocation size from DFT size enables arbitrary RB allocation sizes without increasing DFT complexity.

- 2.1-3: Power, MPR, and A-MPR are already specified in RAN4 for multi-layer DFT-s-OFDM UL in 5G.

- 2.1-4: Enabling multi-rank UL for DFT-s-OFDM will have minimal impact on RAN4 specifications.

Section 2.2 – Advanced Spectral Confinement Techniques

Observation:

- 2.2-1: Advanced spectral confinement techniques can improve spectrum efficiency by enabling additional RBs within the same channel bandwidth. RF trade-offs need further study.

Proposal:

- 2.2-1: RAN4 to study benefits of larger NRB in the same channel bandwidth and investigate RF performance trade-offs of advanced spectral confinement techniques.

Section 2.3 – Channel Bandwidth and FFT Size for 6G

Proposal:

- 2.3-1: RAN4 to study increasing channel bandwidth up to 400 MHz within the new FR1 frequency range (up to 8.4 GHz).

Section 2.4 – UE Rx/Tx Chain Count

Proposals:

- 2.4-1: Study how to define scalable requirements for different numbers of Rx chains per band.

- 2.4-2: For 6G, RAN4 to consider MPR specification based on cumulative power across all antenna connectors, independent of Tx chain count.

Section 2.5 – Channel and Sync Rasters

Proposals:

- 2.5-1: Adopt SCS-based raster as the baseline in all current and future 6G bands.

- 2.5-2: Study migration of NR bands currently using 100 kHz raster to SCS-based raster.

- 2.5-3: Optimize sync raster design to improve initial acquisition time and reduce UE power consumption.

# 4.	References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R1-2506216, “Overview of 6GR air interface”, Qualcomm, 3GPP TSG RAN WG1 #122, Bengaluru, India, Aug 25th – 29th, 2025

[5] 	R1-2506218, Waveforms for 6GR, RAN1#122, 25th-29th August 2025