3GPP TSG-RAN WG4 Meeting #116bis                                         R4-2513021

# Prague, Czech Republic, October 13-17, 2025

Title:	Qualcomm views on 6G General and UE RF

Source:	Qualcomm Incorporated

Agenda item:	8.4

Release:	Rel-20

SI: 	FS_6G_Radio

Document for:	Approval

# Introduction

In this paper we discuss items related to General and UE RF issues as defined in RAN4 chair guidance in [3]. We will discuss non spectrum aspects with LO and IQ Image, Power class evolution, Phase continuity and MPR for TX and RX requirements and MSD framework improvements for receiver side. We will also discuss FR3 architecture, Spectrum aggregation and verification aspects related in the core requirements and very important aspect of feature agnosticism of requirements.

# 2	Discussion

## 2.1	 Non-spectrum UE RF

### 2.1.1		UE TX RF

#### 2.1.1.1	LO and IQ image

LO or DC sub-carrier in RAN4 UE RF specifications is treated as an exception from IBE requirements. This is because minimum requirement for UE to operate on the system is agreed to have finite implementation accuracy and LO passes through the modulator. Similarly, IQ image exception implies that the minimum implementation is allowed some imbalance and therefore some energy is leaked to the mirror frequencies of the allocation via the LO frequency. This was same handling for 4G but 5G removed the ban of data on the subcarrier that is exactly on the LO frequency. 5G also allowed free placement of the LO frequency and three different mechanisms were created to report the location with uplinkTxDirectCurrentList, uplinkTxDirectCurrentTwoCarrierList and uplinkTxDirectCurrentMoreCarrierList.

In EVM test for UE, the DC is removed to test the RF hardware linearity but corresponding BS demodulation requirement where BS would be required to read the location of the LO and remove it does not exists. In fact, there is no LO in the signal for the BS demodulation. In field operation, having LO has been shown to cause throughput problem and it can be concluded that the BS receivers are not accommodating this relaxation.

Observation 2.1.1-1: LO location signalling mechanisms are not used in 5G deployments

LO and IQ image are extensively used in AMPR simulations. Some folding products are justified by the presence of these imperfections. In these simulations, the LO is always assumed in the center of the CC or the center most subcarrier as defined in the clause 5.4.2.2. These folding products are used to justify AMPR in many cases and they mask some more complicated phenomena in the implementations and implementing those complicated phenomena in RAN4 work may be too complicated. Current LO and  IQ image set a baseline expectation for the transmitter performance across the industry. For 3GPP RAN4, it is beneficial to use the same approach for AMPR simulations to what was used in 4G and 5G to ensure the similar .

Proposal 2.1.1-1: For the MPR study define LO and IQ image exceptions in the same way as it was used in the 4G and 5G.

This would be a good early agreement since many new waveform and modulation and coding proposals will need to be studied from needed back off, or MPR, point of view where impairments may play a role. RAN4 can further study the impact and then conclude what is the best approach for the normative work.

If some implementation wants to position the LO to a different frequency than the center, then that should be allowed but no signalling nor relaxation mechanism should be defined for that case.

Proposal 2.1.1-2: Do not use any other position than the centre of the channel BW for LO or IQ image exception in 6G MPR study.

#### 2.1.1.2	Power class evolution

##### 2.1.1.2.1	Motivation

In 5G, the UE power class requirements were expanded to cover for CA and for single CC cases and 1Tx, 2Tx cases with and without MIMO and intra CA as well as handheld and FWA aspects. ULFPTx brought in an interesting flavour to UE power class with UL MIMO with rank-1 transmissions for non-coherent multiport UE. This created a lot of work and specification complexity since these features can be used in conjunction with other features especially the SRS antenna switching max power relaxation got fairly complicated requirement. There are also some problems with the power class capability handling, for example RAN4 has a Table 6.2D.1-1 for UL MIMO power class but there is no capability for MIMO power class therefore if UE wants to support UL MIMO then UE has to declare the lower of the power classes in between Table 6.2D.1-1 and Table 6.2.1-1 despite if the UE implements 26 dBm PAs.

Power class is an important requirement, without it, network design would be difficult and having a standardized  minimum performance requirement which defines how much power at least UE has to be able to provide to close uplink gives a good foundation of minimum performance criteria for a UE to be allowed to enter and operate in a network.

Observation 2.1.1-2: Power class is an important requirement for defining minimum allowed performance output power for UE to operate in a healthy network.

##### 2.1.1.2.2	Single CC power class improvements

One fundamental aspect of power class that limits UE performance and system performance in some cases is the upper limit for the configured maximum output power that is defined for each power class. This limit is defined in the Table 6.2.1-1 in the “Tolerance” column :

And also then enforced in the clause 6.2.4 as PCMAX_H:

It is our view that this was inherited from GSM era when there was no power control in the first deployments and the power during the burst had to be controlled for the limited BS receiver linearity and for the UE SAR reason. Since then, the power control has become much better in UE and OFDM receiver is more tolerant to power differences and if UE implements any other radio such as Bluetooth or Wi-Fi, the SAR must be managed otherwise outside 3GPP power control anyway.

Observation 2.1.1-3: Existence of upper limit for each power class set by PCMAX_H seems not to have a justification anymore.

The urge to create new power classes comes from industry to create more reliable networks by having higher output power from the UE. In many cases, networks are limited by UL coverage, either for high density data or mere cell range. Also, DL is limited by UL performance since if ACK/NACKs for DL transmissions do not reach the BS, then network concludes that UE did not receive the data and stops transmissions to it. Removing the upper limit would enable UEs to unleash the potential for more power when it is feasible, for example in cases when RFFE IL is smaller than anticipated by the initial UE RF link budget design. Also, in some implementations, UE may be able to send very short, one or two-symbol ACK/NACK’s with higher power than what is the declared nominal output power for the declared power class.

Observation 2.1.1-4: Removing the upper limit of the power class would help UE and networks to improve coverage and data throughput.

Removing the limit may be problematic in some cases such as places where there is a regulatory limit disallowing high power transmissions from mobile terminals. There are two mitigation methods for this relatively simple problem: 1) Use P-max in SIB to advertise the allowable upper limit or 2) limit the power in UE based on the geographical location. RAN4 can consider adding additional parameter to limit the power depending on the allowed maximum power from the regulation but assigning this parameter needs to come from the regulation. UE has to satisfy the compliance to the regulatory requirements despite what is written in the 3GPP requirements and likely all implementations implement the method 2) already.

In some cases interference increase might also become a problem if some implementation is able to put out much more power than other UEs in same frequency. It is unlikely that an implementation would be able to put out much more (> 3 dB) power than the nominal power for the power class and a mitigation is same as above, p-max either from SIB or UE and cell specific p-max.

Observation 2.1.1-5: Problems related relaxing upper limit of the output power for the power class can be mitigate with p-max

For network, allowing more relaxed upper limit would provide more power from the UE which has a positive impact to the network health but reference to the PHR may become less clear based on specification since the flexibility between PCMAX_H and PCMAX_L becomes larger. 5G developed methods to report the PHR reference better but it is unclear if these are used in real networks is unclear and proposals to make those methods exposes the fact the reference is unclear anyway.

RAN4 should study if the upper limit can be removed or alternatively relaxed from the value of 2 dB and what and what its benefits and drawbacks would be.

Alternative approach could be also investigated such as define the power class for 100 % dutycycle and allow for higher power in the ratio of lower dutycycle. For example, 50 % would mean 3 dB more power is allowed and 5 % would mean 13 dB more power is allowed.

Proposal 2.1.1-3: RAN4 should study if for a power class, the specification could allow UE to deliver for more output power by re-evaluating the requirements imposed by PCMAX_H

MPR impacts only PCMAX_L and therefore there is no change for that.

##### 2.1.1.2.3	CA power class and combined limits

Similarly for CA, the power sharing concept seems to be artificial and possibly unnecessary at least for inter-band uplink CA. This is described in clause 6.2A.4.1.3 where UE is limited for a combined power to CA power class. Further, in clause 7.5 of TS 38.213 a priority rules are defined for overlapping transmissions. In short, it means that the UE caps it power to something and then limits per band power to some combined value. Why UE has to limit power on grant basis is unclear and it would be better for the network and for the individual UE to allow UE to transmit up to it desired output per band.

Observation 2.1.1-6: Power class for CA seem to put unnecessary limit to the UE output power

RAN4 should study how PCMAX_H is defined for CA and if there is a need to CA power class at all. And cover similarly DC and related cases when UE is operating in more than one uplink CC simultaneously.

Proposal 2.1.1-4: RAN4 should study power class definition for CA by re-evaluation the need for CA power class

#### 2.1.1.3	Phase continuity for enhanced Tx

Phase continuity in uplink (UL) transmissions is critical for coverage enhancement, particularly when leveraging PUSCH repetition. In 5G, repetition efficiency can be limited by DMRS overhead, as DMRS is statically configured per UE. For use cases like Fixed Wireless Access (FWA), where channel conditions are relatively stable, enhancements such as applying DMRS across slots may be beneficial. This requires the UE to maintain phase continuity across repeated PUSCH transmissions, often spanning multiple slots.

A related 6G feature under discussion in RAN1 is long-SLIV transmissions (SLIV = start length indication value), where a single PUSCH spans durations longer than one slot. Reliable phase continuity at the UE is essential to support such extended transmissions.

In summary, improving UL performance depends on expanding the operational envelope where phase continuity is expected. This may involve both UE design considerations and network-side constraints, such as minimizing interruptions from downlink scheduling or SRS sounding.

Proposal 2.1.1-5: RAN4 to investigate how to enable phase continuity over long transmissions (across one or more slot boundaries)

#### 2.1.1.4	MPR for 6G

Current MPR is specified for many different cases such as 1Tx, 2Tx and 4Tx , different power classes and also in one case, separately for FWA. Additional flavour are the various boost concepts that were added. The lowest PAPR signal is pi/2 BPSK [4] with FDSS and shaped DMRS. This was the Rel-18 boosted signal and if 5G MPR table would have been constructed now, that would be the obvious choice for the MPR 0 waveform.

Observation 2.1.1-7: 5G MPR 0 waveform is not the one that has the potential for most output power from same hardware considering enhancements up to Rel-18.

It is a bit too early to define such MPR 0 waveform for 6GR and while the choice of MPR 0 waveform may belong to the normative part of the 6G work, for study phase some simulations assumption would need to be put in place for inter-company evaluation results comparison reasons.

Proposal 2.1.1-6: RAN4 to study candidates for MPR 0 and calibration waveforms

### 2.1.2	UE RF RX

#### 2.1.2.1	Evolving RX requirements

Current understanding is that most 5G bands will be used in 6G as well, i.e. band specified as 5G band will also be specified as 6G band sometime during 6G era. Hence the existing RX requirements set forth for 5G bands serve as a reference for 6G RX requirements study for the respective frequencies and channel bandwidths.

Now it is right time to study if all current RX requirements for 5G are still relevant i.e., if they are needed or not for 6G. For instance, RAN4 should study the relevance of Narrow Band Blocking given GSM is not used in all markets and all bands anymore.

REFSENS is naturally in the heart of UE RF RX requirements. Most UE RF RX requirements are tied into it and the REFSENS itself is the baseline for everything.

It would be good to study the 6G REFSENS level, together with UL RB allocations especially for FDD bands using respective 5G REFSENS as reference to figure out if current 5G REFSENS level and UL RB allocations for FDD are appropriate or if adjustments for 6G are justified. We note that the REFSENS consists of “RF part” and “Demod part”, latter meaning assumed SNR in REFSENS. RAN4 needs to agree on all parameter settings which relate to REFSENS and to the other RX requirements including everything in RMC’s and verification, they should not be left to RAN5 as partially happened with 5G. Along with this, the group should review whether the level of current RF RX requirements is appropriate or if adjustments are justified.

It would be good to discuss the number of RX for which the REFSENS is explicitly specified for. In 5G the explicit REFSENS for all bands was specified using 2Rx, supplemented by deltas for 4Rx, 6Rx, and 8Rx. From the beginning of 5G, most UE’s support 4Rx starting from MB’s (1.7-2.3GHz) despite 4Rx is optional for those bands and on higher frequencies even 6Rx is being supported by some devices. The group should discuss what is the best way to specify REFSENS given 6G will recognise, pending RAN outcome, several different device types which will most likely support different number of RX per each band.

Given the maximum channel BW increases from 100 MHz likely up to 400 MHz, the concept of RX requirements for these wider BW’s must be studied.

Concept for RX requirements for new frequencies, especially those above 7.125GHz must be studied as well when permitted by the regulatory status.

We propose to first study these aspects for non-CA, and then look at aggregation once there is more clarity on non-CA.

Proposal 2.1.2-1: At least the following aspects must be studied for 6G RX RF requirements

- Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz BW

- Study what how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities(e.g. different number of Rx chains, etc) potentially addressing different use cases

- Study the 6G REFSENS level and respective UL RB allocations as well as other RX requirements using respective 5G requirements as reference

- RAN4 must agree on all parameter settings which relate to REFSENS and to the other RX requirements including everything in RMC’s and verification, they should not be left to RAN5 as partially happened with 5G

- Study how to define relevant RX requirements for >100MHz BW’s

- Study concept for RX requirements for new frequencies, especially those above 7.125GHz when permitted by regulatory status

Proposal 2.1.2-2: Study single carrier first and study aggregation cases once single carrier is more stable

#### 2.1.2.2	MSD framework improvements

In this section we concentrate on assessing the current MSD framework and on the proposal how to improve it. We cover more band combination specific aspects in our 6G spectrum submission [5].

Assessment on the current MSD framework:

MSD requirements have been specified in 3GPP from LTE Rel-10 (Carrier Aggregation) onwards. The purpose of the MSD has been, as indicated by the acronym, to show Maximum Sensitivity Degradation for a band combination due to a certain interference mechanism (Harmonics, Harmonic Mixing, Cross-band, Inter-modulations)

The MSD requirement is valid for certain test point(s), and no requirements apply for other test points within that band combination. One can easily envision this being a problem for operator whose UL/DL’s are different than those for the test point; even >30dB MSD may be 0dB for most CC allocations within the band; it can also be anywhere between >0dB and <30dB for some CC allocations, even depending on the exact RB allocations within UL and DL CC’s.

The requirements are derived assuming (in almost all cases) 10dB isolation between antennas, to mimic performance in the field. We note that, especially when UE is equipped with >2 RX antennas, the isolation between at least one TX-RX antenna pair often is >10dB, even 20dB or so.

MSD as well as other RX requirements are verified in conducted mode i.e., antennas are not present in conformance testing. Hence the said antenna isolation is represented by the test equipment system isolation which is so high that the impact of coupling between the connectors is negligible. Hence e.g., in case of 2Rx band the DRX does not see any interference except via PCB coupling and Pin-to-Pin coupling even if the PRX would see significant desense. If TX antenna is different than any of the RX antennas, it could be that neither PRX nor DRX see desense even if the calculated desense using finite (10dB) antenna isolation would be significant.

During LTE era, 10 years ago, there was once a serious attempt to address the discrepancy between the current way of specifying REFSENS/MSD based on assumption of finite antenna isolation and conductive testing. Then, there was discussion on whether some couplers between antenna connectors could be used in testing to achieve more realistic situation, but that was deemed infeasible due to issues on test set-up side [6]. All in all, it is not possible to find good agreement between MSD defined in conductive mode vs MSD measured in OTA.

There are several parameters which impact the MSD. Traditionally, 3GPP sets the minimum requirements which in part has led to situations where the parameters impacting the MSD value are chosen in rather conservative way, i.e. in reality they are often better. The resulting MSD may become pessimistic when multiple conservatively chosen parameters are used together (i.e., all parameters are worst case which seldom happens in device).

LowerMSD WI was done allowing to report lower MSD values, but unfortunately the overall benefit due to many uncertainties and the need to maintain large information database in the NW may not be high enough to warrant large scale deployments of that feature.

The worst-case MSD number serves as a good design reference, as well as gives rough information on how challenging band combination could be in worst case.

Observation 2.1.2-1: Current MSD concept can be summarized as follows.

- MSD is valid for certain test point(s) within band combination, giving no or limited indication to other UL/DL frequency combinations

- MSD cannot be correctly verified in conductive environment as the isolation between antenna connectors is >> than the real isolation between antennas

- There is no good agreement between MSD defined in conductive mode vs MSD measured in OTA.

- MSD numbers especially for certain cases are so high due to pessimistic assumptions used to cover worst case, that deployments are not done even the real observed MSD in the field could be significantly lower

- LowerMSD feature introduced recently allows reporting lower MSD for the worst case, but it does not address the three first issues listed above

Considerations on possible MSD framework improvement:

For 6G, the group should concentrate to find a solution for MSD which benefits all stakeholders; Operators who want to deploy spectrum as efficiently as possible, Infra who may want to get accurate instantaneous MSD information from device beyond what it specified as worst case in the specifications, Chipset vendors who want to be competitive in the products they compete in, and Device vendors obviously want to offer devices operating efficiently using maximum portion of the spectrum. We underline that it makes sense to further improve the MSD concept only if improvements are beneficial and such that they would be utilized. Hence getting infra vendors view on what kind of MSD concept would be extremely beneficial.

RAN4 should study how to improve current MSD framework. In our view, the potential improvement should address MSD for any CC frequency combinations, i.e. not only for the worst case for which the MSD is explicitly specified for.

For instance, RAN4 should study if it would be feasible to establish a more instantaneous, measurements-based concept for MSD which would ideally show the desense within the present radio conditions in a precise way, based on actual frequencies and RB allocations. This would give much more accurate information on the desense in present radio conditions than worst case MSD number in specifications. Even with some overhead due to signalling, this information could be beneficial for the NW for its scheduling decisions. Other alternative methods for improved MSD framework should be considered as well, such as static reporting of MSD for the configuration accounting CC frequencies.

There is still good reason to maintain some kind of worst-case MSD requirements in the specification as that serves as good minimum design target as well as gives a high-level indication of how challenging the band combination would be vs other band combinations in worst case.

As a high-level view, one could think that in 6G there could be worst case requirements in the specification but in addition there could be measurements-based instantaneous MSD reporting or other improvements, if found beneficial.

For the 6G SI, we propose to study the following aspects for MSD cases

Proposal 2.1.2-3: Study what kind of MSD concept would benefit all stakeholders, i.e., Operators, Infra Vendors, Chipset Vendors, Device vendors

Proposal 2.1.2-4: To amend the worst-case MSD number(s) in the specification, consider the possibility of using some kind of measurements-based instantaneous MSD reporting or static reporting of MSD for the configuration accounting CC frequencies as candidate solutions

## 2.2	FR3 Architecture

FR3 spans the 7–24 GHz range and sits between FR1 and FR2, each of which employs distinct UE RF front-end solutions. From a performance standpoint, the upper portion of FR3 may benefit from an FR2-like front-end architecture, while the lower portion may be better served by an FR1-like solution due to factors such as antenna size and alignment with existing FR1 bands.

Notably, TR 38.820 from the FR3 Study Item (SI) suggests that FR3 could be divided into three sub-bands, reinforcing the idea that FR3 should not be treated as a single, uniform frequency range.

Spectrum studies from WRC-23 and those planned for WRC-27 have identified the 7–8 GHz and 13–15 GHz ranges as potential candidates for mobile use. These are not the only possibilities—other FR3 bands are also being considered by various regulators, and upper 6 GHz is also a candidate band for 6G deployment. As a result, each newly available FR3 band warrants its own dedicated analysis and discussion, rather than being grouped under a single umbrella

Proposal 2.2-1: Each spectrum band in FR3 warrants its own separate discussion on UE RF front end choice. For example, discussion of < 8.4 GHz is decoupled from discussion on 13-15 GHz band.

For < 8.4 GHz bands, our studies suggest many synergies can be exploited by aligning UE RF front end choice with FR1 practice, but this can be discussed further.

## 2.3.	Spectrum aggregation

RAN#109 agreed that 6G will support Carrier Aggregation. The discussion on whether Dual Connectivity and/or Dual Stack is/are used in addition to CA continues in RAN and is about to begin in WG’s after a few months. RAN4 should defer at least too detailed studies before hearing from RAN/RAN1 outcomes. RAN4 should begin studies from Carrier Aggregation by studying how to specify band combinations in general in the specification.

RAN4 needs to follow duplexing discussion and its outcomes  in RAN1. The aspect of simultaneous/non-simultaneous TX/RX should be included from the very beginning as changing that for a given band combination is extremely difficult in real deployment. The challenge is that if you start deployment with non-simultaneous TX/RX (say TDD-TDD), eventually you will have a big base of devices supporting that but not supporting simultaneous TX/RX for that combination. Then even if you bring in devices supporting simultaneous TX/RX for that combination, you may want to shift into simultaneous TX/RX as then the big legacy device bases could not support that band combination. In hindsight, 5G simultaneous TX/RX was addressed in haste, resulting in partially ambiguous requirements for especially TDD-TDD CA combinations.

Proposal 2.3-1: RAN4 should begin spectrum aggregation studies from Carrier Aggregation, and look into other schemes upon confirmation of support from RAN/RAN1

Proposal 2.3-2: RAN4 should study all applicable duplexing modes, according to the outcomes of upcoming studies in RAN1

Proposal 2.3-3: Support of Simultaneous TX/RX between the bands should be addressed properly from the beginning to avoid ambiguity in specifications

## 2.4	Testing and verification of the requirements

For 5G, FR2 represented a significant requirement and testing challenge, because it deviated from legacy practice of conducted domain testing. During the Rel-15 work phase, RAN5 and RAN4 boundaries were difficult to determine, and as a result, the RAN4 core requirement is scattered with inconsistent test details like test grid specification, non-RAN1/RAN2 defined behavior (aka test modes). Several releases later in Rel-20, there is much better understanding of how testing and requirements can be delineated between the work groups.

In general (not just specific to FR2), test-related functions (‘test modes’) exist to facilitate testing in some way (RAN5 domain). UE core requirements (RAN4 domain) on the other hand are meant to establish design goals for field performance for UEs.

Observation 2.4-1: UE RF core requirements are design goals for the UE and represent a minimum performance expectation that can be used for network design.

Proposal 2.4-1: RAN4 to discuss whether it is justified to specify test modes in the core requirement. A test mode in this context is UE behavior that is not captured in RAN1 or RAN2 design and is therefore not exercised by the network in the field.

Core requirements in their capacity as a design goal are not meaningful to specify only apply over some subset of the expected environmental conditions. Test equipment challenges tend to be temporary, and should be handled in RAN5 anyway.

Proposal 2.4-2: UE RF core requirement apply over the full range of environmental conditions.

Both proposals above seem to make it ‘harder for the UE’, but that is relative to an inconsistent baseline in 5G. Moreover, if a clear principle is established at the outset, company concerns can be addressed during the requirement derivation process.

## 2.5	Feature orthogonality

In this section we discuss about a problem of orthogonality of the requirements between two features. The issue can be described with a question: When requirements for UL MIMO and Carrier Aggregation were defined independently of each other, what requirements apply when UE is configured for UL MIMO and CA simultaneously? For 5G both were defined in Rel-15 and it could have been assumed that for combined operation of these features the possible relaxation or dependencies between operation that might be needed in specifications would have been brought up during Rel-15. The section 4 of 38.101-1 implies this but is not perfectly clear since it states the relations between general requirement and additional requirement.

In the absence of a specific requirement for the combined operation, UE has to meet both requirements and Rel-15 UE can support the combination of CA with UL MIMO. This could be made clear for example by amending the clause 4 by sentence in the lines of:

“In case UE supports features where requirements are under more than one suffix providing additional requirements, UE has to meet general and all the additional requirements simultaneously. In case there are overlapping requirements, UE has to meet the tighter requirements.”

The problem RAN4 will face is that is also well explained by continuing the example case: Despite the UL MIMO and CA requirements both were in Rel-15,  For Rel-17 a WI objective was agreed for “CA with UL MIMO” and this then made the combination of CA and UL MIMO a Rel-17 feature. This is unique characteristics of RAN4. Other WGs typically define requirements for features at a time and intend to solve issues of the combined features at the time of introduction. It maybe so by design since RAN4 deals with the implementation challenges and problems may only come up in RAN4.

Observation 2.5-1: RAN4 is the WG that in many cases concludes requirements for combinations of features.

To avoid ambiguity in 6G era, RAN4 should discuss what combinations of features are discussed and confirmed to be feasible and study how the specifications could be written with better flexibility. This would also help with the discussions on WI prioritization that have reflections of roadmap discussion.

Proposal 2.5-1: RAN4 should study how to define requirements in such manner that dependencies are minimised or at least are unambiguously understood and specified when multiple features are simultaneously configured during the introduction of new features

# 3. 	Conclusion

For Non-spectrum UE TX RF LO and IQ image we observed

Observation 2.1.1-1: LO location signalling mechanisms are not used in 5G deployments

And proposed:

- Proposal 2.1.1-1: For the MPR study purposes, define LO and IQ image exceptions in the same way as it was used in 4G and 5G.

- Proposal 2.1.1-2: Do not use any other position than the center of the channel BW for LO or IQ image exception in 6G MPR study.

For Power class evolution we observed that:

- Observation 2.1.1-2: Power class is an important requirement for defining minimum allowed performance output power for UE to operate in a healthy network.

- Observation 2.1.1-3: Existence of upper limit for each power class set by PCMAX_H seems not to have a justification anymore.

- Observation 2.1.1-4: Removing the upper limit of the power class would help UE and networks to improve coverage and data throughput.

- Observation 2.1.1-5: Problems related to relaxing the upper limit of the output power for the power class can be mitigated with p-max.

- Observation 2.1.1-6: Power class for CA seems to put unnecessary limit to the UE output power

And proposed:

- Proposal 2.1.1-3: RAN4 should study if for a power class, the specification could allow UE to deliver more output power by re-evaluating the requirements imposed by PCMAX_H.

- Proposal 2.1.1-4: RAN4 should study power class definition for CA by re-evaluating the need for CA power class.

For phase continuity over slots we proposed:

- Proposal 2.1.1-5: RAN4 to investigate how to enable phase continuity over long transmissions (across one or more slot boundaries).

And for MPR discussion we proposed

- Proposal 2.1.1-6: RAN4 to study candidates for MPR 0 and calibration waveforms.

For UE RX general requirements we proposed that

- Proposal 2.1.2-1: At least the following aspects must be studied for 6G RX RF requirements 7:

  - Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz BW 8.

  - Study how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities.

  - Study the 6G REFSENS level and respective UL RB allocations as well as other RX requirements using respective 5G requirements as reference.

  - RAN4 must agree on all parameter settings which relate to REFSENS and to the other RX requirements including everything in RMC’s and verification.

  - Study how to define relevant RX requirements for >100MHz BW’s.

  - Study concept for RX requirements for new frequencies, especially those above 7.125GHz when permitted by regulatory status 13.

- Proposal 2.1.2-2: Study single carrier first and study aggregation cases once single carrier is more stable 14.

For UE RF RX MSD requirements we observed that

- Observation 2.1.2-1: Current MSD concept can be summarized as follows:

  - MSD is valid for certain test point(s) within band combination, giving no or limited indication to other UL/DL frequency combinations.

  - MSD cannot be correctly verified in a conductive environment as the isolation between antenna connectors is much greater than the real isolation between antennas 9.

  - There is no good agreement between MSD defined in conductive mode vs MSD measured in OTA.

  - MSD numbers, especially for certain cases, are so high due to pessimistic assumptions used to cover the worst case, that deployments are not done even though the real observed MSD in the field could be significantly lower.

  - LowerMSD feature introduced recently allows reporting lower MSD for the worst case, but it does not address the three first issues listed above

And proposed:

- Proposal 2.1.2-3: Study what kind of MSD concept would benefit all stakeholders, i.e., Operators, Infra Vendors, Chipset Vendors, Device vendors.

- Proposal 2.1.2-4: To amend the worst-case MSD number(s) in the specification, consider the possibility of using some kind of measurements-based instantaneous MSD reporting or static reporting of MSD for the configuration accounting CC frequencies as candidate solutions

For FR3 Architecture discussion we proposed

- Proposal 2.2-1: Each spectrum band in FR3 warrants its own separate discussion on UE RF front end choice.

For Spectrum Aggregation discussion we proposed;

- Proposal 2.3-1: RAN4 should begin spectrum aggregation studies from Carrier Aggregation, and look into other schemes upon confirmation of support from RAN/RAN1.

- Proposal 2.3-2: RAN4 should study all applicable duplexing modes, according to the outcomes of upcoming studies in RAN1.

- Proposal 2.3-3: Support of Simultaneous TX/RX between the bands should be addressed properly from the beginning to avoid ambiguity in specifications.

For Testing and Verification of the Requirements we proposed

- Proposal 2.4-1: RAN4 to discuss whether it is justified to specify test modes in the core requirement.

- Proposal 2.4-2: UE RF core requirement apply over the full range of environmental conditions.

And the important topic of Feature Orthogonality discussion we proposed

- Proposal 2.5-1: RAN4 should study how to define requirements in such manner that dependencies are minimized or at least are unambiguously understood and specified when multiple features are simultaneously configured during the introduction of new features 23.

# References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R4-2513020, “Qualcomm Views on System parameters”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025

[5]	R4-2513022, “Qualcomm views on 6G spectrum”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025

[6]	R4-157052, “On Introduction of Antenna Isolation for RF Tests”, Rohde & Schwarz, RAN4-77