3GPP TSG-RAN WG4 #116bis	R4-2513041

Prague, Czech Republic, October 13-17, 2025

Agenda item:	8.8		Demodulation

Source:	Qualcomm Incorporated

Title:	On demodulation aspects of 6GR

Document for:	Discussion

# Introduction

The 6G study item was approved in RAN #108 [1]. It also includes a study on the 6GR core and performance requirements. Topic (5) e) covers demodulation and performance aspects and topic (5) f) handles aspects related to testability to be driven by RAN4.

- 6GR core and performance requirements

- General RF aspects [RAN4]

    - Intra-3GPP co-existence studies

    - Study RF related system parameters and requirements

- BS RF requirement aspects including band [RAN4]

    - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

    - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

- UE RF requirement aspects including band and band combination [RAN4]

    - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR

    - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination

    - Study UE RF capabilities considering different device types and implementations

- RRM aspects for 6GR [RAN4, RAN1, RAN2]

    - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

    - Study how to improve 6G requirement specification, including structure and drafting principles

- Demodulation and performance aspects [RAN4]

    - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

    - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

- Aspects related to the testability [RAN4]

    - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Other aspects

    - Handling irregular channel bandwidths including the definition [RAN4, RAN1]

    - Definition of ‘frequency range(s)’[RAN4]

In addition, a study on the 6G scenarios and requirements is conducted in RAN plenary [2]. Its outcome is summarized in the technical report 38.914 [3].

In this contribution, we discuss our views of the goals of the RAN4-driven radio resource management aspects of the 6G study item including enhancements for the requirement framework and the applicability rules.

# Views for the 6G Study Item for Demodulation Performance

A new study item for 6G radio has been approved in [1]. The justification for the study emphasizes the practical user experience, systems extensibility and performance excellence of the new radio access technology. In particular, the new 6GR must ensure an acceptable performance-complexity trade-off with enhanced spectral efficiency and improved spectrum utilization with the goal to deliver superior performance with respect to 5G NR. Regarding the 6G core and performance requirements, the 6G study item states that the framework and the key assumptions for defining demodulation and performance requirement should be re-assessed aiming at improvements and/or simplification compared to 5G NR for UE and BS [1].

In our view this means that RAN4 should focus on enhancing the test framework to ensure that functionality and performance is tested under conditions that reflect field-relevant scenarios as close as possible. For example, existing demodulation tests verify performance at BLER = 30%. This allows verifying the functionality of the demodulation chain but is not a typical BLER operating point that is seen in the field. Hence, existing demodulation tests provide limited insight into the performance in field-relevant scenarios.

Observation 1: Existing RAN4 tests do not always ensure testing under field-relevant conditions.

Proposal 1: RAN4 should focus on enhancing the test framework to ensure that functionality and performance is tested under conditions that reflect field-relevant scenarios.

RAN4 should also ensure that the devices are tested under modes of operation that are encountered in field deployments. Good test coverage for a wide range of scenarios and SNR regions observed in the field should be provided. On the other side, the test framework should ensure that functional testing and debugging in case of failures is supported. That means that the tests should not be overly complex and should support the identification of root causes of test failures. To ensure that the 6GR design supports enhancing spectral efficiency and improving spectrum utilization, RAN4 should also ensure that the test framework allows verifying these enhancements in 6GR.

Proposal 2: RAN4 should ensure that the devices are tested under modes of operation that are encountered in field deployments to provide test coverage for a wide range of scenarios.

Proposal 3: RAN4 should ensure that the test framework allows verifying enhanced spectral efficiency and spectrum utilization in 6GR.

Proposal 4: The test framework should support the identification of root causes of test failures.

The study item description states that 6GR and RAN shall ensure that the 6G system can also operate without AI/ML [1]. Hence, non-AI/ML processing should be the baseline for defining demodulation performance requirements.

Proposal 5: RAN4 should assume non-AI/ML processing as baseline for defining demodulation performance requirements.

In the subsequent sections we outline our view for the enhancements of the test framework in RAN4 for demodulation and channel state feedback performance in 6GR.

## Enhancements of Test Framework

In the following subsections, we share our view on the existing test framework for demodulation and channel state feedback and how it should be enhanced in our opinion. The goal is to ensure that functionality and performance is tested under conditions that reflect field-relevant scenarios and to test functionality of state-of-the-art implementations in the devices. In our opinion it is important that RAN4 enhances the baseline assumptions of 5G NR and defines a new baseline for 6GR that takes the implementation and deployment enhancements during the lifetime of 5G NR into account. In our view the following topics should be addressed in the test framework

- Channel state feedback – Existing PMI fail to show the usefulness of advanced precoding codebooks and existing RI tests are overly relaxed in many cases. The existing framework of separate CQI/PMI/RI testing should be enhanced keeping in mind that any new framework should also support simple debugging of functionality in terms of errors as the current framework does.

- Link adaptation – Existing FMCS tests do not reflect the most common operation regime of BLER = 10%. Enhancing the framework towards link adaptation tests may close this gap.

- Extended test equipment (TE) support – an increasing number of 5G NR features in RAN1 cannot be tested in RAN4 at UE side in terms of functionality and performance since the TE does not support the corresponding Tx signal processing. Extending the framework to support simple Tx signal processing in the TE may help closing this coverage gap.

### Channel State Feedback

In the following subsections we share observations and proposals for PMI and Rank Indicator (RI) tests.

#### PMI requirements

RAN4 measures precoding gain of a codebook as the throughput ratio between following UE reported PMI and a random precoder. While specifying requirements for a codebook, it shall be purposeful if RAN4 chooses settings that demonstrate relative gains with advanced codebooks (vs Type-I codebook). For instance, Rel-18 Doppler codebook is supposed to perform better than Rel-16 eType-II codebook around 30-50 kmph speeds. However, Rel-18 Doppler codebook requirements are so relaxed that Rel-16 codebook too can meet them. With the current requirements, the true utility of Rel-18 Doppler codebook to gain higher throughput with reduced UL PMI overhead in field scenarios is not demonstrated.

Observation 2: Rel-18 Doppler codebook RAN4 requirement does not demonstrate superior performance with respect to Rel-16 eType-II codebook.

Proposal 6: RAN4 shall choose scenarios that demonstrate genuine performance gains realizable in the field.

#### Rank Indicator (RI) requirements

The objective of Rank Indicator (RI) tests is to verify that the reported rank indicator accurately represents the channel rank. The RI requirement metric is a ratio of throughputs between “follow RI” and fixed rank. This metric intends to measure the accuracy of reported rank based on the relative increase in throughput. Although the metric is apparently reasonable, there are few concerns, e.g., in Test 4 of 6.4.3 .1 in 38.101-4:

- For a 4Rx device, Rank 4 reporting is not verified.

- The threshold (0.9) does not expect throughput gains by following reported rank

In Test 4 of section 6.4.3.1 in [4], the RI metric threshold is so low that a UE which always reports fixed rank 2 in a follow RI mode too can meet the threshold. This means that a UE needs not report Rank4 or Rank3 and can still pass the test.

Observation 3: No RI requirement that explicitly requires reported rank 4 for a 4Rx device.

Observation 4: Consistency of reported rank is not verified in current RAN4 tests

In our view, due to the above reasons the current RAN4 approach is insufficient to serve twin purposes of reported rank verification, and its accuracy. The approach using a fixed rank might be reasonable for 2Rx devices but is not complete for higher (e.g. 4Rx) Rx devices. To achieve the stated objectives for RI requirements, a new approach is needed.

Extending approach used for CQI requirements, the ratio of throughput between “follow RI” and “median reported RI” is a simple metric for a given SNR. This approach does not fix any reference rank and can truly measure accuracy and consistency of reported rank. Moreover, it allows us to independently specify requirements for each rank indicator. With suitable SNR points for each reported rank such as 2,3, and 4, both accuracy and consistency can be verified.

To illustrate, an SNR point can be chosen where the median reported rank is 4, and a threshold for the ratio of throughput between follow RI and median rank is chosen. The expectation is at that SNR point, a Rank 4 shall be reported consistently ~ 90% of the time, and the throughput achievable by follow RI is at least slightly higher than that by only reporting Rank 4. This approach in-a-way is a link adaptation test where the reported rank, and CQI are used for DL transmissions with no HARQ retransmissions.

Proposal 7: RAN4 to investigate alternative metric that measures consistency and accuracy of Rank Indicator (RI).

Proposal 8: RAN4 shall discuss throughput ratio between “follow rank” and “median rank” as a metric for RI requirements.

### Link Adaptation Testing

In the current RAN4 requirements, demodulation requirements are specified with fixed MCS, and typically at 70% of maximum throughput. This implies the first transmission (1st Tx) BLER is ~ 30%. However, 30% 1st Tx BLER is different from ~ 10% seen in most field deployments.

Observation 5: Typical 1st transmission BLER of 10% seen in field deployments is not used in RAN4 specifications.

Although there can be views where in 10% BLER is not a good measure of HARQ combining during MAC retransmissions, and the difficulty in alignment due to flattish throughput curves in this region, we still hold the view to specify requirements for realistic settings that are close to field use cases.

Proposal 9: RAN4 should study to choose test settings that are close to field use cases.

All CSI reporting requirements tests are based on link adaptation wherein one or more among the parameters CQI, PMI and RI are applied by gNB as “follow parameter”. The objective was to specify requirements for a given parameter based on relative throughput gains. In section 5.6 of [4], link adaptation approach was used where in MCS follows reported CQI to measure throughput for demod performance. All CSF and current demodulation requirements intend to specify minimum performance requirements of a single (isolated) parameter, but don’t measure spectral efficiency – the most critical measure of a link. Moreover, it is also found in studies that the current R-ML demodulation requirements based on fixed MCS (FMCS) are sometimes too relaxed.

To measure spectral efficiency, our considered view is to have new tests where in gNB has the flexibility to change the MCS to achieve the highest throughput. At a given set SNR where a consistent reported rank is ensured, gNB can change the scheduled PDSCH MCS to measure the maximum throughput achievable. Set outer loop first transmission BLER target such as 10%, often seen in the field, can be used for such requirements.  Moreover, for each reported rank, suitable SNR points can be chosen, and throughput requirements can be specified based on outer-loop link adaptation (OLLA). We believe this new approach comprehensively captures the capability of a receiver along with other ILLA based requirements.

Observation 6: Current link adaptation-based RAN4 requirements don’t measure the highest achievable throughput/spectral efficiency of the link.

Proposal 10: RAN4 to investigate Outer-Loop (gNB controlled) Link Adaptation (OLLA) and/or Inner-Loop (ILLA) methods to specify requirements for throughput.

### Testing with Extended TE Support

In RAN4, performance requirements are typically not introduced for features that rely on network-side signal processing, such as transmission-side beamforming due to the variability in gNB implementations and deployment-specific configurations. While this approach ensures fairness in UE testing and avoids standardizing efforts that are vendor-dependent, it also presents notable drawbacks. The absence of performance benchmarks for such features can hinder efforts to quantify the benefits of advanced network-side signal processing techniques. Furthermore, the lack of standardized requirements may limit the ability to effectively assess UE performance in realistic deployment scenarios.

Observation 7: In RAN4, performance requirements are typically not introduced for features involving network-side signal processing. While this allows flexibility for vendors and avoids standardizing deployment-specific behavior, it limits the ability to effectively assess UE performance in realistic deployment scenarios.

For example, in the case of SRS port grouping, RAN4 defines a fixed mapping of SRS ports to codewords (CW0 and CW1) for UEs with 6Rx or 8Rx configurations. However, the effectiveness of this feature heavily depends on how the gNB performs beamforming across these ports. Since beamforming is a network-side function and varies by deployment, RAN4 does not introduce performance requirements for this feature. While this avoids constraining vendor-specific implementations, the absence of performance requirements makes it difficult to assess UE performance under realistic deployment conditions.

Observation 8: In RAN4, no performance requirements are introduced for SRS port grouping because its effectiveness depends on network-side beamforming. Due to the absence of such requirements, there is no guarantee for UE performance in realistic deployment scenarios.

Similarly, in the case of coherent joint transmission (CJT) calibration reporting using delay and frequency offsets, the UE reports offset values to assist the network in pre-compensating for timing and frequency misalignments across TRPs. However, the actual performance of this feature depends heavily on how the gNB applies these offsets during beamforming and scheduling, which varies across network implementations and deployment scenarios. As there is no test framework to evaluate it, RAN4 refrains from introducing performance requirements for this feature.

Observation 9: RAN4 did not introduce performance requirements for CJT delay and frequency offsets reporting, as its effectiveness depends on how the gNB applies these offsets during beamforming and scheduling, both of which vary across network implementations and deployment scenarios. Due to variability in network implementations and the absence of a test framework, RAN4 does not define performance requirements for this feature.

Another example is the port-selection codebook, which enables the UE to report preferred CSI-RS ports to support downlink beamforming, allowing the gNB to optimize transmission based on UE feedback. An attempt to define performance requirements was discussed in Rel-17; however, RAN4 ultimately did not specify performance requirements for this feature due to its strong reliance on gNB-side beamforming behavior, which varies across deployments. Since the effectiveness of the port-selection codebook is determined by how the gNB selects and applies CSI-RS ports, which may vary across deployments, UE performance cannot be reliably assessed in realistic deployment scenarios.

Observation 10: RAN4 did not define performance requirements for the port-selection codebook due to its strong dependence on gNB-side beamforming behavior, which varies across implementations and deployment scenarios. This variability makes it impractical to guarantee UE performance in deployment scenarios or functionality of this this feature at the UE side.

While RAN4 avoids introducing performance requirements for features involving network-side signal processing, such as SRS port grouping and CJT delay and frequency offsets reporting and port-selection codebook, it should still consider verifying these features using test equipment algorithms. This approach would enable validation of real-world performance without restricting flexibility in network implementations.

Proposal 11: RAN4 should explore utilizing test equipment algorithms to evaluate features that rely on network-side processing, enabling realistic UE performance assessment without limiting network implementation flexibility.

Proposal 12: RAN4 should identify features dependent on network-side processing and aim to define corresponding performance requirements, where feasible, using test equipment-based evaluation methods.

## Advanced Receivers

One important consideration for enhancing the test framework in RAN4 to conditions that reflect field-relevant scenarios is the receiver type for downlink MIMO. Table 2-1 lists the SU-MIMO receivers that have been defined for 5G NR over various releases.

Table 2-1: MIMO Receiver Types in 5G NR

| Receiver type | Release | Type | Release |
| --- | --- | --- | --- |
| MMSE | Rel-15 | Mandatory | Default receiver in 5G NR. MMSE without interference rejection capability |
| Enhanced receiver type I | Rel-15 | Optional | R-ML receiver with enhanced inter-stream interference suppression for SU-MIMO transmissions |
| MMSE-IRC | Rel-17 | Mandatory in Rel-17 Optional in Rel-15/16 | MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference |
| Simplified 8Rx Receiver | Rel-18 | Optional | 8Rx receiver for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors |
| Baseline 8Rx Receiver | Rel-18 | Optional | 8Rx receiver for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector |

The default receiver in 5G NR is a plain MMSE receiver without inter-cell and intra-cell inter-user interference rejection capabilities. MMSE-IRC receivers have been introduced in Rel-17 and are optional for Rel-15/16 devices. The simplified and baseline 8Rx receivers that have been introduced in Rel-18 are MMSE-IRC type of receivers. Demodulation requirements for interference have been introduced for 8Rx in Rel-19 without introducing a new receiver type. Meanwhile, MMSE-IRC receiver is state of the art for all number of Rx antennas.

Observation 11: The default receiver in 5G NR is MMSE. MMSE-IRC receivers are mandatory since Rel-17.

The enhanced receiver type I is a R-ML (reduced Maximum-Likelihood) receiver with enhanced inter-stream interference suppression for SU-MIMO transmission. In contrast to the MMSE-IRC receiver, a R-ML receiver is a non-linear receiver. This receiver type has been introduced as an optional feature in Rel-15 and provides significant performance gains over MMSE-IRC receiver in case of more severe inter-stream (inter-layer) interference. R-ML receivers are widely available in real world UE devices and should be the baseline receiver for 6GR as well.

Observation 12: Non-linear R-ML receivers are widely available in real world UE devices.

Both MMSE-IRC and R-ML receivers are important for real world scenarios. For scenarios with less inter-layer interference, a R-ML receiver does not provide performance gains over a MMSE-IRC receiver. In such a scenario, applying a R-ML receiver would not be power-efficient. Hence, it is important to define both MMSE-IRC and R-ML receiver as the baseline for 6GR to allow for an optimal performance-power trade-off.

Proposal 13: Both R-ML receiver and MMSE-IRC receiver should be baseline receivers for 6GR, independent on the number of Rx antennas.

## SNR Operating Points

As it was outlined in the introduction of this section, one of the goals of the 6G study item should be to enhance the test framework in such a way that functionality and performance is tested under wide range of conditions that reflect field scenarios. In this context also the SNR operating point should be discussed both for single carrier as well as for multiple component carriers.

In 5G NR, the SNR for conducted testing with external noise source is defined as follows [4]:

SNR=j=1NRxEsjj=1NRxNocj

NRx denotes the number of receiver antenna connectors, and the index j indicated the jth antenna connector. NOC is the power spectrum density and typically -134 dBm/Hz in the conducted tests. This SNR definition assumes that the resource elements are not precoded and does not account for any gain that can be associated to the precoding operation. The SNR refers to the secondary synchronization signal in 5G NR.

Observation 13: The SNR in the 5G NR demodulation requirements is the non-beamformed SNR measured on the secondary synchronization signal of the SSB.

The SNR is the non-beamformed SNR measured on the SSB. The SNR measured on the DMRS of the PDSCH can be significantly higher because of beamforming gains. Since RAN4 makes use of the SNR measured on the SSB, it is important that the demodulation requirements are defined based on the SNR that are observed in the field and can be expected in upcoming 6G deployments. Otherwise, the coverage of the RAN4 demodulation tests is limited.

Proposal 13: To ensure good field coverage, the SNR values in the demodulation requirements should follow the SSB SNR values that can be measured in existing 5G NR scenarios and those SSB SNR values that can be expected in upcoming 6G deployments.

In field measurements, easily SSB SNR values of 35 dB and above can be observed per Rx antenna. However, in the RAN4 demodulation requirements for single component carriers, the SNR is upper bounded by 30 dB. The SNR requirement that is closest to 30 dB is defined for 1024QAM and 2 Rx antennas. For most other demodulation requirements, the SNR is even upper bounded by around 25 dB. We feel that due to that restriction a performance range of around 10 dB is not well tested in RAN4, and UE performance is not guaranteed for SNR above 25 dB. Hence, RAN4 should study whether the coverage range for defining demodulation requirements can be extended to larger values.

Observation 14: In field measurements, easily SSB SNR values of 35 dB and above can be observed per Rx antenna.

Observation 15: For single component carriers, UE demodulation performance is not well tested for a wide range of SSB SNR values above SNR = 25 dB.

Proposal 15: RAN4 should study whether the coverage range for relevant field scenarios can be extended by defining demodulation requirements for larger SNR values as currently being used in 5G NR.

A topic closely related to the SNR is the TxEVM of the basestation and what assumptions for the TxEVM are made to derive the demodulation requirements. TxEVM is typically modelled as additional noise in the simulations to derive the demodulation performance requirements. In linear domain, SNR and TxEVM in the demodulation tests is related by SNR=1TxEVM2, i.e., the TxEVM of the base station bounds the achievable SNR.

Table 2-2 shows the TxEVM requirement for different modulation schemes for the base station type 1-C and type 1-H [5]. For example, for 256QAM, the required TxEVM = 3.5% that roughly corresponds to SNR = 29. Since in field measurements, SNR values far beyond 29 dB can be observed, it is evident that commercial base stations are exceeding this minimal requirement significantly.

TxEVM severely limits the SNR range to define demodulation requirements. If RAN4 wants to extend the coverage range for defining demodulation requirements, it should also be discussed in RAN4 whether the TxEVM, at least in the simulations for deriving the demodulation requirements, should be tightened.

Table 2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

Observation 16: Since in field measurements, SSB SNR values far beyond 29 dB, corresponding to 3.5% TxEVM, can be observed, it is evident that commercial base stations are exceeding this minimal TxEVM requirement significantly.

Proposal 16: RAN4 should study whether the TxEVM requirements for the base station, at least in the simulations for deriving the demodulation requirements, could be tightened.

In case of multiple component carriers, it can be noticed that the coverage hole for SNR values that can be observed in the field, is even larger. Most of the SNR values of the demodulation requirements for carrier aggregation are below SNR = 20 dB since the channel coding rate is set to less than 0.5 and the modulation alphabet is limited to 16QAM and 64QAM. RAN4 should study whether the coverage range for relevant field scenarios can be extended for carrier aggregation. It should also be studied whether the SNR requirement should be dependent on the number of component carriers.

Proposal 17: RAN4 should study whether the coverage range for relevant field scenarios can be extended for carrier aggregation. It should also be studied whether the SNR requirement should be dependent on the number of component carriers.

## Channel Modelling

In the following subsections we provide our view about the channel models to be used in 6GR.

### Channel Modelling for RAN4 Demodulation Requirements in 6GR

In the history of 5G NR, Tapped Delay Line (TDL) models have served as the primary reference for performance evaluation, particularly within RAN4 testing scenarios. These models offer temporal resolution but lack spatial coherence, which limits their ability to capture the full benefits of advanced MIMO schemes. Despite these limitations, TDL remains an extremely valuable benchmark for continuity and comparative analysis, and its inclusion in future evaluations is essential. As we have seen with the recent developments in 5G NR Rel-19 that led to the publication of TR 38.753, Spatial Channel Modeling (SCM) is gathering increasing attention for the purposes of the definition of demodulation performance requirements.

Observation 17: TDL models, while foundational for 5G NR, are spatially agnostic and in some cases have been shown not to sufficiently characterize spatially dependent features for the purpose of testing certain scenarios (e.g. advanced MIMO schemes, multiple codeword scenarios).

To address these limitations, SCM models have gained prominence, and within them the Clustered Delay Line (CDL) model. CDL models introduce spatial determinism and coherence, making them more suitable for evaluating spatial processing performance. Their structured nature allows for an increased level of modeling of certain real-world propagation phenomena (e.g. distribution of SNR per layer, spatial properties influencing precoding choices), which is especially beneficial for demodulation studies that rely on spatial diversity and channel predictability.

Proposal 18: Maintain both TDL and CDL models in future 6GR performance requirements. TDL should continue to serve as a reference and comparison, while CDL provides the spatial features that can be used to test advanced demodulation scenarios.

As AI/ML-based receivers become increasingly relevant in the context of 6G demodulation, the choice of spatial channel models plays a critical role in ensuring realistic and reliable performance evaluations. While CDL (Clustered Delay Line) models offer deterministic spatial behavior and structured propagation paths, this determinism introduces risks when used in conjunction with learning-based receivers.

Observation 18: The deterministic nature of CDL models can lead AI/ML-based receivers to learn the channel behavior during testing, resulting in artificial performance that does not generalize to real-world conditions.

This phenomenon arises because CDL models, based on sum of sinusoids with stable spatial directions, might allow receivers to infer and memorize channel patterns. In a test environment, this can produce misleading results, as the receiver may exploit the predictability of the model rather than demonstrating adaptability. Consequently, CDL may not be suitable for evaluating AI/ML receivers until methods are developed to mitigate this inference risk.

Proposal 19: Consideration of CDL modeling in 6GR for AI/ML receiver evaluations should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

While CDL models offer valuable spatial realism for traditional demodulation studies, their deterministic properties pose significant challenges for AI/ML-based receiver evaluations. TDL models, with their stochastic nature, may remain the safer choice for benchmarking robustness and generalization in AI/ML contexts. A similar concern applies to PMI (Precoding Matrix Indicator) prediction tasks. CDL models maintain stable spatial directions, which can be exploited by learning algorithms to predict PMI values with high accuracy in test scenarios. However, this does not reflect the variability and unpredictability of real-world channels, potentially leading to over-optimistic conclusions about receiver performance.

Observation 19: Stable spatial directions in CDL models can distort PMI prediction results, making them appear more effective than they would be in dynamic environments.

Proposal 20: Consideration of CDL modeling in 6GR for PMI report evaluation should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

Another area of improvement lies in the antenna correlation assumptions used in TDL models. Current implementations often rely on idealized or overly simplified correlation matrices that do not reflect practical deployments. Enhancing TDL with more realistic spatial correlation values would improve its utility in scenarios where legacy compatibility is required.

Proposal 21: Extend the current TDL framework to include more realistic antenna correlation values, enabling more advanced modeling while preserving backward compatibility.

In summary, spatial channel modeling for 6G demodulation should embrace both legacy and forward-looking approaches. TDL remains essential for continuity with 5GNR, while CDL offers spatial behavior that can be exploited in testing next generation receivers. Enhancements to TDL and strategic use of CDL will ensure that demodulation performance is accurately and comprehensively evaluated across diverse deployment scenarios.

### CDL Modelling for 6GR-NTN

RAN4 has recently completed a study on spatial channel modeling, focusing on CDL channel models for TN, with conclusions documented in TR 38.753. Building on this work, we propose initiating a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the predominantly LOS nature of NTN links. Aligned with terrestrial network modeling practices, this extension would improve the realism of NTN channel modeling and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

Observation 20: RAN4 has recently concluded a study on CDL channel modeling for terrestrial networks, documented in TR 38.753.

Proposal 22: We propose initiating a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the LOS-dominant nature of NTN links and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

## Demodulation Tests for the Base Station

For 6G demodulation performance for base stations, one of the limitations of the 5G base station demodulation performance requirements is the limitation by the 8Rx configuration for all demodulation requirements/ tests. Moving to 6G, it is viable also to consider more than 8Rx antennas to address massive MIMO deployments, which is a key pillar for 5G and 6G deployments. As base station demodulation requirements are typically capped at 8Rx, it would be desirable to understand the feasibility from RAN4 point of view to study more than 8Rx demodulation requirements.

Proposal 23: RAN4 to study the feasibility of considering higher than 8Rx scenarios when defining the BS demodulation requirements.

Another important aspect to be considered for base station demodulation performance in 6G, is the consideration of spatial channel modelling for base station demodulation performance. Although the work done in Rel-19 study item on spatial channel modelling focused on downlink and the impact on UE demodulation performance, spatial channel modelling and its impact are expected to be considered as well for uplink and thus, it is expected that base station demodulation performance will be studied and provided adopting CDL channel models.

Observation 21: CDL channel models are expected to be considered in RAN4 for uplink to address base station demodulation performance.

## Device Types

The introduction of device types is an ongoing discussion in the plenary and in RAN1. RAN1 studies the device types form physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN. Hence, RAN4 should wait for further clarification in RAN and RAN1 what device types may get defined. Afterwards RAN4 should discuss how device types can be covered in the test framework of RAN4.

Observation 22: The introduction of device types is an ongoing discussion in the plenary and in RAN1. RAN1 studies the device types form physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

Proposal 24: RAN4 should wait for further clarification in RAN and RAN1 what device types may get defined. Afterwards RAN4 should discuss how device types can be covered in the test framework of RAN4.

# Applicability Rules

We recommend RAN4 to review how applicability rules are currently handled in RAN4 demodulation specifications (i.e., 38.101-4 for UE demodulation and 38.104/38.141-1/38.141-2 for BS demodulation) and to identify opportunities for improvement in 6G. This specification, while comprehensive, presents challenges in clarity and consistency that affect both interpretation and implementation.

One key observation is that the applicability of tests varies significantly across device types. Some devices are assigned dedicated sections, others are embedded within broader clauses, and some rely on tables that differ in structure and completeness. This fragmented approach makes it difficult for stakeholders to quickly determine which tests apply to which configurations, especially as new device categories emerge.

Some applicability rules are structured by antenna configuration, others by optional features, and many are buried in tables with conditional logic. This complexity underscores the need for a harmonized and streamlined approach

Observation 23: The current structure of applicability rules in TS 38.101-4 is fragmented and sometimes inconsistent across device types, making it difficult for stakeholders to interpret test requirements reliably, especially for different types of devices.

There are many examples of the inconsistencies above, but we can limit the list to a glaring example, regarding the applicability of demodulation requirements for 2RX UEs. Indeed, this is presented in a way that reveals inconsistencies that complicate understanding.

In Table 5.1.1.2-1, TS 38.101-4, the specification states that for UEs supporting only 2RX, “all tests in Clause 5.2.2” apply. This suggests a unified and complete applicability of demodulation requirements for 2RX devices.

However, Clause 5.2.2 is subdivided into multiple subclauses each addressing different requirements and configurations. Some of these subclauses include tests that are only relevant when certain features are signaled (e.g., enhanced receiver types, HST-SFN, HST-DPS, RedCap), which are not necessarily supported by all 2RX UEs and are reported via additional applicability tables or conditions.

This fragmented structure means that the reader needs to manually verify each subclause and its conditional logic. So, applicability is not centralized, and the reader needs to navigate multiple layers of specification to determine the correct test scope.

Proposal 25: RAN4 should discuss improving Demodulation spec in 6GR by replacing broad applicability statements with clear, centralized mappings of test coverage. This would enhance consistency and reduce ambiguity across device types and configurations.

With the emergence of new application scenarios, novel device types are being introduced, and RAN4 is actively defining their corresponding performance requirements. It is important to recognize that while some devices may support TN functionality, others may operate independently without supporting it. Therefore, when determining test applicability across device types, it is essential to consider both the device capabilities and their intended application scenarios. Specifically, for devices that do not support legacy TN, test applicability should be structured to avoid dependence on legacy TN test cases.

Observation 24: RAN4 is defining performance requirements for emerging device types, recognizing that some devices may support TN functionality while others may operate independently without supporting it.

Proposal 26: RAN4 should implement a capability-aware test applicability framework that considers device functionality. For devices lacking legacy TN support, test applicability should be designed to avoid dependency on TN-related procedures and corresponding test cases.

# Conclusions

In this paper we provided our views for the demodulation performance in the 6GR study item. Our observations and proposals are summarized as follows:

Views for the 6G Study Item for Demodulation Performance

Observation 1: Existing RAN4 tests do not always ensure testing under field-relevant conditions.

Proposal 1: RAN4 should focus on enhancing the test framework to ensure that functionality and performance is tested under conditions that reflect field-relevant scenarios.

Proposal 2: RAN4 should ensure that the devices are tested under modes of operation that are encountered in field deployments to provide test coverage for a wide range of scenarios.

Proposal 3: RAN4 should ensure that the test framework allows verifying enhanced spectral efficiency and spectrum utilization in 6GR.

Proposal 4: The test framework should support the identification of root causes of test failures.

Proposal 5: RAN4 should assume non-AI/ML processing as baseline for defining demodulation performance requirements.

Channel State Feedback

Observation 2: Rel-18 Doppler codebook RAN4 requirement does not demonstrate superior performance with respect to Rel-16 eType-II codebook.

Proposal 6: RAN4 shall choose scenarios that demonstrate genuine performance gains realizable in the field.

Observation 3: No RI requirement that explicitly requires reported rank 4 for a 4Rx device.

Observation 4: Consistency of reported rank is not verified in current RAN4 tests.

Proposal 7: RAN4 to investigate alternative metric that measures consistency and accuracy of Rank Indicator (RI).

Proposal 8: RAN4 shall discuss throughput ratio between “follow rank” and “median rank” as a metric for RI requirements.

Link Adaptation Testing

Observation 5: Typical 1st transmission BLER of 10% seen in field deployments is not used in RAN4 specifications.

Proposal 9: RAN4 should study to choose test settings that are close to field use cases.

Observation 6: Current link adaptation-based RAN4 requirements don’t measure the highest achievable throughput/spectral efficiency of the link.

Proposal 10: RAN4 to investigate Outer-Loop (gNB controlled) Link Adaptation (OLLA) and/or Inner-Loop (ILLA) methods to specify requirements for throughput.

Testing with Extended TE Support

Observation 7: In RAN4, performance requirements are typically not introduced for features involving network-side signal processing. While this allows flexibility for vendors and avoids standardizing deployment-specific behavior, it limits the ability to effectively assess UE performance in realistic deployment scenarios.

Observation 8: In RAN4, no performance requirements are introduced for SRS port grouping because its effectiveness depends on network-side beamforming. Due to the absence of such requirements, there is no guarantee for UE performance in realistic deployment scenarios.

Observation 9: RAN4 did not introduce performance requirements for CJT delay and frequency offsets reporting, as its effectiveness depends on how the gNB applies these offsets during beamforming and scheduling, both of which vary across network implementations and deployment scenarios. Due to variability in network implementations and the absence of a test framework, RAN4 does not define performance requirements for this feature.

Observation 10: RAN4 did not define performance requirements for the port-selection codebook due to its strong dependence on gNB-side beamforming behavior, which varies across implementations and deployment scenarios. This variability makes it impractical to guarantee UE performance in deployment scenarios or functionality of this this feature at the UE side.

Proposal 11: RAN4 should explore utilizing test equipment algorithms to evaluate features that rely on network-side processing, enabling realistic UE performance assessment without limiting network implementation flexibility.

Proposal 12: RAN4 should identify features dependent on network-side processing and aim to define corresponding performance requirements, where feasible, using test equipment-based evaluation methods.

Advanced Receivers

Observation 11: The default receiver in 5G NR is MMSE. MMSE-IRC receivers are mandatory since Rel-17.

Observation 12: Non-linear R-ML receivers are widely available in real world UE devices.

Proposal 13: Both R-ML receiver and MMSE-IRC receiver should be baseline receivers for 6GR, independent on the number of Rx antennas.

SNR Operating Points

Observation 13: The SNR in the 5G NR demodulation requirements is the non-beamformed SNR measured on the secondary synchronization signal of the SSB.

Proposal 14: To ensure good field coverage, the SNR values in the demodulation requirements should follow the SSB SNR values that can be measured in existing 5G NR scenarios and those SSB SNR values that can be expected in upcoming 6G deployments.

Observation 14: In field measurements, easily SSB SNR values of 35 dB and above can be observed per Rx antenna.

Observation 15: For single component carriers, UE demodulation performance is not well tested for a wide range of SSB SNR values above SNR = 25 dB.

Proposal 15: RAN4 should study whether the coverage range for relevant field scenarios can be extended by defining demodulation requirements for larger SNR values as currently being used in 5G NR.

Observation 16: Since in field measurements, SSB SNR values far beyond 29 dB, corresponding to 3.5% TxEVM, can be observed, it is evident that commercial base stations are exceeding this minimal TxEVM requirement significantly.

Proposal 16: RAN4 should study whether the TxEVM requirements for the base station, at least in the simulations for deriving the demodulation requirements, could be tightened.

Proposal 17: RAN4 should study whether the coverage range for relevant field scenarios can be extended for carrier aggregation. It should also be studied whether the SNR requirement should be dependent on the number of component carriers.

Channel Modelling

Observation 17: TDL models, while foundational for 5GNR, are spatially agnostic and in some cases have been shown not to sufficiently characterize spatially dependent features for the purpose of testing certain scenarios (e.g. advanced MIMO schemes, multiple codeword scenarios).

Proposal 18: Maintain both TDL and CDL models in future 6GR performance requirements. TDL should continue to serve as a reference and comparison, while CDL provides the spatial features that can be used to test advanced demodulation scenarios.

Observation 18: The deterministic nature of CDL models can lead AI/ML-based receivers to learn the channel behavior during testing, resulting in artificial performance that does not generalize to real-world conditions.

Proposal 19: Consideration of CDL modeling in 6GR for AI/ML receiver evaluations should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

Observation 19: Stable spatial directions in CDL models can distort PMI prediction results, making them appear more effective than they would be in dynamic environments.

Proposal 20: Consideration of CDL modeling in 6GR for PMI report evaluation should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

Proposal 21: Extend the current TDL framework to include more realistic antenna correlation values, enabling more advanced modeling while preserving backward compatibility.

Observation 20: RAN4 has recently concluded a study on CDL channel modeling for terrestrial networks, documented in TR 38.753.

Proposal 22: We propose initiating a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the LOS-dominant nature of NTN links and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

Demodulation Tests for the Base Station

Proposal 23: RAN4 to study the feasibility of considering higher than 8Rx scenarios when defining the BS demodulation requirements.

Observation 21: CDL channel models are expected to be considered in RAN4 for uplink to address base station demodulation performance.

Device Types

Observation 22: The introduction of device types is an ongoing discussion in the plenary and in RAN1. RAN1 studies the device types form physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

Proposal 24: RAN4 should wait for further clarification in RAN and RAN1 what device types may get defined. Afterwards RAN4 should discuss how device types can be covered in the test framework of RAN4.

Applicability Rules

Observation 23: The current structure of applicability rules in TS 38.101-4 is fragmented and sometimes inconsistent across device types, making it difficult for stakeholders to interpret test requirements reliably, especially for different types of devices.

Proposal 25: RAN4 should discuss improving Demodulation spec in 6GR by replacing broad applicability statements with clear, centralized mappings of test coverage. This would enhance consistency and reduce ambiguity across device types and configurations.

Observation 24: RAN4 is defining performance requirements for emerging device types, recognizing that some devices may support TN functionality while others may operate independently without supporting it.

Proposal 26: RAN4 should implement a capability-aware test applicability framework that considers device functionality. For devices lacking legacy TN support, test applicability should be designed to avoid dependency on TN-related procedures and corresponding test cases.

# References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, RAN #108, June 2025

- TR 38.914, Study on 6G Scenarios and Requirements, v0.1.0, June 2025

- TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: performance requirements, v18.8.0, June 2025

- TS 38.104, Base Station (BS) radio transmission and reception, v18.10.0, June 2025

- R4-2512622, Way forward for [116][322] NR_demod_Ph5_Part2_UE, RAN4 #116, August 2025

The simplified SU-MIMO 8Rx receiver was renamed to simplified 8Rx receiver in Rel-19 [8].

The baseline SU-MIMO 8Rx receiver was renamed to baseline 8Rx receiver in Rel-19 [8].