3GPP TSG-RAN WG4 Meeting #116-bis														R4-2513052

Prague, Czech, 13th – 17th October, 2025

Agenda item:			8.12

Source:	Samsung

Title:	Discussion on testability and OTA for 6GR

Document for:	Approval

# 1	Introduction

The objectives of testability and OTA for the work group level 6G SI are shown as following [1, RP-252912]:

（5） 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

Testability plays an important role in the verification of RF, RRM, Demodulation and other requirements before commercialization of 6G products. Especially the OTA requirements and testing are more and more needed to meet the future market demand and to guarantee the performance in field. In this contribution, we review the testability and OTA work in 5G, discuss the testability challenges in 6G and share our views on how to handle the testability work in 6G Day 1.

# 2	Discussion

## 2.1	testabitly and OTA work in 5G

The testability and OTA work in 5G can be roughly separated into two stages.

In the 1st stage, FR2 testability which is grand new 5G spectrum and FR1 basic SISO and MIMO OTA are the focus

- FR2 testability: basic RF/RRM/Demode test method study

- SISO OTA: single carrier TRP TRS

- MIMO OTA: static TRMS

In the 2nd stage, new feature, new functionality, new device type and dynamic testing are gradually involved

- FR2 testability: multi-RX, STxMP

- SISO OTA: 2Tx UL-MIMO, 2Tx TxD, TAS, carrier aggregation, NTN with partial sphere metric, XR

- MIMO OTA: dynamic CMT

- Others: Ambient IoT, AI beam management, etc.

It can be observed that the trend of mobile communication is to be more and more broaden in terms of functionalities, device types and usage scenarios. Different from previous generations when telecommunication was the focus, versatile requirements and scenarios are involved and correspondingly the testability needs also to be adapted. In 6G Day 1, the testability study should take the new situation into account.

Observation 1:	the trend of mobile communication is to be more and more broaden in terms of functionalities, device types and usage scenarios, the testability study should take the new situation into account in 6G Day 1.

## 2.2	testing challenges in 6G

In 6G, new spectrum will be introduced, more diverse device types are supported from Day 1, more feature and functionalities are to be involved:

- New 6G spectrum: up to 15GHz or higher

- More diverse device types: handheld UE, FWA UE, VSAT, XR, Robot, IoT, V2X, etc.

- More feature and functionalities: AI native support, integrated TN and NTN, integrated sensing and communication, etc.

All these bring new challenges to testability. It is beneficial to study the testability framework by taking all these into account.

Proposal 1:	RAN4 to take new spectrum, diverse device types, new feature and functionalities into account in the testability framework study from 6G Day 1.

## 2.3	testability of 6G new spectrum

6G new spectrum is located between 7.125GHz and 24.25GHz. Typically, both ‘around 7GHz’ and ‘around 15GHz’ are under discussion in both RAN level and WG level study.  For ‘around 7GHz’, it can be confirmed that ‘FR1-like’ requirements would be specified according to RAN4 study on IMT in TR 38.922 [2]:

### 5.4.2 UE antenna characteristics The outcome of the RAN WG4 study for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.125 - [10-13] GHz would have "FR1 like" requirements. Therefore, the UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

For ‘around 15GHz’, there was no conclusion in the IMT study. However, it is for sure that the “the antenna module doubles in size going from 28 GHz to 20 GHz and further doubles at 14 GHz” if antenna array with analog beam forming is adopted. Larger antenna panel makes it not practical to be installed in handheld UE, and it would be even worse if more than one panel is to be equipped in order to provide better spherical coverage. Discrete antennas with digital beam forming are also studied including both patch antennas and metal frame antennas. The patch antennas show some antenna gain only at certain direction; the metal frame antennas show no clear gain. Moreover, discrete antennas with digital beamforming can still be measured with conductive method similar to coherent UL MIMO where various TPMIs play the similar role as digital beam forming. In 5G TRP TRS work item, coherent UL MIMO with various TPMIs has been designed with a new TRP OTA metric to accommodate such digital beamforming like performance gain in radiation.

Observation 2:	previous RAN4 study on IMT at around 15GHz shows that analog beam forming is not practical to be adopted by handheld UE due to large antenna panel size.

Observation 3:	discrete antennas with digital beam forming is similar with coherent UL MIMO where conductive requirements and testing are still applicable. A new TRP OTA metric to accommodate such digital beamforming like performance gain in radiation was adopted in TRP TRS work item.

Based on above discussion, it is more practical to test RF requirements in conductive way for the new 6G spectrum around 15GHz. An additional OTA metric can be adopted to accommodate such digital beamforming like performance gain in radiation.

Proposal 2:	conductive testing is still applicable for RF requirements at around 15GHz. Additional OTA metric can be further studied to accommodate potential multi-antenna performance gain in radiation.

According to previous RAN4 experience, frequency is not the only factor to determine the conductive or radiative requirement and test, but UE type and form factor also play important role. E.g., for ATG in FR1, there are both conductive and radiative requirements based on UE reporting:

For ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18, the measured maximum output power Pmax,c,AC shall remain within +2 dB and -2 dB of the rated maximum output power Prated,c,AC reported by the ATG UE. For ATG UE with antenna array indicating the capability antennaArrayType-r18, the measured maximum output power Pmax,c,TABC shall remain within +2 dB and -2 dB of the rated maximum output power Prated,c,TABC reported by the ATG UE.

The example indicates that form factor is also determination factor between conductive and radiative requirements and testing. As to handheld UE, due to form factor limitation, it seems conductive requirements and testing is the only choice.

Observation 4:	frequency is not the only factor to determine the conductive or radiative requirement and test, but UE type and form factor also plays important role, as indicated by the ATG example.

Proposal 3:	For handheld UE, only conductive testing is considered up to around 15GHz, except traditional OTA requirement and testing.

## 2.4	testability of diverse device type

It is consensus that diverse device type will be supported in 6G Day 1. The test method and performance metric could be different for different device types due to different form factors, different characteristics and so on. It should be noted that handheld UE especially smart phone is still the dominant device type motivating the development of mobile communication industry. Consequently, handheld UE should be the 1st priority and other device types can be included based on commercial needs in 6G testability and OTA work.

Observation 5:	The test method and performance metric could be different for different device types due to different form factors, different characteristics and so on.

Proposal 4:	handheld UE should be the 1st priority and other device types can be included based on commercial needs in 6G testability and OTA work.

## 2.5	performance metric for 6G OTA

For decades the popular OTA performance metrics are SISO TRP TRS and MIMO static TRMS. Among them the TRP TRS has been widely used in both development and certification testing. Even in 4G LTE stage, 3GPP failed to specify TRP TRS requirements for mobile phone but TRP TRS was still widely adopted in practice, as it is an effective metric to verify the antenna performance, which can reflect user experience in field implicitly.

Along with new features and functionalities emerging, there is also voice to introducing test method and performance metric more aligned with real usage scenario such as OTA testing with dynamic channel model. This implies another direction, i.e., to verify the overall performance under various field conditions, focusing on user experience directly.

Observation 5:	The OTA performance metric can be divided into two categories, one is to verify hardware especially antenna performance, and the other is to verify the overall performance under various field conditions.

There are proposals to consider new OTA test cases e.g. throughput test case and/or user-centric testing with Quality of Experience (QoE) taken into account [3, 6GWS-250191]. We are open for that. Actually the MIMO OTA is already kind of such test case. It should be noted that OTA performance metric simulating various field conditions may not replace the antenna performance based metric. At least for handheld UE, the TRP TRS metric is well justified and verified for a long time, RAN4 should be cautious to replace TRP TRS with new OTA metrics if there is no fully justification. In our understanding, antenna performance based metric and user-centric performance based metric are not contradicted to each other but can co-exist.

Proposal 5:	antenna performance based metric and user-centric performance based metric are not contradicted to each other but can co-exist. TRP TRS metric for handheld UE should not be replaced with new OTA metrics if there is no fully justification.

## 2.6	Realistic testing methodology

When designing test methodology for new device or new functionalities, a balance between complexity and test time/cost should be considered. In initial discussion of FR2 testability in 5G, the measurement requires so long time that the battery could not last until test ends, which is not affordable for the industry. During the OTA work in 5G, RAN4 spent many efforts to simply the test procedure and reduce the test time for both FR2 and FR1 OTA testing. In 6G, the same principle should be followed.

Proposal 6:	testability and OTA in 6G should be designed in a realistic manner with reasonable test cost and test time.

## 2.7	testability and OTA work management

Usually BS testing is discussed together with BS work items, while UE testing is discussed in a dedicated SI/WI. E.g., FR2 requirements, multi-RX & STxMP, NTN are discussed in core WI and the corresponding testing are discussed in sperate testing SI/WI. There are also precedent cases in 5G that the OTA testing is conducted in the core SI/WI directly, e.g., Ambient IoT, AIML beam management. There are pros and cons where to discuss each detailed OTA topic and it is also up to RAN plenary, but it would be good to clarify how to handle similar cases in 6G, especially for AIML, ISAC, etc.

Proposal 7:	RAN4 to discuss whether UE OTA for some features should be discussed in dedicated OTA SI/WI or discussed in corresponding core SI/WI (e.g., AIML, ISAC, etc.) individually.

## 2.8	harmonized standard

In 5G MIMO OTA discussion, RAN4 suffered a lot when discussing the environmental condition (SIR vs noise-free) and finally standard fragmentation occurs between 3GPP and CTIA. Standard fragmentation is not beneficial for the industry. A global harmonized standard is highly expected in 6G, and coordination with other SDOs (e.g., CTIA, CCSA, etc.) are necessary.

Proposal 8:	RAN4 to target for a global harmonized standard in conformance testing and OTA.

# 3	Conclusion

Observation 1:	the trend of mobile communication is to be more and more broaden in terms of functionalities, device types and usage scenarios, the testability study should take the new situation into account in 6G Day 1.

Proposal 1:	RAN4 to take new spectrum, diverse device types, new feature and functionalities into account in the testability framework study from 6G Day 1.

Observation 2:	previous RAN4 study on IMT at around 15GHz shows that analog beam forming is not practical to be adopted by handheld UE due to large antenna panel size.

Observation 3:	discrete antennas with digital beam forming is similar with coherent UL MIMO where conductive requirements and testing are still applicable. A new TRP OTA metric to accommodate such digital beamforming like performance gain in radiation was adopted in TRP TRS work item.

Proposal 2:	conductive testing is still applicable for RF requirements at around 15GHz. Additional OTA metric can be further studied to accommodate potential multi-antenna performance gain in radiation.

Observation 4:	frequency is not the only factor to determine the conductive or radiative requirement and test, but UE type and form factor also plays important role, as indicated by the ATG and NTN Ku band examples.

Proposal 3:	For handheld UE, only conductive testing is considered up to around 15GHz, except traditional OTA requirement and testing.

Observation 5:	The test method and performance metric could be different for different device types due to different form factors, different characteristics and so on.

Proposal 4:	handheld UE should be the 1st priority and other device types can be included based on commercial needs in 6G testability and OTA work.

Observation 5:	The OTA performance metric can be divided into two categories, one is to verify hardware especially antenna performance, and the other is to verify the overall performance under various field conditions.

Proposal 5:	antenna performance based metric and user-centric performance based metric are not contradicted to each other but can co-exist. TRP TRS metric for handheld UE should not be replaced with new OTA metrics if there is no fully justification.

Proposal 6:	testability and OTA in 6G should be designed in a realistic manner with reasonable test cost and test time.

Proposal 7:	RAN4 to discuss whether UE OTA for some features should be discussed in dedicated OTA SI/WI or discussed in corresponding core SI/WI (e.g., AIML, ISAC, etc.) individually.

Proposal 8:	RAN4 to target for a global harmonized standard in conformance testing and OTA.

# Reference

[1] RP-252912	Revised SID: Study on 6G Radio		RAN#109

[2] TR 38.922      Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz;

[3] 6GWS-250191    Keysight views on 6G testability