3GPP TSG RAN4 Meeting #116bis		R4-2513068
Prague, Czech Republic, 13th – 18th October, 2025

Title:	RAN4 6GR SI related high level principles

AI:	8.2

Source:	Nokia

Document for:	Discussion

# 1			Introduction

This paper discusses general aspects of 6GR RAN4 SI related work.

# 2			Discussion

## 2.1	6GR RAN4 BS RF

For BS RF, we identify number of aspects which should be taken into account once RAN4 progresses 6GR work. They are summarized under BS RF requirements specification and specifications simplification below:

BS RF requirements specification:

  - Carefully check each conducted/radiated requirement currently defined for legacy RATs and confirm/justify they are applicable for 6GR (e.g. reconsider EVM definition, window size and the method for averaging over the time)

  - Take into account NR enhancements of co-location requirements and adopt also for 6GR related requirements (i.e. Tx ON/OFF, Tx IM, out-of-band blocking and co-location spurious emissions)

  - Define essential number to test configurations/cases to sufficiently verify the performance of the BS

  - Specify relevant MSR capability set(s), including configurations with LTE and NR

Specifications simplification:

  - Define requirements in such a way they do not need to be updated whenever new frequency band is defined (take into account also ongoing RAN task work in RAN4 on co-existence and co-location)

  - Requirements should be defined in generic way that no new channel bandwidths are needed to be introduced later

  - Optimize the specification structures to efficiently accommodate different BS types and classes

## 2.2	6GR RAN4 UE RF

We see two major themes emerging for UE RF in general for 6GR. First is the need to unleash all possible UE Tx power available. This is a task for RAN4 UE RF in 6GR as RAN1 has already defined PHY to be so efficient in 5G that it is virtually impossible to enhance PHY in 6GR substantially to get more UE Tx power. Second major theme is to keep 6GR specifications simple but sufficient.

Maximize UL Tx power, do not set artificial bounds --> leverage full power potential

  - Optimize (minimize) MPR, A-MPR

  - Default power class is PC2, also for FDD

  - Power boosting built in from start, not as a dedicated feature

  - Avoid Tx power relaxations as much as possible

  - FWA more stringent MPR simulation assumptions compared to handheld

Keep it Simple, do not over specify things

  - Focus on key requirements which impact 6GR system performance and consider only necessary regulatory requirements to be included in specification.

  - Carefully assess if each individual legacy (LTE/NR) requirement is still relevant and necessary for 6GR, i.e. do not specify requirement just because it has always been specified.

  - Do not specify band combinations specific requirements such as MSD, dTic etc on per individual band combination level. As an example, if MSD is still needed, a look up table or another generic approach could be considered. In our view BCSs and dRibs are not needed at all.

  - Specification should be defined such that new channel bandwidths are not needed to be introduced in later releases. Meaning that additional channel bandwidths for existing bands basket or irregular channel bandwidth WIs are not needed.

## 2.3	6GR RAN4 RRM

For 6GR we have identified a number of areas where we see that 6GR can do better than either of 4G or 5G. One of the most important area we want to look at is the overall measurement framework, which is built on principles from before 3GPP introduced Carrier Aggregation. Additionally, the aspects of energy efficiency and UE autonomous interruptions are highly important for differentiating enhancements in 6GR.

Measurement Framework:

Categorization of measurements: 6GR is the 2nd RAT defined by 3GPP which is CA native. During NR, RAN4 has worked extensively on measurement and CA related challenges. Based on this work and related work in 4G, we think RAN4 should discuss how to account the learnings and work done during 4G and 5G into the 6G measurement framework design.

- We suggest RAN4 to study categorizing measurements and requirements based on reason for requesting the measurements.

Measurements across States and Cell Changes: In 4G and 5G, 3GPP have introduced measurement across states which can improve delay related to CA configuration at connection setup.

- We suggest RAN4 discussing general UE measurement assumptions during State and Cell changes.

Simplified Measurement Gap Structure: It is common understanding in RAN4 RRM that NR gap requirements are overly complex in NR.

- We suggest studying how to reduce this complexity by reducing the number of gap patterns and measurement gap types.

With this work we aim at having simplified UE measurements and related requirements with improved network visibility to the UE delays.

Energy Efficiency:

Network energy efficiency must be accounted from Day-1 6GR design. This is essential as we have seen from earlier RAT’s that if this is not done at the beginning, it is very difficult later on to define efficient NES features which can give practical energy saving on network side. UE power saving is also important to account from Day-1. Furthermore, it is important that both network and UE power savings operate well together.

- RAN4 to discuss possible UE impact from NES and defining requirements accordingly.

UE Autonomous Interruptions:

Interruptions were introduced in Rel-10 with CA. Later on, RAN4 has allowed further UE autonomous interruptions. However, in NR the amount of DCI control message has increased and the interruptions have now started having negative impact on system level, impacting system operation and efficiency (e.g. BWP misalignments). It has been experienced in field deployments that missed DCI messages impacts features defined in other WGs. Hence, UE autonomous interruptions allowed in RAN4 is no longer a RAN4 internal issue.

- RAN4 to discuss interruptions, aiming at removing all UE autonomous interruptions in 6GR.

Connected Mode Mobility:

Interruption reduction is an important aspect of handover in general. We expect handover interruption is one important aspect of 6G handover as well. The handover delay and interruption requirement is defined by RAN4, and we suggest RAN4 to study ways to achieve improvement to the handover delay.

- RAN4 to study at least HO interruption reduction and HO requirements based on target cell status.

Beam Management and MIMO:

MIMO features in 5G NR have deep connection to the beam management and TCI switching requirements. Additionally, TCI switching is used when considering multi-TRP operation. In all aspects of TCI switching, the delay and common understanding between UE and network is important. In NR we have identified a number of aspects which we see would need to be discussed discussed in 6G to ensure an efficient 6G design related to TCI state switching.

- RAN4 to study TCI state switching to ensure common set of requirements in 6G, and address requirements with long delays.

Procedure Related RRM:

In LTE and 5G, cell activation including SCell activation and PSCell addition and activation has been discussed heavily in RAN4 RRM enhancement discussions. Since the Rel-15 framework for SCell activation delay, several enhancements have been specified to reduce the activation delay in different scenarios. Several of these enhancements gives both gain and simplification and should be accounted into the Day-1 6G. To do this we propose RAN4 to consider following:

- RAN4 to discuss the necessary delay components to be considered when considering SCell activation.

## 2.4	6GR RAN4 demod

In this section we give a quick summary of our view concerning 6GR demodulation-led study objectives. There are two main categories of issues that we think the 6GR SI should be adressing:

Firstly, fixing foundational assumptions for 6GR demodulation requirements. There are several foundational assumptions that have been made to create demodulation performance requirements in previous generations. We think that some of them should be re-thought, and some others need to be re-confirmed or re-aligned within the group.

Secondly, bringing higher layer aspects into 6GR demodulation requirements and honouring DUT feedback. Current demodulation and CSI reporting requirements are often neglecting to dynamically react to DUT feedback and to consider scheduling aspects. This leads the DUT to be tested at operating points that are not in line with deployment and do not match receiver implementation performance in the presence of deployment resource allocation and scheduling. Additionally, this makes it impossible to predict and lower bound performance scaling in deployment environments.

To summarize the specific objectives, we are proposing in our “Views on 6G SI demodulation topics” contribution:

- Fixing foundational assumptions for 6GR demodulation requirements

  - Spatial channel models for MIMO requirements

    - Use SCM for all requirements with >1 layer. Less, but higher quality, requirements.

  - SNR operating point range (extension)

    - Deployed UEs regularly see up to 35dB of receive SNR. Have requirements for such high TPUT/stress cases.

  - UE classifications and applicability

    - Depending on RAN1/2 decisions, UE classifications can be used to replace (or very much limit) applicability rules.

  - Feedback less channels and signals

    - Broadcast/feedback-less channels to be assumed testable.

  - Performance requirements vs. functional requirements

    - Reaffirm the mission of RAN4 DMD to produce performance requirements, not functional requirements.

- Higher layer aspects in demodulation requirements (MAC layer) and honouring DUT feedback

  - Requirements at dynamic deployment operating points and aligning with deployment performance scaling.

  - TE changes behaviour based on DUT feedback, e.g., PMI, RI.

  - TE takes decisions (via known implementation) to align with deployment operating points, e.g., SU/MU scheduling, dynamic resource allocation/slots, applying timing offset reports (CJT), OLLA, etc.

- Other

  - UE Baseline and advanced receivers

    - Important for low number of Rx branches (UEs), ineffective for large number (BS).