3GPP TSG RAN4 Meeting #116bis		R4-2513069
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	6GR Considerations on BS RF

Agenda item:		8.5 BS RF and coexistence

Document for:	Approval

# 1	Introduction

RAN#108 Plenary meeting agreed 6G study item in [1] with following objectives related to RAN4 and BS RF area:

BS RF requirement aspects including band [RAN4]

    - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

    - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

At RAN4#116, a plan for the 6GR study in RAN4 was presented by the RAN4 Chair [2]. In this contribution we discuss our considerations on “BS RF” 6GR topic with a scope in Q4’25 and Q1’26 focused on:

- non-spectrum and non-AI BS RF,

- MSR,

- coexistence study.

# 2	Discussion

2.	1		General

This document addresses which aspects should be taken into account for defining 6GR BS RF requirements (system parameters are discussed in a separate document [3]). First of all, we identify the following general considerations to be take into account when drafting new specifications:

BS RF requirements specification:

  - Carefully check each conducted/radiated requirement currently defined for legacy RATs and confirm/justify they are applicable for 6GR

  - Take into account NR enhancements of co-location requirements and adopt also for 6GR related requirements

  - Define essential number to test configurations/cases to sufficiently verify the performance of the BS

  - Specify relevant MSR capability set(s), including configurations with LTE and NR

Specifications simplification:

  - Define requirements in such a way they do not need to be updated whenever new frequency band is defined

  - Requirements should be defined in generic way that no new channel bandwidths are needed to be introduced later

  - Optimize the specification structures to efficiently accommodate different BS types and classes. The specification clauses should be organized in logical order and paying attention to readability for audience outside of 3GPP organization.

Specification structure for BS RF:

  - As 6GR specification structure for BS RF will be decided in the work item phase, we would like to share here our initial view on this issue. For NR approach with single BS core specification for conducted and radiated requirements (TS 38.104) and two separate specifications for BS conformance testing (TS 38.141-1 and TS 38.141-2) was established. Overall, it seems that split, which was done in Rel-15 for NR, worked fine up to now and can continue also for 6GR.

It is proposed to take into account above considerations for further work related to 6GR BS specifications structure and simplification.

2.2	Transmitter requirements

We expect a similar set of BS transmitter requirements are needed to be defined for 6GR BS than for NR BS. Both conducted and radiated (OTA) BS transmitter are expected to be introduced. The following transmitter requirements need to be discussed and agreed:

Conducted BS transmitter characteristics:

BS output power

Base Station output power is a fundamental RF characteristic that directly impacts coverage, capacity, and network energy efficiency. Defining appropriate output power requirements for 6GR will be crucial for optimizing network performance and deployment strategies. Further studies are required to evaluate 5G BS output power requirement for 6GR BS, balancing performance, energy efficiency, and deployment flexibility across the diverse range of anticipated 6GR use cases and frequency bands.

Output power dynamics

Base Station output power dynamics refer to the ability of a BS to rapidly and precisely adjust it transmit power level in response to varying network conditions, traffic demands, and environmental factors. It is proposed that investigations into 6GR BS output power dynamics should commence by thoroughly analysing and building upon the established 5G NR requirements.

Transmit ON/OFF power

The transmitter OFF power requirement is crucial for co-located base stations primarily to manage interference, optimize energy consumption, and ensure efficient network operation. For co-location it is a mandatory requirement. Current specification defines conducted requirement at either antenna connector or TAB connector.

Transmitted signal quality (EVM)

Error Vector Magnitude (EVM) is a critical metric for assessing the quality of a transmitted modulated signal, quantifying the difference between the ideal and measured constellation points. Maintaining stringent EVM performance is essential for ensuring reliable communication and achieving high spectral efficiency. For 6GR, several factors will likely necessitate a re-evaluation and potential evolution of EVM requirements and measurement methodologies. Current EVM definition, which still includes impairments from time synchronization and equalization require 6GR reconsideration of EVM definition, window size and the method for averaging over the time. These impairments had been established at a time when 64QAM was LTE's highest modulation order, and the tightest EVM requirement was 8 %.

Further study is needed to define appropriate EVM requirements for 6GR, considering aspects above.

Unwanted emissions

Unwanted emissions requirements (like SEM) that are regulatory in nature should continue to be respected in the 6GR specifications, while those that are related to coexistence (like ACLR and transmitter spurious emission) should be decided by the outcomes of coexistence studies. The outcomes of the coexistence studies as recorded in TR 38.921 and TR 38.922 can be used as a starting point, but RAN4 should examine whether the simulation parameters and assumptions used in these coexistence studies would continue to reflect real-life deployment for 6GR or they should be revised. Specification structure should also take into account if all relevant frequency bands shall be listed in this Clause for specific SEM, to reduce the specification impact whenever new frequency band is introduced.

Transmitter intermodulation

Transmitter intermodulation is a mandatory requirement for co-location. Securing co-located BSs via reasonable requirements is highly important because it offers significant economic, operational, and environmental benefits for network operators. As Rel-20 work item on enhancement of co-location requirements focuses on OTA requirements, it is important to keep in mind that if there are findings affecting also the conducted requirements, they might be considered in 6GR especially for higher frequencies. Current minimum requirements assume 30dB coupling loss between two co-located BS, which might be a pessimistic value for the higher 6GR frequencies. Intra-system minimum requirements are important to keep also in 6GR. Regional requirements especially for new bands need to be evaluated.

Radiated BS transmitter characteristics

Radiated transmit power

The fundamental principles and definitions of 5G NR radiated output power, such as EIRP, provide a robust and adaptable framework that can largely be reused for 6GR. While 6GR introduces new technologies and operational paradigms, the core concepts of characterizing power emitted from a BS antenna system remain relevant and can be extended or refined. EIRP have to be manufacturer declarable value.

OTA base station output power

For 6GR Over-the-Air (OTA) base station output power, the principles outlined for conducted output power remain equally fundamental. Further studies are required to evaluate how existing 5G NR radiated BS output power requirements can be adapted or redefined for 6GR BS.

OTA output power dynamics

For 6GR OTA base station output power, it is crucial to investigate similar dynamic characteristics as those defined for conducted output power requirements.

OTA transmit ON/OFF power

The OTA transmitter OFF power requirement is crucial for co-located base stations primarily to manage interference, optimize energy consumption, and ensure efficient network operation. For co-location it is a mandatory requirement. Current specification defines this as Co-location reference antenna operation requirement. Rel-20 work item on enhancement of co-location requirements focuses on OTA requirements. We expect that the work done there will also be reflected in 6GR OTA transmit ON/OFF power requirement. Rel-20 work item on enhancement of co-location requirements has focus on FR1 frequencies above 3.3GHz. For 6GR and higher frequencies, similar work needs to be done to reflect reasonable requirements above the FR1 frequencies.

OTA transmitted signal quality

A similar approach to defining and evaluating EVM requirements and measurement methodologies should be considered for OTA as for conducted requirements. For 6GR, several factors will likely necessitate a re-evaluation and potential evolution of OTA EVM requirements and measurement methodologies as for conducted requirements.

OTA unwanted emissions

Same views as expressed above for the conducted unwanted emissions. Considering the time and complexity of OTA testing, extra care should be taken to limit the number of OTA test configurations and test points, for example in transmitter spurious emission, particularly those that would not notably enhance the test coverage.

OTA transmitter intermodulation

OTA transmitter intermodulation is a mandatory requirement for co-location. Securing co-located BSs via reasonable requirements is highly important because it offers significant economic, operational, and environmental benefits for network operators. Rel-20 work item on enhancement of co-location requirements focuses on OTA requirements. We expect that the work done there will also be reflected in 6GR OTA transmitter intermodulation requirement. Rel-20 will evaluate all co-location requirements with focus on FR1 frequencies above 3.3GHz. For 6GR and higher frequencies, similar work needs to be done to reflect reasonable requirements above the FR1 frequencies. In the Rel-19 work item on CLTA (co-location test antenna) there were initial proposal to remove OTA transmitter intermodulation requirement for higher frequencies e.g. 4.9GHz. This was not agreed but expectation is that it will come up again in the Rel-20 work item. We see that there is a possibility that this requirement can be redundant for higher frequencies, but the actual boundary needs to be evaluated first. One problematic issue is also that OTA transmitter intermodulation is mandatory requirement for co-location and cannot be easily removed.

OTA spatial emission

Whilst OTA spatial emission is defined for band n104 in NR, careful consideration should be taken whether this case applies or not for other higher frequency band.

It is proposed to take into account above considerations for further work related to 6GR BS Tx requirements.

2.	3		Receiver requirements

We expect a similar set of BS receiver requirements are needed to be defined for 6GR BS than for NR BS. Both conducted and radiated BS receiver are expected to be introduced. The following receiver requirements need to be discussed and agreed:

Conducted BS receiver characteristics:

Reference sensitivity level

The Reference sensitivity level (REFSENS) is a fundamental receiver requirement that defines the minimum signal power level at the receiver antenna connector (or equivalent for radiated measurements) at which a Base Station (BS) can achieve a specified performance target, typically a Block Error Rate (BLER) for a given reference measurement channel. Maintaining stringent REFSENSE is crucial for ensuring adequate coverage, maximizing cell range, and supporting high data rates, especially at the cell edge. 5G NR REFSENS requirements, as defined in 3GPP TS 38.104, serve as the baseline for defining 6GR BS REFSENS.

Dynamic range

The dynamic range of a BS receiver is a critical performance metric that defines its ability to correctly process a desired signal in the presence of strong interfering signals, or when the desired signal itself varies significantly in power. It is proposed that the dynamic range requirements as defined for 5G NR in 3GPP TS 38.104, serve as the initial baseline for defining 6GR BS dynamic range requirements. This approach leverages the established and proven methodologies from 5G NR, providing a solid foundation for 6GR.

In-band selectivity and blocking

In-band selectivity (ACS) and blocking requirements are related to coexistence (like ACLR and transmitter spurious emission) should be decided by the outcomes of coexistence studies. The outcomes of the coexistence studies as recorded in TR 38.921 and TR 38.922 can be used as a starting point, but RAN4 should examine whether the simulation parameters and assumptions used in these coexistence studies would continue to reflect real-life deployment for 6GR or they should be revised.

Out-of-band blocking

Out-of-band blocking is crucial for co-located base stations to prevent interference, ensure efficient spectrum utilization, and maintain optimal network performance for all systems on the shared site. Out-of-band blocking is fundamental to ensuring that 6GR networks can deliver higher capacity, lower latency, and better reliability in increasingly complex and spectrally dense environment. Conducted out-of-band blocking requirement need to be evaluated for Prefsens for 6GR and wider bandwidths. Interfering signal type and mean power could be adopted from NR but when introducing new bands, special attention must be given to offsets. Co-location minimum requirements must also be evaluated as they assume 30dB coupling loss between interfering transmitter and victim receiver. This might be pessimistic value for higher 6GR frequencies.

Receiver spurious emission

Same views as expressed above for transmitter spurious emissions.

Receiver intermodulation

Receiver intermodulation is a critical conducted receiver requirement that characterizes the ability of a BS receiver to process a desired signal without significant degradation when two or more strong interfering signals are present. It is proposed that the receiver intermodulation requirements, including the test methodology, interferer power levels, frequency offsets, and performance criteria, as defined for 5G NR, serve as the initial baseline for defining 6GR BS conducted receiver intermodulation requirements.

In-channel selectivity

In-channel selectivity defines the ability of a BS receiver to receive a desired signal at its reference sensitivity level in the presence of an adjacent channel interferer. This requirement is crucial for ensuring robust communication performance in spectrally dense environments where adjacent channels may be occupied by other services or operators. It is suggested that the fundamental principles and test methodology for in-channel selectivity, as defined for 5G NR, serve as the initial baseline for defining 6GR BS in-channel selectivity requirements.

Radiated BS receiver characteristics:

OTA sensitivity

For OTA sensitivity, it shall be evaluated what is the frequency range this requirement is needed. For lower frequency bands it is recommended to be transparent to corresponding NR band i.e. need to be defined. And in such case, it is to be manufacturer declarable value.

OTA reference sensitivity level

It is proposed that the principles and performance targets of 5G NR REFSENS requirement, as defined in 3GPP TS 38.104, serve as the initial baseline for defining 6GR BS OTA REFSENS.

OTA dynamic range

Similarly to conducted requirement for dynamic range, OTA dynamic range requirement defined for 5G NR, should serve as the initial baseline for defining 6GR requirement.

OTA in-band selectivity and blocking

Same views as expressed above for the conducted in-band selectivity and blocking. Considering the time and complexity of OTA testing, extra care should be taken to limit the number of OTA test configurations and test points, for example in narrowband blocking, particularly those that would not notably enhance the test coverage.

OTA out-of-band blocking

OTA out-of-band blocking is crucial for co-located base stations to prevent interference, ensure efficient spectrum utilization, and maintain optimal network performance for all systems on the shared site. OTA out-of-band blocking is fundamental to ensuring that 6G networks can deliver higher capacity, lower latency, and better reliability in increasingly complex and spectrally dense environment. OTA out-of-band blocking requirement need to be evaluated for Prefsens for 6GR and wider bandwidths. Interfering signal type and RMS field strength could be adopted from NR but when introducing new bands, special attention must be given to offsets. Rel-20 work item on enhancement of co-location requirements will evaluate all co-location requirements with focus on FR1 frequencies above 3.3GHz. For 6GR and higher frequencies, similar work needs to be done to reflect reasonable requirements above the FR1 frequencies.

OTA receiver spurious emissions

Same views as expressed above for OTA transmitter spurious emissions.

OTA receiver intermodulation

As discussed above for conducted receiver intermodulation, for OTA it is proposed as well that the fundamental principles and performance criteria for receiver intermodulation, as defined for 5G NR, serve as the initial baseline for defining 6GR BS OTA receiver intermodulation requirements. This approach extends the proven methodologies from a conducted framework to a radiated measurement environment, acknowledging the unique challenges and complexities introduced by OTA testing.

OTA in-channel selectivity

OTA In-channel selectivity defines the ability of a BS receiver to receive a desired signal at its reference sensitivity level in the presence of an adjacent channel interferer, considering the complete radio system including the antenna array in a radiated environment. Similar to conducted in-channel selectivity for 6GR OTA in-channel selectivity 5G requirements should be a baseline.

It is proposed to take into account above considerations for further work related to 6GR BS Rx requirements.

2.	4		MSR

Whenever new RAT is introduced, decision needs to be made on new MSR capability set(s) to be defined. In our opinion, there is no need to introduce single-RAT 6GR capability set as that would be covered by single-RAT specifications (as done for NR). Having in mind both LTE and NR will be deployed for number of years, it is proposed RAN4 define new capability sets including relevant configurations using 6GR together with LTE and NR.

Define new capability sets including relevant configurations using 6GR together with LTE and NR.

Similarly as for NR, do not define single-RAT 6GR capability set in MSR specifications.

2.	5		Coexistence studies

Coexistence studies will be performed to decide the related 6G requirements (ACLR, ACS, transmitter spurious emission for coexistence, receiver in-band blocking) for BS and UE in the targeted frequency bands. During the NR SI, coexistence studies were performed reusing largely the simulation parameters and assumptions (refer to TR 38.803) from LTE (refer to TR 36.942) and AAS (TR 37.840 and TR 37.842). RAN4 should examine whether these simulation parameters and assumptions would continue to reflect real-life deployment for 6GR or they should be revised.

For example, the NR throughput over a channel with a given SINR was approximated by an attenuated and truncated form of the Shannon bound (which was derived in LTE) with the maximum SINR (and thus throughput) extended to cover higher order modulations (compared to LTE). RAN4 should discuss and decide whether this SINR to throughput mapping equation should continue be used for 6GR, and if so, what parameters of this equation should be adjusted to better reflect the expected link level performance for 6GR.

Other simulation parameters and assumptions for consideration include, for example, base station vertical coverage range which was included in LS to ITU-R but not used in the coexistence simulation, and applying a fixed beam grid for user specific beamforming (i.e., using code-book beamforming instead of assuming the beam always formed exactly towards the target user) which is the typical implementation in real-life deployment.

RAN4 should examine whether the simulation parameters and assumptions used for LTE and NR coexistence studies would continue to reflect real-life deployment for 6GR or they should be revised.

During the NR SI, coexistence studies were performed using largely the simulation parameters and assumptions (refer to TR 38.803) from LTE (refer to TR 36.942) and AAS (TR 37.840 and TR 37.842). These simulation parameters and assumptions may not continue to reflect real-life deployment for 6GR.

# 4	Conclusion

In this contribution we present our view on BS RF aspects for 6GR. The following proposals have been made:

Proposal 1: It is proposed to take into account above considerations for further work related to 6GR BS specifications structure and simplification

Proposal 2: It is proposed to take into account above considerations for further work related to 6GR BS Tx requirements.

Proposal 3: It is proposed to take into account above considerations for further work related to 6GR BS Rx requirements.

Proposal 4: Define new capability sets including relevant configurations using 6GR together with LTE and NR.

Proposal 5: Similarly as for NR, do not define single-RAT 6GR capability set in MSR specifications.

Proposal 6: RAN4 should examine whether the simulation parameters and assumptions used for LTE and NR coexistence studies would continue to reflect real-life deployment for 6GR or they should be revised.

# References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

[2] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

[3] R4-2513144, 6GR System parameters, RAN4#116bis, Nokia