3GPP TSG-RAN WG4 Meeting #116bis	R4-2513072

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	AI/ML use cases and general aspects in 6G

Agenda item:	8.9

Document for:	Discussion

Introduction

During RAN4#116 meeting, RAN4 6G topics were presented by RAN4 chair [1]. AI related discussions are part of the topics to be discussed covering following aspects:

RAN4 driven use cases, RAN4 impacts of other WG driven use cases, RAN for AI

In this paper, we provide our views on the following main topics:

- RAN4-led AI/ML studies

- RAN4 impacts from RAN1-led AI/ML use cases

- Discussion

RAN4-led AI/ML studies

Post-deployment enhancements

In the last RAN1 meeting (RAN1#122) [2] the following agreement was reached regarding the feature selection for the study in 6G.

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

As it is mentioned in the above agreement that online training/finetuning are part of training assumptions for 6G, it becomes clear that more frequent updates in AI/ML functionalities will occur, and these updates may impact the behaviour of AI/ML functionalities in a significant manner. In the existing 5G-A, conventional (pre-deployment) conformance testing methods, the UE-side AI/ML enabled functionality configurations are tested against the set of KPIs for different requirements and test scenarios. However, due to more frequent (over-the-air) changes/updates in the implementation versions of the AI/ML features combined with the radio environmental conditions, the performance behaviour of the AI/ML functionalities can differ from what was tested earlier in lab conditions with previous implementation versions. Hence, the introduction of any testing/validation approach which can be applied in post-deployment, i.e., after initial performance certification in the lab, would be beneficial for 6GR AI/ML use cases.

In 5G-A Rel-19, it was agreed to use the performance monitoring framework to handle post-deployment testing. It was sufficient for Rel. 19 use cases since the scope was only AI/ML use cases with offline training. Now in 6GR the assumption on training types has widened. As can be seen from the RAN1#122 agreement [2], assumptions explicitly include new continuous learning methods such as online training/finetuning, dynamic label construction. This requires more than observation and monitoring, especially when the UE-side AI/ML enabled functionality has the capability of online training/finetuning.

Apart from the regular functionality updates and UE feedback reports, these cases require more from both the UE and the NW to ensure that the AI/ML functionalities are performing as expected throughout the updated functionality life cycle, e.g., exploration and exploitation phases due to online training.

For 6G AI/ML use cases, we can envisage a pre- and post- deployment testing and validation workflow as illustrated in Figure 1.

Figure 1: Post deployment testing and validation, as envisaged for 6GR UE-side AI/ML enabled features and use cases.

The figure is divided into 3 phases as described below.

- As a part of the pre-existing standard conformance tests, the AI/ML enabled functionality in the device is tested and conformed. This is the legacy way. Once this is done, the devices are deployed in the field.

Phases marked 2 and 3 in the Figure 1 is what we refer to as the Post Deployment Workflow. These phases 2 and 3 can happen in tandem based on the nature of update to the AIML enabled functionality.

- In-field post deployment validation:

  - Proactively, conduct the test of the update in the field and based on the results enable the AIML enabled functionality.

  - Reactively, enable the updated AIML enabled functionality and monitor it for the performance degradation and take suitable actions.

- In-lab post deployment testing: Where the devices re-conformed in the lab for the specific Features or Feature Groups before enabling the updated functionality in the field.

Now with the kind of new 6G features that different working groups are considering, and the complexity of testing AI/ML enabled functionalities involving online training and finetuning, the post deployment testing & validation procedures should be robust enough to cater to all kind of in-the-lab and in-the-field scenarios.

This could result in:

- A significant update to the in-lab test framework – test requirements, conformance testing and certification – to cater to online training/finetuning.

- A post deployment test (in-the field validation, in-the-lab test) framework.

- Updates to ML management procedures (“LCM framework” in 5G-A) required to support in-the-field validation of functionalities.

To achieve this, we propose a new RAN4 study on post-deployment testing and validation and its enhancement in tight collaboration with RAN2, to ensure a robust post-deployment testing and validation framework and its requirements are in place for 6G use cases from Day 1.

RAN4 to further discuss and consider studying post-deployment testing and validation enhancements in 6G in collaboration with RAN2 from early phases of the 6GR SI.

      - AI/ML UE performance behaviour

The specifications of the post-deployment testing and validation framework (right side of Figure 1), as starting point can rely on the RAN1/RAN2 signalling procedures which will be developed for supporting the new 6GR use cases, such as configuration and monitoring procedures. When the in-field post-deployment procedure is needed (e.g. based on MNO policies, automated network optimisations, etc) during normal network and UE operations, it is beneficial to have a well specified and inter-operable, mechanism to trigger and retrieve the validation results from the UEs. It would also allow to have certain options for the NW to take proactive decisions, rather than just reactive actions based on monitoring information received from UE. This indicates, the need to further enhance and extend the UE-side AI/ML monitoring procedures defined in 5G-A.

A post deployment testing and validation framework for the UE-side AI/ML functionalities can allow NW to take more proactive decisions, rather than just reactive actions based on monitoring information received from UE.

One possible direction for enhancements, which can be explored in this study, is the design of mechanisms for the NW to retrieve from the UEs additional information about the most likely ‘reasons’ or ‘explanations’ for their AI/ML functionality (configuration) performance and behaviour in particular radio conditions/scenarios.

The availability of this type of information from the UE would allow the NW to better understand, validate, and ultimately trust, the AI/ML behaviour of the UE across diverse real-world deployment scenarios.  The NW analysis based on such information facilitates rapid identification of UE’s unexpected behaviours and enables the NW to correlate its decisions with the performance metrics and user experience indicators. Furthermore, the ability to interpret UE behaviour/prediction using explanations provides actionable insights for other over-the-air configurations, which may include finetuning, re-training and data collection, ensuring that AI/ML-enabled UE devices maintain optimal performance while contributing to overall network efficiency and user satisfaction throughout their operational lifecycle.

RAN4 should consider, as part of the post-deployment testing and validation study, solutions for enhanced UE-side AI/ML functionality behaviour/performance reporting.

RAN4 study on the reported measurements for AI/ML use cases

The application of AI/ML to RAN use cases has several advantages such as measurement reduction, improved and timely decisions for operations such as handover, radio link management and beam switching etc. However, these use cases have also added predicted measurements in the reports in addition to the physically performed measurements. There are several open points regarding these reports which may contain either predicted measurements or physically performed measurements or both. This issue requires further RAN4 attention since different types of measurements might have different delay and accuracy requirements.

      - Impact on prediction accuracy

During previous study/work items on beam management and AI/ML Mobility, it has been observed that UE may report to the network either the predicted measurements or the physically performed measurements. For the physically performed measurements report, the common understanding is to follow the Non-AI/ML measurement accuracy requirements for both L1-RSRP and L3-RSRP measurements. However, for predicted RSRP accuracy requirements, it is not agreed to follow the same requirements as measured RSRP accuracy. The predicted RSRP accuracy may differ from the physically performed measurement accuracy due to the following reasons:

- The AI/ML model would be likely trained on a dataset that consists of physically performed measurements from the field or the synthetic data. Therefore, the physically performed measurement would be the upper bound for the performance of the AI/ML model in case of best predictions. Hence, their accuracy would be always less than the physically performed measurements.

- Although in case of skipped measurements, where skipped measurements may be replaced by predicted measurements, the lack of accuracy in prediction might not be justified. The argument would be that other non-AI/ML based techniques may also result in similar lack of accuracy while applying measurement reduction. Therefore, the additional complexity of AI/ML based solutions may not be justified. However, in case of future predictions (such as Temporal domain Case-A prediction of AI/ML BM/Mobility use cases) some reduced accuracy would be expected and it is understandable.

The above-described loss of accuracy in reported predicted measurements may impact Network sided decisions and therefore it should be studied thoroughly in RAN4. RAN4 should consider the following aspects in order to study any relaxation in predicted measurement reports:

- Is it feasible to consider different accuracy classes for different reports?

  - More accurate reports for physically performed measurements

  - Less accurate reports for predicted measurements

- How may NW use more and less accurate reports?

- What level of relaxation can be allowed for predicted measurement reports without impacting overall performance?

- How to ensure that predicted measurements and physically performed measurements, having different level of accuracy, can be distinguished on the network side? Network may need to be indicated about different types of reports as listed below:

  - Reports based only on physically performed measurements

  - Reports based on physically performed measurements mixed with predicted results

  - Reports based only on predicted results

- How to mitigate, using network-controlled mechanisms, any potential negative impact on the overall performance due to loss of accuracy in the reports?

The loss of accuracy in predicted reports may have adverse effects on Network decisions for operations such as handover and beam switching etc.

      - Potential improvement of prediction accuracy

As discussed in the above section, one of the reasons of loss of accuracy in predicted measurements is the training dataset consisting of physically performed measurements containing measurement errors and/or synthetic data as well as the use of physically performed measurement as input to AI/ML during inference phase. These errored measurements become the upper bound for the prediction performance. During the study on AI/ML BM and Mobility use cases, it was observed that if an improvement in prediction accuracy is desired, then a further enhancement in physically performed measurement accuracy requirements would be needed. However, RAN4 didn’t really discuss how much relaxation in prediction accuracy may be acceptable in order to ensure correct Network sided decisions and how to improve prediction accuracy if the required accuracy is not achieved. RAN4 may consider the following aspects to improve prediction accuracy:

- Enhancement in physically performed measurement accuracy requirements

- Preference to cluster-based approach wherever possible to guarantee minimum required performance

- Capabilities of UE supporting improved accuracy for physically performed measurements

Since physically performed measurements would be used for initial training of AI/ML functionalities before deployment as well for in-field fine-tuning and retraining, therefore, they become the upper bound of the prediction performance.

Stricter requirements for physically performed measurements, prioritization of cluster-based approaches and support of UEs capable of improved accuracy for physically performed measurements are some of the potential ways to improve the predicted measurement accuracy.

      - Impact on reporting delay requirements

As discussed in Section 2.1.2.1, we may have measurements with lower accuracy when using AI/ML-based measurement prediction. In case different classes of measurements are defined based on their accuracy, RAN4 needs to study whether and how to define different reporting delay requirements for such measurements. The following aspects can be considered by RAN4 when studying the impact on reporting delay requirements:

- Enhanced reporting delay requirements for all measurement classes (i.e., with lower and higher accuracies)

- Enhanced reporting delay requirements for measurements with lower accuracy compared to measurements with higher accuracy

Reporting delay requirements may differ for different types of measurement classes with lower or higher accuracies.

      - Impact on measurement gaps configuration

During measurement gaps, the UE must temporarily stop communicating with the serving cell to measure, e.g., other frequencies. AI/ML techniques can help reduce the number of measurements a UE needs to perform. Hence, the impact on measurements gap will be significant. Below we highlight a few. RAN4 needs to study whether and how to define different requirements for such measurement gap configurations. The following aspects can be considered by RAN4 when studying the impact on measurement gap configurations:

- Reduction in Measurement Gaps Configurations: With the help of AIML predictions, less measurement gaps configurations will be required. One of the factors that may impact measurement gap configuration is the prediction horizon of AIML model.

- Dynamic measurements gap configuration: Instead of static, periodic gaps, NW could use AI/ML-assisted predictions to configure gaps dynamically, only when necessary.

- Simplifications of Measurement Gap Patterns (Table 9.1.2-1, TS38.133: Gap Pattern Configurations):

  - Depending on AIML capability on UE side, measurement gap pattern might be configured accordingly.

  - Measurement Gap length (MGL) and Measurement Gap Repetition Period (MGRP) might be impacted as well.

  - Table 9.1.2-1 might be extended or reduced depending on AIML capabilities.

RAN4 to consider the study on the reported measurements for AI/ML use cases and further discuss the following aspects as part of this study:

- Impact on prediction accuracy

- Potential ways to improve prediction accuracy

- Impact on reporting delay requirements

- Impact on measurement gaps configuration

RAN4 impacts from RAN1-led AI/ML use cases

CSI-RS overhead reduction

For AI/ML-based CSI prediction using a UE-side model, both training and inference are fully localized at the UE (with training assumed at the UE-side server), with performance monitoring supported by the UE through calculated metrics reported to the network. AI/ML-based CSI prediction specification work reused most of the non-AI/ML-based CSI prediction framework which was already specified in Rel-18. Therefore, the specification work was mainly limited to performance monitoring, data collection, and some other aspects such as UE AI/ML processing units.

Given the recent work on AI/ML-based CSI prediction and CSI compression, it is reasonable to expect that these Rel-19/20 AI/ML CSI use cases are strong candidates for normative 6G standardization work. Additionally, as the next steps in AI/ML for CSI feedback, spatial and/or frequency domain CSI-RS overhead reduction could be considered during the 6G study phase.

In the CSI-RS overhead reduction use case, AI/ML techniques can be leveraged to design intelligent and adaptive CSI-RS schemes that reduce redundancy, optimize resource allocation, and enhance channel estimation accuracy even in complex environments.

One effective approach is to divide the full set of antenna ports into smaller, more manageable subsets. Each subset is then assigned to a dedicated resource, which can be allocated in the frequency domain (using PRBs), the time domain (using slots), or combination of both. These dedicated resources are specifically used to transmit the CSI-RS corresponding to each subset of ports.

AI/ML techniques can then be applied effectively to merge the channel estimates obtained from the different subsets and enable sparser CSI-RS configurations. This allows for accurate reconstruction of the complete spatial-frequency-temporal characteristics of the channel, while significantly reducing overhead.

In a UE-side solution, an AI/ML model is implemented at the UE to reconstruct full CSI from reduced-density CSI-RS. The AI/ML model at the UE can adopt various architectures, including MLP-based models, CNN-based models, and transformer-based models (e.g., encoder-only, decoder-only, or autoencoder models), building on the methodologies explored in Rel-18 and Rel-19.

As soon as RAN1 agrees to consider the use case of CSI-RS overhead reduction, RAN4 should consider starting the study on the impacts on RAN4. This should be the case particularly for UE sided model scenario.

DMRS design with AI receiver

The use case of DMRS design with AI receiver received significant support from many companies in last RAN1#122 meeting [2]. The current 5G DMRS design is fully orthogonal to data symbols and is also orthogonal among UEs in a MU-MIMO scenario. Hence, increasing MIMO size would imply supporting increasingly large number of DMRS ports. Furthermore, the current DMRS design also lacks in flexibility to cater real-world deployment scenarios such as extreme mobility scenarios. AIML receivers may enable a flexible and/or robust DMRS patterns, allowing us to overcome the limitations of the legacy DMRS design. Such solutions may reduce the transmission overhead by enabling new designs/sparsity patterns of the DMRS.

In summary, DMRS can either follow the conventional approach (i.e., 5G legacy) or adopt a new reduced density configuration, which requires specification changes. Moreover, as a special case of the latter, it is also possible to utilize a dynamic DMRS configuration to reduce DMRS density based on the bandwidth – particularly appealing in use cases with an increased number of TX antenna ports. The reason for this is that with AIML receivers, lower DMRS density is sufficient when operating with larger bandwidths.

As soon as RAN1 agrees to consider the use case of DMRS design with AI receiver, RAN4 should consider starting the study on the impacts on RAN4.

Inter-cell beam prediction

Although beam prediction in an intra-cell scenario was considered in AI/ML BM use case of rel.19 (WI: NR_AIML_air), the applicability or benefits of beam prediction for inter-cell beam management weren’t considered.

In case of inter-cell beam prediction, the UE can be configured with measurement and report configurations to obtain measurements from RSs/beams of multiple cells (or TRPs associated with different PCIs), where each cell may transmit multiple beams. Depending on the number of beams per cell and the number of cells involved in the measurement, inter-cell beam measurements can result in high RS overhead and increased latency (in receiving reports for multiple cells).

Sub-use cases of inter-cell beam prediction:

Similar to the Rel-19 AI/ML beam prediction, RAN1 might consider different sub-use cases for inter-cell beam prediction:

- Spatial domain inter-cell beam prediction:

  - Measurements of a subset of beams (Set B) from M cells are used as model input, and the model output provides the Top-1/K beams from the full set of beams (Set A) across M cells (or per cell).

- Temporal domain inter-cell beam prediction :

  - A history of measurements of a subset/set of beams (Set B) from M cells is used as model input, and the model output provides the Top-1/K beams for future time instances from the full set of beams (Set A) across M cells (or per cell).

High-level details on model input/output for spatial domain inter-cell beam prediction are further explained in Figure 2 and 3. In both Figures, 8 DL Tx beams per cell are considered (white and colored small boxes), where the prediction set (Set A) may consist of 8*M beams. The model uses measurements of 4 beams (colored small boxes) per cell as input (meaning that measurement Set B may consist of 4*M beams).

- Figure 2 shows a case where the ML model predicts the best DL Tx beam (additionally the corresponding L1-RSRP) across all M cells

- Figure 3 shows a case where the ML model predicts the best DL Tx beam (additionally the corresponding L1-RSRP) for each cell.

Figure 2 : Best beam prediction across cells. Model input is L1-RSPRs of DL Tx beams of multiple cells. The solid colour box on the right-hand side represents the best predicted beam.

Figure 3 : Best beam prediction per cells. Model input is L1-RSPRs of DL Tx beams of multiple cells. The solid colour boxes on the right-hand side represent the best predicted beams (best beam per cell).

As soon as RAN1 agrees to consider the use case of inter-cell beam prediction, RAN4 should consider starting the study on the impacts on RAN4 for both spatial domain and temporal domain prediction scenarios.

Conclusion

In this contribution, we share our views on AI/ML use cases and general aspects in 6G. Specifically, we cover following aspects:

- RAN4-led AI/ML studies

- RAN4 impacts from RAN1-led AI/ML use cases

The following Observations and Proposals were made.

Proposal 1: RAN4 to further discuss and consider studying post-deployment testing and validation enhancements in 6G in collaboration with RAN2 from early phases of the 6GR SI.

Observation 1: A post deployment testing and validation framework for the UE-side AI/ML functionalities can allow NW to take more proactive decisions, rather than just reactive actions based on monitoring information received from UE.

Proposal 2: RAN4 should consider, as part of the post-deployment testing and validation study, solutions for enhanced UE-side AI/ML functionality behaviour/performance reporting.

Observation 2: The loss of accuracy in predicted reports may have adverse effects on Network decisions for operations such as handover and beam switching etc.

Observation 3: Since physically performed measurements would be used for initial training of AI/ML functionalities before deployment as well for in-field fine-tuning and retraining, therefore, they become the upper bound of the prediction performance.

Observation 4: Stricter requirements for physically performed measurements, prioritization of cluster-based approaches and support of UEs capable of improved accuracy for physically performed measurements are some of the potential ways to improve the predicted measurement accuracy.

Observation 5: Reporting delay requirements may differ for different type of measurement classes with lower or higher accuracies.

Proposal 3: RAN4 to consider the study on the reported measurements for AI/ML use cases and further discuss the following aspects as part of this study:

- Impact on prediction accuracy

- Potential ways to improve prediction accuracy

- Impact on reporting delay requirements

- Impact on measurement gaps configuration

Proposal 4: As soon as RAN1 agrees to consider the use case of CSI-RS overhead reduction, RAN4 should consider starting the study on the impacts on RAN4. This should be the case particularly for UE sided model scenario.

Proposal 5: As soon as RAN1 agrees to consider the use case of DMRS design with AI receiver, RAN4 should consider starting the study on the impacts on RAN4.

Proposal 6: As soon as RAN1 agrees to consider the use case of inter-cell beam prediction, RAN4 should consider starting the study on the impacts on RAN4 for both spatial domain and temporal domain prediction scenarios.

- References

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, 3GPP TSG RAN WG4 Meeting #116, Bengaluru, Aug 25th – 29th, 2025.

- R1-2506456, Moderator summary #2 on AI/ML for 6GR, Samsung, 3GPP TSG RAN WG1 #122, Bengaluru, Aug 25th – 29th, 2025.