3GPP TSG-RAN WG4 Meeting # 116bis                                       R4-2513086

Prague, CZ, Oct. 13 – 17, 2025

Agenda Item: 6.17.2.3

Source: MediaTek Inc.

Title: Discussion on test cases for conditional LTM

Document for: Discussion

# Introduction

In this contribution, we would like to discuss the RRM performance requirements to be defined for R19 mobility.

# Discussion

According to WID RP-250339, the scope is limited to non-DC. So we don’t need to define test cases for conditional PSCell switch.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

Given that UE needs to pass legacy LTM test cases, there is no need to verify all aspects of cell switch procedure. We only need to focus on the delta part, i.e., cell switch is triggered by UE. Given the fact that the delay and interruption of conditional LTM is exactly the same as CHO if T/F tracking cannot be skipped. We propose the following test list for CLTM and subsequent CLTM.

- CLTM from FR1 to FR1 with early TCI state activation

- CLTM from FR2 to FR2 with early TCI state activation

- subsequent CLTM from FR1 to FR1

- subsequent CLTM from FR2 to FR2

Proposal 1: introduce the following test cases for CLTM and subsequent CLTM.

- CLTM from FR1 to FR1 with early TCI state activation

- CLTM from FR2 to FR2 with early TCI state activation

- subsequent CLTM from FR1 to FR1

- subsequent CLTM from FR2 to FR2

As subsequent CLTM naturally include a complete CLTM procedure. Thus for UE which can pass subsequent CLTM, test case for CLTM can be skipped.

Proposal 2: For UE which passed subsequent CLTM, test cases for CLTM can be skipped.

# Summary

In this paper, we provide some view on performance requirements of R19 mobility. We have the following proposals:

Proposal 1: introduce the following test cases for CLTM and subsequent CLTM.

- CLTM from FR1 to FR1 with early TCI state activation

- CLTM from FR2 to FR2 with early TCI state activation

- subsequent CLTM from FR1 to FR1

- subsequent CLTM from FR2 to FR2

Proposal 2: For UE which passed subsequent CLTM, test cases for CLTM can be skipped.

# 4 Reference

RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.